3GPP TSG-RAN WG4 Meeting#117                               R4-2521753

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.9

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6G MRSS study

Document for:	Approval

Introduction

In RAN#108 meeting, a new SID on Study on 6G Radio [1] was approved. In this contribution, we would like to share some further understandings on 6G MRSS study.

Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Discussion

## 2.1. Numerology

- RAN4#116bis: - Issue 1-2-0: General consideration Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling - Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

As shown in the following table 1, we compared the commercialized 5G data/SSB numerology and agreement for 6G data/sync signal numerology, it can already guarantee the orthogonality between 5G and 6G in theory with the same numerology assumption.

Table 1. the summary of 5G commercialized numerology and agreement for 6G numerology

| Operating frequency | Commercialized 5G numerology | Agreement for 6G numerology |
| --- | --- | --- |
| FR1 FDD | 15KHz | 15KHz, FFS for 30KHz FDD e.g.1-2.5GHz- [NOTE 1] FFS for 7.5KHz for sub1GHz [NOTE 2] |
| FR1 TDD | 30kHz including sub-6GHz | 30kHz including sub-6GHz and around 7GHz |
| FR2-1 TDD | 120 kHz | 120 kHz |
| NOTE 1: 30kHz in FR1 FDD bands is not proposed for MRSS and proposed for the same numerology for FDD+TDD inter-band to simplify the HARQ process and save some baseband memory. For MRSS case, NOTE 2: 7.5kHz in sub1GHz is proposed for larger coverage with large CP length from our understanding. This is also not for 5G-6G MRSS in field. This is also up to RANP decision. |  |  |

Proposal 1: for numerology for MRSS between 5G and 6GR, propose to follow the agreement reached in RAN1 with the assumption of the same numerology between the commercialized 5G and 6GR.

## 2.2. Channel raster

- Issue 1-2-1: Channel rater - Proposals from companies - P1 (Apple): 7.5kHz shifting is not needed for 6G-5G MRSS. - P2 (CMCC): - For MRSS between NR and 6GR, it is proposed to discuss whether 100KHz channel raster for low band are still needed. - No need to consider 7.5KHz uplink shift for MRSS between NR and 6GR. - P3 (Spreadtrum, UNISOC): - For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz for MRSS between 5G and 6G - There is no need to define UL shift 7.5 kHz for MRSS between 5G and 6G - P4 (OPPO): For the 5G-6G MRSS bands, the 6G channel raster design shall make sure it is compatible with 5G. Besides of that, no other issue is foreseen in MRSS channel raster design - P5 (Xiaomi): Support 5kHz channel raster in day 1 for below 3GHz - Remove per band channel raster concept with following value per sub-frequency range basis Frequency range; Channel raster. <3GHz; 5kHz. 3GHz ~ 24.25kHz; 30kHz. 24.25GHz ~ 52GHz; 120kHz. - Way forward - Discuss following impact - Whether 7.5kHz shifting is needed for 5G-6G MRSS - Whether 100KHz channel raster for low band are needed for 6G-5G MRSS - How to handle the bands in NR can support 10 kHz for MRSS between 5G and 6G - Other impacts are not precluded

As shown in the following Figure 2, for above 3GHz, it is clear that SCS based channel raster for better spectrum utilization for intra-band contiguous CA case still work for both 5G and 6GR. In other words, for the 5G refarming bands with SCS based channel raster defined, this kind of channel raster should be also reused for 6G.

Figure 1. the 5G channel raster for FR1 and FR2-1 TN bands

Figure 2. the 5G channel raster for FR1-NTN and FR2-1 NTN bands

Proposal 2: for 5G refarming bands with SCS based channel raster to 6G operation, SCS based channel raster should be applied.

The question left is how to define channel raster for FR1 NR bands defined with 100kHz and 10kHz enhanced channel raster. From the 5G-6G MRSS perspective and ensure the even/odd PRB alignment between BS and UE side, then the option 2 is preferred to simplify the s design.

- Option 1: 100kHz channel raster;

- Option 2: 10kHz channel raster;

- Option 3: 15kHz channel raster which is mainly used for intra-band contiguous CA;

- Option 4: 5kHz channel raster which can support both option 2 and option 3;

For option 3 15kHz SCS based channel raster, this can be useful for the case that whole 5G band or spectrum is released and used to deploy 6GR directly, then SCS based channel raster would be beneficial for intra-band contiguous CA deployment in theory. However it should also noted that FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster as illustrated in Figure 3 and Figure 4, the largest TDD bands n40 is limited within 100MHz frequency spanning range and largest FDD band band n65 is limited within 90MHz frequency spanning range. In other words, according to the increasing 6GR bandwidth definition for 15kHz SCS from 50MHz to 100MHz, for those bands as mentioned with 10kHz/100kHz channel raster, single carrier operation (e.g. 100MHz with 30KHz SCS for band 40 or 90MHz with 15kHz SCS for band n65) is more appropriate configuration and deployment in field.

Figure 3. band definition below 1GHz

Figure 4. band definition between 1GHz and 2.4GHz

Observation 1: FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster, the frequency range of band itself is limited and don’t see the strong motivation/necessity to deploy the intra-band contiguous CA.

In the following Figure 5, we illustrated how the candidate channel raster impacts on the channel arrangement. It could be found that center frequency of the carrier should be shifted with 5kHz offset if 15kHz channel raster is adopted. For other candidate channel raster, the center frequency of the carrier could be kept unchanged.

Figure 5. the illustration of different options for channel raster

In the following Figure 6, we also illustrated the actual frequency shifting if we go with option 3, if the allocated 5G NR spectrum cannot be changed from the regulator perspective, that means the emission requirement outside of the legacy carrier position is still valid for the new channel raster, the guard band should be correspondingly reduced with 5kHz which might cause some emission compliance problems especially from UE side. If we consider the MRSS deployment with the new 15kHz channel raster, this basically means additional 5kHz Doppler shift similar as ATG doppler shift case will be imposed on the legacy 5G NR UE which will have big impacts on the frequency/timing tracking and throughput performance at the end.

Figure 6. the illustration of channel placement for option 3 15kHz based channel raster

Option 4 seems workable to accommodate both option 2 and option 3, however this might cause some confusion for different deployment assumption, e.g. whether 10kHz channel raster is allowed for intra-band contiguous CA case or whether 15kHz channel raster is allowed for MRSS case. From the standardization perspective, it is preferred to specify the single channel raster without any ambiguity and performance problem for certain deployment.

Proposal 3: for 5G refarming bands with 100kHz or 10kHz based channel raster to 6G operation, propose to define 10kHz channel raster in 6G day 1.

## 2.3. Sync raster

First of all, the sync raster of SSB in MRSS scenario is highly dependent on RAN1’s physical layer design. From RAN4 perspective, it might be better to postpone the discussion until there are sufficient progress in RAN1. The exact sync raster design could be done directly in the WI phase.

From RAN4 perspective, if 5G and 6G is designed with different SSB signal as shown in the following Figures, in order to improve the coverage for the certain cell specific broadcast channel (e.g. SSB channel), power boosting is one useful and efficient solution to enable it. For MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol should be considered.

Figure 2.3-1. The illustration of 5G-6G spectrum sharing for FDD low bands below 1GHz

Figure 2.3-2. The illustration of 5G-6G spectrum sharing for FR1 FDD/TDD middle bands around 2GHz

Figure 2.3-3. The illustration of 5G-6G spectrum sharing for FR1 TDD high bands

Proposal 4: for MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol(s) should be considered.

Proposal 5: for sync raster of MRSS BS, propose to postpone the discussion in RAN4 until RAN1 has reached sufficient progress for the initial access for MRSS deployment.

## 2.4. Waveform

Agreement: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Similar as numerology discussion for MRSS design, regarding the waveform for MRSS between 5G and 6GR, we propose to follow the agreement in RAN1 e.g. CP-OFDM and DFT-s-OFDM.

Proposal 6: for waveform for MRSS between 5G and 6GR, propose to follow the agreement reached in RAN1.

## 2.5. Modulation

LP-WUS signal was introduced in Rel-19 for both FR1 and FR2-1 as one of important 5G-A feature to enable ultra-low power consumption. However during the Rel-19 discussion, the technical concerns on the potential EVM degradation on NR DL signal due to the simultaneous transmission of LP-WUS signal was raised especially for high modulation order (e.g. 256QAM) which is almost the mandatory feature of the commercial NR BS, therefore we propose to conduct more technical discussions in 6G day1 to figure out the problems instead of reusing the agreement/requirements from 5G-A in Rel-19 and leave up to the implementation.

Proposal 7: if LP-WUS signal is supported in 6G day1, propose to further discuss the impacts on potential EVM degradation of NR signal due to the simultaneous LP-WUS signal transmission.

## 2.6. Channel bandwidth

- Proposals: - P1(Apple): Do not consider MRSS between 6G&5G for small channel bandwidths, e.g., 3MHz (or even 5MHz). - P2(CMCC): it is proposed to consider the impact on MRSS when discussing irregular channel bandwidth. The solution to support irregular channel bandwidth may have impact on MRSS support - Way forward - Discuss whether to have some restriction on supported bandwidth for 5G-6G MRSS

Usually there is not much relationship between channel bandwidth and MRSS. In addition, the irregular channel bandwidth is also not specified in 5G-A except for 7MHz and 6MHz, therefore we don’t see the necessity of discussing MRSS between 5G and 6G. Even though for some regions with 6MHz and 8MHz deployed via the proprietary solution without any spec impacts, it is also very difficult to discuss the MRSS between 5G and 6G for these cases.

Proposal 8: for irregular channel bandwidth except for 6MHz and 7MHz, propose not to discuss the MRSS between 5G and 6G.

## 2.7. Coexisting with NB-IoT and eMTC in 6GR carrier

### 5.4.3 Massive Communication (IoT) The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT): - 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT - [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device. Editor note: “6G should support coexistence with NB-IoT (all deployment modes) and eMTC via semi-static configuration” is moved to 5.2 (migration and architecture)

In the last RAN4 meeting, there were some discussion on 6GR coexistence with NB-IoT and eMTC. This TP to TR 38.914 was agreed in 6G RAN SI [3], however this is not reflected in 6G SI yet. From our understanding, this could be discussed until RAN updates the 6G SI or approve one WID in Rel-22 similar as the coexistence between 5G and NB-IoT and eMTC in Rel-16 with two separate WIDs approved.

Proposal 9: for 6GR coexisting with in-band NB-IoT and eMTC, propose to postpone the discussion until RAN has any agreement to guide the WG’s action.

## 2.8. RF requirements

For MRSS BS supporting both 5G and 6G in the NR refarming bands, it is expected that they may have the RF hardware upgrade and different baseband processing for 5G and 6G. Therefore, it is reasonable to have the unified RF requirement for MRSS BS supporting both 5G and 6G. In addition, considering the applicability of RF requirement for MRSS BS, it is more reasonable to apply the 6GR BS requirement for MRSS BS instead of 5G BS requirement due to the coming detailed 6GR RF requirement discussion (e.g. in-band/out of band blocking requirement) and improvement of 5G BS RF requirement (e.g. co-location relevant requirements etc ).

Proposal 10: for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G.

Proposal 11: for MRSS BS, propose to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification.

## 2.9. Inter-RAT mobility between 6GR and NR

For inter-RAT mobility between 6GR and NR, both the NR measurement and handover are important procedures for the mobility. Based on 5G standardization experience, inter-RAT E-UTRA measurement without gap was introduced in Rel-18 which is a bit late for the commercialization from 5G and 4G deployment perspective, Inter-RAT NR measurement without gap should be introduced in 6G day 1 and meanwhile to minimize the handover delay between 5G and 6G should be also considered in 6G day 1.

Proposal 12: for MRSS BS, propose to consider the inter-RAT NR measurement without gap and minimize the handover delay between NR and 6GR in 6G day1.

## 2.10. Switching time

- Proposals from companies - P1(OPPO): For MRSS operation via TDD/dynamic sharing, the switching time between 5G configuration and 6G configuration need study. - Way forward - Further discuss

For mobility between 5G and 6GR, this is not done in DCI manner and this is executed with RRM measurement/reporting and handover procedure, therefore we don’t see the necessity to define the switching time between 5G and 6GR.

Proposal 13: for MRSS BS, propose not to consider the switching time between NR and 6GR.

Conclusions

In this contribution, we shared some further views on the 6G MRSS BS and proposals are made as following:

Proposal 1: for numerology for MRSS between 5G and 6GR, propose to follow the agreement reached in RAN1 with the assumption of the same numerology between the commercialized 5G and 6GR.

Proposal 2: for 5G refarming bands with SCS based channel raster to 6G operation, SCS based channel raster should be applied.

Observation 1: FR1 NR bands below band n41 specified with 100kHz channel raster and 10kHz enhanced channel raster, the frequency range of band itself is limited and don’t see the strong motivation/necessity to deploy the intra-band contiguous CA.

Proposal 3: for 5G refarming bands with 100kHz or 10kHz based channel raster to 6G operation, propose to define 10kHz channel raster in 6G day 1.

Proposal 4: for MRSS BS, if the power boosted channel between 5G and 6G is overlapped in the time domain, the maximum power limitation of MRSS BS and to minimize the impacts on other DL channels on the same symbol(s) should be considered.

Proposal 5: for sync raster of MRSS BS, propose to postpone the discussion in RAN4 until RAN1 has reached sufficient progress for the initial access for MRSS deployment.

Proposal 6: for waveform for MRSS between 5G and 6GR, propose to follow the agreement reached in RAN1.

Proposal 7: if LP-WUS signal is supported in 6G day1, propose to further discuss the impacts on potential EVM degradation of NR signal due to the simultaneous LP-WUS signal transmission.

Proposal 8: for irregular channel bandwidth except for 6MHz and 7MHz, propose not to discuss the MRSS between 5G and 6G.

Proposal 9: for 6GR coexisting with in-band NB-IoT and eMTC, propose to postpone the discussion until RAN has any agreement to guide the WG’s action.

Proposal 10: for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G.

Proposal 11: for MRSS BS, propose to consider the TN BS with 5G-6G TN MRSS in the existing TN MSR specification and NTN SAN with 5G-6G NTN MRSS in the new NTN MSR specification.

Proposal 12: for MRSS BS, propose to consider the inter-RAT NR measurement without gap and minimize the handover delay between NR and 6GR in 6G day1.

Proposal 13: for MRSS BS, propose not to consider the switching time between 5G and 6GR.

References

- RP-251881, New SID: Study on 6G Radio, Approved.

- R4-2513269, Views on 6G system parameter, ZTE Corporation.

- R4-2514646, WF on [116bis][108] 6G spectrum sharing, CMCC, approved.

- RP-252960, pCR for 38.914 on Requirements of new and existing services for 6G, CMCC, approved.