3GPP TSG-RAN WG4 Meeting#117                                R4-2521760

Dallas, USA, 17-21 Nov, 2025

Agenda item:	6.13.2.2

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on conformance testing of the legacy RF requirement for SBFD BS

Document for:	Approval

Introduction

In the RAN#102 meeting, new WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD) has been approved. It’s expected to start the RAN4 meeting from April meeting. Since we have already reached good progress on the SBFD BS RF impacts in SI phase and outcome was captured in TR 38.858. In this contribution, we would like to share some further views on conformance testing for SBFD BS.

Discussion

## 2.1. Test signal and test configuration

In the last RAN4 meeting, there were some initial discussions on test signal and test configuration. For test signal for configuration-limited SBFD capable BS, it is understandable to test all declared channel bandwidth to verify the hardware design implementation e.g. front-end filter, etc. For the subcarrier spacing, the proposed value in the square bracket is somehow aligned with NR commercial network and aligned with 6G design. However in the NR specification, we still allow the different sub-carrier spacing for each bandwidth and certain bands. In order to keep the consistency with the existing the NR specification, we propose to keep the same wording for sub-carrier spacing.

Issue 1-1-4: Test signal used to build Test Configurations when testing existing Tx requirements Agreement: - For configuration-limited SBFD-capable BS, Table 1 shall be used. FFS SCS in the table. - Table 1: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-limited. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; Smallest supported subcarrier spacing. Subband configuration; All declared subband configurations. - For configuration-flexible SBFD-capable BS, use Table 2 as a starting point. FFS SCS in the table. - Table 2: Signal to be used to build NR SBFD TCs SBFD BS type; Configuration-flexible. TC signal characteristics; BWchannel; All declared channel bandwidths. Subcarrier spacing; Smallest supported subcarrier spacing. Subband configuration; DUD; DU/UD. - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widese UL subband size, and the largest NRB,SBFD,DL,1/2.; - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL..

For the configuration-flexible SBFD-capable BS, in order to simplify the test burden similar as other NR/LTE BS, we propose to use the following test signal for SBFD BS conformance testing, otherwise we might end up with different testing coverage between non-SBFD symbols (following the legacy approach) and SBFD symbols (with new approach).

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 1:For the configuration-flexible SBFD-capable BS, propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 2: For the sub-carrier spacing of test signal for both configuration-limited and configuration-flexible SBFD-capable BS, propose to smallest supported subcarrier spacing aligned with NR specification.

RAN4#116bis Issue 1-1-5: Test configuration Agreement: - Consider single carrier RAN4#108 Issue 2-1-3: Multi-carrier BS analysis - Agreement: - During Rel-18 SI, RAN4 will only discuss the interpretation-1 of multi-carrier support for SBFD-capable BS, i.e., SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier. - RAN4 didn’t study the feasibility for the case on interpretation-2 of multi-carrier support for SBFD-capable BS i.e., SBFD operates in more than one BS carriers, and legacy TDD operates in the other intra-band BS carrier(s) (if any), which is contiguous or non-contiguous to the SBFD carriers.

In the last RAN4 meeting, it was agreed to use single carrier as test configuration for SBFD capable BS, however it is not correct from my understanding. According to the Rel-18 SI and Rel-19 core requirements, for the same bands, both intra-band contiguous and non-contiguous is allowed with the SBFD operation confined within single carrier. In order to align with Rel-18 SI agreement and Rel-19 core requirement (multi-carrier with or without inter-RF gap at the receiver requirements).

Proposal 3: propose to define the test configuration for SBFD capable BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |

## 2.2. Test model

For test model for SBFD symbol in the specification, the existing test model in the specification is based on the transmission bandwidth configuration of the test signal, however for SBFD symbols, test model should be updated according to DL-UL sub-configuration correspondingly.

In the following Figure 2.2-1, we would like to share the understanding the test mode for NR-FR1-TM1.1 in the visualized way firstly.

- Figure 2.2-1. the visualized resource allocation for the legacy TM1.1

Considering the DU/UD and DUD configurations, the test model could be defined as shown in Figure 2.2-2.

- Figure 2.2-2. the visualized resource allocation for the updated TM1.1 for SBFD operation.

- Proposal 4: Propose to consider the following updated test model for NR-FR1-TM1.1 and NR-FR2-TM1.1 for SBFD conformance testing.

| Parameter | Value [DU] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

| Parameter | Value [DU case] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB -NRB,SBFD,DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

| Parameter | Value [DUD] |
| --- | --- |
| # of PRBs PDSCH nRNTI=03 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=03 | QPSK |
| Starting RB location of PDSCH nRNTI=03 | NRB -NRB,SBFD,DL,2 |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

In the following Figure 2.2-3, we would like to share the understanding the test mode for NR-FR1-TM2 and NR-FR2-TM2 in the visualized way firstly.

- Figure 2.2-3. the visualized resource allocation for the legacy TM2

- Figure 2.2-4. the visualized resource allocation for the updated TM2 for SBFD operation.

- Proposal 5: Propose to consider the following updated test model as shown in Figure 2.2-4 for NR-FR1-TM2 and NR-FR2-TM2 for SBFD conformance testing.

- In the following Figure 2.2-4, we would like to share the understanding the test models with power boosting/de-boosting in the specification.

- Figure 2.2-4. the visualized resource allocation for the legacy TM with power boosting and de-boosting

- Figure 2.2-5. the visualized resource allocation for the legacy TM with power boosting and de-boosting

- for SBFD operation.

- Proposal 6: Propose to consider the following updated test model as shown in Figure 2.2-5 for test models with power boosting/de-boosting relevant test models.

Table 4.9.2.2.2-1: Specific physical channel parameters of NR-FR1-TM1.2

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs boosted | x=40% |
| # of QPSK PDSCH RBGs which are boosted | NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of PDSCH RBGs which are boosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of boosting (dB) | 3 |
| # of QPSK PDSCH PRBs which are deboosted | NRB−3−PNRBG |
| Locations of PDSCH RBGs which are deboosted | Starting at RB#3 and excluding PRBs of RBGs which are boosted |
| Level of deboosting (dB) | 10log10NRB−3−103/10PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 | QPSK |

| Parameter | Value |
| --- | --- |
| Target percent of 16QAM PDSCH PRBs deboosted | x = 60% |
| # of 16QAM PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB−3P,12NRB−3P−NRB−3Pmod2+1NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Locations of 16QAM RBGs which are deboosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of deboosting (dB) | -3 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB−3−PNRBG |
| Locations of PDSCH RBGs which are boosted | Starting at RB#3 and excluding PRBs of RBGs which are deboosted |
| Level of boosting (dB) | 10log10NRB−3−10−3/10PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 for which EVM is not measured | QPSK |

Table 4.9.2.2.8-1: Specific physical channel parameters of NR-FR1-TM3.3

| Parameter | Value |
| --- | --- |
| Target percent of QPSK PDSCH PRBs deboosted | x = 50 % |
| # of QPSK PDSCH RBGs within a slot for which EVM is measured | NRBG=minxNRB−3P,12NRB−3+3modP−PP−NRB−3+3modP−PPmod2+1, where P is determined from table 5.1.2.2.1-1 from TS 38.214 [18], configuration 1 column using NBWPsize=NRB−3 as the size of the bandwidth part and NBWPstart=3 |
| Level of deboosting (dB) | -6 |
| Locations of QPSK RBGs which are deboosted | NRB−3+3modPP−1 and if NRBG>1, 1, 3, …, 2NRBG−2+1 |
| Level of deboosting (dB) | -6 |
| # of QPSK PDSCH PRBs within a slot for which EVM is not measured (used for power balancing only) | NRB−3−PNRBG |
| Locations of QPSK RBGs for power balancing | Starting at RB#3 and excluding PRBs of RBGs which are deboosted |
| Level of boosting (dB) | 10log10NRB−3−10−610PNRBGNRB−3−PNRBG |
| Modulation of PDSCH PRBs with nRNTI=2 for which EVM is not measured | QPSK |

## 2.3. RF channel

For RF channels for conformance testing, usually this should be discussed case by case for different RF requirement to achieve the balance between test coverage and conformance testing of wors case. In addition, for FR1 WA and higher power MR above 36 dBm with the implementation of Rx analog filter, it is not possible to have multiple RF channels for the conformance testing.

- Proposal 7:For RF channels for configuration-flexible SBFD-capable BS conformance testing, propose to discuss the case by case e.g. follow the legacy NR RF channels if the type of RF requirements are the same between non-SBFD symbols and SBFD symbols.

## 2.4. Test procedure

Regarding the test procedures for SBFD symbols, except for additional In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, other RF requirement for SBFD symbols are aligned with non-SBFD symbols, in other words, the existing test procedures for non-SBFD symbols could be also reused for SBFD symbols.

Proposal 8: For the test procedure for SBFD symbols, except for In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, propose to reuse the existing test procedures for the SBFD symbols’ conformance testing.

## 2.5. MU and test tolerance

Regarding the test procedures for SBFD symbols, except for additional In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, it’s expected to reuse the same test setup or test chamber for the conformance testing between non-SBFD and SBFD symbols. If the transmitter turn on during Rx conformance testing don’t have the impacts on Rx MU analysis, then it should be fine to reuse the existing MU for SBFD symbols’ testing requirement.

Proposal 9: If the activated transmitter during Rx conformance testing don’t have the impact on Rx MU analysis, propose to reuse the existing MU for SBFD symbols’ testing requirement.

Conclusions

In this contribution, we want to share some further views on conformance testing for SBFD BS and proposals/observations are made as following:

Proposal 1:For the configuration-flexible SBFD-capable BS, propose to define the test signal for SBFD conformance testing in the following way:

Table 4.7.2-1: Signal to be used to build NR SBFD TCs

| Operating Band characteristics |  | Band n39 | Other FR1 bands |
| --- | --- | --- | --- |
| TC signal | BWchannel | 20 MHz (Note 1) | 40 MHz (Note 1) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing |  |
| NOTE 1: If this channel bandwidth is not supported, the narrowest supported channel bandwidth shall be used. |  |  |  |

Table 4.7.2.1-2: Signal to be used to build NR TCs for BS type 2-O

| Operating band characteristics |  | FDL_high – FDL_low ≤ 3250 MHz |
| --- | --- | --- |
| TC signal | BWchannel | 100 MHz (Note 1, Note 2) |
| characteristics | Subcarrier spacing | Smallest supported subcarrier spacing declared per operating band (D.7) |
| NOTE 1: If this BS channel bandwidth is not supported, the narrowest supported BS channel bandwidth declared per operating band (D.7) shall be used. |  |  |

Proposal 2: For the sub-carrier spacing of test signal for both configuration-limited and configuration-flexible SBFD-capable BS, propose to smallest supported subcarrier spacing aligned with NR specification.

Proposal 3: propose to define the test configuration for SBFD capable BS in the following way:

| NRTC1: Contiguous spectrum operation | NRTC1: Contiguous spectrum operation |
| --- | --- |
| NRTC2: Contiguous CA occupied bandwidth | NRTC2: Contiguous CA occupied bandwidth |

- Proposal 4: Propose to consider the following updated test model for NR-FR1-TM1.1 and NR-FR2-TM1.1 for SBFD conformance testing.

| Parameter | Value [DU] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

| Parameter | Value [DU case] |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB -NRB,SBFD,DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

| Parameter | Value [DUD] |
| --- | --- |
| # of PRBs PDSCH nRNTI=03 | NRB,SBFD,DL,2 |
| Modulation PDSCH nRNTI=03 | QPSK |
| Starting RB location of PDSCH nRNTI=03 | NRB -NRB,SBFD,DL,2 |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- Proposal 5: Propose to consider the following updated test model as shown in Figure 2.2-4 for NR-FR1-TM2 and NR-FR2-TM2 for SBFD conformance testing.

- Figure 2.2-4. the visualized resource allocation for the updated TM2 for SBFD operation.

- Proposal 6: Propose to consider the following updated test model as shown in Figure 2.2-5 for test models with power boosting/de-boosting relevant test models.

- Figure 2.2-5. the visualized resource allocation for the legacy TM with power boosting and de-boosting

- for SBFD operation.

- Proposal 7:For RF channels for configuration-flexible SBFD-capable BS conformance testing, propose to discuss the case by case e.g. follow the legacy NR RF channels if the type of RF requirements are the same between non-SBFD symbols and SBFD symbols.

Proposal 8: For the test procedure for SBFD symbols, except for In-channel adjacent subband leakage and In-channel adjacent subband blocking for SBFD and Transient period between SBFD and non-SBFD operation, propose to reuse the existing test procedures for the SBFD symbols’ conformance testing.

Proposal 9: If the activated transmitter during Rx conformance testing don’t have the impact on Rx MU analysis, propose to reuse the existing MU for SBFD symbols’ testing requirement.

References

- RP-234035,New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, Samsung, Approved.

- TR 38.858

- R4-2321091, TP to TR 38.858 Impact on BS RF requirements, ZTE, Agreed

- R4-2409958, Way forward for [111][313] NR_duplex_evo, Approved.

- R4-2413283, On modification of existing TX RF requirements for SBFD, Huawei

- R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF, Approved

- R4-2411640,On the potentially new requirements for SBFD operation for FR1 and FR2-1, Samsung

- R4-2416585, Way Forward for [112bis][307] NR_duplex_evo_BSRF, Approved.

- R4-2419907	Way Forward for [113][306] NR_duplex_evo_General, Approved

- R4-2419908	Way Forward for [113][307] NR_duplex_evo_BSRF, Approved.

- R4-2502310	Way Forward for [114][306] NR_duplex_evo_General, approved.

- R4-2502380	Way Forward for [114][307] NR_duplex_evo_BSRF, approved.

- R4-2512665,Way Forward for [116][306] NR_duplex_evo_General

- R4-2512680,Way Forward for [116][307] NR_duplex_evo_BSRF

- R4-2515158, Way Forward for SBFD BS conformance, Ericsson, approved.

- R4-2313867, WF for feasibility and BS requirements impacts on SBFD operation, approved.