3GPP TSG-RAN WG4 Meeting #117	R4-2521771

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on Rel-20 NR demodulation with CDL models

Agenda item:	7.5.3

Document for:	Discussion

# Introduction

In RAN4#116-bis meeting, companies have agreements on following issues [1].

Angle scaling for rCDL

- Agreement:

  - Update the rCDL channel model based on the latest angular scaling procedure as in TR 38.901 v19.1.0 and further discuss the details.

Frequency generalization for rCDL channel modeling

- Agreement:

  - Define the requirements based on the same channel model for all different FR1 frequencies for this WI

- The maximum Doppler fD should be fixed as X Hz

    - As baseline for Rel-20 demod requirement, X =10

    - FFS if other fD can be considered

  - On the note to Maximum Doppler:

- The ratio between the UE speed v, i.e., magnitude of the velocity vector, and the wavelength is normalized to be X Hz (maximum Doppler).

Test scope for PDSCH demodulation requirements with SCM

- Agreement:

  - Cover PDSCH mapping type A demodulation requirements with 8T8R 8 layer case and the 4T4R 4 layer case.

Other issues, such as test configurations and metrics for PDSCH and PMI tests, have not achieved agreements. In this contribution, we deliver our view on open issues in WF.

# Discussion

## 2.1 	Potential model update

UE antenna modelling

- <For INFORMATION> It is RAN4 common understanding that the following were added in Rel-19 version of TR 38.901:

  - #1: Candiadate location for UE antenna placements

  - #2: Directional antenna assumption for UE

- Further discuss the two options in the next meeting:

  - Option 1: Update the UE antenna placement and assumption of rCDL based on v19.0.0 of 38.901.

  - Option 2: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

  - The moderator requests proponents of option 1 to provide detailed simulation assumptions in the next meeting.

It is a common understanding that the new UE antenna models would lead to new channel model profile derivation. The UE antenna boresight orientation and element radiation pattern would change the channel cluster truncation, then impact the final performance. But how to It might be a risk that companies can’t reach alignment within several meetings and the timeline of the WI would be delayed.

On the other hand, the issue would impact 6G channel model derivation if we don’t have conclusion now. The corresponding effort of investigation in RAN4 can’t be avoid anyway if a practical or feasible channel model is necessary for 6G requirements. One possible way is to consider the new UE antenna models in Rel-20 SI and would be better to have corresponding channel model profiles for further comparison and alignment.

The new UE antenna model might have large impact on current model and the corresponding effort on comparison and alignment would be very high.

Proposal 1	Don’t consider new UE antenna model in Rel-20 WI, and possibly study the impact in Rel-20 NR MU-MIMO SI stage.

Delay scaling after cluster truncation

- Candidate options:

  - Option 1: Consider omitting the post-truncation delay scaling step in rCDL model derivation. Methods other than linear scaling and/or metrics other than RMS delay spread could be investigated.

  - Option 2: Reuse the Rel-19 SI approach captured in TR38.753

Based on our analysis in Rel-19, the frequency coherence bandwidth is reduced after truncation during rCDL-C1 derivation, see figure below which is aligned with figure 6.4.4-2 in TR38.753. The reason is the almost half of cluster powers are removed, and the delay offset between most dominant clusters are increased if same RMS delay is applied.

If we want to have same delay offset among strong clusters to keep frequency coherence, one possible solution is to apply target RMS delay (e.g., 365ns) only on 20 clusters CDL-C model and do not recalculate cluster delay after truncation. Take rCDL-C1 as an example, the equivalent RMS delay would be 211ns if no delay adjustment is applied after truncation, see figure 2.1-1.

Figure 2.1-1 Remove cluster and not adjust the cluster delay

The frequency coherence is reduced after truncation if re-do delay power adjustment with same target RMS delay as before truncation.

The RMS delay is reduced from 365ns to 211ns if no delay power adjustment is applied after truncation.

The other solution could be combining removed cluster power to the remaining cluster with the closest delay regardless of angular character of clusters. This is similar as truncated TDL model derivation method which could keep the total cluster power and delay offset the same after truncation. By this method, the delay offset among the strongest clusters would be kept after truncation and the frequency coherence would be kept. Taking the derivation of rCDL-C1 as an example, the delay and power of cluster #6 and #14 from original 20 clusters will be updated, and the RMS delay of final 12 clusters model is 365ns, see figure 2.1-2.

Figure 2.1-2 Merge delay and power of removed clusters to remaining clusters

Based on the method in Figure 2.1-2, we can have new rCDL-C model (a.k.a. rCDL-Cx) as below. The PDSCH 8 layers performance comparison between rCDL-C1 and rCDL-Cx can be seen in Figure 2.1-3. Based on the results, the rCDL-Cx has exactly same FC as 20 cluster model while the RMS delay is also same. But corresponding spatial domain power density is changed on cluster #6 and #14, then the impact on performance might need more investigation.

Figure 2.1-3 Frequency coherence comparison

The frequency coherence of merging cluster model rCDL-Cx is the same as 20 cluster model.

Before the meeting, companies had some discussions by email, some companies propose to consider more than 12 clusters (e.g., 16 clusters) for truncation to minimize the impact by removing clusters, and also some proposals on considering AE radiation impact on cluster power and then impact on delay scaling. It would be better that TE vendors could improve the current implementation to support more filter taps to avoid losing more channel richness by truncation.

As for AE radiation impact, we think it is reasonable to consider its impact on cluster power and corresponding delay spread, but current AE radiation pattern has minor difference than isotropic on these clusters based on previous study. In that case, the impact might not huge even we don’t consider AE radiation pattern in current assumption.  On the other hand, the impact might need some investigation if we don’t use current assumption, such as using subarray virtualization rather than a single AE.

Regarding the CDL model would be applied in whole 6G demodulation requirements, it is worthy to take more effort to have better understanding the impact of different truncation methods, and TE vendors are also expected to give input on necessary of increasing cluster numbers supported by equipment.

Proposal 2	Companies take 20 clusters model as baseline and check the impact of different truncation methods within two meetings.

Further rCDL-C1 channel alignment

- Candidate options:

  - Option 1: Measure and align channel statistics such as Spatial Domain Power Density (SDPD), Time Coherence (TC), and Frequency Coherence (FC) as described in TR 38.753 Section 6.4.

  - Option 2: Not to do further rCDL-C1 channel alignment

On rCDL-C1, we have analysis on the channel properties in option 1 during Rel-19 SI stage and the derivation formula are captured in section 6.4 in TR 38.753. It could be necessary for companies to check channel properties once the rCDL-C model is updated in Rel-20 together with simulation results alignment among companies.

Proposal 3	Companies to check channel properties together with simulation results alignment once the rCDL-C model is updated according to TR38.901 v19.

## 2.2	Test scope

Antenna Array Virtualization (AAV) assumptions

- Candidate options:

  - Option 1: Only consider Ms=Ns=1

Parameters for CDL modelling

- Candidate options:

  - Option 1: Use CDL parameters listed in Table 6.1-2 of TR 38.753 as baseline

Duplex mode and SCS

- Agreement:

  - Cover TDD 30kHz SCS with 40MHz CHBW and FFS whether to also cover FDD 15kHz SCS with 10MHz CHBW

Companies had a lot of discussion on the antenna virtualization method during Rel-19 SI stage. One of issues is that the subarray gain will be normalized at the receiver side which will lead to worse performance than no subarray approach. But we also see some performance benefit in some special cases, such as eType-II PMI, if we apply larger element separation (e.g., extending from 0.5λ to 4 λ) to represent subarray impact (the vertical radiation pattern should be reduced, but not considered in Rel-19 stage) [3].

It is OK to reuse current one-to-one mapping method in Rel-20 WI, but it is worth studying more practical study more practical antenna array virtualization approach in 6G stage to address massive MIMO benefit in real network from demodulation perspective.

Proposal 4 	Only consider Ms=Ns=1 for antenna virtualization in Rel-20 WI on NR demodulation enhancement.

Proposal 5	RAN4 should further study practical method on subarray virtualization for SCM in 6GR.

### 2.2.1	SU-MIMO PDSCH

Precoding, MCS and test metric

- Agreement and candidate options:

  - For 4T4R 4L test, it is agreed to use the test metric: SNR @ 70% max TP.

  - For 8T8R 8L test, it is agreed to use test metric: Define single SNR and further discuss the SNR is based on:

    - Option a: Based on per CW thput with Different MCS per CW

    - Option b: Based on total thput with different MCS per CW

    - Option c: Based on total thput with same MCS per CW

    - Further discuss: How to define the SNR for option a. How to select the MCS for different CW.

The motivation of this issue is the target SNR for worse CW in 8 layers transmission would be very high and not practical for test. In that case, companies propose to use lower MCS for worse CW to make it more practical since UE would report ACK/NACK and CQI separately. But in existing test setup, it is not possible to directly configure different MCS for two CW. Thus, twice of tests might be needed with different MCS.

Based on our simulation results [2], we can see the target SNR@70% NThp on better CW with MCS13 is very close to SNR on worse CW with MCS10. Thus, it could be possible to use MCS13 for better CW and MCS10 for worse CW and take 70% NThp SNR as metric.

Figure 2.2-1 NThp comparison among different MCS configurations

Proposal 6	Apply MCS13 for better CW and MCS10 for worse CW. Take SNR@70% NThp per CW as throughput.

PMI bias

- Candidate options:

  - Option 1: RAN4 shall exploit approaches to address the PMI bias issue

- Option 1a: Apply the existing simple beam steering approach specified in TS 38.101-4 B.2.3.2.3

- Option 1b: time variant mean angles (AOA/AOD/ZOA/ZOD)

- Other options are not precluded

  - Option 2: No need to consider time-wise evolution for PMI reporting tests

  - Option 3: Defer a consideration of a comprehensive long-term solutions to the 6G Radio Study Item

2.2.2	SU-MIMO PMI

To have a better PMI report requirement, multiple channel power angular distributions would be necessary to check UE report correct PMI accordingly. Currently, beam steering matrix on Tx side together with TDL model is applied for existing PMI report requirements. From the test setup perspective, it could have similar impact when CDL model is applied, but it could be complex for simulator implementation on top of CDL model regarding angular scaling. Another risk is the alignment among companies, especially for eType-II cases regarding to quite diverse results in Rel-19 SI.

For time variant mean angles changing, our understanding is that multiple channel model profiles would be implemented in TE in advance, and profiles are applied by a certain order during the tests. It means companies should derive these profiles and could get alignment among these profiles. The effort would also be high as previous option.  To simplify, maybe limited number (e.g., 4) of different channel models can be derived and implemented in TE.

Considering the evaluation effort would be high for this issue and existing PMI report requirements have consider beam steering method, the Rel-20 WI on PMI report performance could skip this issue and focus on the alignment of eType-II since this requirement is assumed as optional on top of existing PMI requirements.

But it could be better to consider this in Rel-20 SI scope. It might be possible to have limited profiles according to relative larger variant steps either by beam steering or mean angular changing (especially on AoA and AoD), then to check if different PMI would be reported. Another way is to have channel coefficients data sets based on simulations including different strong channel angles, then TE need to update coefficients by looking up data sets.

Proposal 7 	Consider PMI bias issue in Rel-20 NR MU-MIMO SI scope.

Whether to define eType II PMI reporting requirements based on SCM

- Candidate options:

  - Option 1: Specify requirements for PMI performance with eType II precoding

- Option 1a: Select the lowest γ value among the ones companies provided.

- Option 1b: Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

  - Option 2: Further simulation result alignment is required

- Option 2a: Collect both TP gain and SNR @ 90% Max TP to evaluate alignment

- Option 2b: RAN4 can take 8Tx 4 layers Type-I PMI case for initial alignment if rCDL model is updated.

Test metric for eType II PMI reporting (if introduced)

- Candidate options:

  - Proposal 1: Reuse the same test metric for the existing eType II requirement based on TDL, i.e., Thput of following PMI (eType II)/ Thput of Random PMI (Type I codebook) with SNR @ 90 % of the maximum throughput for following PMI.

  - Proposal 2: Check PMI simulation results on both normalized throughput and gamma values and do down selection.

The eType-II code book is expected to have better performance then type-I codebook for high layer transmission due to smaller spatial resolution and power concentration. But the benefit of eType-II is limited and companies’ results are not aligned well according to simulation campaign in Rel-19. This is the main concern from companies.

From demodulation perspective, it is better to have eType-II requirements with CDL model. Regarding the potential rCDL-C model updates, we suggest companies trying to have alignment on eType-II again after model modifications. The throughput curves for follow PMI and random PMI, γ value could be delivered, then companies could check the alignment status and make decision. The Type-I PMI case could be captured either to check the alignment level and benefit from eType-II.

Proposal 8	Make discission on whether and how to define eType-II PMI report requirements after the alignment campaign once rCDL-C model is updated.

Test scope for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: RAN4 needs to define requirements for both 8T4R and 4T4R cases as captured in the WID

Rank for eType II PMI reporting (if introduced)

- Candidate options:

  - Option 1: Only consider rank 2

  - Option 2: Consider rank 4

For eType-II, higher layer transmission would be more typical scenario especially for 8T4R configuration. It would be better to capture 8T4R with 4 layers and 4T4R with 2 layers in the simulation scope. We think the key issue is still about the alignment level among companies if 4 layers case is considered. As we discussed above, companies could make decision after simulation alignment compaign with updated rCDL-C channel model.

Proposal 9	Considering 8T4R for rank 4 and 4T4R for rank 2 in the simulation scope, and make decision on requirement definition based on simulation result alignment for updated rCDL-C model.

## 2.3	Test applicability

Test applicability for SCM SU-MIMO requirements

- Candidate options:

  - Proposal 1: Introduce new declaration for the SCM testing. RAN4 needs to discuss whether to define one single declaration on SCM requirements or define multiple declarations for each individual SCM test cases.

  - Proposal 2: Introduce new optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules

  - Proposal 3: If requirements with rCDL are introduced, their applicability should be optional and based on UE declaration to support these requirements.

Generally, the receiver algorithm should be independent of channel model, but the new defined demodulation requirement with SCM should be optional according to companies’ consensus. In that case, capability declaration and corresponding applicability rule might be necessary. Based on the requirement characters, proposal 2 could be a common way for Rel-20 scope. But for 6G, it might not be necessary to consider such kind of declaration and applicability.

Proposal 10	Take proposal 2 as the temporary UE capability to define applicability rule for demodulation requirements introduced in Rel-20 SCM WI.

Release independent for SCM SU-MIMO requirements

- Candidate options:

- Proposal 1: Discuss whether to define release independent rules for the new SCM requirements.

It seems a bit early to discuss this at current stage regarding the scope is not completely settled. We propose postponing the discussion until the requirements are confirmed.

Proposal 11	Postpone discussion on release independent till the requirements scope are confirmed.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The new UE antenna model might have large impact on current model and the corresponding effort on comparison and alignment would be very high.

Observation 2	The frequency coherence is reduced after truncation if re-do delay power adjustment with same target RMS delay as before truncation.

Observation 3	The RMS delay is reduced from 365ns to 211ns if no delay power adjustment is applied after truncation.

Observation 4	The frequency coherence of merging cluster model rCDL-Cx is the same as 20 cluster model.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Don’t consider new UE antenna model in Rel-20 WI, and possibly study the impact in Rel-20 NR MU-MIMO SI stage.

Proposal 2	Companies take 20 clusters model as baseline and check the impact of different truncation methods within two meetings.

Proposal 3	Companies to check channel properties together with simulation results alignment once the rCDL-C model is updated according to TR38.901 v19.

Proposal 4 	Only consider Ms=Ns=1 for antenna virtualization in Rel-20 WI on NR demodulation enhancement.

Proposal 5	RAN4 should further study practical method on subarray virtualization for SCM in 6GR.

Proposal 6	Apply MCS13 for better CW and MCS10 for worse CW. Take SNR@70% NThp per CW as throughput.

Proposal 7 	Consider PMI bias issue in Rel-20 NR MU-MIMO SI scope.

Proposal 8	Make discission on whether and how to define eType-II PMI report requirements after the alignment campaign once rCDL-C model is updated.

Proposal 9	Considering 8T4R for rank 4 and 4T4R for rank 2 in the simulation scope, and make decision on requirement definition based on simulation result alignment for updated rCDL-C model.

Proposal 10	Take proposal 2 as the temporary UE capability to define applicability rule for demodulation requirements introduced in Rel-20 SCM WI.

Proposal 11	Postpone discussion on release independent till the requirements scope are confirmed.

# References

[1] 	R4-2514832, WF on NR_demod_Ph6_Ph5, China Telecom

[2]	R4-2521772, Simulation results on Rel-20 NR demod with SCM, Ericsson

[3]	R4-2510713, Simulation results for Rel-19 SCM for demodulation, Ericsson