3GPP TSG-RAN WG4 Meeting #117	R4-2521772

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Simulation results for Rel-20 SCM for demodulation

Agenda item:	7.5.3

Document for:	Information

# Introduction

In RAN4#116bis meeting, the general view on rCDL model updates and test cases modifications are discussed. In this contribution, some potential modification impacts are compared according to options in WF [1].

# Simulation

Table 2-1: Common simulation assumptions for PDSCH

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |

Table 2-2: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Codebook for DMRS and PDSCH |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

## Frequency coherent comparison

Figure 2-1 Frequency coherence comparison

## Requirement metric for 8 layers PDSCH

Table 2-3 Simulation results summary for SU-PDSCH with rank 8

| Channel model | PMI configuration | Equivalent Doppler [Hz] | CBW/SCS [MHz/kHz] | MCS (Table 1) | Per CW SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 30% |  | 70% |  |
|  |  |  |  |  | CW0 | CW1 | CW0 | CW1 |
| rCDL-C1 in 38.753 | random | 10 | 40 / 30 | 13 | 7.6 | 10.2 | 17.7 | 24.1 |
|  |  |  |  | 11 | 3.2 | 8.5 | 13.6 | 20.1 |
|  |  |  |  | 10 | 2.6 | 7.8 | 11.8 | 18.9 |

Figure 2-2 NThp comparison among different MCS configurations

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2514832, WF on NR_demod_Ph6_Ph5, China Telecom

# Annex A

Table A-1 CDL-C A2_20clusters

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 232 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 240 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 290 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 300 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 340 | -14.1 | -73 | 43.1 | 92.3 | 50.8 |
| 9 | 448 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 476 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 11 | 792 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 12 | 988 | -16.2 | 64.6 | 26.3 | 105.2 | 121 |
| 13 | 1552 | -16.9 | 62.2 | -22.6 | 94 | 53.9 |
| 14 | 1678 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |
| 15 | 2002 | -18.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 16 | 2044 | -20.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 17 | 2300 | -19 | 58.1 | 0.2 | 91.7 | 35.8 |
| 18 | 2420 | -18.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 19 | 2568 | -24.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 20 | 3154 | -25.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-2: Channel model parameters for rCDL-C1

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-3: Channel model parameters for rCDL-Cx

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 232 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 240 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 294 | -8.5 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 340 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 448 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 476 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 792 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 988 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 1992 | -10.0 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |