3GPP TSG-RAN WG4 Meeting #117	R4-2521774

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on 6G operation efficiency on demodulation

Agenda item:	8.9.2.3

Document for:	Discussion

# Introduction

A new meeting agenda is assigned for RAN4#117 to discuss how to improve the operation efficiency in 6GR including specification and modernization in general and ideas for specific RAN4 area. In RAN4#116-bis meeting, companies proposed some ideas on how to improve current demodulation specifications.

In this contribution, our views for RAN4 Demod operation efficiency improvement are discussed.

# Discussion

Demodulation specification principles

- Proposals

  - Option 1: For 6G Demodulation specification structures, take TS38.101-4 as a starting point.

  - Option 2: For 6G Demodulation specification drafting principles, the descriptions of test parameters should be aligned with RAN1/RAN2 descriptions as much as possible, in order to avoid ambiguous understanding.

  - Option 3: For FRCs in 6G Demodulation specification, prefer to use a formula-based or pseudo-code-based definition for FRCs instead of table-based approach listing every parameter combination.

    - Option 3A: RAN4 needs to discuss how to specify FRC table in the specification for both BS and UE demodulation performance, considering the discussion in SI modernization of specification format and procedures for 6G.

3GPP is now discussing the modernization of specification format and procedures for 6G in Rel-20 SA, RAN and CT joint SI. This SI studies to maintain the 3GPP specifications not in Word files, but in a version management system hosted in 3GPP (https://forge.3gpp.org/rep/fs_6gspecs_new). In the new management system, the technical specifications would be described by new formats such as Markdown or OpenDocument. The conclusion will be published as TR 21.802.

The big challenge of RAN4 demodulation requirements is to maintain many complex tables to define the test configurations and measurement channels. For the measurement channel tables, RAN4 has spent a large amount of time to specify and maintain the tables. Since most parameters in the tables are derived from RAN1 specifications, we think it is possible to specify them in a simpler way, for example, formula-based definition.

Proposal 1	Interested companies are encouraged to keep eyes on the discussion on Rel-20 SI Modernization of Specification Format and Procedures for 6G. If necessary, companies should contribute to this SI from RAN4 demodulation requirement point of view.

Proposal 2	RAN4 should discuss further how to specify the demodulation performance requirements and measurement channel tables in the new specification formats.

FRC tables are widely used in both BS and UE demodulation specifications. There are many maintenance CRs to correct small errors in FRC tables in almost every meeting. For BS demodulation specifications, how to align FRC table numbering among specifications was discussed but finally failed due to un-synchronized CR submission for a feature. If new specification format is agreed, the more flexible FRC definition should be captured to address concerns above.

For UE PDSCH FRC, it is specified the channel bits and TBS per slot. Although such fixed scheduling is useful to calculate the throughput, it is not suitable for dynamic scheduling scenario, for example, link adaptation or adaptive PRB size, and non-slot-based scheduling, for example, repetition in Cat-M/NB-IoT. RAN4 should study more flexible FRC considering such scenarios.

Proposal 3	Take FRC table improvement (e.g., more flexible FRC) as start point for demodulation specification improvement.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Interested companies are encouraged to keep eyes on the discussion on Rel-20 SI Modernization of Specification Format and Procedures for 6G. If necessary, companies should contribute to this SI from RAN4 demodulation requirement point of view.

Proposal 2	RAN4 should discuss further how to specify the demodulation performance requirements and measurement channel tables in the new specification formats.

Proposal 3	Take FRC table improvement (e.g., more flexible FRC) as start point for demodulation specification improvement.

# References

[1]