# 3GPP TSG-RAN WG4 Meeting #117	R4-2521781

Dallas, USA, Nov. 17 – 21, 2025

Agenda Item:	4.21.1

Source: 	Ericsson

Title:	Remaining issues on Rel-19 NES

Document for:	Discussion

# Introduction

In RAN4 #116 meeting, the Rel-19 NES enhancement WI has completed. One of maintenance issue was identified in last meeting[1]. In this contribution, we will discuss this maintenance issue on Rel-19 NES.

- OD-SSB

Interruption

In RAN4 #116 meeting, RAN4 agreed the interruption requirement directly in the CR. However, we noticed that the interruption occasions and length are not aligned in different sections. From our understanding, when UE supports Rel-19 OD-SSB capability, UE can follow the OD-SSB periodicity to perform the SCell activation. In other words, UE doesn’t need to wait the SMTC and the max SMTC among active serving cells to perform the SCell activation.

In Rel-15, the interruption is extended with SMTC duration due to the AGC retuning should based on the SMTC occasions together with all the active serving cells [2]. When UE supports Rel-19 OD-SSB capability, UE can response the OD-SSB immediately. The related RF retuning action can also follow the same method to retune the AGC based on OD-SSB directly. Therefore, the OD-SSB based SCell activation interruption for intra-band should follow the same principles of inter-band SCell activation.

RAN4 #113 Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue

Proposal 1: In Intra-band SCell activation scenario, RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

- Summary

In this contribution, we have discussed the remaining issues of NES enhancement requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: In Intra-band scenario, RAN4 to align the interruption length of OD-SSB based deactivated SCell measurement with SCell activation as the RF retuning time without the SMTC duration.

- References

- R4-2514808, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R4-1810013, “Misalignment Issue on SCell Activation Delay and Interruption requirements”, MediaTek Inc.