3GPP TSG RAN WG4 Meeting #117	R4-2521785

# Dallas, USA, 17 November 2025 – 21 November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.921 | CR | - | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (FS_NR_IMT_param) (NR_BS_RF_req_evo-Core) draftCR to TR 38.921 Addition of BS Transmitter Spurious emission co-existence work |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IMT_param, NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-28 |  |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |
| Reason for change: |  | RAN4 received a LS from ETSI MSG TFES in R4-2414917 on the Tx spurious emission asking RAN4 to re-evaluate co-existence requirements limits for high frequency bands. ETSI also asked to provide feedback on the assumptions taken for the derivation of the limits in BS specifications. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause 10 – BS Tx spuirous emission co-existence re-evaluation is added to TR 38.921 with following sub-clauses: - General - Co-existence study - Co-existence simulation results - Summary of results |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The essential simulation work carried out with relevant assumptions and the new methodology will not be documented, which can be analyzed or re-visited for future work. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 10 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200513: "Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz"

[3]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[4]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[5]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[6]	ERC Recommendation 74-01, "Unwanted emissions in the spurious domain"

[7]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[8]	3GPP TR 25.942, “Radio Frequency (RF) system scenarios”

[9]	3GPP TR 38.922, “Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14800-15350 MHz”

<End of change 1>

<Start of change 2>

## 10	BS Transmitter spurious emissions: co-existence requirements limits re-evaluation for high frequency bands

### 10.1	General

#### 10.1.1	Scope

This clause presents the outcome based on the request from ETSI MSG TFES [7] to re-evaluate Transmitter spurious emission co-existence requirement levels. The focus of this work is on bands in the upper region of FR1 with priority above 5.9 GHz.

#### 10.1.2	Background

The spurious emission requirements in current RAN4 specifications for lower bands in FR1 region were originally evaluated considering passive sector antenna having a fixed main beam with peak gain of 17 dBi, where they are two networks in the same geographical area. The aggressor BS has interfering link towards the wanted UE in the same network and victim BS of the other network. The reference simulation assumptions can be found in TR 25.942 [8].

Based on the duplexing method FDD and TDD, the requirement levels apply as listed in Table 10.1.2-1 below –

Table 10.1.2-1: Requirement levels

| Duplex method | BS-to-BS (dBm/MHz) | BS-to-UE (dBm/MHz) |
| --- | --- | --- |
| FDD | -49 | -52 |
| TDD | -52 | -52 |

##### 10.1.2.1	Protection of UE receiver (BS-to-UE)

The transmitter co-existence spurious emission requirement for BS is defined to protect the downlink of another network in the same geographical area. According to TR 25.942 [8], subclause 5.1.4.1, the coupling loss between BS and UE was set to -

- Minimum Coupling Loss (MCL) equal to 70 dB for the Macro-cellular environment,

- Minimum Coupling Loss (MCL) equal to 53 dB for the Microcell environment.

However, the MCL for Microcell environment was adopted for requirement level derivation. The Microcell MCL was selected since it was seen as the worst-case MCL.

The assumed values are listed in Table 10.1.2.1.-1.

Table 10.1.2.1.-1: Original assumptions for BS-to-UE coupling

| Parameter | Variable | Value |
| --- | --- | --- |
| UE receiver noise figure | F | 9 dB |
| BS-to-UE coupling loss | L | 53 dB |

The current requirement level is derived based on a link budget calculation on a logarithmical scale:

Pem=kT+B+F+L

where kT is equal to -174 dBm/Hz and B is equal to 1 MHz. Assuming the following values for the UE noise figure, F equal to 9 dB and L equal to 53 dB, the maximum allowed spurious emission level, Pem at the BS transmitter can be calculated to -52 dBm/MHz.

##### 10.1.2.2	Protection of BS receiver (BS-to-BS)

Traditionally the requirement level was determined by a static link budget calculation based on assumptions of BS noise figure and Macrocell BS-to-BS coupling loss. The assumed values are listed in Table 10.1.2.2-1 below -

Table 10.1.2.2.-1.: Original assumptions for BS-to-BS coupling

| Parameter | Variable | Value | Note |
| --- | --- | --- | --- |
| BS receiver noise figure | F | 7 dB |  |
| Macrocell BS-to-BS coupling loss | L | 67 dB | The current requirement level is derived based on an assumption of MCL between two base stations operating in different networks in the same geographical area of 67 dB (see TR 25.942 [8], subclause 10.2.1). |
| Protection criteria | M | 9 dB | Assumed value corresponds to 0.5 dB degradation of victim receiver sensitivity. |

The current requirement level is derived based on logarithmical scale as:

Pem=kT+B+F+L-M

where kT is equal to -174 dBm/Hz, B is equal to 1 MHz and parameter values in Table 2.1.2-1. With the assumption the maximum allowed spurious emission level in another band is calculated to -49 dBm/MHz.

#### 10.1.3	Modelling of out-of-band antenna gain

##### 10.1.3.1	Method 1: Based on roll-off profile curve with correlation factor 0.5 [9]

The model equation for analyzing decorrelation effects in advanced antenna systems (AAS) was first introduced in 3GPP Release-11 as part of the AAS Study item documented in TR 37.840. This equation provided a mathematical framework to capture the relationship between spatial separation, correlation, and system performance. However, at that time, RAN4 could not reach agreement on the most appropriate roll-off profile, which describes how correlation decreases as spatial separation increases. The absence of a consensus meant that the model remained incomplete in this respect, limiting its practical applicability for performance evaluations and simulations.

In Release-19, a profile was finally introduced in TR 38.922 [9] providing a reference curve for modeling decorrelation in certain deployment scenarios. The equation below from [9] describes the composite far field antenna gain for an AAS BS in the presence of partial spatial correlation – particularly relevant for out-of-band emissions.

AAθ,φ=Asubθ,φ+10log101+ρm=1Mn=1Nwm,nvm,n2-1   [9]

As can be seen in the equation, the composite antenna pattern is the sum of the subarray element gain and an array gain component which depends on the array geometry, beamforming vector and element correlation. This model accounts for spatial decorrelation due to spectral separation from the carrier, which affects the coherent summation in the array factor. The key addition in this model is the ρ(f) where f=offset from carrier center, which models the roll-off of coherent gain as a function of offset from the carrier center frequency. The parameterized piece-wise linear roll-off model is captured in TR 38.922 Clause 7.3.2.1.2 [9], which depicts how the fundamental aspects of the array antenna gain drops outside the wanted signal.

The out-of-band emissions consist of a mix of two components:

- Correlated intermodulation distortion (IMD) products, which are partially coherent across antenna elements because they originate from the same nonlinear processes.

- Uncorrelated wideband transmitter noise, which is random and independent at each antenna element.

Because of this mixture, the correlation factor ρ(f) does not simply jump from 1 (fully correlated) inside the carrier to 0 (fully uncorrelated) immediately outside it. Instead, ρ(f) gradually decreases with frequency offset from the carrier edge as noise becomes more dominant and coherence diminishes. Assuming ρ(f)=1 outside the carrier tends to overestimate the coherent array gain, potentially underestimating interference levels, while assuming ρ(f)=0 can be too conservative by ignoring any partial correlation that exists.

The full array doesn’t make sense for integer multiples of the fundamental frequency as the antenna array and beamforming are primarily optimized for the fundamental frequency. Thus, the antenna efficiency drops reducing the effective radiated power at integer multiples regardless of the array size. We can also not ignore the fact that the physical size of the antenna elements is fixed but the wavelength λ changes with frequency which further drastically changes the radiation pattern, impedance input and efficiency. These combined effects of wavelength mismatch, reduced antenna efficiency and incoherent nature of the integer multiples of the fundamental frequency eliminates the possibility for the antenna array to achieve full array gain. As a tradeoff due to above reasons, 0.5 (partial coherent) is suggested as a way to model the partial coherence between the array elements.

### 10.2	Co-existence study

#### 10.2.1 	Co-existence simulation scenarios

Table 10.2.1-1 summarizes the simulation scenarios.

Table 3.1-1: Summary of simulation scenarios

| No. | Aggressor Simulating frequency | Victim Simulating frequency | Direction | Priority |
| --- | --- | --- | --- | --- |
| 1 | 2 GHz | 6.4 GHz | DL – BS Micro to UE |  |
| 2 | 2 GHz | 6.4 GHz | UL – BS Macro to BS Macro |  |
| 3 | 2 GHz | 6.4 GHz | UL – BS Macro to BS Micro | Down-prioritized |
| 4 | 2 GHz | 6.4 GHz | DL – BS Macro to UE | Down-prioritized |
| 5 | 2 GHz | 6.4 GHz | UL – BS Micro to BS Micro | Down-prioritized |
| Note 1: It is agreed that for DL, the most critical scenario is Micro BS to UE interference. Note 2: It is agreed that for UL, the most critical scenario is Macro BS to Macro BS interference. |  |  |  |  |

#### 10.2.2 	Co-existence simulation assumption

##### 10.2.2.1	Network layout model

###### 10.2.2.1.1 	Urban Macro

Details on urban macro network layout model are listed in Table 10.2.2.1.1 -1 and 10.2.2.1.1 -2.

Table 10.2.2.1.1 -1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 500 m | Similar to TR 25.942 assumptions |
| BS antenna height |  | 25 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |

Table 10.2.2.1.2 -2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | Option 1: Un-coordinated operation - 100% Grid Shift (Baseline) Option 2: Un-coordinated operation - 50% grid shift | Option 1 is baseline and is used for result derivation, primarily to replicate similar cell arrangement as in TR 25.942. |

Figure 10.2.2.1.1 -1: Uncoordinated operation

###### 10.2.2.1.2 		Dense Urban

Details on dense urban network layout model are listed in Table 10.2.2.1.2 -1 and 10.2.2.1.2 -2.

Table 10.2.2.1.2 -1: Single operator layout for dense urban

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | Fixed cluster circle within a macro cell. | note1 |
| Number of micro BSs per macro cell |  | 3 | 3 cluster circles are in a macro cell. 1 cluster circle has 1 micro BS. |
| Radius of UE dropping within a micro cell |  | < 72.17 m | 3÷4×ISD÷3 |
| BS antenna height |  | 10 m |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 80 % |  |
|  | 50% low loss, 50% high loss | Low/high Penetration loss ratio |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMi in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 3m |  |
| Channel model |  | UMi |  |
| Shadowing correlation |  | Between Micro site: 0.5 |  |
| Note 1: Micro BS is randomly dropped on an edge of the cluster circle. All UEs communicate with micro BS, i.e. macro cell is only used for determining position of micro BS. As a layout of macro cell, hexagonal grid, 19 macro sites, 3 sectors per site model with wrap around with ISD = 500 m is assumed. |  |  |  |

Figure 10.2.2.1.2 -1: Network layout for dense urban

Table 10.2.2.1.2 -2: Multi operators layout for dense urban

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Cluster circle is coordinated | Note 1 |
| Minimum distance between micro BSs in different operator | 10 m |  |
| Note 1: Macro cell is collocated. Micro BS itself is randomly dropped. |  |  |

##### 10.2.2.2 	Propagation Model

The path-loss model is described in Clause 4.2.2. and can be used all propagations unless stated.

For modelling BS-to-BS propagation, the free space path loss (FSPL) in dB should be used, where the separation distance d between BSs is in meter and frequency fc in GHz is given by

##### 10.2.2.3 	Antenna characteristics

###### 10.2.2.3.1	BS antenna characteristics

The BS antenna is modelled as described in Clause 8.1.1. However, the antenna parameters for 6.4 GHz can be taken from Clause 8.1.2 and for 2GHz for different BS deployments are listed in Table 10.2.2.3.1-1 below –

Table 10.2.2.3.1-1: IMT parameters relevant for 1710 to 4990 MHz [9]

| Parameter | Macro Rural | Macro suburban | Macro urban | Micro urban |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. | 65 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 6.4 dBi |
| Msub | 3 | 3 | 3 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | N/A |
| subtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 4 | 4 | 4 | 8 |
| N | 8 | 8 | 8 | 8 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.1l m | 2.1l m | 2.1l m | 0.7l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤120 deg. |
| escan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 46 dBm | 37 dBm |
| mech | 3 deg. | 6 deg. | 6 deg. | N/A |

###### 10.2.2.3.2	UE antenna characteristics

The UE is expected to have a conducted interface assuming an isotropic radiation pattern antenna and without beamforming.

##### 10.2.2.4 	Other simulation parameters

Table 10.2.2.4-1: Other simulation parameters

| Parameters | BS-to-BS coupling: Urban macro | BS-to-UE coupling: Dense urban |
| --- | --- | --- |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |
| The number of active UE (UL) | 1 UE per slot | 1 UE per slot |
| Additional Polarization gain | 0 dB | 0 dB |
| Gain at Rx | Full array again |  |
| BS Noise figure in dB | 6 dB for 6.4 GHz |  |
| UE Noise figure in dB |  | 9 dB for 6.4 GHz |

##### 10.2.2.5 	Evaluation metric

The Release-99 study in TR 25.942 to evaluate the minimum coupling loss refers to 100%tile on the coupling loss CDF curve which is very sensitive to simulation and can fluctuate across different simulation run. The results presented are summarized for the following CDF %tiles on the coupling loss curve –

- 99.9%tile

- 99.95%tile

- 99.99%tile

##### 10.2.2.6	Co-existence simulation methodology

Adopt the following simulation steps -

Step 1 – Generate Aggressor and Victim networks based on the deployment parameters, defined for the Network layout. The aggressor network operates at 2 GHz, while the victim network operates at 6.4 GHz. The propagation of the spurious emissions happens at 6.4 GHz.

Step 2 – UEs are randomly generated inside the two networks deployment.

Step 3 – The BF weights are adjusted to point to the LOS direction between the serving BSs and served UE, in the two networks.

Step 4 – Apply BF weights to the subarray ports.

Step 5 – Calculate the Coupling Loss between the aggressor BS antenna, dimensioned to operate at 2 GHz, and the victim UE or BS antenna, which are operating at 6.4 GHz.

### 10.3	[Co-existence simulation results]

#### 10.3.1	BS-to-BS coupling: Urban Macro

Table 10.3.1-1 Coupling loss CDF %tile with correlation factor 0.5 and 100% grid shift for BS-to-BS coupling Urban Macro

|  | Coupling Loss CDF %tile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 99.99% | 99.95% | 99.9% |  |  |
| ZTE |  |  |  |  |  |
| Huawei |  |  |  |  |  |
| Nokia |  |  |  |  |  |
| Ericsson |  |  |  |  |  |

Table 10.3.1-2 Coupling loss CDF %tile with correlation factor 0.5 and 50% grid shift for BS-to-BS coupling Urban Macro

|  | Coupling Loss CDF %tile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 99.99% | 99.95% | 99.9% |  |  |
| ZTE |  |  |  |  |  |
| Huawei |  |  |  |  |  |
| Nokia |  |  |  |  |  |
| Ericsson |  |  |  |  |  |

#### 10.3.2	BS-to-UE coupling: Dense Urban

Table 10.3.2-1 Coupling loss CDF %tile with correlation factor 0.5 for BS-to-UE coupling Dense Urban

|  | Coupling Loss CDF %tile |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 99.99% | 99.95% | 99.9% |  |  |
| ZTE |  |  |  |  |  |
| Huawei |  |  |  |  |  |
| Nokia |  |  |  |  |  |
| Ericsson |  |  |  |  |  |

### 10.4 	[Summary of results]

<End of change >