3GPP TSG-RAN WG4 Meeting #117	R4-2521796

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on Rel-19 LP-WUS Performance Requirements

Agenda item:	4.22.3

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

Issue 1-1-3 LP-WUR operation with RedCap <Agreement> The MR wake up delay of the 2Rx RedCap UE applies for 1Rx RedCap UE Issue 1-1-9: On the serving cell and interference cell LP-SS with opposite sequences for the case when {M, L}={1,6} <Agreement> Capture the following note in LP-WUR core requirements: Note: Higher margin may be required for UE to consider an entry or exit criteria fulfilled when LR related thresholds are configured, if the serving cell and neighbour cell are using opposite binary sequences for LP-SS, and LP-SS are transmitted on the same time and frequency resource in this two cells.

Discussion

RAN2 MR relaxation exit

In RAN2#131 the following definition for entry was agreed:

<RAN2 agreement>: The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

The above means that there is no configuration for relaxation exit threshold from the network side.

Figure 1: Only entry threshold is configured based on RAN2 agreement for Relaxation.

The above only applies for relaxation scenario, for MR offloading RAN2 has both entry and exit thresholds.

Our understanding is that the RAN4 has had the working assumption that there are both entry and exit thresholds configured for MR relaxation. Hence, the RAN4 has specified the following:

The UE shall evaluate and consider an entry or exit criteria is fulfilled within Tevaluate-LP-WUR-PSS/SSS, provided that the corresponding criteria is met by a margin of 6 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR1 and by a margin of 7.5 dB for SS-RSRP and/or 3.5 dB for SS-RSRQ in FR2 when SSB Ês/Iot ≥ -3dB

The RAN4 margin is related to measurement accuracy which may vary based on radio conditions causing ping-pong. One option to avoid this ping pong is to define time to trigger-type of time condition but it’s not clear if this would help, as it would just delay the ping-pong effect.

Network cannot configure exit threshold for RRM relaxation and with the margin, the exit may cause ping-pong.

Define static exit threshold based on entry threshold configuration. Discuss what should be the value, e.g. 4dB (in addition to margin).

When the UE exits the relaxation, we understand that there may be need for wake-up time as the UE may sleep between the measurements. In this sense, the relaxation does not differ much from offloading.

The existing wakeup delay is re-used when UE exits RRM relaxation mode due to exit condition being met.

LP-WUR and SDT

Related to SDT, RAN2 agreed the following in the last meeting:

RAN2 agreement: - Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed.

Small data transmission (SDT) was specified in R17 and R18. SDT is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). SDT can be initiated either by the UE or by the network. MO-SDT is initiated by the UE only if less than or equal to a configured amount of UL data awaits transmission across all radio bearers for which SDT is enabled, the DL RSRP is above a configured threshold, and a valid SDT resource is available. MT-SDT is initiated by the network with an indication to the UE in a paging message when DL data awaits transmission for radio bearers configured for SDT; based on the indication, the UE initiates the MT-SDT only if the DL RSRP is above a configured threshold. SDT was specified for UE power saving purposes like LP-WUS and therefore it can be assumed that they should work together.

The following is capture in the TS 38.304 running CR:

5.2.4.x.3 Serving cell measurement offloading rules LP-WUS UE may choose to perform serving cell measurement offloading (i.e., serving cell measurement is fully offloaded to LR and no serving cell measurement via MR is required) according to requirements specified in TS 38.133 [8] if the entry condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled. LP-WUS UE is not required to perform serving cell measurement offloading according to requirements specified in TS 38.133 [8] if the exit condition for serving cell measurement offloading in clause 5.2.4.x.4 is fulfilled.

If the UE evaluates conditions for SDT initiation when LP-WUS UE is performing serving cell measurement offloading, then MR RSRP measurement is not available since measurements are performed by LR. It needs to be decided whether LP-WUS UE can use RSRP measurement for SDT condition evaluation in this case. Also, MR RSRP measurement can be considered for this case, but using it causes delay (2 times DRX cycle), since RSRP needs to be measured according to RAN4 requirements.

RAN4 to confirm SDT requirements apply also for UE supporting LP-WUS

The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading (i.e., no MR results available).

Issue 1-1-16: LP-WUR operation with EMR

In the last meeting, it was agreed that when T331 is running, UE shall not use LP-WUS if R16 EMR and LP-WUS are configured simultaneously.

There was also online discussion and concerns about EMR R18 behaviour. The following highlights the relevant section in 38.133 for EMR R18 feature:

4.7.3 Measurement Report Requirements A UE shall perform validity check and report valid measurement results: - if the UE supports measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UE supports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in the clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

The following figure illustrates the highlighted cases. If ValidityDuration is configured, it is performed retrospectively. If the validityDuration is not configured, UE may still report.

Figure 2: EMR R-18 operation, UE checks measurements retrospectively

If the UE has been woken up with LP-WUS, at the MSG-1 there may not be measurements. For this case RAN4 may define that if the is woken up by the LP-WUS signal, it may skip the validityCheck.

For Rel-18 eEMR if the UE has been woken up by LP-WUS from fully offloading scenario, UE may skip the validityCheck for the configured carriers (measIdleCarrierListNR or measReselectionCarrierListNR).

Conclusion

The following Observations and Proposals were made:

Observation 1: Network cannot configure exit threshold for RRM relaxation and with the margin, the exit may cause ping-pong.

Proposal 1: Define static exit threshold based on entry threshold. Discuss what should be the value, e.g. 4dB.

Proposal 2: The existing wakeup delay is re-used when UE exits RRM relaxation mode due to exit condition being met.

Proposal 3: RAN4 to confirm SDT requirements apply also for UE supporting LP-WUS

Proposal 4: The UE supporting LP-WUS uses LR-RSRP measurements for condition evaluation when the evaluation is started while the UE is in MR offloading (i.e., no MR results available).

Proposal 5: For Rel-18 eEMR if the UE has been woken up by LP-WUS from fully offloading scenario, UE may skip the validityCheck for the configured carriers (measIdleCarrierListNR or measReselectionCarrierListNR).