3GPP TSG-RAN WG4 Meeting #117	R4-2521816

Dallas, US, Nov. 17 – 21, 2025

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on general issues for 6G RAN4 specification improvement

Agenda item:	8.12.1

Document for:	Approval

- 1 Introduction

An SID on 6G Radio was approved and further revised in [1] to initiate the study on developing a 6G radio access technology. RAN4 has started 6G topic discussion since RAN4#116bis meeting. A dedicated thread [116bis][111] was then set for discussion on 6GR RAN4 specification improvement [2].

In this contribution, we give our considerations on general issues for 6G RAN4 specification improvement.

- 2	Discussion

Issue 1-1-1: General and common Issues for all RAN4 specifications

- Observed issues

  - Issue 1: Requirements for one feature

    - One single feature may have requirements at multiple clauses and across multiple specs.

    - For the same feature, either new clause for one requirement or re-use legacy clause for another requirement.

    - Different approaches are adopted for different features in the structure.

  - Issue 2: Redundancy

    - Many duplications and repetition of requirements for different scenarios and use cases.

    - Duplicated requirements scattered throughout different sections.

  - Issue 3: Consistency

    - Terminologies are not aligned or consistent

  - Issue 4: Structuring

    - Combine core and performance requirements into the same spec.

    - Some of the 3GPP specs are too large and become very difficult even to open.

In last meeting, the following issue on UE RF specifications was observed during the previous 5G RAN4 standardization activities and may provide valuable insights for ongoing discussions [3].

2.1	“Feature-sub-file” spec structure

In current RAN4 UE RF spec, the clauses are organized by suffix which indicates a specific feature. The suffix are shown in Table 2.1-1 and for most cases indicating the feature at 2nd level clause. Some cases have the feature at 3rd level clause. For example, for shared spectrum channel access, suffixes A, B, and D are used for indicating at 3rd level clause, while for V2X, suffixes A and F are used. A terminal which supports a specific feature needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to L) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to L) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause. Note that suffixes like H (CA+UL MIMO) and L (CA+TxD) address multi-feature interactions. For example, a terminal which supports UL CA with UL MIMO shall meet the corresponding requirements in suffix H with all UL CCs with UL MIMO. Figure 2.1-1 illustrates the recursive “feature-suffix” spec structure in 5G NR specs. It can be foreseen that if the “feature-suffix” approach continue to be applied to 6GR specifications, more new features will be added for emerging features just like I for RedCap in Rel-17 and K for UAVs in Rel-18 during 5G NR. Too many recursive multi-level “feature-suffix” sub-clauses will make 6GR specifications more complicated which should be avoided at the beginning of 6GR.

Table 2.1-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplementary Uplink (SUL) |
| D | UL MIMO |
| E | V2X |
| F | Shared spectrum channel access |
| G | Tx Diversity (TxD) |
| H | Carrier Aggregation (CA) with UL MIMO |
| I | (e)RedCap |
| J | ATG |
| K | Aerial UE (UAV) |
| L | Carrier Aggregation (CA) with Tx Diversity |

Figure 2.1-1: The recursive “feature-suffix” spec structure in NR

Observation 1: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. There are also some second level features further recursively categorized into 3rd level features which causes the spec poor readability. Furthermore, some suffixes, e.g. H (CA+UL MIMO), which address multi-feature interactions make the spec more complicated.

Proposal 1: In 6GR RAN4 spec, the recursive multi-level feature sub-clauses should be avoided.

To optimize the spec structure in 6GR, we suggest to re-organize the spec files by features. We call the new structure as “Feature-sub-file” approach. For example, for TS xx.101-1, the zip file may include a serial of sub-files which indicates the features respectively. In each zip file, a common sub-file for all features will be included for the basic aspects. When we introduce a new feature, a sub-file for the new feature will be included in the zip file. A terminal which supports the new feature needs to meet both the requirements specified in the common sub-file and the additional requirements specified in the new feature specific sub-file. If there is a difference in the requirements between the common sub-file and the feature specific sub-file, the requirements specified in the feature specific sub-file will be applied. By this way, when we check the requirements for a new feature in the spec, we only need to open at most two sub-files, i.e., the common sub-file and the feature specific sub-file, and no need to open all the features as did in 5G spec. In addition, the “feature-suffix” structure in the feature specific sub-file will be compressed to at most two levels. And in most cases, there is no need “feature-suffix” any more in the feature specific sub-file, since the sub-file already indicates which feature it specifies.

Proposal 2:  For the structure of 6GR RAN4 specification, it can be optimized with the guidelines as below.

- Re-organize the specification zip file by the features, each of the constituent sub-file specifies a certain feature, such as CA, DC, Redcap etc.

- A common sub-file for the basic aspects will be specified for all features.

- In each sub-file for a certain feature, the clauses could be further specified with a second level sub-clause to reflect the requirements of the feature if needed.

2.2	Redundancy & Consistency

2.2.1	About notes usage and naming

In previous RAN4 meetings, when discussing about the RAN plenary task on RAN4 spec improvement, the issues of redundancy & consistency have also been touched. Some companies complain that the notes in current RAN4 spec tables are too casual and not rigorous for the specifications, which severely reduce the readability of the specification. After long discussion, it captured some guidelines regarding to table notes in PRD#01 as below.

The following are the rules for drafting table notes:

-	For the existing notes in table in current spec, the notes should not be moved outside the table due to the issues of large number of “void” notes and external references to the notes outside of RAN4.

-	For future notes in table,

1)	Do not use NOTEs in tables for requirements that apply every cell/line or general requirements in the table. Use text above the table instead.

2)	If similar notes are to be introduced into a table, a more generic note description should be considered.

3) If a note is intended for terminology, avoid having the note in the table if the terminology is defined in the clauses of symbols and abbreviations in the specification.

We suggest such agreements should also be applied in 6GR phase.

Proposal 3: In 6GR RAN4 spec, it is suggested to apply the following guidelines to table note drafting.

- Do not use NOTEs in tables for requirements that apply every cell/line or general requirements in the table. Use text above the table instead.

- If similar notes are to be introduced into a table, a more generic note description should be considered.

- If a note is intended for terminology, avoid having the note in the table if the terminology is defined in the clauses of symbols and abbreviations in the specification.

Regarding to the notes in current NR spec, we also notice that in many cases the notes were removed and no longer exist. For these void notes, they are specified row by row which looks quite redundant. In future 6GR specification, we suggest if multiple void appears, is it is possible to merge all the void notes to a single row?

(a) The void notes in Table 6.5B.3.3.2-1 in 38.101-3       (b) The reserved sub-clauses in chapter 6 in 38.101-1

Figure 2.2.1-1: Illustration on void notes & reserved sections

Proposal 4: In 6GR RAN4 spec, it is suggested to merge multiple consecutive void notes and reserved sub-clauses into one row if there are such cases, e.g. “NOTE x ~ y: Void”.

Furthermore, we also notice that in current RAN4 spec, the naming conventions for some table titles or sub-clause titles are too general, which should be more specific from the perspective of readability. In 6GR stage, with the goal to be a high quality specification, such cases should be avoided.

(a) Missing specific NS label name in 38.101-1          (b) Missing type for spurious emission in 38.101-3

Figure 2.2.1-2: Illustration on naming conventions for table titles

Proposal 5: In 6GR RAN4 spec, it is suggested to normalize the naming convention of the table / sub-clause titles, too general or confusing name should not be used.

2.2.2	About abbreviations / symbols usage

It is noted that in current RAN4 spec, many symbols or abbreviations have been defined. However, we notice that not all the abbreviations / symbols defined in Section 3 are used reasonably. For example, some abbreviations / symbols only occurred in Section 3 and no appearance in the spec body. Some terminologies used many times in the spec body but missing in Section 3. The worse scenario is that some abbreviations / symbols have different meaning between Section 3 and the spec body. We suggest all such cases should be avoided in 6GR stage. If a symbol / abbreviation is defined in Clause 3, it should be used in the spec body part. Let’s take TS 38.101-1 as an example, we notice that “Measurement bandwidth” is abbreviated as “MBW” in Section 3, however in the spec body more than 205 times it still uses “Measurement bandwidth” instead of “MBW”, although in the spec body sometimes it duplicates the abbreviation definition as “measurement bandwidth (MBW)”. If each time in the body we still use “measurement bandwidth (MBW)”, what is the meaning to define it in Section 3. One more case is that we have the abbreviation “BW” for “bandwidth” in Section 3, however the “bandwidth” appears in the spec body for more than 4800 times!

Observation 2: In 5G RAN4 spec, the usage of abbreviations and symbols are quite casual and not rigorous. In some cases it appears only in Section 3 and no appearance in the spec body, while some other cases are just the opposite. What is worse is that sometimes the abbreviation / symbol has different meaning in Section 3 and spec body.

Proposal 6: In 6GR RAN4 spec, the following usage of symbols and abbreviations should be followed.

- Do not use the abbreviations and symbols only in the definition part (Section 3).

- Do not use the abbreviations and symbols only in the spec body part.

- The meaning of the abbreviations and symbols should be consistent in the whole specification.

- There is no need to repeat the abbreviation and symbol definition in the spec body part whenever it is used.

2.2.3	About RAN2 language usage

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18,....

Additionally, in the existing 5G NR specification, we found the descriptions for some RF requirements are quite complicated and difficult to understand. For example, for the (e)RedCap UE REFSEN requirements as following. The IE names were repeated again and again although RAN2 has a clear definitions for (e)RedCap, RAN4 still use a complicated texts in the specification.

There are some other requirements have the similar situation in the specification. Usually these descriptions use too much RAN2 IE signallings/language but sometimes it may duplicated even discrepancy between RAN2 and RAN4 understanding. We think we should try to reduce the usage of RAN2 language as much as possible in RAN4 specification.

Proposal 7: In 6GR, it is proposed to reduce the usage of RAN2 language in RAN4 specification as much as possible.

2.2.4	About missing (sub-)clause suffix

Currently in NR UE RF spec, the requirements are defined for each feature in different clause suffixes. For TS 38.101-1, using single carrier requirements as the benchmark, we observe that different approaches are used for different features:

-  Several (sub-)clause suffixes are missing for a certain feature, for example for (e)RedCap, there are only three clause suffixes defined.

-  For some other features such as NR CA/DC, each requirement has its corresponding (sub-)clause suffix.

Although some of the (sub-)clause suffixed are marked as ‘Reserved’, we are not fully understanding what is the exact meaning. We could not find the reason why the descriptions are missing in the specification. Then it would cause confusion for the reader who didn’t follow the feature discussion as there are at least two following interpretations:

a)  To reuse the existing requirements as single carrier feature.

b)  The requirements without clause suffix do not apply to that feature.

The above two interpretations are completely opposite in the understanding. We believe the interpretation a) is the main reason for not explicitly define the requirements. However, there are no such descriptions in the specification. When we look at NR DC feature, all of requirements are explicitly define although almost all of the RF requirements are reused from the NR CA requirements. But we also believe there are exceptions that a certain requirement is not applied, for example, MPR requirements are not applied to ATG, so there is no MPR sub-clause defined.

Observation 3: In 5G UE RF specification, different approaches are adopted for different features in the structure. Meanwhile, there are too many (sub-)clause suffixes not defined for RF requirements.

Observation 4: There exist different interpretations for the missing (sub-)clause suffixes requirements which causes misunderstanding.

For 6GR, we think the above issue for the specification improvement should be taken into account. When introducing a new feature into the specification, some general guidance/rules for uniform drafting are required to define the RF requirements.

Proposal 8: It is proposed in 6GR to discuss uniform drafting guidance/rules for the RF requirements definition when introduction of a new feature in the specification.

- 3	Conclusion

In this contribution, we give our views on general issues for 6G RAN4 specification improvement. Issues on “Feature-sub-file” spec structure, redundancy & consistency on notes usage, table naming and abbreviation / symbol usage have been discussed. The conclusions are summarized as below:

Observation 1: In 5G UE RF spec, the structure is organized by the requirements with different features packing into the second level sub-clauses. There are also some second level features further recursively categorized into 3rd level features which causes the spec poor readability. Furthermore, some suffixes, e.g. H (CA+UL MIMO), which address multi-feature interactions make the spec more complicated.

Proposal 1: In 6GR RAN4 spec, the recursive multi-level feature sub-clauses should be avoided.

Proposal 2:  For the structure of 6GR RAN4 specification, it can be optimized with the guidelines as below.

- Re-organize the specification zip file by the features, each of the constituent sub-file specifies a certain feature, such as CA, DC, Redcap etc.

- A common sub-file for the basic aspects will be specified for all features.

- In each sub-file for a certain feature, the clauses could be further specified with a second level sub-clause to reflect the requirements of the feature if needed.

Proposal 3: In 6GR RAN4 spec, it is suggested to apply the following guidelines to table note drafting.

- Do not use NOTEs in tables for requirements that apply every cell/line or general requirements in the table. Use text above the table instead.

- If similar notes are to be introduced into a table, a more generic note description should be considered.

- If a note is intended for terminology, avoid having the note in the table if the terminology is defined in the clauses of symbols and abbreviations in the specification.

Proposal 4: In 6GR RAN4 spec, it is suggested to merge multiple consecutive void notes and reserved sub-clauses into one row if there are such cases, e.g. “NOTE x ~ y: Void”.

Proposal 5: In 6GR RAN4 spec, it is suggested to normalize the naming convention of the table / sub-clause titles, too general or confusing name should not be used.

Observation 2: In 5G RAN4 spec, the usage of abbreviations and symbols are quite casual and not rigorous. In some cases it appears only in Section 3 and no appearance in the spec body, while some other cases are just the opposite. What is worse is that sometimes the abbreviation / symbol has different meaning in Section 3 and spec body.

Proposal 6: In 6GR RAN4 spec, the following usage of symbols and abbreviations should be followed.

- Do not use the abbreviations and symbols only in the definition part (Section 3).

- Do not use the abbreviations and symbols only in the spec body part.

- The meaning of the abbreviations and symbols should be consistent in the whole specification.

- There is no need to repeat the abbreviation and symbol definition in the spec body part whenever it is used.

Proposal 7: In 6GR, it is proposed to reduce the usage of RAN2 language in RAN4 specification as much as possible.

Observation 3: In 5G UE RF specification, different approaches are adopted for different features in the structure. Meanwhile, there are too many (sub-)clause suffixes not defined for RF requirements.

Observation 4: There exist different interpretations for the missing (sub-)clause suffixes requirements which causes misunderstanding.

Proposal 8: It is proposed in 6GR to discuss uniform drafting guidance/rules for the RF requirements definition when introduction of a new feature in the specification.

- 4	Reference

[1]	RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2]  R4-2514518, Topic summary for [116bis][111] 6G operation efficiency, Moderator

[3] 	R4-2514648, Way forward for [116bis][111] 6G operation efficiency, CATT