3GPP TSG RAN WG4 Meeting #117	R4-2521821

Dallas, USA 17th – 21st Nov. 2025

Agenda Item:	7.4.3.2

Source: 	Ericsson

Title:	Discussion on Rel-20 RRM Enhancements

Document for:	Discussion

# Introduction

In RAN#109, NR RRM Phase 6 got approved [1]. According to the Objectives, RAN4 should extend support of less than 5 MHz CBW for RedCap. Also, relaxed RLM/BFD requirements should be extended to (e)RedCap UEs. RAN4#116-bis meeting was the first meeting where we examined the scope of the WI and the impact of supporting those features with two RX [2]. This contribution discusses the 1 RX RedCap UE RRM requirements that may be impacted by the introduction of less than 5 MHz CBW and relaxed RLM/BFD.

# RedCap Less than 5 MHz

In this section, we analyse the impact of less than 5 MHz on RedCap RRM requirements.

## RedCap UE with HD-FDD

For Redcap UE with HD-FDD, the handover requirements are specified as follows:

Hence, less than 5 MHz Support does not impact baseline handover requirements of HD-FDD. It is the same for RLM, BFD and Intra/Inter frequency measurements.

For 2Rx RedCap for HD-FDD, RAN4 can reuse the following requirements from NR less than 5 MHz in Rel-18 as baseline.

- Handover

- RLM/ BFD

- Intra-frequency/Inter-frequency measurement

## Coverage Equivalence between 1Rx and 2 Rx RedCap UE

During the online discussion at the last RAN4 meeting, it was noted that companies have different interpretations of the term ‘coverage equivalence’. From our perspective, coverage equivalence refers to achieving comparable reliability performance (e.g., 90% PDCCH decoding success probability) at the same cell-edge locations for both 1Rx and 2Rx RedCap UEs. As the WID objective limits the PDCCH control bandwidth (CBW) to 15 PRBs and considering CORESET#0 configurations with 2 or 3 symbols, the supported aggregation levels (AL) should be up to 8 for 15 PRBs (with puncturing) to maintain consistency with the intended control-channel design scope. Hence, it is agreed to check whether it is feasible to target coverage equivalence between 1 RX and 2 Rx RedCap UEs.

After reviewing the coverage analysis for Rel-18 NR less than 5 MHz [3]and Rel-17 RedCap/eRedCap [4], it was observed that the UL is generally the limiting link at the cell edge for less than 5 MHz, rather than the DL. For 2 Rx UEs, an SNR gap of more than 8 dB is observed between the UL and DL. In that context, 1Rx RedCap UE introduces ~3 dB of DL diversity loss compared to 2 RX UE. Still, a margin of more than ~5 dB to the UL bottleneck typically remains.

In NR, UL and DL have a >8dB SNR difference for 2 Rx UE.

Considering a ~3 dB gain loss due to 1Rx, UL and DL still have about a >5 dB margin for 1 Rx UE.

Even if we consider a 1.5 dB implementation loss, the DL link budget remains comfortably above the UL coverage limit, even with a 1 RX RedCap UE. The same principle applies to 1 RX RedCap UE with a bandwidth of less than 5 MHz. Hence, it is feasible to target coverage equivalence between 1 RX and 2 RX Redcap UEs.

There is no coverage issue for 1Rx RedCap UE with less than 5 MHz since the coverage is limited to Uplink.

## Hypothetical PDCCH parameters for 1Rx RedCap UE

Hypothetical PDCCH parameters for out-of-sync and in-sync evaluation for 1Rx Redcap UE can reuse the Rel-18 parameters shown in tables 1 and 2, respectively. Additionally, the beam failure instance parameters are listed in table 3 for 1 RX RedCap UE. Based on the WID, DL BW 15 PRB should be specified only. RAN4 should discuss and agree on the following parameters.

Table 1: PDCCH transmission parameters for out-of-sync evaluation for a 1 Rx RedCap UE operating on a cell with less than 5 MHz BW

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3 MHz |
| [DL Transmission BW] | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4 dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote 1 |
| NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication. |  |

Table 2: PDCCH transmission parameters for in-sync evaluation for a 1 Rx RedCap UE operating on a

cell with less than 5 MHz BW

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3 MHz |
| [DL Transmission BW] | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-Distributed |

Table 3: PDCCH transmission parameters for beam failure instance for a 1 Rx RedCap UE operating on a cell with less than 5 MHz BW

| Attribute | Value for BLER |
| --- | --- |
| Channel BW | 3 MHz |
| [DL Transmission BW] | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 Note 1 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0 dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0 dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote2 |
| NOTE 1: PDCCH puncturing as defined in TS 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |

RAN4 should agree on the hypothetical PDCCH parameters of table 1, table 2, and table 3 for out-of-sync, in-sync, and beam failure instance evaluation for 1Rx Redcap UE with less than 5 MHz BW.

# RedCap Relaxed RLM/BFD

In this section, we discuss RLM/BFD relaxation support for RedCap.

## Scenarios and Criteria for Relaxed RLM/BFD

In Rel-17/Rel-18 RedCap 1 Rx UE is supported only for FR1. Hence, for RedCap RLM/BFD Relaxation, we think the following cases should be supported and specified in Rel-20.

    - Case 1: SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

    - Case 2: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

    - Case 3: SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

    - Case 4: CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

    - Case 5: SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

    - Case 6: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

RAN4 should specify following cases for RedCap with RLM/BFD Relaxation in Rel-20.

- Case 1: SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

- Case 2: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

- Case 3: SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

- Case 4: CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

- Case 5: SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

- Case 6: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

Furthermore, in Rel-18 less than 5 MHz is only supported for FR1 and CSI-RS based measurement is out of scope. Hence, RLM/BFD relaxation for RedCap with less than 5 MHz should only support case 1 and case 3.

RLM/BFD relaxation for RedCap with less than 5 MHz should only support case 1 and case3.

## RLM/BFD Relaxation- Entry Criteria and exit Criteria.

In Rel 17, the RLM relaxation criteria for NR are defined in Clause 8.1.1.1 in TS 38.133, and the BFD relaxation criteria are described in Clause 8.5.1.1 in TS 38.133 for serving cells as follows.

for other serving cells, when - the good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is fulfilled for the SpCell if the lowMobilityEvaluationConnected is not configured, or - the UE is also configured with lowMobilityEvaluationConnected and both low mobility criterion defined in clause 5.7.13.1 of TS 38.331 [2] is fulfilled for a period of TSearchDeltaP-Connected and good serving cell quality criterion defined in clause 5.7.13.2 of TS 38.331 [2] is fulfilled for the SpCell.

In Rel-17, two measurement criteria were specified for RRC_CONNECTED state:

- Low Mobility

- Good Serving cell quality.

In the WID, it is stated that relaxation should be specified for RedCap devices, such as wearables. Hence, we consider RedCap low-mobility devices. Hence, for RLM/ BFD relaxation Low Mobility and Good Serving cell quality specified in Rel-17 should be reused as baseline.

RLM/ BFD relaxation Low Mobility and Good Serving cell quality specified in Rel-17 should be reused as baseline for RedCap RLM/ BFD relaxation for both 1RX and 2 RX UE.

Similarly, exit criteria specified in clause 8.1.1.1 for RLM and clause 8.5.1.1 for BFD can be reused in Rel-17.

Exit criteria of RLM/ BFD relaxation, specified in Rel-17, can be reused for RedCap RLM/ BFD relaxation for both 1RX and 2 RX UE.

## RLM/BFD Relaxation- Evaluation Period.

In Rel 17, the relaxation parameter K was introduced to scale the RLM/BFD evaluation windows in stable conditions, thereby achieving energy efficiency while maintaining minimum safety bounds to preserve mobility reliability. As 1 Rx RedCap UE, TEvaluate_out_SSB_Relax,RedCap is already double compared to 2 Rx RedCap UE to maintain the safety bounds for mobility.

TEvaluate_out_SSB_Relax,RedCap should not exceed TEvaluate_out_SSB_Relax defined in Rel-17 RLM/BFD for 1 Rx RedCap UE.

To meet TEvaluate_out_SSB_Relax for 1 Rx RedCap UE, Parameter K1 should be set to 2 for Max(TDRX,TSSB) ≤40ms and K1 = 1 for 40ms<Max(TDRX,TSSB) ≤80ms as stated in table 4. K factor of CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1 needs further study.

Table 4: Evaluation period TEvaluate_out_SSB_Relax, RedCap for FR1for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB_Relax, RedCap (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80ms | Max(400´ K3 NOTE3, Ceil(30 ´ K1NOTE2 ´ P) ´ Max(TDRX,TSSB)) |
| 80ms < Max(TDRX,TSSB) ≤160ms | Ceil(30 ´ P) ´ Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80ms. NOTE 2: K1 = 2 for Max(TDRX,TSSB) ≤40ms and K1 = 1 for 40ms<Max(TDRX,TSSB) ≤80ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

RAN4 to agree on the table for TEvaluate_out_SSB_Relax, RedCap with 1Rx RedCap UE in FR1.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for RedCap Less than 5 MHz and made the following observations and proposals.

Observation 1	In NR, UL and DL have a >8dB SNR difference for 2 Rx UE.

Observation 2	Considering a ~3 dB gain loss due to 1Rx, UL and DL still have about a >5 dB margin for 1 Rx UE.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	For 2Rx RedCap for HD-FDD, RAN4 can reuse the following requirements from NR less than 5 MHz in Rel-18 as baseline.

 Handover

 RLM/ BFD

 Intra-frequency/Inter-frequency measurement

Proposal 2	There is no coverage issue for 1Rx RedCap UE with less than 5 MHz since the coverage is limited to Uplink.

Proposal 3	RAN4 should agree on the hypothetical PDCCH parameters of table 1, table 2, and table 3 for out-of-sync, in-sync, and beam failure instance evaluation for 1Rx Redcap UE with less than 5 MHz BW.

Proposal 4	RAN4 should specify following cases for RedCap with RLM/BFD Relaxation in Rel-20.

Case 1: SSB based RLM/BFD measurement relaxation for 2Rx RedCap UE in FR1

Case 2: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR1

Case 3: SSB based RLM/BFD measurement relaxation for 1 Rx RedCap UE in FR1

Case 4: CSI-RS based RLM/BFD measurement relaxation 1 Rx RedCap UE in FR1

Case 5: SSB based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

Case 6: CSI-RS based RLM/BFD measurement relaxation 2Rx RedCap UE in FR2

Proposal 5	RLM/BFD relaxation for RedCap with less than 5 MHz should only support case 1 and case3.

Proposal 6	RLM/ BFD relaxation Low Mobility and Good Serving cell quality specified in Rel-17 should be reused as baseline for RedCap RLM/ BFD relaxation for both 1RX and 2 RX UE.

Proposal 7	Exit criteria of RLM/ BFD relaxation, specified in Rel-17, can be reused for RedCap RLM/ BFD relaxation for both 1RX and 2 RX UE.

Proposal 8	TEvaluate_out_SSB_Relax,RedCap should not exceed TEvaluate_out_SSB_Relax defined in Rel-17 RLM/BFD for 1 Rx RedCap UE.

Proposal 9	RAN4 to agree on the table below for TEvaluate_out_SSB_Relax, RedCap with 1Rx RedCap UE in FR1.

Table: Evaluation period TEvaluate_out_SSB_Relax, RedCap for FR1for 1Rx RedCap UE

| Configuration | TEvaluate_out_SSB_Relax, RedCap (ms) |
| --- | --- |
| Max(TDRX,TSSB) ≤80ms | Max(400´ K3 NOTE3, Ceil(30 ´ K1NOTE2 ´ P) ´ Max(TDRX,TSSB)) |
| 80ms < Max(TDRX,TSSB) ≤160ms | Ceil(30 ´ P) ´ Max(TDRX,TSSB) |
| NOTE 1: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length and no longer than 80ms. NOTE 2: K1 = 2 for Max(TDRX,TSSB) ≤40ms and K1 = 1 for 40ms<Max(TDRX,TSSB) ≤80ms. NOTE 3: K3 = K1, if K1 ≤ 2; otherwise K3 = 1. |  |

# References

- RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

- R4-2514821, WF on NR_RRM_Ph6, Ericsson

- R1-2301154, NR support of spectrum less than 5MHz for FR1, Ericsson

- TR 38.875, Rel-17 Study on support of reduced capability NR devices

- R4-2410395, BigCR for NR_FR1_lessthan_5MHz_BW, Nokia

- R4-2514865, Simulation assumptions for(e)RedCap UE enhancement Study, Ericsson