# 3GPP TSG RAN WG4 Meeting #117 														R4-2521869

# Dallas, USA, November 12 -17, 2025

Agenda Item:	6.17.2.3

.3Source: 	Ericsson

Title:	Discussion on Rel-19 conditional LTM performance requirement

Document for:	Discussion

# Introduction

In this contribution, we will address several aspects regarding the test case design for conditional LTM performance requirement from the way forward [1].

# Discussion

## Intra-CU conditional LTM

Issue 1-7: early DL sync < Way forward > - Option 1: With early DL sync. - Option 2: With and without early DL sync. - Option 3: For each FR, one test case with and without early DL sync, with early DL sync only for other test cases.

One open issue from last meeting is whether the CLTM without early DL sync shall be tested. Early DL sync is part of the new function introduced from LTM, we think for CLTM is also needs to be verified. For the CLTM candidate cell that being triggered execution without DL sync partially can behave as CHO if it is triggered by L3 measurement, however, due to the fact that CLTM also support L1 measurement triggered execution, the UE can behave different as CHO.

- Proposal 1: RAN4 test scope shall cover with and without early DL sync for CLTM

#### Issue 2-1: Test case list for Subsequent CLTM < Way forward > - Option 1: Define test cases for subsequent CLTM. - For FR1: - If UE support 2 or more test cases for FR1, UE to pass one test case for subsequent CLTM which is different from CLTM. - If UE support 1 test case for FR1, UE is only required to pass the test case for CLTM, and not required to pass the test case for subsequent CLTM. - For FR2: the same proposal as FR1. - Option 2: Not define test case for subsequent CLTM as CLTM.

The relationship between subsequent CLTM and CLTM is that UE have execute on one CLTM candidate and then UE execute on the next CLTM candidate. For test verification, the subsequent CLTM will naturally include a CLTM procedure. We are generally open to reduce the repeated test cases, as long as the FR group, intra-inter frequency , RACH or RACH-less, L1/L3 trigger have been verified in either of CLTM or subsequent CLTM.

For example for UE support 2 or more test case for FR1, the UE need to pass different subsequent test case as CLTM test case. For UE only support 1 test case, then UE need to pass only the CLTM test case.

- Proposal 2: CLMT and subsequent CLTM can be combined together to have full coverage of the test cases.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: RAN4 test scope shall cover with and without early DL sync for CLTM

	Proposal 2: CLMT and subsequent CLTM can be combined together to have full coverage of the test cases.

# References

- R4-2514816, WF on RRM requirements for NR_Mob_Ph4_Part2 , China Telecom, October, 2025.