3GPP TSG-RAN WG4 Meeting #117	R4-2521881

Dallas, USA, 17th – 21st November 2025

Source:	ISSDU, NYCU

Title:	Simulation Study on Adjacent-Channel Coexistence between SBFD Systems for Commercial Deployment

Agenda item:	7.2.4 - SBFD BS to SBFD BS adjacent channel coexistence

Document for:	Discussion

Introduction

Sub-band Full Duplex (SBFD) has emerged as a promising duplex evolution concept in 5G-Advanced and is expected to become one of the formal duplex modes in beyond-5G (Release-20) systems. While 3GPP RAN4 has established coexistence methodologies for SBFD–TDD adjacent channel evaluations under TR 38.858 Annex E[1], the SBFD–SBFD (base station to base station) coexistence scenario has not yet been formally defined or studied in prior releases.

In practice, future Release-20 networks may deploy multiple operators, multiple cells, and heterogeneous duplex configurations in adjacent sub-bands. To anticipate such scenarios, we extend the RAN4 baseline coexistence methodology to evaluate SBFD–SBFD adjacent channel coexistence using the same framework defined in TR 38.858[1] and TR 38.803[2].

This contribution provides concrete observations and recommendations to refine the interference modeling in SBFD environments, ensuring alignment with practical deployments and beyond-5G system assumptions, as well as to inform the specification work (TR38.8xx v0.0.1 skeleton) on SBFD adjacent-channel coexistence.

Discussion

Following the RAN4 procedure in TR 38.858 Annex E under the Coexistence Simulation Methodology (Urban Macro), the simulation framework consists of four main steps:

- Network Generation

- Victim and aggressor networks are built independently.

- UE positions are randomly distributed under the Urban Macro scenario defined in TR 38.803.

- Association between UE and BS is based on coupling loss.

- Select adjacent channel coexistence cases. (see Table 1)

- Scenario Configuration

- Carrier frequency: 4 GHz (FR1)

- ISD: 500 m, 19 sites × 3 sectors = 57 cells

- UE per cell: 1 (UE–BS distance ≥ 35 m, UE–UE ≥ 2 m)

- Antenna array: BS UPA = [1, 1, 8, 8, 2], UE UPA = [1, 1, 1, 1, 1]

- BS ACLR = 45 dBc, BS ACS = 50 dBc

- Co-site interference: 6 dB below noise floor

- Grid shift: 5 % – 100 % (to emulate carrier offset between operators)

- Beamforming & Scheduling

- Round-robin scheduling with ideal beamforming toward the LOS direction.

- One UE per sector ensures deterministic SINR evaluation.

- Throughput Evaluation

- Troughput= αlog21+SINR , with saturation and cutoff thresholds SINRmin,SINRmax

- Interference components include:

  - Intra-SB inter-UE interference

  - Inter-SB inter-BS interference

  - Co-site inter-sector interference

  - Self-interference (noise floor – 6 dB in RAN4 baseline)

  - Adjacent-Channel interference between SBFD Base Stations

| Case | Aggressor | Victim | Slot allocation |
| --- | --- | --- | --- |
| 1 | SBFD {U, D} | SBFD {D, U} |  |
| 2 | SBFD {U, D} | SBFD {D, U, D} |  |
| 3 | SBFD {D, U, D} | SBFD {D, U} |  |
| 4 | SBFD {D, U, D} | SBFD {D, U, D} |  |

Table 1 SBFD-SBFD adjacent channel coexistence cases

According to TR 38.858 Annex E, in the SBFD-TDD adjacent-channel coexistence scenario, the base-station receiver blocking model defines the total receiver input power as

Pin= Pw+Iintra_SB,inter_UE+Iinter_SB,inter_BS+IACI+Iself+Ico_site,inter_sector

where each interference component represents intra- and inter-subband interference, adjacent-channel interference, self-interference, and co-site inter-sector interference, respectively.

- Pw is the wanted signal received from the scheduled UE. The wanted signal level is calculated as:

Pw=PTX,  UECLscheduled UE→serving BS

- Iintra_SB,inter_UE is the interference from UEs transmitting within the UL sub-band in the victim network. The interference level is calculated as:

Iintra_SB,inter_UE=PTX,  UECLother UE→serving BS

- Iinter_SB,inter_BS is the interference from BS transmitting on the adjacent DL sub-band in the victim network. The interference level is calculated as:

Iinter_SB,inter_BS=PTX,  BSCLown network BS→serving BS

- IACI is the adjacent channel interference from BS transmitting on the DL channel in the aggressor network. The interference level is calculated as:

IACI=PTX,  BSCLadjacent network BS→serving BS

- Iself is the victim self-interference power. The interference level is calculated as: (Mself=6dB)

Iself=kTBSBFMself

- Ico_site,inter_sector is the interference from neighboring sectors in a 3-sector configuration. The interference level is calculated as: (Mco_site=6dB)

Ico_site,inter_sector=kTBSBFMco_site

where additional variables are defined as:

- PTX,  UE  is the configured UE transmitter power.

- PTX,  BS is the BS transmitter power.

- BSB is the allocated UL subband bandwidth

- Mself is the noise-floor-to-self interference ratio.

- Mco_site is the noise-floor-to-aggregated co-site inter sector interference ratio

- kT is the thermal noise

- F is the receiver noise figure

The total interference power can be expressed in linear scale as:

Itot=Iintra_SB,inter_UE+Iinter_SB,inter_BSASBIRinterSB,inter_BS  +IACIACIR+Iself+Ico_site,inter_sector

- ASBIRinterSB,inter_BS is the in-channel Adjacent Sub-Band Interference Ratio, which is derived by applicable BS ACLR and BS ACS.

- ACIR is derived by applicable BS ACLR and BS ACS.

The SINR at the victim receiver including total noise and total interference can be expressed in linear scale as:

SINR=PwItot+kTBSBFblock

Self-Interference Modeling

The current self-interference model in RAN4 assumes a constant interference level—6 dB below the thermal noise floor—for all SBFD configurations (DU, UD, DUD). While this simplifies simulation, it overlooks key differences in duplex spacing, filter characteristics, and hardware isolation performance. As a result, the model cannot reflect the unique RF isolation behaviors across configurations, and ASBIR (Adjacent Sub-Band Isolation Ratio) remains undefined.

To improve modeling fidelity and alignment with real-world performance, RAN4 is invited to formally define ASBIR as the ratio between the desired in-band received power and the coupled self-interference from the adjacent sub-band. ASBIR is defined as:

ASBIR=11sub_band ACLRBS,inteference band+1sub_band ACSBS,victim band

where sub_band ACLRBS,inteference band represents the transmitter leakage from the BS downlink sub-band (interfering band), and sub_band ACSBS,victim band the receiver selectivity of the BS uplink sub-band (victim band). It is further proposed that minimum ASBIR values be specified for each SBFD configuration (DU, DUD, UD) to ensure accurate simulation inputs and better alignment with testable RF design criteria.

Proposal 1:	RAN4 to consider adding a configuration-dependent self-interference model (based on SBFD duplex arrangements defined in 5G-Advanced, i.e. DU, UD, DUD) to support more flexible SBFD deployments beyond 5G.

Uplink Interference Components

Current SBFD–TDD coexistence simulations primarily evaluate downlink interference and intra-band uplink interference. However, in SBFD–SBFD coexistence, uplink-to-uplink interference becomes an important factor, particularly under multi-operator and dense-UE scenarios. Although each UE transmits at lower power than the base station, their large number and dense distribution make UL interference a critical issue.

Two key uplink interference sources are not considered in current coexistence evaluations: (1) inter-band uplink interference between UEs of different operators transmitting on adjacent SBFD uplinks, and (2) co-site inter-sector uplink interference between sectors of the same site operating under different SBFD configurations. In addition, current modeling does not reflect sub-band isolation between adjacent uplink sub-bands, which can significantly impact the level of UL–UL interference.

To improve accuracy, RAN4 is encouraged to explicitly include both uplink interference terms in the SINR computation for SBFD–SBFD coexistence simulations. Each UL–UL interference component should be weighted by the corresponding ASBIR value, reflecting the combined transmitter leakage (UE sub-band ACLR) and receiver selectivity (BS sub-band ACS) between adjacent uplink sub-bands:

ASBIR=11sub_band ACLRUE,inteference band+1sub_band ACSBS,victim band

This improvement will enhance the reliability of SINR evaluation and yield results that better represent practical deployment conditions, especially in high-UE-density networks.

Proposal 2:	Include multi-operator SBFD coexistence as a key Release-20 uplink study scenario, specifically where adjacent uplink sub-bands are operated by different operators.

DUD Configuration Interference Evaluation

DU and DUD are two common SBFD configurations. RAN4 currently assumes they exhibit similar interference characteristics; however, DUD’s duplex gap is only half that of DU, leading to stronger sub-band coupling and stricter filtering requirements. Maintaining identical assumptions across the two configurations underestimates the real interference in DUD systems.

Current coexistence simulations do not reflect the distinct interference characteristics of DUD configurations. In particular, the narrower duplex spacing in DUD is not adequately captured, leading to underestimated sub-band isolation impacts and potential system performance degradation.

Based on NR RF baselines, the DUD configuration should be evaluated independently, with a minimum ASBIR requirement defined according to its duplex spacing. This approach ensures that coexistence simulations accurately capture the interference characteristics of DUD and avoid underestimating its impact relative to other SBFD configurations.

Proposal 3:	RAN4 to evaluate Release-20 SBFD coexistence using existing NR RF baselines but with separate assessments for each SBFD configuration.

Heterogeneous Sector Configuration

Most RAN4 coexistence simulations assume a single, uniform SBFD configuration across the entire network or site. However, in practical beyond-5G deployments, operators are expected to adopt heterogeneous SBFD configurations at the sector level to adapt to varying traffic demands and interference environments. For instance, one sector may use DU while another uses DUD on the same site. This flexibility reflects commercial deployment needs but introduces additional interference dynamics.

Current coexistence simulations do not capture the complexity introduced by sector-level diversity. Specifically, the assumption of uniform SBFD configuration overlooks cross-sector interference and fails to reflect dynamic spectrum allocation behavior in multi-sector environments. This gap in modeling may lead to overly optimistic coexistence results and limit the relevance of simulation outputs to real deployments.

RAN4 can discuss incorporating heterogeneous SBFD configuration support into the coexistence simulation framework. This would enable the study of cross-sector interference, adaptive resource coordination, and the evaluation of coexistence strategies in realistic network topologies.

Proposal 4:	RAN4 can discuss heterogeneous SBFD deployments in the Release-20 coexistence simulations by studying cases where different sectors within a site use mixed SBFD configurations.

Conclusion

This study extends the RAN4 coexistence framework from SBFD–TDD to SBFD–SBFD adjacent-channel scenarios, identifying four major modeling gaps and proposing practical solutions. This paper discussed SBFD–SBFD adjacent- channel coexistence and proposed:

Proposal 1:	RAN4 to consider adding a configuration-dependent self-interference model (based on SBFD duplex arrangements defined in 5G-Advanced, i.e. DU, UD, DUD) to support more flexible SBFD deployments beyond 5G.

Proposal 2:	Include multi-operator SBFD coexistence as a key Release-20 uplink study scenario, specifically where adjacent uplink sub-bands are operated by different operators.

Proposal 3:	RAN4 to evaluate Release-20 SBFD coexistence using existing NR RF baselines but with separate assessments for each SBFD configuration.

Proposal 4:	RAN4 can discuss heterogeneous SBFD deployments in the Release-20 coexistence simulations by studying cases where different sectors within a site use mixed SBFD configurations.

- References

- 3GPP TSG RAN, “Study on Evolution of NR Duplex Operation,” TR 38.858, V18.1.0, Technical Report, 3rd Generation Partnership Project (3GPP), March 2024.

- 3GPP TSG RAN, “Study on New Radio (NR) Access Technology: Radio Frequency (RF) and Co-existence Aspects,” TR 38.803, V14.4.0, Technical Report, 3rd Generation Partnership Project (3GPP), June 2024.