3GPP TSG-RAN WG4 Meeting #117	R4-2521898

Dallas, USA, Nov 17th – Oct 21st, 2025

Source:	Nokia

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	6.11.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the performance monitoring and generalization requirements for AI/ML-based CSI prediction use cases.

Discussion

Performance monitoring

Reporting Accuracy Requirements for Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. Initially, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

RAN4#116 meeting has made the agreements on the delay and accuracy requirement of performance monitoring [2].

Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

In RAN4#116bis meeting, it was agreed that no delay requirement would be defined for performance monitoring [3].

Issue 1-1: Requirements for performance monitoring Agreement: No performance monitoring delay requirements are specified in RAN4 for CSI prediction.

While it is agreed that no performance monitoring delay requirements are specified in RAN4 for CSI prediction, an open issue is whether it is feasible to define and test a reporting accuracy requirement for performance monitoring in AI/ML-based CSI prediction. A potential test framework can be specified in RAN4, where the UE calculates and reports SGCS values under static and controlled radio conditions. Since the environment remains unchanged, any variation in reported values indicates potential inaccuracies in the UE’s reporting mechanism.

By evaluating the consistency of SGCS reports across multiple monitoring instances, the test equipment (TE) can determine whether the UE reliably reports the performance monitoring metric. This approach avoids the need for ground-truth measurements or dynamic environmental changes and still provides a valid basis for defining accuracy requirements. Although this framework differs from conventional RAN4 test designs, it is suitable for verifying the stability of UE-reported metrics. Under static conditions, reported values should remain stable, and significant deviations may signal issues in the AI/ML-based prediction or reporting function.

Since RAN1 has already defined the performance monitoring mechanism and SGCS reporting structure, RAN4 must define corresponding test procedures to ensure the reliability of UE-reported metrics. Without such testing, performance monitoring reports used for AI/ML life-cycle management may be unreliable, potentially leading to degraded network performance. RAN4 can define an accuracy requirement using a statistical consistency-based approach. For example, it may require that in at least 90% of test iterations, reported SGCS values remain within a defined tolerance range under static conditions. This ensures the reported values are stable and suitable for performance monitoring.

RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

Generalization

RAN4#116bis meeting has an agreement on the test setup for generalization [3].

Issue 1-8: Generalization Agreement: Test setup for the generalization will be further discussed based on the following options - Option 1: Introduce multiple tests with different MCS, which will lead to different SNR points. - Option 2: Introduce multiple tests with different channel models. The details of the channel models are FFS. - Option 3: the combination of option 1 and 2. It is FFS on how to quantify the generalization performance.

Multiple tests with different MCS levels should be considered as a baseline for generalization testing. Varying the MCS levels naturally leads to different SNR conditions, which helps evaluate how well the AI/ML-based CSI prediction adapts to changes in link quality. This is important because generalization performance must be robust across a range of operating conditions, not just under ideal or fixed scenarios. Including multiple MCS levels ensures that the test setup reflects realistic variations encountered in practical deployments.

Regarding channel models for CSI prediction tests, TDL models can be used as a baseline. As defined in 3GPP TR 38.901, TDL includes multiple profiles such as TDL-A, B, or C. These profiles represent different propagation conditions with varying delay spreads and fading characteristics, covering both NLOS and LOS scenarios. In addition to selecting different TDL profiles, varying Doppler spread values can be applied to reflect different mobility conditions and assess the robustness of CSI prediction under dynamic environments.

From the perspective of field performance, TDL models provide a standardized framework for generalization testing. CSI prediction simulations in RAN4 have so far been based on TDL models, so RAN4 can consider to specific multiple profiles for generalization tests. This approach aligns with ongoing discussions in Rel-19 and provides a practical basis for scalable and realistic testing.

Thus, combining Option 1 and Option 2 enables RAN4 to evaluate AI/ML-based CSI prediction across different SNR levels and channel conditions. This approach supports the goal of verifying generalization gain and ensuring that models perform reliably in practical deployment scenarios. It also aligns with ongoing discussions in Rel-19 and provides a flexible foundation for future test procedure development.

RAN4 should define a test setup by combining Option 1 and Option 2 to verify generalization of the UE-sided AIML configurations in practical scenarios.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- RAN4 can define reporting accuracy requirements for AI/ML-based CSI prediction by using a test framework that monitors SGCS consistency under static radio conditions.

- RAN4 should define reporting accuracy requirements for CSI prediction performance monitoring using a test framework that evaluates the stability of reported values in a fixed environment.

- RAN4 should define a test setup by combining Option 1 and Option 2 to verify generalization of the UE-sided AIML configurations in practical scenarios.

- References

- R4-2508080, “WF on AI/ML air interface part 1,” RAN4#115, Malta.

- R4-2511890, “WF on AI/ML air interface for [116][131],” RAN4#116, Bengaluru, India.

- R4-2514629, “WF on AI/ML air interface for [116][131],” RAN4#116bis, Prague, Czech Republic.