# 3GPP TSG RAN WG4 Meeting #117 														R4-2521910

Dallas, USA, Nov. 17-21, 2025

Agenda Item:	4.8.2

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

There was a good progress in the last meeting, and we were able to agree on most of the essential issues regarding low-band CA via switching and the big CR is agreed. There are few open issues which are left to FFS in the maintenance phase and we discuss them in this paper.

# Discussion

One of the remaining open issues was the activation and deactivation timeline of the switching pattern. In the previous meeting, we had agreed that the RRC-configured switching pattern would become applicable after the SCell is activated. However, we later revised that agreement and decided that the pattern should be applicable from the start of the SCell activation command.

To ensure both the network and UE are aligned, we need to discuss and agree on the exact timeline from which the switching pattern becomes applicable. For this clarification, we need to consider both SCell activation methods:

– MAC CE-based SCell activation

– RRC-based direct SCell activation

For both of these activation methods, there are a couple of open issues pending since the meeting before last:

- First issue: Value of TLBCA

- Second issue: When to apply the switching pattern

During the last meeting, we reached an agreement on the first issue. The agreement is listed below.

•	TLBCA =1ms for MAC CE based, 0 ms for RRC based.

The second issue, when to apply the switching pattern is still open. The WF from the last meeting is listed below.

• For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms+TLBCA)/slot-length. Switching ends at the end of the slot. • For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(TRRC_Process +TLBCA)/slot-length. Switching ends at the end of the slot.

We believe all companies are aligned on the first part of the paragraph, except for the portion highlighted in yellow. The discussion during the last meeting focused on whether this part is actually needed or not. Our understanding is that RAN1 has already captured when shall the switching end in their specification. The RAN1 spec is listed below.

When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration

One view from some companies is that, in the current RAN1 text, the switching pattern was not considered applicable to an SCell that is in activation. We do not disagree with that. The main reason for this, is that, when we initially agreed the switching pattern is applicable from SCell activation completion, we sent an LS to RAN1 stating that the switching pattern is applicable only after SCell activation is completed, and RAN1 captured this accordingly in their specification.

However, RAN4 later revised the agreement, stating that the switching pattern is applicable from the SCell activation command. This updated understanding was not communicated to RAN1 before the last meeting.

With the latest LS from RAN4 to RAN1, RAN1 is now expected to clarify the applicability of the switching pattern during SCell activation. In light of this, we believe our specification should not capture this aspect in a more relaxed or ambiguous way than how it is captured by RAN1. If companies are more concerned about it, we are fine to revisit this if the RAN1 spec do not clarify this or capture this.

If we would like to capture this in RAN4 spec, we need to capture something on the following lines. Captuing it as Switching ends at the end of the slot is not exactly correct in our view.

For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after slot n+(THARQ+4ms)/slot-length.

For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after the slot n+( TRRC_Process)/slot-length.

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after slot n+(THARQ+4ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after the slot n+( TRRC_Process)/slot-length.

Deactivation of Switching Pattern:

In last meeting we agreed on the following.

- The switching pattern is NOT applied after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation - TLBCA =1 ms for MAC-CE based - Further discuss for timer triggered SCell deactivation - If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately after n+ ((THARQ+3ms+ TLBCA)/slot-length) for MAC-CE based SCell deactivation. - Following RAN1 logic, the carrier switching, if applicable, shall be placed at the end of a slot. - FFS RAN4 to capture this sub-bullet in the CR. - Further discuss for timer triggered SCell deactivation

One open issue concerns timer-triggered SCell activation. In the case of timer-based SCell activation, the UE may not know whether data scheduling will occur in the last slot before the timer expires. This uncertainty means the UE cannot be ready in advance to deactivate the pattern. Therefore, it may look reasonable to apply some margin to timer based SCell activation as well. However, current timer based SCell deactivation has 3ms margin and we think the additional 1 ms margin through the TLB-CA is not needed.

- TLB-CA is not needed for the timer based SCell deactivation.

SCell activation delay requirement:

In the last meeting, it was agreed that RAN4 will specify the SCell activation delay for two scenarios: (i) when none of the SSB/SMTC occasions overlap with the switching pattern, and (ii) when some SSB/SMTC occasions are impacted by the switching pattern. However, the specific requirements were not discussed, as expressing them is complex due to the variety of possible cases, as outlined below.

| Pattern and SSB/SMTC overlapping status | Whether to define the requirement or not |
| --- | --- |
| Some SSB overlapped, SMTC not overlapped | Define requirement |
| Some SSB and SMTC overlapped | Define requirement |
| None overlapped | Define requirement |
| All SSB overlapped, some SMTC not | FFS |
| All SMTC overlapped, some SSB not | FFS |
| All SSB and SMTC overlapped | Do not define requirement |

When only one component—either SSB or SMTC—is present during activation, the delay can be defined relatively easily. However, when both SSB and SMTC are present, the number of possible combinations increases significantly, making the requirement definition more complex. To simplify this, we propose not to define the delay for each component individually. Instead, we suggest specifying the activation timeline based on the combined delay for SSB and SMTC, ensuring the earliest possible time when both are available and not overlapping with the switching pattern.

- RAN4 to agree on the following text in the SCell activation delay requirement for the  cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern:

For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies:

-	if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

-	if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained.

-	if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

RLM requirement impact on the PCell

We think couple of procedures such as RLM and CBD on the PCell needs special attention.

RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell.

Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected.

Based on the switch pattern, the availability of RLM-RS on the PCell may be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SDL SCell do not have UL support, HARQ feedback is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM and faster link recovery on the PCell. For example, if any of the following condition is met, UE could prioritize performing RLM and link recovery on the slots where the RLM-RS or SSB is transmitted by ignoring the pattern on those slots.

- N1 number of out of sync are triggered on PCell; or

- T310 timer is started.

Similarly, if the BFD is detected on the PCell, UE could prioritize performing CBD on the slots where the CBD-RS is transmitted by ignoring the pattern on those slots (i.e., even if the slot containing CBD-RS belonging to SCell, UE prioritize the CBD-RS reception). In the case of RLM, NW do not know when the RLM occurs at the UE and in this case we could only define the interruption requirements.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after slot n+(THARQ+4ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell during the RRC configured switching gap (LBCA-SwitchingGap-Duration-PCelltoSCell) ending at the end of the slot immediately prior to the first slot with bit value ‘1’ in the RRC configured switching pattern that occurs after the slot n+( TRRC_Process)/slot-length.

- TLB-CA is not needed for the timer based SCell deactivation.

- RAN4 to agree on the following text in the SCell activation delay requirement for the  cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern:

For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies:

-	if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

-	if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained.

-	if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537

[3]		R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple)