3GPP TSG RAN WG4 Meeting #117	 																R4-2521920

Dallas, USA, 17 Nov. 2025 – 21 Nov. 2025

Agenda Item:	6.17.2.1

Source: 	Ericsson

Title:	Discussion on event triggered reporting test configurations for Mobility Ph 4

Document for:	Discussion

# Introduction

In this contribution we discuss the event triggered reporting test configurations for MIMO Ph 5.

# Discussion

## Event-driven beam management

Test configuration:

For the event-triggered reporting delay requirements, the end point of the measurement period is defined as the first UL transmission, as per the core requirements listed below. According to RAN2, the UE may transmit the first UL either by sending an SR, a special purpose SR, or by using an existing UL grant.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

In the test configuration, using SR as the end point is not verifiable, since the UE can send the SR at any arbitrary time, and the test equipment (TE) cannot determine when the measurement period has actually ended. While using a special purpose SR could be an alternative, but not all UEs may support it. To ensure consistent testing across all UEs, we propose scheduling a DCI with UL grant in every slot once the event is triggered.

- RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

# Summary

The following have been proposed in this contribution.

- RAN4 to agree that TE sends DCI in every slot after the event condition is changed in the test.

# References

[1] 		R4-2508271, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[2]		R4-2507014, On RRM performance aspects and test cases for MIMO Ph 5, Ericsson

[3]		R4-250xxxx, Discussion on CJTC reporting and asymmetric UL TRP requirements, Ericsson

[4]		R4-2317371