3GPP TSG-RAN WG4 Meeting #117	R4-2521928

# Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM performance of R19 ATG

Agenda Item:	6.7.3

Document for:	Discussion

- Introduction

Until last RAN4 meeting, after good progress achieved, the core part of R19 ATG has been completed and almost all issues have been concluded in perf part discussion, only some small issues were left to address in maintenance phase[1]. In this paper, after the review on all progress in perf part, we provide our further discussion on the left issues.

- Discussion

In last meeting, regarding the test case, only one remaining issue was left, i.e. whether and how to introduce Event triggered reporting on SCC with deactivated SCell with the neighbour cell measurement skipping indication.

Since the deployment of ATG in CA case is introduced in R19, so the measurement procedure on SCC is to be verified, together with the test motivation of interruption due to deactivated SCC measurement. Another new feature is the SCC neighbour cell measurement skipping, which is introduced in R19 and need to be verified.

To simplify the test case design, we prefer to verify the basic measurement procedure on SCC(with neighbour cell measurement) and the reduced measurement procedure on SCC(skipping the neighbour cell measurement) in one test case with two sub cases. So that unified test configuration could be used for the two sub cases only except the SCC neighbour cell measurement skipping indication. The advantage of this solution lies in the two sub cases could be fairly verified since unified test configuration is shared. While it should be noted that for the sub case of skipping neighbour cell measurement, the test requirement is kind of unique, different from typical test requirements, i.e. no reporting receiving is deem as the test requirement.

Another solution is to verify the basic measurement procedure on SCC and the reduced measurement procedure on SCC in separate test case respectively. The test configuration of basic measurement procedure on SCC has been provided in [1], i.e. event A6 is configured at the MO of SCC. Regarding how to verify the reduced measurement procedure on SCC, since UE does not measure the neighbour cell(s) of SCC, if any measurement reporting is expected to verify the measurement procedure, Event A1 or A2 is applicable. Since the actual scaling factor N1 for antenna phase array could not be verified in FR1 conduct test, whether UE performs the neighbour cell measurement, which could only be verified through the measurement report. Based on the specification in clause 5.5.5.1 of [2], the UE needs to report the best neighbour cell’s measurement result as long as the IE reportAddNeighMeas is configured in the reportConfig. As a result, this IE could be configured associated with the MO of SCC and Event A1/A2 is used.

1> if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas: - 2> for each measObjectId referenced in the measIdList which is also referenced with servingCellMO, other than the measObjectId corresponding with the measId that triggered the measurement reporting: - 3> if the measObjectNR indicated by the servingCellMO includes the RS resource configuration corresponding to the rsType indicated in the reportConfig: - 4> set the measResultBestNeighCell within measResultServingMOList to include the physCellId and the available measurement quantities based on the reportQuantityCell and rsType indicated in reportConfig of the non-serving cell corresponding to the concerned measObjectNR with the highest measured RSRP if RSRP measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured RSRQ if RSRQ measurement results are available for cells corresponding to this measObjectNR, otherwise with the highest measured SINR; - 4> if the reportConfig associated with the measId that triggered the measurement reporting includes reportQuantityRS-Indexes and maxNrofRS-IndexesToReport: - 5> for each best non-serving cell included in the measurement report: - 6> include beam measurement information according to the associated reportConfig as described in 5.5.5.2;

Based on above analysis, the above two solutions are both feasible to verify the normal measurement procedure and reduced measurement procedure, either is fine to us. While it is necessary to ensure such configurations would not conflict with UE self-checking for all the NW configurations.

Proposal 1: To verify the normal measurement procedure and reduced measurement procedure on SCC, two solutions are both feasible as long as the configurations are confirmed by UE self-checking:

- Solution 1: Verify the two sub cases in one test case and Event A6 is configured, same test configurations are shared for the two sub cases only except the NW indication of neighbour cell measurement skipping.

- Solution 2: Verify the normal measurement procedure and reduced measurement procedure in separate test cases. Event A6 is configured for the former, Event A2 and the IE reportAddNeighMeas are configured for the latter.

- Conclusion

In this contribution, we have the following proposals on the perf part of R19 ATG:

Proposal 1: To verify the normal measurement procedure and reduced measurement procedure on SCC, two solutions are both feasible as long as the configurations are confirmed by UE self-checking:

- Solution 1: Verify the two sub cases in one test case and Event A6 is configured, same test configurations are shared for the two sub cases only except the NW indication of neighbour cell measurement skipping.

- Solution 2: Verify the normal measurement procedure and reduced measurement procedure in separate test cases. Event A6 is configured for the former, Event A2 and the IE reportAddNeighMeas are configured for the latter.

- Reference

- R4-2514795, “WF on NR_ATG_enh_RRM”, CMCC, 3GPP TSG-RAN WG4 Meeting #116bis.

- TS 38.331 v18.7.0