3GPP TSG-RAN WG4 Meeting #117	R4-2521929

Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on maintenance of R19 NES

Agenda Item:	4.2

Document for:	Discussion

- Introduction

During last meeting, good discussion were held around the core part of NES and summarized in [1]. In this paper, we provide our further views on some remaining issue around the OD-SSB of the core part.

- Discussion

- In previous release of NR, the SSB-less operation is allowed in both intra-band contiguous and inter-band cases. The SSB-less operation mainly aims to reduce the NW power consumption and meanwhile reduce the processing delay and UE implementation complexity during the SCell activation procedure. Similarly, OD-SSB is another solution to achieve the NW energy efficiency. Based on existing specification, both SSB-less operation and OD-SSB operation are applicable to SCell.

- From the perspective of NW deployment, it is possible that intra-band or inter-band CA is configured to a UE, and SCell 1 is configured as SSB-less operation, another SCell 2 is configured with OD-SSB. Based on the existing specification, an anchor cell is needed to provide timing/AGC/QCL info for the SSB-less SCell(SCell 1) so that the SSB-less operation is applicable provided the side conditions of SSB-less operation meet. In intra-band CA, such anchor cell could be an active cell which is contiguous to the SSB-less SCell in the same band. In inter-band CA, a reference cell could be configured by the NW through referenceCell-r18 as the anchor cell, otherwise the QCL-typeC source cell is seen as the anchor cell.

- No matter the intra-band or inter-band CA, an OD-SSB SCell is possible to be seen or configured as the anchor cell to help the UE performs the SSB-less operation at the SSB-less SCell. There are multiple concrete cases of OD-SSB SCell, e.g. only OD-SSB is configured to the SCell without AO-SSB(Case 1), both OD-SSB and AO-SSB are configured for the SCell and they could be at the same of different frequency(Case 2). It is necessary to determine which SSB of the anchor cell is used to provide time/AGC info for the SSB-less SCell. So that the TRS of the SSB-less SCell should meet QCL-TypeC with this SSB of the anchor cell. We copy the relevant description in existing spec as below for intra-band CA and inter-band CA respectively.

- For intra-band CA:

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) or SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - - The RTD between the target SCell and the contiguous active serving cell is within ±260ns, and - - The difference of the reception power with the contiguous active serving cell is <= 6 dB, and - - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

- For inter-band CA:

- For a UE supporting scellWithoutSSB-InterBandCA-r18 if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - - The EPRE difference at the UE is smaller than or equal to 30 dB, where EPRE difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell normalized by SCSs of SSB of reference cell and A-TRS/P-TRS of SSB-less SCell, and - - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

- For OD-SSB Case 1 of the anchor cell, i.e. only OD-SSB is configured in the anchor cell, no AO-SSB, based on above RAN4 requirements, the anchor cell must be an active cell, and for active SCell configured with OD-SSB Case 1, RAN1 has the following agreement to guarantee there should be activated OD-SSB on the active OD-SSB SCell. So it is straightforward that the OD-SSB is deem as the SSB of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell.

- For OD-SSB Case 2 of the anchor cell, i.e. both AO-SSB and OD-SSB are configured in the anchor cell, since the AO-SSB is deem as the basic SSB to guarantee all UEs’ access instead of only serving the OD-SSB aware UE, so it is preferred to use the AO-SSB as the SSB of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell regardless the AO-SSB and OD-SSB are in same frequency or not.

- Proposal 1: For the case that an SCell configured with OD-SSB is the anchor cell of the SSB-less SCell, to facilitate the SSB-less operation, it is necessary to decide which SSB(OD-SSB or AO-SSB) of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell.

- Proposal 2: Decide which SSB of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell as below:

- -  If the anchor cell is configured with only OD-SSB(Case 1), the OD-SSB is the SSB to provide timing/AGC/QCL info for the SSB-less SCell.

- -  If the anchor cell is configured with both OD-SSB and AO-SSB(Case 2), the AO-SSB is the SSB to provide timing/AGC/QCL info for the SSB-less SCell regardless the AO-SSB and OD-SSB are in same frequency or not.

- We submitted a relevant CR to capture our proposal in our another contribution[2].

- Conclusion

In this contribution, we have the following proposals on the RRM aspects around NES:

- Proposal 1: For the case that an SCell configured with OD-SSB is the anchor cell of the SSB-less SCell, to facilitate the SSB-less operation, it is necessary to decide which SSB(OD-SSB or AO-SSB) of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell.

- Proposal 2: Decide which SSB of the anchor cell to provide timing/AGC/QCL info for the SSB-less SCell as below:

- -  If the anchor cell is configured with only OD-SSB(Case 1), the OD-SSB is the SSB to provide timing/AGC/QCL info for the SSB-less SCell.

- -  If the anchor cell is configured with both OD-SSB and AO-SSB(Case 2), the AO-SSB is the SSB to provide timing/AGC/QCL info for the SSB-less SCell regardless the AO-SSB and OD-SSB are in same frequency or not.

- Reference

- R4-2514808, “WF on Netw_Energy_NR_enh_RRM_Part1”, Ericsson, 3GPP RAN4#116bis, 2025.

- R4-25219161, (Netw_Energy_NR_enh-Core)CR on OD-SSB of R19 NES, ZTE, 3GPP RAN4#117.