3GPP TSG-RAN WG4 Meeting #117	R4-2521930

Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips, Qualcomm Incorporated

Title:	Discussion on maintenance of R19 SBFD

Agenda Item:	4.19.3

Document for:	Discussion

- Introduction

Until last meeting, good discussion were held around the SBFD and the core part discussion was completed. All relevant progress achieved during the core part discussion were summarized in [1][2][3][4][5][6][7]. In this paper, we provide our further views on the remaining issues of UE-to-UE CLI and SBFD operation impacts.

- Discussion

- Based on the progress summarized in [7] during last meeting, only two issues are still up to further discussions. In this paper, we provide our views on the two open issues.

- Revisit of RAN4 agreements on the collision between Method #1 and #2

- This issue was raised in last meeting, the the conclusion made in RAN4 is to wait for RAN1 conclusion. But it seems that there were no RAN1 discussions on this issue in the last RAN1 meeting. It should be noted that in previous 116 meeting, regarding this issue, RAN4 has already held warm discussion and the following progress was achieved:

Issue 1-2-1: Collision between Method #1 and Method #2 - Agreement (Tuesday AH session) - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol. - Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

- Considering such situation, we prefer to insist on the previous agreements.

- Proposal 1: For the collision between Method #1 and Method #2, insist on the previous agreement, i.e. UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol..

- Simultaneous configuration of SBFD and Dual Connectivity

- Regarding the configuration of SBFD in multi-carrier case, we review all the progress in different WGs.

RAN1#120 has made the following agreement to support the co-existence of multiple carrier and Rel-19 SBFD:

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

Furthermore, even in previous SI phase, RAN4 has already achieved the following agreement to support the co-existence of multiple carrier and Rel-19 SBFD.

Agreement - Rel-19 requirement shall consider both (1) single carrier operation for SBFD (2) SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier.

Which was also captured in the technical report [8] as below.

During Rel-18 SI, for SBFD-capable BS, RF requirement was only studied based on the semi-static configuration of subband time and frequency location, which is supported by SBFD-capable BS. RAN4 mainly focused on the following multi-carrier configuration for SBFD-capable BS: - - SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier.

- Furthermore, the RRM session has achieved consensus on the requirements of SBFD SCell activation in [7] as below.

<Agreement>: - Existing SCell activation requirements apply also for SBFD SCell.

- So the SBFD is confirmed supportive in multi-carrier case with the restriction of only one TDD cell/carrier is configured with SBFD operation.

- Observation 1: SBFD operation is supported in multi-carrier case with the restriction of only one TDD cell is configured with SBFD operation.

- Actually both RAN1 and RAN2 have already discussed the configuration of SBFD in DC and concluded LS to RAN4.

- RAN1 has also confirmed the feasibility of applying SBFD in DC as the highlighted part below in [9]:

Agreement - From RAN1 perspective, considering the specification impact, SBFD and NR-DC can be configured simultaneously only for inter-band NR-DC if a UE is capable of simultaneous transmission and reception indicated by UE capability simultaneousRxTxInterBandCA, where SBFD operation is applicable on only one NR TDD carrier. - - Inform RAN2 that specification impact to support this feature is not restricted to RAN1 but might result in specification impact on RAN4 as well. If supported, at least the following specification change to TS 38.213 should be adopted and the RAN4 specification for the configured transmitted power level for NR-DC in TS38.101-1 Clause 6.2B.4.1 may also need to be updated accordingly.

- Meanwhile, In RAN2 LS [10] sent to RAN1 and RAN4, RAN2 also confirm the feasibility form the perspective of RAN2 specification impact as below.

- RAN2 evaluated the specification impact to support simultaneous configuration of SBFD and DC, and concluded the following: From RAN2 point of view there may be limited specification impact (i.e., stage-2 impact but no need to introduce stage-3 spec impact). Send LS to RAN4 and RAN1 (cc RAN3) to inform this conclusion.

- Furthermore, regarding the feasibility of supporting DC scenario, RAN3 has already approved the coordination between gNBs via XnAP message for the sake of SBFD time/frequency configuration. The XnAP setup message is also the initial step to establish DC between MN and SN.

RAN3#126 agreement - The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

- Observation 2: Regarding the support of SBFD in DC, other WGs have the following progress:

- -  Based on RAN1 agreement, the specification impact lies in three points:

- -  only applicable for inter-band NR DC if UE capable of simultaneousRxTxInterBandCA;

- -  SBFD operation is only configured on one NR TDD carrier;

- -  The support of SBFD in DC is not restricted to RAN1;

- -  Based on RAN2 LS, there may be limited specification impact;

- -  RAN3 has already approved the coordination between gNBs via XnAP message for the sake of SBFD time/frequency configuration.

- Based on current status, we want to share our understanding on the specification impact from RAN4 perspective including both RF part and RRM part.

- After reviewing the whole discussion throughput the SI and WI phases and the endorsed big CR in RF part, firstly there are no spec impacts on the UE RF requirements from SBFD operation with DC. For the SBFD capable BS RF requirements, as [10] mentioned, all of the specified BS RF requirements are applicable to SBFD carrier, no matter DC is configured or not. We don’t see additional efforts needed to support the SBFD in DC case, it is straight forward to leverage the assumption in multi-carrier case, i.e. support SBFD operation on one TDD carrier in DC scenario.

- From RRM perspective, there were no assumption for the single carrier or multi-carrier in the past. After reviewing all of the RRM impact/requirements for SBFD, we don’t identify any specification impacts from RRM perspective. Company mentioned that the RLM/BFD for PSCell measurement of CSI-RS based L1 measurement may need to be added to the spec if SBFD operation in DC is supported in [11]. From the functionality perspective, it is necessary to support such measurement on PSCell, however it does not imply any specification impact since, RLM/BFD is applicable to both PCell and PSCell, which is captured in Clause 8.1.1 and 8.5.1 as below in the existing spec. The approved SBFD relevant RRM requirements do not impact any applicability rule of the RLM/BFD.

8.1 Radio Link Monitoring 8.1.1 Introduction The requirements in clause 8.1 apply for radio link monitoring on: - PCell in SA NR, NR-DC and NE-DC operation mode, - PCell operating with less than 5 MHz BW in SA NR (single carrier) operation mode, - PSCell in NR-DC and EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode, when configured. - The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell, PSCell and deactivated PSCell if configured with bfd-and-RLM with value true as specified in TS 38.331 [2].

8.5 Link Recovery Procedures 8.5.1 Introduction The UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set q0 as specified in TS 38.213 [3] in order to detect beam failure on: - PCell in SA, NR-DC, or NE-DC operation mode, - PSCell in NR-DC and EN-DC operation mode, - SCell in SA, NR-DC, NE-DC or EN-DC operation mode, - Deactivated PSCell in NR-DC and EN-DC operation mode - The RS resource configurations in the set q0 on PCell, PSCell or deactivated PSCell (if configured with bfd-and-RLM with value true) can be periodic CSI-RS resources and/or SSBs. RS resource configuration in the set q0 on SCell shall be periodic CSI-RS.

- Both inter-band FR1+FR1 DC and FR1+FR2 DC are typical NR DC deployments, especially for the case of FR1+FR2 DC, which is an important deployment with FR2 cell/carrier involved in. Regarding the scheduling restriction for inter-band case, it is captured in Clause 8.1.7 of TS38.133 as below for the RLM measurement, it can be shown there is no scheduling restriction on the serving cell of another band. For BFD, it is also same situation.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

There are no scheduling restrictions on FR1 serving cell(s) due to radio link monitoring performed on FR2 serving PCell and/or PSCell. There are no scheduling restrictions on FR2 serving cell(s) due to radio link monitoring performed on FR1 serving PCell and/or PSCell.

- We also review the endorsed big CR for SBFD, currently only intra-band case is mentioned around the scheduling restriction of L1 CLI measurement. Similar as above, the scheduling restriction for inter-band case should also be added into the LI CLI measurement and the above traditional principles could be reused.

- Proposal 1: In order to support inter-band CA/NR-DC for SBFD operation, the scheduling restriction of L1 CLI measurement is to be captured in the specification and reuse the legacy principle in L1 measurement.

-  When inter-band FR1+FR1 CA/NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the band in which the serving cell(s) L1-CLI measurement is configured.

-  When FR1+FR2 NR-DC (PCell is FR1 only) is configured, there are no scheduling restrictions on FR1 serving cell(s) due to L1 CLI measurement performed on FR2 serving cell(s).

-  When FR1+FR2 NR-DC (PCell is FR1 only) is configured, there are no scheduling restrictions on FR2 serving cell(s) due to L1 CLI measurement performed on FR1 serving cell(s).

- Therefore, whether to allow SBFD in DC scenario, after reviewing the whole RAN4 discussion and endorsed big CR, there are no specification impacts identified from both RF and RRM aspects.

- Proposal 2: In order to support SBFD in DC scenario, no specification impacts is identified from the RF aspect. For RRM part, the scheduling restriction for inter-band case needs to be captured in the specification.

- All other WGs have provided a thorough analysis on the specification impact, from RAN4 perspective, we provide the analysis on RAN4 specification impact as above. We don’t see any restriction to allow the SBFD in DC case with only one SBFD carrier.

- Proposal 3: Inter-band DC is a important NR scenario in both specification and field deployment, based on all WGs’ inputs, the SBFD in DC is confirmed to feasible and applicable in Rel-19.

- In this meeting, we also provide a draft on the LS reply to RAN2 in our another paper[12].

- Proposal 4: Send LS reply to RAN2 to provide RAN4 specification impact analysis and confirm SBFD in DC.

- Conclusion

In this contribution, we have the following proposals on the RRM aspects around SBFD:

- Proposal 1: For the collision between Method #1 and Method #2, insist on the previous agreement, i.e. UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol..

- Observation 1: SBFD operation is supported in multi-carrier case with the restriction of only one TDD cell is configured with SBFD operation.

- Observation 2: Regarding the support of SBFD in DC, other WGs have the following progress:

- -  Based on RAN1 agreement, the specification impact lies in three points:

- -  only applicable for inter-band NR DC if UE capable of simultaneousRxTxInterBandCA;

- -  SBFD operation is only configured on one NR TDD carrier;

- -  The support of SBFD in DC is not restricted to RAN1;

- -  Based on RAN2 LS, there may be limited specification impact;

- -  RAN3 has already approved the coordination between gNBs via XnAP message for the sake of SBFD time/frequency configuration.

- Proposal 1: In order to support inter-band CA/NR-DC for SBFD operation, the scheduling restriction of L1 CLI measurement is to be captured in the specification and reuse the legacy principle in L1 measurement.

-  When inter-band FR1+FR1 CA/NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) configured in other bands than the band in which the serving cell(s) L1-CLI measurement is configured.

-  When FR1+FR2 NR-DC (PCell is FR1 only) is configured, there are no scheduling restrictions on FR1 serving cell(s) due to L1 CLI measurement performed on FR2 serving cell(s).

-  When FR1+FR2 NR-DC (PCell is FR1 only) is configured, there are no scheduling restrictions on FR2 serving cell(s) due to L1 CLI measurement performed on FR1 serving cell(s).

- Proposal 2: In order to support SBFD in DC scenario, no specification impacts is identified from the RF aspect. For RRM part, the scheduling restriction for inter-band case needs to be captured in the specification.

- Proposal 3: Inter-band DC is a important NR scenario in both specification and field deployment, based on all WGs’ inputs, the SBFD in DC is confirmed to feasible and applicable in Rel-19.

- Proposal 4: Send LS reply to RAN2 to provide RAN4 specification impact analysis and confirm SBFD in DC.

- Reference

- R4-2413900, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#112, 2025.

- R4-2416858, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#112bis, 2025.

- R4-2420097, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#113, 2025.

- R4-2502597, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#114, 2025.

- R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#115, 2025.

- R4-2512133, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, 3GPP RAN4#116, 2025.

- R4-2515170, “Way Forward for [116bis][304] NR_duplex_evo_Maintenance”, Huawei, HiSilicon, 3GPP RAN4#116bis, 2025.

- 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

- R1-2504858, Reply LS on simultaneous configuration of SBFD and DC, RAN1, LS in, 2025-05.

- R4-2513787, “On general aspects for SBFD operation”, Samsung, 3GPP RAN4#116bis, 2025-10.

- R4-2513798, “Discussion on RRM core maintenance of Rel-19 SBFD”, Samsung, 3GPP RAN4#116bis, 2025-10.

- R4-2521934, “Reply LS on simultaneous configuration of SBFD and DC”, ZTE, 3GPP RAN4#117, 2025-11.