3GPP TSG-RAN WG4 Meeting #117	R4-2521947

# Dallas, USA, 17th – 21st November 2025

Agenda item:		7.6.3

Source: 		Qualcomm Inc.

Title:	 	NB-IoT NTN PC1.5 requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements for NR NTN and IoT NTN phase 2 has been approved [1]. One of the objectives is PC1.5 requirements for NB-IoT. In this contribution related UE RF requirements are discussed.

## 2	Discussion

2.1 Component power handling capability

Ideally, High power UE requirements should maximize the possibility for high power UE transmissions. However, also UE RF component feasibility needs to be considered. In real UE implementations, it is highly possible that UE could support e.g. NR NTN and IoT NTN and use the same signal path for both. Given the differences in NR and NB-NTN waveforms, it may be possible that Tx chain is used to support e.g. PC2 NR and PC1.5 NB-IoT. This is even more so given that RAN4 agreed that NR PC2 PA is used for deriving the PC1.5 NB-IoT requirements.

If the same NR PC2 components are used, supporting PC1.5 puts higher strain on them, as the components may not be from the beginning designed to sustain PC1.5 power level. Even though NB-IoT is HD-FDD, RAN1 specifications allow up to 256ms contiguous UL transmission, after which UE has to stop to receive DL for 40 ms. This results in slightly over 86% Tx duty cycle.

Furthermore, if the Same Tx path is used for both NR PC2 and NB-IoT PC1.5, the stress on the components is further increased as NR duplexer is part of the signal path, meaning the post-PA loss can be higher resulting in higher output power at PA output to achieve the same output power at antenna.

Observation 1: It is possible to use the same signal path to support e.g. NR NTN PC2 and NB-IoT NTN PC1.5.

Observation 2: Up to 256ms contiguous UL transmission is possible for NB-IoT with ~86% duty cycle.

Observation 3: Supporting PC1.5 with PC2 hardware puts additional strain on the components.

At the same time, it can be recognized that this is not the only implementation options. It is theoretically also possible to used dedicated PC1.5 hardware and standalone support for NB-IoT with NR NTN. Therefore, accommodations should not penalize UEs that do better.

Observation 4: Power handling related accommodations should not penalize UEs that can do better.

To enable re-use of same technologies and components, it would be beneficial not to mandate UE to follow higher than 25% duty cycle for PC1.5. This aligns with the existing NR TN and NR NTN specifications which specify default 50% duty cycle after which power reduction is made. Unlike the TN solution, we see power reduction should be optional: UEs with higher power handling capability can transmit longer times.

Proposal 1: Allow PC1.5 NB-IoT NTN UE to reduce transmit power to

- PC2 when Tx duty cycle in a radio frame is between 25 and 50%

- PC3 when Tx duty cycle in a radio frame is above 50%

2.2 MPR

MPR study was conducted with the assumptions agreed in [2]. PC2 NR PA was pushed beyond it’s normal operating conditions to reach 29 dBm at antenna connector, after which emission compliance to NB-IoT general SEM, UTRA ACLR and spurious emissions were evaluated. The results are captured in Table 1 and Table 2. Main observation from the results is that there is in general near-perfect alignment with the starting point agreed in [2], only 6-tone allocation has a 0.5 dB adjustment.

Observation 5: Given the increased post-PA loss assumption compared to PC1 evaluation, there is 0.5 dB adjustment for 6-tone allocation compared to the starting point in [2].

Proposal 2: Adopt the MPR values in Table 1 and Table 2 for PC1.5

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | ≤ 1.5 dB | 0 dB | ≤ 1.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

2.3 A-MPR

For band n256 there is NS_24N requirement for -50 dBm/MHz protection at 5 MHz frequency offset. The total backoff specified for PC2 and PC1 is shown in Table 3, which also includes the proposal for PC1.5.

Table 3: A-MPR for NS_24N

| Power class | Minimum MPR [dB] | Maximum MPR [dB] | A-MPR [dB] | Total backoff |
| --- | --- | --- | --- | --- |
| PC2 (specified) | 0 | 2 | 5.5 | 5.5…7.5 |
| PC1 (specified) | 0 | 4 | 10 | 10…14 |
| PC1.5 (proposal) | 0 | 2 | 8 | 8..10 |

Observation 6: For multi-tone allocations PC1 is allowed to transmit lower power than PC2 or proposed PC1.5

Proposal 3: Adopt 8 dB A-MPR backoff for NB-IoT assuming MPR values are

Proposal 4: Consider reduction of A-MPR for PC1 multi-tone allocations for PC1 in maintenance to remove the scenario where PC1 output power can be lower than PC2 or PC1.

## 3	Conclusion

In this contribution UE RF requirements for NB-IoT NTN for PC1.5 were discussed. Following observations and proposals were made.

2.1 Component power handling capability

Observation 1: It is possible to use the same signal path to support e.g. NR NTN PC2 and NB-IoT NTN PC1.5.

Observation 2: Up to 256ms contiguous UL transmission is possible for NB-IoT with ~86% duty cycle.

Observation 3: Supporting PC1.5 with PC2 hardware puts additional strain on the components.

Observation 4: Power handling related accommodations should not penalize UEs that can do better.

Proposal 1: Allow PC1.5 NB-IoT NTN UE to reduce transmit power to

- PC2 when Tx duty cycle in a radio frame is between 25 and 50%

- PC3 when Tx duty cycle in a radio frame is above 50%

2.2 MPR

Observation 5: Given the increased post-PA loss assumption compared to PC1 evaluation, there is 0.5 dB adjustment for 6-tone allocation compared to the starting point in [2].

Proposal 2: Adopt the MPR values in Table 1 and Table 2 for PC1.5

Table 1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 0 dB | ≤ 1.5 dB | 0 dB | ≤ 1.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |

Table 2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0 dB | ≤ 0 dB |  | ≤ 0 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

2.3 A-MPR

Observation 6: For multi-tone allocations PC1 is allowed to transmit lower power than PC2 or proposed PC1.5

Proposal 3: Adopt 8 dB A-MPR backoff for NB-IoT PC1.5 assuming MPR values are within 0 to 2 dB.

Proposal 4: Consider reduction of A-MPR for PC1 multi-tone allocations for PC1 in maintenance to remove the scenario where PC1 output power can be lower than PC2 or PC1.

## 4	References

[1] 	RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)

[2]	R4-2515157, WF for [116bis][318] NR_IoT_NTN_Ph2, Moderator (CHTTL)