3GPP TSG-RAN4 Meeting #117	R4-2521949

# Dallas, USA, November 17th – 21st 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3141 | rev | - | Current version: | 17.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_PC2_UE_FDD-Core) CR to TS 38.101-1: Addition of missing RMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia, Huawei, Ericsson, Skyworks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_UE_FDD-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RMC for PC2 FDD is missing from the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Active UL slots for PC2 FDD are added to Annex 2. Clarification is included for Rx requirements to ensure measurements are performed when UL transmissions is active. The FDD active slots are also used for receiver tests in which the UL power is set at PCMAX_L,f,c – 4 dB, the actual power level may still exceed PC3 with the power window for PUSCH power control added (assumed for conformance tests) . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 FDD test parameters are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, Annex A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

The applicability of receiver requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured (Table 6.2.3-1).

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annex A.2.2, the DL reference measurement channels specified in Annex A.3.2 and using the set-up specified in Annex C.3.1. For FDD bands, the minimum requirements shall be verified during active UL transmission, unless otherwise stated.

The minium requirements specified in clauses 7.5, 7.6, 7.7 and 7.8 for NR band n48 refer to the minimum requirements for NR bands < 2.7 GHz.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency, or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of these carriers j = 1,2, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

where FInterferer (offset),j for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier j as specified in clause 7.5, clause 7.6.2 and clause 7.6.4 for the respective requirement and BWChannel(j) the channel bandwidth of carrier j. FInterferer (offset),j for a sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5A, 7.6A.2 and 7.6A.3. The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow- band blocking shall be tested separately with a single in-gap interferer at a time.

For the additional requirements for operation with shared spectrum channel access, the receiver requirements apply under the assumption that all 20 MHz sub-bands and all RB’s of each sub-band within the downlink channel are allocated with intra-cell guard bands configured to zero.

## 7.1A	General

<End of Change 1>

<Start of Change 2>

## A.2.1	General

The measurement channels in the following clauses are defined to derive the requirements in clause 6 (Transmitter Characteristics) and clause 7 (Receiver Characteristics). The measurement channels represent example configurations of physical channels for different data rates.

The measurement channels in the following clauses are applicable to both FDD and TDD.

The active uplink slots for TDD configurations are specified in table A.2.1-1. TDD slot patterns defined for reference sensitivity tests will be used for TDD UL RMCs, unless otherwise stated. The active uplink slots for additional TDD configuration specified in Table A.2.1-2, with the TDD pattern defined in Table A.2.1-3, is used for the phase continuity tests for DMRS bundling with 15 kHz SCS. The active uplink slots configuration specified in Table A.2.1-4 and the additional TDD pattern in Table A.2.1-5 are used for shorter transient period capability EVM tests at 15 kHz SCS.

Table A.2.1-1: TDD active uplink slots

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 4, 9 |
| 30 kHz | 8, 9, 18, 19 |
| 60 kHz | 16, 17, 18, 19, 36, 37, 38, 39 |

Table A.2.1-2: TDD active uplink slots for additional TDD configuration

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 8, 9 |

Table A.2.1-3: TDD pattern for additional TDD configuration

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 7DS2U |
| Special Slot Configuration (Note 2) |  | 6D+4G+4U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 10 ms |
|  | nrofDownlinkSlots | 7 |
|  | nrofDownlinkSymbols | 6 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 4 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

Table A.2.1-4: TDD active uplink slots for shorter transient period capability

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 3,4 |

Table A.2.1-5: Additional TDD pattern for shorter transient period capability

| Parameter |  | Value |
| --- | --- | --- |
|  |  | SCS 15 kHz (µ0) |
| TDD Slot Configuration pattern (Note 1) |  | 2DS2U |
| Special Slot Configuration (Note 2) |  | 10D+2G+2U |
| referenceSubcarrierSpacing |  | 15 kHz |
| UL-DL configuration | dl-UL-TransmissionPeriodicity | 5 ms |
|  | nrofDownlinkSlots | 2 |
|  | nrofDownlinkSymbols | 10 |
|  | nrofUplinkSlot | 2 |
|  | nrofUplinkSymbols | 2 |
| NOTE 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. NOTE 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. |  |  |

The active uplink slots for PC2 FDD are specified in Table A.2.1-6.

Table A.2.1-6: FDD active uplink slots for Power Class 2

| SCS | Active Uplink slots |
| --- | --- |
| 15 kHz | 3 to 6 |
| 30 kHz | 6 to 13 |
| 60 kHz | 12 to 27 |

NOTE:	To allow flexibility for signal configuration for PC2 FDD, e.g. PUCCH and SRS may be transmitted also during other slots as long as no UL duty cycle exceedence will happen in a radio frame.

## A.2.2	Reference measurement channels

<End of Change 2>