3GPP TSG-RAN WG4 Meeting #117	R4-2521981

# Dallas, US, 17th Nov 2025 – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0103 | rev | - | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates unwanted emissions specifications for the IoT NTN band 253 to align with ETSI emissions regulations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Viasat, Thuraya, Apple, Aalyria |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_extLband-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the follo8ing releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ETSI out of band emissions were not captured for bands IoT NTN L-band 253. More recently, two sets of ETSI requirements (current and future) were captured for n250, the uplink of which operates in the same frequency range as band 253. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the NS flags specified for n250 to band 253 as well to capture the ETSI emission requirements for the uplink frequency range 1668 – 1675 MHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ETSI requirements for the uplink frequency sub-band 1 range, 1626.5 – 1660.5 MHz, will be inconsistent across 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3, 6.2B.3, 6.5A.3.3, 6.5A.4.4, 6.5B.3.3, 6.5B.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Once approved, this CR shall be implemented in the specification before the CR#XXX in R4-25XXXXX. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3A-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3A-1 | PC3 | PC5 |
|  |  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3A-1 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | Table 5.3A-1 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_09N | 6.5A.3.3.4, 6.5A.4.4.10 | 253 | Table 5.3A-1 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | Table 5.3A-1 | N/A |  |
| NS_13N1 | 6.5A.4.4.11 |  | Table 5.3A-1 | N/A |  |
| NOTE 1: NS_09N (band 253) is currently in force. NS_13N may supersede NS_09N when the new ETSI requirements get ratified. |  |  |  |  |  |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5A.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5A.3.3.1, 6.5A.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | N/A |  |
| NS_05N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | N/A |  |
| NS_09N | 6.5A.3.3.4, 6.5A.4.4.10 | 253 | N/A |  |
| NS_11N | 6.5A.3.3.2, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_12N | 6.5A.3.3.3, 6.5A.4.4.5 | 254 | PC3 | PC5 |
|  |  |  | ≤ 3 | 0 |
| NS_13N1 | 6.5B.4.4.11 |  | N/A |  |
| NOTE 1: NS_09N (band 253) is currently in force. NS_13N may supersede NS_09N when the new ETSI requirements get ratified. |  |  |  |  |

<< NEXT CHANGE >>

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2-1.

Table 6.2B.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC3 | PC5 |
|  |  |  | ≤ 3.5 | ≤ 0.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |
| NS_03N | 6.5B.3.3.2, 6.5B.4.4.4 | 254 | N/A |  |
| NS_04N | 6.5B.3.3.3, 6.5B.4.4.5 | 254 | 6.2B.3.1 |  |
| NS_05N | 6.5B.3.3.4, 6.5B.4.4.5 | 254 | 6.2B.3.2 |  |
| NS_09N | 6.5B.3.3.4, 6.5B.4.4.10 | 253 | N/A |  |
| NS_11N | 6.5B.3.3.3, 6.5B.4.4.5 | 254 | 6.2B.3.3 |  |
| NS_12N | 6.5B.3.3.4, 6.5B.4.4.5 | 254 | 6.2B.3.4 |  |
| NS_13N1 | 6.5B.4.4.11 |  | N/A |  |
| NOTE 1: NS_09N (band 253) is currently in force. NS_13N may supersede NS_09N when the new ETSI requirements get ratified. |  |  |  |  |

<<Unchanged Text Skipped>>

<< NEXT CHANGE >>

#### 6.5A.3.3	Additional Spectrum Emission Mask for category M1

##### 6.5A.3.3.1	Requirements for network signalling value "NS_02N” and “NS_03N”

When "NS_02N" or "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.1-1.

Table 6.5A.3.3.1-1: Additional requirements for "NS_02N" and "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

##### 6.5A.3.3.2	Requirements for network signalling value "NS_04N"

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.2-1 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.2-1: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

##### 6.5A.3.3.3	Requirements for network signalling value “NS_05N"

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-1: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

##### 6.5A.3.3.4	Requirements for network signalling value “NS_09N"

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.4-1 for any channel bandwidth configured within the corresponding band.

Table 6.5A.3.3.4-1: Additional requirements for "NS_09N" and “NS_10N”

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth | Note (measurement method) |
| --- | --- | --- | --- |
| ± 0-25 | 30 to 15 | 3 kHz | Average |
| ± 25-125 | 15 to -20 |  |  |
| ± 125-425 | -20 |  |  |
| ± 425-1500 | -20 to -35 |  |  |
| ± 1500-36000 | -25 | 30 kHz |  |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

<< NEXT CHANGE>>

#### 6.5A.4.4	Additional spurious emissions

##### 6.5A.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5A.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5A.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

##### 6.5A.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3-1.

Table 6.5A.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |

##### 6.5A.4.4.4	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5.4.4.4-1: Additional out-of-band requirements for "NS_03N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5A.4.4.5	Minimum requirement (network signalled value "NS_04N" and "NS_05N")

When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.5-1: Additional out-of-band requirements for "NS_04N" and "NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

##### 6.5A.4.4.6	Void

##### 6.5A.4.4.7	Void

##### 6.5A.4.4.8	Void

##### 6.5A.4.4.9	Void

##### 6.5A.4.4.10	Minimum requirement (network signalled value "NS_09N")

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.10-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.10-1: Additional out-of-band requirements for "NS_09N"

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| 1660.5 to 1666.5 | -25 | 30 kHz | Average |
| 1666.5 to 1667.575 | -35 to -20 | 3 kHz | Average |
| 1667.575 to 1667.875 | -20 | 3 kHz | Average |
| 1667.875 to 1667.975 | -20 to 15 | 3 kHz | Average |
| 1667.975 to 1668 | 15 to 30 | 3 kHz | Average |
| 1675 to 1675.025 | 30 to 15 | 3 kHz | Average |
| 1675.025 to 1675.125 | 15 to - 20 | 3 kHz | Average |
| 1675.125 to 1675.425 | -20 | 3 kHz | Average |
| 1675.425 to 1676.5 | -20 to -35 | 3 kHz | Average |
| 1676.5 to 1677 | -25 | 30 kHz | Average |
| 1677 to 1680 | -30 | 30 kHz | Average |
| 1680 to 1685 | -30 | 100 kHz | Average |
| 1685 to 1695 | -30 | 300 kHz | Average |
| 1695 to 1705 | -30 | 1 MHz | Average |
| 1705 to 2250 | -30 | 3 MHz | Average |
| 2250 to 12750 | -30 | 3 MHz | Peak hold |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

##### 6.5A.4.4.11	Minimum requirement (network signalled value "NS_13N")

When "NS_10N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.11-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.11-1: Additional out-of-band requirements for "NS_13N”

| Frequency range (MHz) | Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE (measurement method) |
| --- | --- | --- | --- |
| 30 to 1000 | -36 | 100 kHz | Peak hold |
| 1000 to 1559 | -31 | 1 MHz | Average |
| 1559 to 1605 | -40 | 1 MHz | Average |
| 1605 to 1612.5 | -40 to -28.5 | 1 MHz | Average |
| 1612.5 to 1616.5 | -25 to -20 | 1 MHz | Average |
| 1616.5 to 1621.5 | -20 to -16 | 1 MHz | Average |
| 1621.5 to 1624.5 | -30 to -27.5 | 30 kHz | Average |
| 1624.5 to 1625.125 | -27.5 to -27.2 | 30 kHz | Average |
| 1625.125 to 1625.8 | -27.2 to -20 | 30 kHz | Average |
| 1625.8 to 1626 | -20 to -17 | 30 kHz | Average |
| 1626 to 1626.2 | -17 to -10 | 30 kHz | Average |
| 1626.2 to 1626.5 | -10 | 30 kHz | Average |
| NOTE 1: Spectrum emissions are linearly interpolated versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. NOTE 3: The average measurement method shall apply except that an averaging period of 20ms shall be used in the sub-band 1573.42 MHz to 1580.42 MHz |  |  |  |

<< NEXT CHANGE>>

#### 6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

##### 6.5B.3.3.1	Requirements for network signalling value "NS_02N”

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.1-1.

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

##### 6.5B.3.3.2	Requirements for network signalling value "NS_03N"

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.2-1.

Table 6.5B.3.3.2-1: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± 0.85–1.7 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.38 MHz.

##### 6.5B.3.3.3	Requirements for network signalling value "NS_04N"

As specified in 6.5A.3.3.2.

##### 6.5B.3.3.4	Requirements for network signalling value “NS_05N"

As specified in 6.5A.3.3.3.

##### 6.5B.3.3.5	Requirements for network signalling value “NS_09N”

As specified in 6.5A.3.3.4.

<<NEXT CHANGE >>

#### 6.5B.4.4	Additional spurious emissions

##### 6.5B.4.4.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5B.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5B.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

##### 6.5B.4.4.3	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.3-1.

Table 6.5B.4.4.3-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band 256. |  |  |

##### 6.5B.4.4.4	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

##### 6.5B.4.4.5	Minimum requirement (network signalled value "NS_04N" and "NS_05N")

When "NS_04N" or "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.5-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

##### 6.5B.4.4.6	Void

##### 6.5B.4.4.7	Void

##### 6.5B.4.4.8	Void

##### 6.5B.4.4.9	Void

##### 6.5B.4.4.10	Minimum requirement (network signalled value "NS_09N")

When "NS_09N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.10-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

##### 6.5B.4.4.11	Minimum requirement (network signalled value "NS_13N")

When "NS_13N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.11-1 where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

<<END OF CHANGES >>