# 3GPP TSG-RAN WG4 Meeting #117 															R4-2522008

Dallas, US,  November 17 - 21, 2025

Agenda Item:	8.13.1

Source:	Ericsson

Title:	On UE/BS specification handling

Document for:	Discussion

# 1	Introduction

In this paper, we present our view on the UE/BS specification handling.

# 2	Discussions

## 2.1.1	On release-independency specification of 3x.307

MCC points out the how RAN4 should handle the release independent feature in 38.307 in RAN contribution [RP-240905] and this should be clarified further in day 1 in 6G. RAN4 has added many new aspects for release independent since the first introduction of the release independent specification in UMTS time. At beginning only bands are in specification, then other features added, e.g CA combination, channel bandwidth, power class, band for different feature etc. in particular, there are some “release independent” feature which we think may not belong to 3x.307. In our opinion, these “release independent" features should be handled in RAN2 in TS 38.331 instead of RAN4 38.307. RAN2 has “early implementation” concept for release independent from Rel-N and has a rule that no interoperability issue "Implementation of this CR from Rel-N will not cause interoperability issues". Any early implementation CR agreed is tracked in annex C in TS 38.331.

Observation 1: Release independent from Rel-N in RAN2 is defined as "Implementation of this CR from Rel-N” and RAN2 also has a rule of “not cause interoperability issues”.

Below we lists some example in 38.307 to illustrate our view.

Example 1: UL 7.5KHz shift for band n40 in 38.307 and quoted below. RAN2 has CR (R2-2011066) for early implementation according to RAN4 LS.

Table 5.3-2: UL 7.5KHz shift for band n40 in FR1

| Feature | DL/UL | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the configuration was introduced) |
| --- | --- | --- | --- | --- |
| 7.5KHz UL shift for band n40 in FR1 | UL | TDD | Rel-15 | Table B.4.7-1 |

Example 2: High speed train and quoted below. RAN4 sent LS (R4-1915916) to RAN2 and RAN2 CR (R2-2006188 ) for early implementation.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |

Example 3: DMRS bundling for GSO scenario. The feature is specified in RAN1 in Rel-17 while RAN4 add the core requirement in Rel-18. This may not be in line with MCC expectation.

“F.	Where can the REL-independent approach not be used?

The REL-independent approach is strictly limited to bands and band combinations, it is not a mechanism to ignore 	the REL-freeze by squeezing whatever late features into frozen releases.

The REL-independent approach shall also not be misused to shift RAN4 work (for which there is currently no time 	available) into the next REL but still squeeze in the corresponding RAN1/2/3 work into the current REL because 	acc. to 3GPP procedures the incomplete feature (RAN1/2/3 work) has to be removed from the current REL at the 	REL-freeze as it cannot work without missing RAN4 part and RAN5 can not test incomplete features.”

Table 9.2-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| DMRS bundling for GSO scenario | Rel-17 | Table F.1-4 | Rel-18 WI NR_NTN_enh introduced phase continuity requirements for DMRS bundling for the GSO scenario: see Table C.1-1 |

Observation 2: Some “release independent” aspects in TS 38.307 overlap with early implementation of CR in RAN2 TS 38.331.

Use RAN2 release independent from Rel-N with early implementation concept for “release independent” feature instead of the 3x.307 if such feature has other working group impact, e.g signalling in RAN2. Following the MCC guidance on release independent handling in RAN4, only allow the band related feature in 3x.307.

## 2.1.2	Consistency of the specification wording

For 5G, RAN4 specified 3 specifications in RF, namely 38.101-1/2/3. The clause numbering of the requirements at the specification was aligned, e.g. Clause 6 for transmit characteristics, so RAN5 conformance test specification clause numbering could be aligned with RAN4 for all frequency ranges. This should also be used for 6GR.

the clause numbering in RAN4 specification of 6GR for different FR range should be aligned.

The suffix structure of the 5G specification can be reused in 6G. However, the requirement relation between general requirement and suffix requirement is stated in clause 4:

“A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional clause (suffixes A to L) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional clause requirements (suffixes A to L) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional clause.”

The rule of suffix requirement in above clarification may need to consider more cases in 5G specification. For example, in 38.101-1 there are configured output power for CA in clause 6.2A.4 and the general clause 6.2.4.  In general clause 6.2.4, there is a ΔTIB,c in Pcmax,c formular which pointing to the clause 6.2A.4:

“∆TIB,c is the additional tolerance for serving cell c as specified in clause 6.2A.4.2 for NR CA”

This example is not the case of the “tighter requirements” as the requirement are different for general and suffix requirements (i.e the general is Pcmax,fc and suffix requirement for total aggregated power from all carriers, Pcmax) and for CA feature, both general requirement and suffix requirement apply. When configured with CA the UE configures a maximum power Pcmax,fc per serving cell and a total aggregated power Pcmax at power prioritization according to 38.213 occurs when the UE is power limited. However, it is not easy to understand the UE power configuration behaviour as the CA suffix requirement specify the total aggregated power Pcmax from different CC and general requirement specify the signal CC output power, it is not clear to reader if the single CC power PCMAX,c would be capped when the aggregated power from each Pcmax,c exceeds the total power PCMAX. And if so, which carrier power PCMAX,c would be reduced?

In case to specify the feature requirements at both general clause and suffix clause, and the requirement in general clause and suffix clause relating to each other, it is preferable to state which requirement should be met in what condition.

In 38.101-1, the different wording is used for a UE to do something, it is summarized below:

| Next word after “UE is” | example | comments |
| --- | --- | --- |
| configured | “UE is configured” 53 occurrence | Most used, could be interpreted with RRC configuration |
| scheduled | “UE is scheduled” 32 occurrence | Most used in context of DCI scheduling from network, can be MAC signaling (MAC-CE) for e.g. ending a semi-persistent scheduling |
| Configured or scheduled | “Configured or scheduled” 2 occurrence | Either RRC or DCI, or semi-persistent scheduling |
| associated | “UE is associated” 3 occurrence | Only used in “UE is associated with a serving cell on PS carrier”, could be inter-changeable with “configured”? |
| allowed | “UE is allowed” 17 occurrence | “UE is allowed to set “ or “UE is allowed to reduce”, should follow the 21.801 drafting rule, Annex E, for permission, “may” is used and equivalent expressions for use in exceptional cases “is permitted is allowed is permissible” |
| required | “UE is required” 5 occurrence | Most used with the number of Rx antenna. “ATG UE is required optionally to be equipped with four Rx antenna ports.” should follow the 21.801 drafting rule, Annex E, use “shall” or when it related to UE capability, rewording to “if the UE indicates support of “UE capability”, it shall meet the [RAN4] requirements of [suffix or general requirement] |
| capable | “UE is capable” 2 occurrence | should follow the 21.801 drafting rule, Annex E, use “can” |
| within | “ UE is within” 4 occurrence | Only used as “UE is within the protected zone”, no ambuiguity |

For another example on consistent wording, the “antenna”, “antenna connector”(275 occurrence) and “antenna ports”(167 occurrence) are all used in requirements. the specification quality could be improved if the wording is consistent and even better if such terminology is defined in clause 3. In the clause of receiver requirements, it seems the “antenna port” and “antenna connector” can be inter-changeable, for example:

“7.2	 Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports.”

And

“Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).”

But in transmitter requirement, sometimes the “antenna port” is refereeing to antenna ports logically associated with specific physical channels/signals used in RAN1 specification, for example:

“If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the requirements in clause 6.2F.2 apply for the power class as indicated by the ue-PowerClass field in capability signaling.”

And below in the conext of single layer or two-layer

“The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2 as specified in [38.306].”

It is less obvious if the layer information is not mentioned and both antenna connector and antenna port are used at the same time.

“In Figure 6.3A.3.3.6-5, the uplink transmission on band X and band Y is with one transmit antenna connector and one antenna port, and the uplink transmission on band Z is with two transmit antenna connectors and two antenna ports.”

Observation 3: the wording in RF specification is not consistent in different requirement in 5G specification

the wording consistency can be improved with drafting rules and clearly defined terminology.

## 2.1.3	7GH, 8Ghz and 15GHz the specification structure

When the 7GHz , 8 GHz and 15GHz frequency range is to be specified in 6G, it should be discussed if the conducted specification or OTA specification should be used. As the UE and BS may have different spec structure such discussion can be separately discussed for UE and BS.

discuss which specification is used to specify the 7GHz, 8GHz and 15GHz, for UE and BS respectively.

## 2.1.3	UE EMC specification structure

The development of 6G standards presents a suitable opportunity to review the structure of Electromagnetic Compatibility (EMC) specifications. Unlike the network nodes, the current UE EMC specifications are relatively straightforward, consisting of a per-RAT structure:

- TS 36.124, Evolved Universal Terrestrial Radio Access (E-UTRA); Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

- TS 38.124, NR; Electromagnetic compatibility (EMC) requirements for mobile terminals and ancillary equipment

For 6G, there are two potential options.

One option is to consolidate all UE EMC Specifications into a single UE EMC specification that would contain EMC requirements for all RATs, which provides a single reference point of UE EMC requirements. Make maintenance work simple. A single change request can update requirements for all RATs.

Another option is to discontinue 3GPP UE EMC specification for 6G, i.e., the existing specifications (TS 36.124, TS 38.124) would remain for legacy technology support but would not be extended for 6G. This approach questions the necessity of developing a new 3GPP UE EMC specification for 6G, considering low interest from UE vendors in 5G. Of course, this would represent a significant change in 3GPP’s scope for UE specifications.

It is recommended that RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification. If the industry consensus confirms a need for a 3GPP specification, then option 1 would be the way forward. If the consensus confirms that external standards are sufficient and that a 3GPP specification is redundant, then option 2 should be seriously considered to avoid unnecessary standardization effort.

Observation 4: The following two options could be considered for 6G EMC specification structure regarding UE:

- Option 1: Create a single, consolidated 6G UE EMC specification.

- Option 2: Discontinue developing 3GPP UE EMC specification for 6G and maintain legacy UE EMC specification (TS 36.124 and TS 38.124).

RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification

## 2.1.3	BS specification structure

#### 2.1.3.1 6G core and conformance specification

As already discussed in our previous contribution ([1]), the AAS BS core specification TS 37.105 will also be enhanced with the introduction of the new 6G RAT (Hybrid AAS BS and OTA AAS BS), and similar to TS 37.104, there will be support only for E-UTRA, NR and 6G.

A second alternative approach would be to not create a new single RAT specification for 6G, instead extend the scope of MSR BS specification 37.104 to include full support for single RAT requirements as well. Such an approach can lead to the withdrawal of single RAT specifications 36.104 and 38.104 from Rel-21 and have only two multi-standard specifications enabled instead: 37.104 (for BS with up to 8 antenna ports) and 37.105 (for hybrid and OTA BS).

When introducing 6G RAT, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.104 and TS 37.105) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.104 and TS 37.105).

The alternative of dropping completely the single RAT conformance specifications and rely totally on TS 37.141 (for BS with conducted requirements) and 37.145-1/-2 (for hybrid and OTA AAS BS) is appealing and proposed for further discussion.

Similar and aligned with the core specifications decision, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2).

#### 2.1.3.2 EMC specification structure

As exposed in our previous contribution ([1]), the development of 6G standards presents a suitable opportunity to review the structure of Electromagnetic Compatibility (EMC) specifications. The current framework has evolved with each new generation of technology:

- TS 36.113, Evolved Universal Terrestrial Radio Access (E-UTRA); BS (BS) and repeater Electromagnetic Compatibility (EMC)

- TS 37.113, NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) BS (BS) Electromagnetic Compatibility (EMC)

- TS 37.114, Active Antenna System (AAS), BS (BS) Electromagnetic Compatibility (EMC)

- TS 38.113, NR; BS (BS) ElectroMagnetic Compatibility (EMC)

- TS 38.114, NR; Repeaters ElectroMagnetic Compatibility (EMC)

- TS 38.175, NR; Integrated access and backhaul ElectroMagnetic Compatibility (EMC)

A rationalized structure for 6G could improve clarity, ease of use, and long-term maintainability. The following text outlines three potential options for structuring the 6G EMC specifications for network nodes. Each option offers a different approach with advantages and disadvantages.

One option is to consolidate all EMC requirements into one comprehensive specification, i.e., a single specification would contain EMC requirements for all RATs for BS, Repeater, and IAB. Equipment-specific requirements/test setup/configurations would be in separate clauses.

Another option is to merge all BS-related EMC specifications while maintaining separate specifications for other type of equipment, i.e., one specification for BS including single RAT, AAS and multi-RAT operation (covering TS 36.113, TS 37.113, TS 37.114, TS 38.113 and 6G BS), and separate specifications for Repeaters and IAB nodes.

The third option is to continue the existing methodology by creating new, separate specifications for each 6G equipment type, i.e., new individual EMC specification for 6G BS/Repeater/IAB, and existing specifications for previous technologies would remain unchanged.

Table 1: Comparison of the pros and cons of the three options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1: A single unified specification | - Provides a single reference point for EMC requirements for all RATs for all types of network nodes. Make maintenance work simple. A single change request can update requirements for all equipment types. | - May result in a large document, potentially making it difficult to navigate. - Careful construction is needed to prevent the requirements/test setup/configurations of one equipment type from being incorrectly applied to another. |
| Option 2: A consolidated BS specification with separate specification for Repeaters and IAB | - Reduce the overlap that currently exists between the various BS EMC specifications. - Maintains a more focused scope for each individual specification. | - While reduced, the effort to maintain multiple specifications remains. |
| Option 3: Maintain the current structure | - Follows a well-established and understood process. - Allows each document to be focused on a single equipment type. | - Continue the current maintenance overhead with updating repeated text in multiple documents. - Lead to an increase in the number of specifications over time. |

Observation 5: The following three options could be considered for 6G EMC specification structure regarding network nodes:

- Option 1: A single unified specification

- Option 2: A consolidated BS specification with separate specifications for Repeaters and IAB

- Option 3: Maintain the current structure

Our preference is Option 2, as it maintains a more focused scope for each type of network node and ensures a reasonable workload on the EMC specifications within RAN4.

Adopt Option 2 for the 6G EMC specification structure: a consolidated BS EMC specification with separate EMC specifications for Repeaters and IAB.

# 4	Summary

Observations:

Observation 1: Release independent from Rel-N in RAN2 is defined as "Implementation of this CR from Rel-N” and RAN2 also has a rule of “not cause interoperability issues”.

Observation 2: Some “release independent” aspects in TS 38.307 overlap with early implementation of CR in RAN2 TS 38.331.

Observation 3: the wording in RF specification is not consistent in different requirement in 5G specification

Observation 4: The following two options could be considered for 6G EMC specification structure regarding UE:

- Option 1: Create a single, consolidated 6G UE EMC specification.

- Option 2: Discontinue developing 3GPP UE EMC specification for 6G and maintain legacy UE EMC specification (TS 36.124 and TS 38.124).

Proposals:

Proposal-1:Use RAN2 release independent from Rel-N with early implementation concept for “release independent” feature instead of the 3x.307 if such feature has other working group impact, e.g signalling in RAN2. Following the MCC guidance on release independent handling in RAN4, only allow the band related feature in 3x.307.

Proposal-2:the clause numbering in RAN4 specification of 6GR for different FR range should be aligned.

Proposal-3: In case to specify the feature requirements at both general clause and suffix clause, and the requirement in general clause and suffix clause relating to each other, it is preferable to state which requirement should be met in what condition.

Proposal-4:the wording consistency can be improved with drafting rules and clearly defined terminology.

Proposal-5: discuss which specification is used to specify the 7GHz, 8GHz and 15GHz, for UE and BS respectively.

Proposal-6:RAN4 seeks feedback from UE vendors and other relevant stakeholders on the necessity of a dedicated 3GPP 6G UE EMC specification

Proposal-7:When introducing 6G RAT, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.104 and TS 37.105) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.104 and TS 37.105).

Proposal-8:Similar and aligned with the core specifications decision, RAN4 should further investigate the following 2 alternatives:

- Alt1: Create a new TS for 6G single RAT and update the existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2) adding 6G support for MSR configurations.

- Alt2: Add support for 6G single RAT and MSR configurations in existing MSR specifications (TS 37.141, TS 37.145-1 and TS 37.145-2).

Proposal-9:Adopt Option 2 for the 6G EMC specification structure: a consolidated BS EMC specification with separate EMC specifications for Repeaters and IAB.

# References

- R4-2514648; Way Forward for [116bis][111] 6G operation efficiency, CATT

- 3GPP TSG RAN WG2 Handbook (11/2023)