3GPP TSG-RAN WG4 Meeting #117		R4-2522023

Dallas, USA, 17 – 21 November, 2025

Agenda item:		6.12.2.1

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO UEIBM

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

In last RAN4#116bis meeting, RAN4 achieved agreements for most of the test cases in the latest WF [2]. There are FFS for only two test cases. In this contribution, we provide our view of the FFS part.

- Discussion

In last RAN4#116bis meeting, RAN4 agreed to define 5 test cases as below. There are still two FFS on TC4 and TC8 for “with window time based and eventInstanceCount-r19>1”.

Issue 2-1: Test case list and CR split

Agreement:

| TC index | TC name | Company name |
| --- | --- | --- |
| TC1 | Event-2, FR2, Inter-cell SSB based, no window time based, mode-A | Samsung |
| TC2 | Event-2, FR2, CSI-RS based, no window time based, mode-A, with ”QCL-info” for the SSBs with the same cell ID | vivo |
| TC3 | Event-2, FR2, Intra-cell SSB based, no window time based, mode-A | Ericsson |
| FFS on TC4 | Event-2, FR2, FFS Intra-cell SSB or CSI-RS based, with window time based, and eventInstanceCount-r19 > 1 |  |
| TC5 | Event-2, FR1, Inter-cell SSB based, no window time based, mode-A | CMCC |
| TC7 | Event-2, FR1, Intra-cell SSB based, no window time based, mode-A | Nokia |
| FFS on TC8 | Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 |  |

For test case TC4, according to the valid configuration defined in RAN1/RAN2:

eventCountWindow-r19                     SEQUENCE {

eventInstanceCount-r19                   INTEGER (2..16),

eventDetectionTimeWindow-r19             ENUMERATED {ms4, ms5, ms8, ms10, ms16, ms20, ms40, ms80, ms160, ms320, ms640, ms1280}

If eventInstanceCount-19 = 3, for typical SSB periodicity 20ms, the minimum measurement period shall be at least 480ms. For the test in FR2, the total length is a little long.

For test case TC8, if we choose eventInstanceCount-19 = 3, for typical SSB periodicity 20ms, the minimum measurement period shall be at least 60ms.

Proposal 1: For two FFS test cases, RAN4 to only add one test case for TC8.

- Conclusion

In this contribution, we analyze the test cases of Rel-19 MIMO UEIBM and provide our proposals:

Proposal 1: For two FFS test cases, RAN4 to only add one test case for TC8.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2514803, WF on NR_MIMO_Ph5_RRM_Part1, Samsung