3GPP TSG-RAN WG4 Meeting #117		R4-2522027

Dallas, USA, 17 – 21 November, 2025

Agenda item:		6.6.3.2

Source:		Samsung

Title:	Discussion on RRM impacts by using NGSO in the test

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of NTN testing for NGSO are described in [1] as:

The objectives of performance part for enhanced requirements and test methodology for NR and IoT NTN include: NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In RAN4#116bis meeting, there are open issues for NTN tests by using NGSO in performance part. In this contribution, we provide our view of the RRM performance part of NTN testing for NGSO.

- Discussion

In the WF [2], RAN4 achieved the agreements for R19 UL transmit timing test cases but FFS on timing advance tests, shown as below:

#### Issue 2-1-1: R19 UL timing tests and test requirement impact Agreement - Apply the new FR1-NTN channel model for R19 UL transmit timing test cases - No impacts for the test procedure or test requirements; apply the following adaptation - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts” - Clarify that NTA,adjUE in test requirements is calculated based on the generated UL channel with time varying Doppler and delay shifts. - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself. - Do not add new additional margins in the test case. Test tolerance is left for RAN5 - FFS applicability of the timing advance test and its additional margin

In the existing test cases, for NR in 38.133 Annex, there are two series of TCs for NR-NTN:

#### A.14.3.1.1 NR UE Transmit Timing Test for FR1 ### A.14.3.2 Timing advance for satellite access #### A.14.3.2.1 SA FR1 timing advance adjustment accuracy

For the timing advance adjustment, in the test, it verified that the UE adjust the timing of its transmissions with the relative accuracy shall be better than or equal to the UE timing advance adjustment accuracy. In this relative accuracy, there is no timing drifting from T0 to T1. The relative accuracy is at the point when UE adjusts the timing of its transmission.

The impacts by applying NGSO channel model for timing advance tests are the similar as which RAN4 agreed for UL transmit timing test cases.

Proposal #1:

Apply the new FR1-NTN channel model for R19 UL timing advance adjustment accuracy test cases

- No impacts for the test procedure or test requirements; apply the following adaptation

  - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

  - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

- Conclusion

In this contribution, we analyze the impact on RRM performance requirements for NTN testing for NGSO and provide our proposals:

Proposal #1:

Apply the new FR1-NTN channel model for R19 UL timing advance adjustment accuracy test cases

- No impacts for the test procedure or test requirements; apply the following adaptation

  - Change the “Propagation condition” from “AWGN” to “AWGN with time varying Doppler and delay shifts”

  - During the tests, UE position is needed to be added and set by AT command at the beginning of T1 or other suitable method. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

- Reference

[1] RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] R4-2515055, Way Forward for [116bis][310] Rel-19 RRM performance_Part2, MediaTek