3GPP TSG-RAN4 Meeting #117	R4-2522035

# Dallas, United States, 17th - 21st November, 2025

Agenda item:		7.3.3

Source:				Keysight Technologies UK Ltd

Title:					On NTN OTA Testability Aspects for Ka/Ku Bands

Document for:		Approval

# Introduction

This contribution is discussing various testability topics for NTN OTA testing for Ka/Ku Bands. This topic was agreed to be studied within the new Rel-20 WI [1], i.e.,

Objectives for this core part work item are as follows - Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

# Discussion

In the following, the undefined testability aspects for radiated Tx/Rx characteristics from 38.101-5 [7] are highlighted with observations and proposals presented. Some aspects are repeated from [2] as they have not been concluded in RAN4#116bis [3].

### Testability Clarifications to 38.101-5

In [2], various discussions were triggered that resulted in some testability aspects to be captured in the RAN4#116bis WF [3]. Others were concluded implicitly. The following topics, largely discussed and concluded in RAN4#116bis, were taken as a baseline for clarification CRs [5], [6] to [7].

#### OTA Test Condition

While the ‘OTA Test Condition’ topic from [2], see excerpt below, was not explicitly endorsed in the WF [3], the initial TR skeleton [4] only included the IFF methodology, i.e., the Proposal 1 from [2] seems agreeable. The Cat-A and Cat-F CRs [5], [6] to [7] include the necessary far-field radiated interface boundary statements similar to those in [8].

#### OTA Test Condition The applicability of requirements to conducted vs radiated testing is defined in Table 4.2-1 of [7] which clearly indicates that any NTN testing beyond 10 GHz is performed in a radiated environment. Table 4.2-1: Requirement set applicability Requirement; Requirement set. FR1-NTN bands; FR2-NTN bands. Below 10 GHz with antenna connector; Above 10 GHz. Conducted transmitter characteristics; Clause 6; NA; NA. Conducted receiver characteristics; Clause 7; NA; NA. Conducted performance requirements; Clause 8; NA; NA. Radiated transmitter characteristics; NA; Clause 9; Clause 9. Radiated receiver characteristics; NA; Clause 10; Clause 10. Demodulation performance requirements (Radiated requirements); NA; Clause 11; Clause 11.. However, it has not been defined anywhere in [7] that measurements shall be performed in the far field as it was done for FR2, e.g., Clause 3.1 of TS 38.101-2 [8] TRP(Link=TX beam peak direction, Meas=TRP grid): measurement of the TRP of the UE such that the measurement angles are aligned with the directions of the TRP grid points within an acceptable measurement uncertainty while the link angle is aligned with the TX beam peak direction NOTE: For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region. Observation 1 While NTN testing beyond 10 GHz is defined in a radiated environment, the need for far-field conditions has not been specified yet Since the same principles should apply to NTN as to TN testing, it is proposed to include in [7] that the measurements shall be performed with FF conditions. Proposal 1: Include a statement in TS 38.101-5 [7] that for radiated requirements, the radiated interface boundary is associated to the far-field region as stated in TS 38.101-2 [8]

#### Antenna Performance

The ‘Antenna vs end-to-end Performance Testing: Non-Signalling vs Signalling’ topic from [2] was discussed extensively in RAN4#116bis and it was concluded that the antenna performance/gain, defined in Clause 9.6.2 of [7], should be considered a system level requirement and thus non testable given the lack of antenna/RF connectors for all NTN VSAT types.

### Sub-topic 2-12: Antenna gain measurement The antenna gain is not testable from a conformance perspective and should be considered as a system level requirement. Clarification of core spec for this antenna gain topic should be done from Rel-18/19 NTN WIs.

The Cat-A and Cat-F CRs [5], [6] to [7] include the corresponding editor’s note to declare this test case not testable.

#### Pointing Stability

The pointing stability, defined in Clause 9.6.1.2.2 of [7], requires environmental conditions of wind/wind gusts in an OTA test system which are simply not feasible in an OTA test environment.

#### Untestable Requirements in OTA Test System The following requirements are untestable in conventional OTA test systems: - Pointing Stability, see Clause 9.6.1.2.2 [7]: the following environmental conditions (“Under the condition of 100 km/h maximum wind speed, with gusts of 130 km/h lasting 3 seconds”) cannot be emulated in an OTA test system. This requirement would require a separate environmental test environment and is not commonly part of radiated conformance test requirements Proposal 9: Define the pointing stability as a system-level requirement rather than a radiated conformance test requirement as it cannot be tested in conventional OTA test systems

The Cat-A and Cat-F CRs [5], [6] to [7] include the corresponding editor’s note to declare this test case not testable.

#### Vibration

The environmental condition for vibration in Clause E.2.3 of [7] suggests the need to test VSATs for be tested under vibration conditions.

## E.2.3 Vibration The UE shall fulfil all the requirements when vibrated at the following frequency/amplitudes. Table E.2.3-1: Vibration conditions Frequency; ASD (Acceleration Spectral Density) random vibration. 5 Hz to 20 Hz; 0,96 m2/s3. 20 Hz to 500 Hz; 0,96 m2/s3 at 20 Hz, thereafter –3 dB/Octave. Outside the specified frequency range the UE, if powered on, shall not make ineffective use of the radio frequency spectrum. In no case shall the UE exceed the transmitted levels as defined in TS 38.101-1[6] for extreme operation.\

Similar to pointing stability, it is suggested to declare the vibration environmental conditions a system level requirement and thus non testable from a conformance perspective since it is not feasible to emulate vibration conditions in the OTA test environment.

Proposal 1: Consider the environmental condition for vibration in Clause E.2.3 of [7] a system level requirement and thus non testable from a conformance perspective

The Cat-A and Cat-F CRs [5], [6] to [7] have not included the proposed change in Proposal 1 yet as it has not been discussed or proposed previously.

### Polarization

In RAN4#116bis, the topic of polarization was discussed extensively, and the following decisions were captured in the WF [3].

### Sub-topic 2-6: Polarizaion RAN4 start to work on whether and how to support of CP testing in IFF system. Linear polarization is supported as basis based on the WID scope. The core requirements of NTN will be investigated to define the applicability of polarization on the requirements in the next RAN4 meeting.

While some verbal feedback was provided regarding the applicability of polarization (RHCP, LHCP, LP) to certain satellite orbit scenarios, it is requested that VSAT vendors and/or operators clearly clarify which polarization state apply to TX and RX test cases and satellite orbit scenarios so that the required antenna configurations can be studied in detail.

As a starting point, it would be beneficial for VSAT vendors and/or operators to provide feedback which of the polarization combinations in UL/DL from the Table 1 below need to be supported. From a test vendor perspective, it would be very beneficial to limit the number of polarization combinations required in each UL/DL band.

Table 1: Polarization Combinations for VSAT OTA Testing

| Satellite Orbit Scenario | Band | VSAT UL | VSAT DL |
| --- | --- | --- | --- |
| GSO | Ka | - RHCP - LHCP - LP | - LHCP - RHCP - LP |
|  | Ku | - RHCP - LHCP - LP | - LHCP - RHCP - LP |
| NGSO | Ka | - RHCP - LHCP - LP | - LHCP - RHCP - LP |
|  | Ku | - RHCP - LHCP - LP | - LHCP - RHCP - LP |

Proposal 2: VSAT vendors and/or operators to clearly clarify which polarization state (RHCP, LHCP, LP) apply to TX and RX test cases and satellite orbit scenarios so that the required CP&LP antenna configurations can be studied in detail, e.g., by providing feedback on the polarization combinations in Table 1 .

Additionally, it would be very beneficial to get feedback on the prioritization of GSO Ka, GSO Ku, NGSO Ka, and NGSO Ku.

Proposal 3: VSAT vendors and/or operators to provide feedback on prioritization of GSO Ka, GSO Ku, NGSO Ka, and NGSO Ku.

In order to justify the internal CP developments in 2026, it is requested that the ‘baseline’ statement in the WID [1] regarding polarization be modified in the upcoming RAN Plenary meeting. The current statement (“Linear polarization is the baseline”) clearly suggests that despite the intent to “Study whether and how to support circular polarization measurements” linear polarization will be the de-facto measurement approach for NTN VSAT conformance testing.

Proposal 4: Interested parties in CP for NTN OTA testing to update the statement regarding linear polarization being the baseline in the WID in the upcoming RAN Plenary meeting.

Preliminary CP measurements of an in-house designed feed probe integrated in an IFF system designed to support the Ka band were recently performed with the Axial Ratio (AR) results summarized in Table 2. This system was designed with one CP handedness, e.g., RHCP, in the UL and the reverse handedness, e.g., LHCP, in the DL.

Table 2: AR Results for Ka-band IFF System

| Band | Frequency Range | Axial Ratio (Handedness #1) [dB] | Axial Ratio (Handedness #2) [dB] |
| --- | --- | --- | --- |
| Ka UL (SAN receive/ UE transmit) | low | 0.6 |  |
|  | high | 1.6 |  |
| Ka DL (SAN transmit/ UE receive) | low |  | 2.0 |
|  | high |  | 1.9 |

Clearly, these initial results look promising and further improvements, e.g., support for additional frequency bands and polarization states, can be expected over time.

### Need for Beam Steering/Beam Lock Function

Another topic captured in the last WF [3] was the need for beam steering and/or a beam lock function

### Sub-topic 2-10: Beamlock RAN4 further check whether beam steering is needed for VSAT conformance testing.

This topic was initially also discussed in our previous contribution [2] with the excerpt shown below.

#### Automatic Beam Steering/Beam Lock Test Function As discussed earlier, the expectation is that VSATs employ electronic or mechanical beam steering for normal operation. It is not clear whether these VSATs are expected to automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester. Clarification from VSAT vendors/NTN operators is requested. Proposal 7: VSAT vendors/NTN operators to clarify whether VSATs automatically steer the UL&DL beams towards the DL direction inside the OTA test system where the DL direction corresponds to the IFF probe (reflector direction) connected to the DL port of the communication tester. If VSATs indeed support automatic beam steering inside OTA test systems, the test procedures for EIRP/EIS measurements could be closely aligned with those currently used for FR2 UE RF testing. In that case, a test mode would be required for TRP testing to lock the beam in the beam peak direction to avoid the beam continuously changing its UL beam, similar to the UE Beamlock test Function (UBF), defined in Clause 5.4 of [6]. Proposal 8: If VSATs support automatic beam steering towards the DL probe in the OTA test system, a beam lock test function required for TRP measurements must be defined and supported by VSAT vendors. If VSATs do not support automatic beam steering inside OTA test systems, the OTA test system, specifically the SW test executive, will require constant communication with the VSAT to make sure that the UL&DL beam directions are aligned with the measurement probe test direction for EIRP/EIS measurements. In that case, no beam lock test function is required. However, to reduce the development complexities and to optimize test times of OTA test systems, a standardized beam steering interface between VSATs and test system integrators is highly desirable. Observation 3: If VSATs do not support automatic beam steering inside OTA test systems, a standardized beam steering interface between VSATs and test system integrators is highly desirable to reduce the development complexities and to optimize test times

It was commented that the beam peak direction could be assumed the broadside direction or declared by the manufacturer. However, there is no such guidance/allowance currently in [7]. Quite the contrary, the spec clearly suggests the need to verify the Tx beam peak direction, e.g., in Clauses 9.2.1.1 (Minimum requirements for Fixed VSAT) and 9.2.1.2 (Minimum requirements for Mobile VSAT),

The requirement should be verified with test metrics of EIRP (Link=Tx beam peak direction, Meas=Link angle). The peak EIRP of Tx beam peak direction should be verified within the declared minimum elevation angle supported for transmitting. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type.

Similarly, the spec suggests the need to verify the Rx beam peak direction, e.g., Clause 10.3.2 (Minimum requirement)

The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle). The EIS of Rx beam peak direction should be verified within the declared minimum elevation angle supported for receiving. The steered beam peak directions can be achieved by mechanical steering and/or electronic steering according to VSAT Type.

The referenced three clauses each include drawings that clearly visualize and mention a beam peak search measurement grid, e.g.,

Figure 9.2.1.1-1 Example measurement grid for min peak EIRP with the declared supported minimum elevation angle

Given the clear guidance for beam peak searches in [7] for Tx and Rx testing, it is proposed to assume that NTN VSATs need to support beam steering for conformance testing unless the requirements in [7] are relaxed/modified to allow a vendor declaration for the BP direction and/or a fixed measurement direction.

Proposal 5: Assume that NTN VSATs must support beam steering for conformance testing unless the requirements in [7] are relaxed/modified to allow a vendor declaration for the BP direction and/or a fixed measurement direction.

Given the assumption in Proposal 5, Proposal 8 from [2] is applicable and repeated here:

Proposal 6: A beam lock test function required for TRP measurements must be defined and supported by VSAT vendors.

### Worst Case Antenna Assumptions

As discussed in [2], various measurement grids are required for various testing, e.g., beam peak searches, TRP, spurious emissions, etc. To assess the minimum required measurement grid spacing as well as the measurement uncertainty, the worst-case antenna assumptions for each NTN VSAT type are required. The proposals from [2] are thus repeated here:

Proposal 7: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [7] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits.

Proposal 8: Define the worst-case antenna assumptions of the spurs for each of the NTN VSAT Types in Table 9.2.1.0-1 of [7] to assess the minimum required measurement grid spacing as well as the measurement uncertainty for the spurious emissions TRP.

### Special Test System Consideration

#### Localized Cooling

Given the very high transmit power requirements and excessive heating of select components of the VSAT, VSAT vendors are requested to provide feedback whether the OTA test system needs to provide localized cooling directed at dedicated areas of the VSAT during testing, e.g., how much power is expected to be dissipated in terms of heat.

Proposal 9: VSAT vendors are requested to provide feedback whether the OTA test system needs to provide localized cooling directed at dedicated areas of the VSAT during testing, e.g., how much power is expected to be dissipated in terms of heat

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: Consider the environmental condition for vibration in Clause E.2.3 of [7] a system level requirement and thus non testable from a conformance perspective

Proposal 2: VSAT vendors and/or operators to clearly clarify which polarization state (RHCP, LHCP, LP) apply to TX and RX test cases and satellite orbit scenarios so that the required CP&LP antenna configurations can be studied in detail, e.g., by providing feedback on the polarization combinations in Table 1 .

Proposal 3: VSAT vendors and/or operators to provide feedback on prioritization of GSO Ka, GSO Ku, NGSO Ka, and NGSO Ku.

Proposal 4: Interested parties in CP for NTN OTA testing to update the statement regarding linear polarization being the baseline in the WID in the upcoming RAN Plenary meeting.

Proposal 5: Assume that NTN VSATs must support beam steering for conformance testing unless the requirements in [7] are relaxed/modified to allow a vendor declaration for the BP direction and/or a fixed measurement direction.

Proposal 6: A beam lock test function required for TRP measurements must be defined and supported by VSAT vendors.

Proposal 7: VSAT vendors to provide the worst-case antenna assumptions and beam steering assumptions for each of the NTN VSAT Types in Table 9.2.1.0-1 of [7] to assess the minimum required measurement angular grid spacing as well as the measurement uncertainty for BP searches, TRP, and off-axis EIRP emission density limits.

Proposal 8: Define the worst-case antenna assumptions of the spurs for each of the NTN VSAT Types in Table 9.2.1.0-1 of [7] to assess the minimum required measurement grid spacing as well as the measurement uncertainty for the spurious emissions TRP.

Proposal 9: VSAT vendors are requested to provide feedback whether the OTA test system needs to provide localized cooling directed at dedicated areas of the VSAT during testing, e.g., how much power is expected to be dissipated in terms of heat

# References

- RP-252903, Enhancement of UE OTA test method and requirements for NR, RAN VC (vivo), 3GPP TSG RAN Meeting #109, September 2025

- R4-2514328, On NTN OTA Testability Aspects for Ka/Ku Bands, Keysight Technologies, 3GPP TSG-RAN4 Meeting #116-bis, October 2025

- R4-2514820, WF on NR_UE_OTA_Enh, Eutelsat Group, 3GPP TSG-RAN4 Meeting #116bis, October 2025

- R4-2514843, Initial TR skeleton for OTA testing of NR NTN above 10GHz, vivo, 3GPP TSG-RAN4 Meeting #116bis, October 2025

- R4-2522033, Corrections based on NR_UE_OTA_Enh Testability Discussions, Keysight Technologies, 3GPP TSG-RAN4 Meeting #117, November 2025

- R4-2522034, Corrections based on NR_UE_OTA_Enh Testability Discussions, Keysight Technologies, 3GPP TSG-RAN4 Meeting #117, November 2025

- TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements

- TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone