3GPP TSG-RAN WG4 Meeting #117				                           R4-2522042

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Further analysis on coexistence of outdoor AIoT system

Agenda item:	7.11.3

Document for:	Approval

# Background

The simulation assumption on co-existence study of Rel-20 outdoor AIoT deivces has been agreed in RAN4#116bis [1], and in this contribution, we share some analysis based on the initial simulation results we have obtained.

# Study cases on the coexistence scenario for AIoT

In total, 16 coexistence study cases were agreed in RAN4#116bis to be evaluated as shown below in Tabl.1. In this contribution we particularly focus on device C in standalone deployment scenario.

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | device | R2D |
| In-band | Device 2b | 9 | Device | NR UL | D2R |
|  |  | 10 | NR UL | reader | D2R |
|  |  | 11 | Reader | NR DL | R2D |
|  |  | 12 | NR DL | device | R2D |
|  | Device C | 13 | Device | NR UL | D2R |
|  |  | 14 | NR UL | reader | D2R |
|  |  | 15 | Reader | NR DL | R2D |
|  |  | 16 | NR DL | device | R2D |

For the coexistence study cases, the following cases need to be considered under the coexistence study of outdoor AIoT in Rel-20:

| Group | case | direction | Aggressor | victim | notes |
| --- | --- | --- | --- | --- | --- |
| Group 1 | 1, 5, 9, 13 | DL (R2D) | A-IoT reader | NR UE | Derive the AIoT reader ACLR, which can be down-prioritized if it is assumed to be similar to NR BS |
| Group 2 | 2, 6, 10 ,14 | DL (R2D) | NR BS | AIoT device | Derive the AIoT device ACS |
| Group 3 | 3, 7, 11, 15 | UL (D2R) | AIoT device | NR BS | Derive the AIoT device ACLR |
| Group 4 | 4, 8, 12, 16 | UL (D2R) | NR UE | A-IoT reader | Derive the AIoT reader ACS |

Group 1 are to be studied to derive the required ACLR of AIoT readers. For AIoT reader, it can be assumed that the ACLR performance would be similar to a NR BS since it should be able to co-site with NR BS and it also transmits with an OFDM waveform (e.g., OOK waveform generated by OFDM modulator). Therefore, this case can be down prioritized since it will be basically the same as intra NR system coexistence and the coexistence can be guaranteed by defining the SEM and ACLR requirements for AIoT reader to be the same as NR BS.

Observation 1: it can be assumed the ACLR performance of AIoT reader is similar to NR BS.

Proposal 1: down prioritize the co-existence on AIoT reader as aggressor to NR UE.

Group 2 is about investigating the AIoT device ACS requirements. Although, before we consider the inter-system interference between NR and AIoT system, it is necessary to first verify the performance of intra system for AIoT. Our initial simulation results showed in Fig. 1, where the simulation assumption was based on the agreed WF in [2].

Figure. 1. The CDF of SINR for R2D without inter system interference.

Although no clear conclusion on the threshold SINR yet for decoding, but without loss of generality, above 10 dB SINR can be expected. Therefore, around 50% of AIoT devices would be below SINR threshold with such a assumption. Therefore, it is necessary to consider some mechanism to improve the intra system interference of R2D signals, e.g., TDM or FDM between different readers and/or solution to reduce the SINR decoding threshold needs to be specified.

Observation 2: assuming the R2D SINR decoding threshold in the range about 10 dB, around 50% AIoT devices would be below such a threshold.

Proposal 2: RAN 4 shall study mechanisms to reduce the intra system interference, e.g., TDM or FDM between different readers.

Group 3 is mainly focus on deriving the AIoT device ACLR, to minimize its impact to NR system. It is proposed to consider the 5% TP loss of NR system as the performance metric to evaluate the needed ACLR level of AIoT devices.

Proposal 3: considering the 5% TP loss of NR system as the performance metric to evaluate the needed ACLR level of AIoT devices.

Finally, for the simulation case in group 4, which aims to derive AIoT device ACS requirement. Similar to group 2, we also observed that even without the inter system interference, the achievable SINR is relatively low considering the potential SINR decode threshold, as shown in Fig. 2. Therefore necessary enhancement on improving the intra system SINR performance is needed.

Figure. 2. The CDF of SINR for D2R without inter system interference.

Proposal 4: RAN4 shall study the mechanisms to improve the intra system SINR performance of D2R link

# Consideration on the RF assumption of AIoT devices

## ACS/ACLR modelling in co-existence study

How to model the ACS and ACLR for inter system interference still remains open. The agreed spectrum allocations for co-existence simulation for UL&D2R and DL &R2D are illustrated in Fig. 3.

Figure. 3. Spectrum allocation assumption for NR and AIoT system in the co-existence study

Based on this assumption, the corresponding ACLR and ACS model can be assumed based on TR36.942, where examples on how a ACLR and ACS is model are quoted as in Fig. 4 and Fig.5:

Figure. 4. Examples on ACLR model of a aggressor UE with larger bandwidth than victim UE in 36.942

Figure. 5. Examples on DL ACS model in 36.942

Therefore, it is proposed to adopt the following table for ACLR and ACS modelling for NR BS and UE.

|  | ACLR | ACS |
| --- | --- | --- |
| NR UE | ACLR 1 = 30 dB + 10*log10 (3.3MHz/15 kHz) ACLR 2 = 46 dB10*log10 (3.3MHz/15 kHz) | 33 dB |
| NR BS | 45 dB+10*log10 (3.3MHz/180 kHz) | 46 dB |

On the other hand, for the active AIoT devices, still multiple parameters/factors remain unclear, for example, the final waveform design and filter assumptions. Therefore, for sake of simplicity, it is proposed to adopt a flat model for AIoT device when perform the co-existence evaluation to derive the required ACLR and ACS level. How the final requirements of ACLR and ACS to be specified can be left to normative work phase.

Proposal 5: for NR BS and UE, adopt the following model for ACS and ACLR:

|  | ACLR | ACS |
| --- | --- | --- |
| NR UE | ACLR 1 = 30 dB + 10*log10 (3.3MHz/15 kHz) ACLR 2 = 46 dB10*log10 (3.3MHz/15 kHz) | 33 dB |
| NR BS | 45 dB+10*log10 (3.3MHz/180 kHz) | 46 dB |

Proposal 6: for AIoT device, it is proposed to assume a flat model in the co-existence study, while further discuss how the actual ACS and ACLR requirement would be specified during the normative phase stage.

## CFO/SFO

In RAN1#122bis meeting, the CFO/SFO assumption for device C and device 2B has been agreed:

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | 50 ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | 10 ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | 50 ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | 10 ppm |

Taking device C as a example, 10 ppm means 9 kHz frequency shift at 900 MHz which is significantly considering the transmission BW of AIoT device is around 15 kHz only. RAN4 needs to study how to accommodate such a large CFO for outdoor AIoT devices

Observation 3: very large CFO is assumed in RAN1 for both device 2b and device C.

Proposal 7: RAN4 shall study how to handle such a large CFO for outdoor AIoT devices.

# Conclusion

In this contribution, we share our analysis based on some initial co-existence simulation results, where the following observations and proposals are given:

Observation 1: it can be assumed the ACLR performance of AIoT reader is similar to NR BS.

Observation 2: assuming the R2D SINR decoding threshold in the range about 10 dB, around 50% AIoT devices would be below such a threshold.

Observation 3: very large CFO is assumed in RAN1 for both device 2b and device C.

Proposal 1: down prioritize the co-existence on AIoT reader as aggressor to NR UE.

Proposal 2: RAN 4 shall study mechanisms to reduce the intra system interference, e.g., TDM or FDM between different readers.

Proposal 3: considering the 5% TP loss of NR system as the performance metric to evaluate the needed ACLR level of AIoT devices.

Proposal 4: RAN4 shall study the mechanisms to improve the intra system SINR performance of D2R link

Proposal 5: for NR BS and UE, adopt the following model for ACS and ACLR:

Proposal 6: for AIoT device, it is proposed to assume a flat model in the co-existence study, while further discuss how the actual ACS and ACLR requirement would be specified during the normative phase stage.

Proposal 7: RAN4 shall study how to handle such a large CFO for outdoor AIoT devices.

# References

- R4-2515152		Way Forward for [116bis][321] Study of A-IoT for outdoor	LGE