3GPP TSG-RAN WG4 Meeting #117				                             R4-2522049

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	views on general and UE RF of 6GR

Agenda item:	8.3

Document for:	Approval

# Background

In RAN4#116, the general and UE RF of 6G was discussed [1], and a WF was agreed [2]. In this contribution, we share our further views on the general and UE RF discussion based on the agreed WF.

# UE reference architecture

For UE reference architecture, the following agreements were made in RAN4#116bis:

- Strive to agree UE reference architecture on case-by-case manner for features being studied. Discuss reference architecture together with the feature itself as necessary.(first 6G agreement in RAN4)

  - Base evaluations on agreed reference architecture, while resulting requirements should apply for any feasible implementation architecture as much as possible

- If no reference architecture is agreed, recommend companies to mention assumed architecture together with evaluation results for easier comparisons between inputs

In this section, we discuss the reference architecture, including antenna and RF front end, particularly focused on the antenna architecture of handheld devices in FR3 (around 7 GHz and 15 GHz) and RF front end of massive IoT devices.

## Antenna reference architecture at “FR3” for handheld devices

One of the main differences between 5G NR and 6GR is the adoption of spectrum between FR1 and FR2, which is commonly referred to as “FR3” for now. As a transition spectrum from sub-6 GHz to mmWave, the antenna architecture, especially on the UE side, has not been agreed upon. Currently, in FR1, multiple single antennas are integrated to achieve high spatial diversity. On the other hand, multiple antenna arrays are used in FR2, mainly to target a high gain to mitigate the high pathloss in mmWave.

For the lower end of FR3, e.g., around 7GHz, it can be seen as an extension of FR1, and thus it is natural to consider the same antenna architecture around 7GHz as in upper 6GHz as in 5G NR. Currently, 4Rx is mandated as minimum numbers in 5G NR at upper 6GHz, considering the physical limitation of handheld devices, we expect similar number of antennas to be required in 6GR.

Observation 1: Frequencies around 7 GHz can be seen as an extension of FR1, where similar antenna reference architecture as in FR1 can be expected.

Proposal 1: FR1-like antenna architecture with 4 antennas can be assumed for handheld devices around 7GHz.

At around 15 GHz, the device architecture can be debated even further. For devices like smartphones, considering the form factor limitation, it will be very difficult to fit any array structure. Therefore, multiple single-antenna systems, as in FR1, are a more promising antenna architecture to consider. Regarding the number of antennas, it needs to be balanced between the implementation limits in the UE and the system performance. From the UE aspect, it needs to be considered that existing antennas, at least for the current FR1 bands, still need to be supported in 6GR. A schematic drawing of a typical antenna topology in FR1 today is shown on Fig. 1. It can be observed that adding additional antennas is very challenging due to the limited space. The possibility of adding more antennas for FR3 can be achieved by creating more resonant structures in higher frequencies on existing antennas or reusing some existing structures in the mobile phone as antennas.

Observation 2: The space in a smartphone for placing additional antennas around 15 GHz is extremely limited; therefore, reusing the physical structure of the existing antenna around 15 GHz might be necessary.

Fig. 1: a typical antenna topology for a smartphone in FR1

Meanwhile, from the system perspective, based on the investigation captured in Section 6.3.1 of TR38.820, the typical rank of the channel at 15 GHz is in the range of about 2-4 under an indoor environment as shown in Fig. 2. In general, it can be expected that the rank of the channel will be reduced with increased frequencies, due to the usage of higher beamforming gain on the BS side and also the increased shadowing losses in the channel. Therefore, from a system perspective, beyond 4 UE antennas may not provide much additional gain.

Observation 3: The achievable rank around 15 GHz would be in the range of 2-4 for an indoor environment, implying that a UE with 4 antennas can almost exploit the full rank of the channel, and the system performance with more than 4 UE antennas could be limited.

Fig .2: The measurement of channel rank with the UE antennas prototype at 15 GHz (source: TR38.820)

Therefore, considering the factors above, it is proposed to four antennas in FR3 with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones.

Proposal 2: It is proposed to consider four antennas around 15 GHz with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones, as the starting point.

## SAW-less HD-FDD on massive IoT front-end assumptions

From the duplex aspect, HD-FDD is the most important mode for 6G massive IoT devices, which offers improved power efficiency, reduced device complexity, and enhanced global scalability by eliminating band- or regional-specific SAW/BAW duplexers/filters from the RF front end of the devices.

However, based on the lessons learned from 5G NR (e)Redcap implementation, the full advantages of HD-FDD are not realised since band-specific SAW filters would still be required to fulfil the band-to-band protections (-50 dBm), especially for selected bands in sub 1GHz where the offset between the aggressor band and victim band are small (below 12 MHz-ish). Therefore, achieving a true SAW-less HD-FDD design is a key consideration for reducing RF domain complexity in 6G massive IoT, which can offer:

- A true single stock-keeping unit (SKU) design for global operations is possible, allowing for true economies of scale.

- Additionally, a fully SAW-less HD-FDD design can achieve a reduced chip footprint and a smaller IoT module.

- Last, but not least, the insertion loss on Tx and Rx chains can be reduced, and the power efficiency of the system can be enhanced.

Observation 4: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, with improved efficiency in Tx/Rx and a reduced chip footprint/module size.

The design of a true SAW-less device is illustrated in Fig. 3. A similar design principle has been widely adopted in 4G IoT commercial production, e.g., cat-M1 and NB-IoT, thanks to 4G IoT's even smaller CBW. However, it is not the case in 5G NR as larger BW needs to be supported, even for (e)Redcap devices, which is one of the factors that creates a significant implementation feasibility difference between 4G IoT and 5G (e)Redcap devices in terms of SAW-less aspect.

Proposal 3: 6G massive IoT design shall target a truly SAW-less design as baseline.

Fig. 3. Comparison of massive IoT device with SAW filter/duplexer design vs a fully SAW-less design.

The impact on the standard to achieve a true SAW-less implementation is still pending on the basic system parameters required for 6G massive IoT devices. For example, if the device only needs to support a small bandwidth comparable to legacy devices, e.g., 3MHz, it can be designed naturally as a true SAW-less design without any further impact. If a large bandwidth needs to be supported, especially in the uplink direction, other approaches, e.g., limited uplink RB length, allowed power backoff for certain bands and/or certain RB allocations, may need to be considered instead. We summarize the illustrate solutions in appendix of the paper for information. Overall, we see that the impact is very limited and would not jeopardize the overall 6GR design, especially for 6GR broadband services.

Observation 5: The impact to enable a true SAW-less design is very limited and will not jeopardize the overall 6GR design.

Observation 6: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while power backoff or restriction in UL transmission may be needed if larger uplink bandwidth needs to be supported (see Appendix A for detail discussion).

Proposal 4: Depending on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less.

# Single carrier output power requirements

Among the lessons learned from 5G NR, UE behavior in terms of actual output power and the power control framework is one aspect that has been debated frequently in RAN4. Multiple factors in the power domain that a UE applies in the field are unknown to the network, for example, the A-MPR value. These factors may cause more complications and a more conservative design of the network scheduler. Therefore, RAN4 may need to collaborate with RAN1 to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

Proposal 5: RAN4 may need to collaborate with RAN1 in 6GR to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

In addition, SAR/MPE issue particularly concerns the potential new spectrum around 7 GHz and around 15 GHz needs to be investigated. In ICNIPR, the absorbed power density (APD) is adopted as the corresponding basic restrictions rather than SAR or incident power density (IPD) in above 6GHz. The study in [3] shows that very limited output power, e.g., around 15 dBm accepted power into antenna connector can be allowed based on some simple antenna models under 100% duty cycle, due to APD limitation in ICNIRP. Therefore, it implies that either power backoff or duty cycle-based mechanism would still be needed in 6GR for the potential new bands between FR1 and FR2-1.

Observation 7: Very limited output power level is observed around 7GHz and 15 GHz with 100% duty cycle due to the MPE (APD) limitation in ICNIRP.

Proposal 6: RAN4 shall investigate the SAR/MPE solution around 7GHz and 15 GHz with consideration both power backoff and duty cycle based on approach, with target also to achieve a more clear UE behavior in terms of output power than 5G NR.

The HPUE framework is also something that warrants further consideration in 6GR. Currently, RAN4 defines requirements on a case-by-case basis to cover all possible implementations for HPUE, such as 1Tx, 2Tx …. or even multiple TX with different output power. In our view, such an HPUE framework should be reconsidered entirely in the 6GR design. For example, RAN4 can define unified requirements for each power class, which are agnostic to the UE Tx implementation. The feasibility of such an approach needs to be studied in RAN4.

Proposal 7: RAN4 shall study whether it is feasible to define a unified requirement for each power class that is agnostic to the UE Tx implementation, thereby simplifying the HPUE requirement framework.

# Tx and Rx requirements

In general, one of the key aspects in 6GR when considering the RF core requirements including both Tx and Rx is to accommodate diverse device types. From RAN4 perspective, for a given device type, a set of features with corresponding minimum required values needs to be agreed e.g., the number of antennas, and etc. Then, RAN4 can define the minimum requirement for a given feature for a given device type based on the assumption.

## Tx requirements

The Tx requirements should be based on with the applicable ITU-R recommendations, which makes RAN4 specifications more credible outside 3GPP. Local regulatory requirements could be handled based on existing mechanisms for NR, which does not preclude improvements for 6GR. However, the general Tx/emission requirements should be global.

Observation 8: algin the 3GPP requirements with ITU-R increase the credibility of 3GPP requirements globally.

Proposal 8: define the general Tx/emission requirements based on ITU-R recommendations, while the local regulators can be taken into account case by case.

Therefore, similar to 5G NR, it is proposed to define the spurious emission limits in line with SM.329. Particularly, the stricter Category B limits defined in SM.329 shall be applied for UE considering that UEs are intended for global circulation. Meanwhile, the OOB boundary to be line with ITU-R SM.1541 with BW+ 5MHz.

Proposal 9: Defining the OOB boundary to be line with ITU-R SM.1541 with BW+ 5MHz, while the spurious emission limits in line with SM.329 Cat. B.

## Rx requirements

The MSD framework for CA is one area that can be improved in the 6GR design as well. Currently, RAN4 spends a significant amount of time defining the MSD for every band combination; however, the fact is that these values are usually far from the actual implementations. In 6GR, to make MSD useful for the network, a more dynamic MSD reporting mechanism could be studied. Whether to still define any MSD minimum requirements can also be further discussed.

Proposal 10: RAN4 shall study improving the MSD requirement framework with possibilities to support a dynamic MSD reporting mechanism, making the feature useful for the network.

Moreover, to ensure the requirements to be more meaningful for the field operation, OTA requirements can be considered for MSD

Proposal 11: RAN4 to study the OTA requirement for MSD in 6GR if feasible.

## Other considerations

One issue was particularly debated during 5G NR time was that if the core requirement is coupled to certain test mode, e.g., if beam lock mode. In our view, the RAN4 core requirement shall be defined in such a way that it is independent of test method, and it should be met under real UE behavior in the field. The actual test method and configuration is up to RAN5 discussion with understanding that the applicable test method fulfill certain test tolerance requirements.

Proposal 12: For the core requirement defined in 6GR, RAN4 shall make it clear from the beginning that the core requirement itself is not coupled to certain test mode but rather shall be met under actual UE behavior in the field.

# EVM relaxation

EVM relaxation is one of the items that has been agreed upon in RAN#109 to be studied under the 6G track but can possibly be applied to 5G NR if feasible [5]. The aim of EVM relaxation is to allow UE to transmit on each modulation order with relaxed EVM to lower the required MPR. It is worth pointing out that such a scheme is mostly effective with high-order modulation schemes, e.g., 256QAM and 1024QAM, as the UE MPR is dominated by EVM rather than emission level. Considering this study affect both 5GA and 6G track, it is proposed that RAN4 may consider 256 QAM as starting point to carry out the study.

Observation 9: EVM relaxation is effective in reducing MPR for high-order modulations.

Proposal 11: it is proposed that RAN4 focus on 256QAM as starting point to carry out the study.

From the UE perspective, the necessary EVM relaxation needs to be investigated with a certain MPR reduction value. Considering that such relaxation would inevitably place a greater burden on network decoding, it must guarantee a sufficient MPR reduction gain from the UE side. Therefore, RAN4 needs to identify how much EVM relaxation could result in a sufficiently high-performance gain in terms of MPR first. In our view, at least a 1.5 dB MPR reduction shall be ensured; otherwise, it may not be meaningful to consider any EVM relaxation.

From the BS side, how to demodulate the uplink data with a relaxed EVM level needs to be studied as well. It is agreed that RAN4 will study both AI-based and non-AI-based solutions. Considering the overall AI framework of 6GR is still in its very early stage, it is proposed that RAN4 focuses on non-AI-based solutions at this stage. It is necessary to clarify how BS can decode the data with a relaxed EVM value so that proper requirements and tests can be specified.

Proposal 12: Based on the agreed PA model, RAN4 shall consider the following two aspects when determining the feasibility of EVM relaxation:

1) the EVM relaxation level needed to achieve a meaningful MPR reduction, e.g., larger than 1.5 dB based on 5G NR UE PA model as starting point.

2)  The feasibility from BS side how to demod with the corresponding relaxed EVM level

Supposing it is feasible to support a relaxed EVM for a given modulation order, RAN4 may consider defining the minimum requirements for EVM in 6GR from the beginning, aiming for a more relaxed value than 5G NR, to enable better uplink coverage for high order modulations.

Proposal 13: If approved as feasible, RAN4 may consider relaxing the EVM requirement for higher-order modulation schemes in 6GR compared to 5G NR.

# LP-WUR

The low-power wake-up signal and receiver (LP-WUS and LR) were specified in 5G NR Rel-19. Energy efficiency and power saving are considered key motivations for the 6GR design, and it is proposed that the LP-WUS design and LR be considered from day-1 of 6GR, making it more likely to be deployed in the field. In addition, in our view, the usage scenario and the corresponding power saving gain of 5G NR might be too limited, to make the feature more attractive for UE to support it, we propose to study if more RRM measurement can be offloaded to 6G LR design, to allow more usage scenarios of such a power saving feature.

Proposal 14: 6G to support LP-WUS and LR from day-1, with consideration of more usage scenario, e.g., RRM measurement being offloaded to 6G LR design, to increase the power saving gain in real life.

# Conclusion

In this contribution, we share our initial views on selected topics regarding general and UE RF aspect in  6GR study

Observation 1: Frequencies around 7 GHz can be seen as an extension of FR1, where similar antenna reference architecture as in FR1 can be expected.

Observation 2: The space in a smartphone for placing additional antennas around 15 GHz is extremely limited; therefore, reusing the physical structure of the existing antenna around 15 GHz might be necessary.

Observation 3: The achievable rank around 15 GHz would be in the range of 2-4 for an indoor environment, implying that a UE with 4 antennas can almost exploit the full rank of the channel, and the system performance with more than 4 UE antennas could be limited.

Observation 4: a true SAW-less HD-FDD designed massive IoT device can enable a true single stock-keeping unit (SKU) design for global operations, with improved efficiency in Tx/Rx and a reduced chip footprint/module size.

Observation 5: The impact to enable a true SAW-less design is very limited and will not jeopardize the overall 6GR design.

Observation 6: with a limited uplink bandwidth or RB length, e.g., 3MHz, the 6G massive IoT devices can be implemented as SAW-less natively, while power backoff or restriction in UL transmission may be needed if larger uplink bandwidth needs to be supported (see Appendix A for detail discussion).

Observation 7: Very limited output power level is observed around 7GHz and 15 GHz with 100% duty cycle due to the MPE (APD) limitation in ICNIRP.

Observation 8: algin the 3GPP requirements with ITU-R increase the credibility of 3GPP requirements globally.

Observation 9: EVM relaxation is effective in reducing MPR for high-order modulations.

Proposal 1: FR1-like antenna architecture with 4 antennas can be assumed for handheld devices around 7GHz.

Proposal 2: It is proposed to consider four antennas around 15 GHz with an FR1-like antenna architecture for a broadband device with a limited form factor, e.g., smartphones, as the starting point.

Proposal 3: 6G massive IoT design shall target a truly SAW-less design as baseline.

Proposal 4: Depending on the system parameters choice, RAN4 shall study the most proper approach to address the design of true SAW-less.

Proposal 5: RAN4 may need to collaborate with RAN1 in 6GR to develop an improved power control framework, with clearer UE behavior defined in terms of actual output power.

Proposal 6: RAN4 shall investigate the SAR/MPE solution around 7GHz and 15 GHz with consideration both power backoff and duty cycle based on approach, with target also to achieve a more clear UE behavior in terms of output power than 5G NR.

Proposal 7: RAN4 shall study whether it is feasible to define a unified requirement for each power class that is agnostic to the UE Tx implementation, thereby simplifying the HPUE requirement framework.

Proposal 8: define the minimum requirements based on ITU-R recommendations, while the local regulators can be taken into account case by case.

Proposal 9: Defining the OOB boundary to be line with ITU-R SM.1541 with BW+ 5MHz, while the spurious emission limits in line with SM.329 Cat. B.

Proposal 10: RAN4 shall study improving the MSD requirement framework with possibilities to support a dynamic MSD reporting mechanism, making the feature useful for the network.

Proposal 11: RAN4 to study the OTA requirement for MSD in 6GR.

Proposal 12: For the core requirement defined in 6GR, RAN4 shall make it clear from the beginning that the core requirement itself is not coupled to certain test mode but rather shall be met under actual UE behavior in the field.

Proposal 11: it is proposed that RAN4 focus on 256QAM as starting point to carry out the study.

Proposal 12: Based on the agreed PA model, RAN4 shall consider the following two aspects when determining the feasibility of EVM relaxation:

1) the EVM relaxation level needed to achieve a meaningful MPR reduction, e.g., larger than 1.5 dB based on 5G NR UE PA model as starting point.

2)  The feasibility from BS side how to demod with the corresponding relaxed EVM level

Proposal 13: If approved as feasible, RAN4 may consider relaxing the EVM requirement for higher-order modulation schemes in 6GR compared to 5G NR.

Proposal 14: 6G to support LP-WUS and LR from day-1, with consideration of more usage scenario, e.g., RRM measurement being offloaded to 6G LR design, to increase the power saving gain in real life.

# References

- R4-2514509		Topic Summary for [116bis][102] 6G general RF and UE RF	Moderator (Qualcomm)

- R4-2514642		WF on [116bis][102] 6G general RF and UE RF	Qualcomm

- D5.5 Final design of enabling technologies for 6G devices and infrastructure   Hexa-x-II

- RP-252912		Revised SID: Study on 6G Radio		NTT DOCOMO, CMCC, AT&T, Vodafone.

- RP-252952 	  	New WID: UE RF enhancements for NR FR1, Phase 5	Moderator (vivo)

# Appendix A. Potential solutions to enable a SAW-less implementation on 6G massive IoT

In this appendix, we take an example band pair band 13 UL emission fall into the protected band 14 to illustrate the possible solutions to enable a SAW-less implementation of 6G massive IoT:

- Reference case: presume a 5MHz 6G massive IoT device in UL direction, the IMD7 production fall into the protected band 14 when the UE is configured to transmit by the lower edge of band 13.

- Solution 1: Don’t configure the UE near the lower edge of band 13 to avoid the IMD 7 production fall into the band 14.

- Solution 2a: reduce the RF UL BW of 6G massive IoT to 3MHz so that the IMD7 production is moved out of the protected band.

- Solution 2b: reduce the BB UL BW of 6G massive IoT to 3MHz so that the IMD7 production is moved out of the protected band.

- Solution 3: allow power backoff when the UE is configured on the lower edge of band 13.