3GPP TSG-RAN WG4 Meeting #117 	R4-2522069

Dallas, US, Nov 17th – 21st, 2025

Agenda Item: 6.18.2

Source: MediaTek inc.

Title: Discussion on RRM performance requirements for XR

Document for: Discussion

# Discussion on performance part

In this contribution, we provide further discussion on the performance part issues captured in the WF [1].

## Discussion on Test case list

### Issue 1-1-5: List of test cases <Agreement> - Define test case for: - Inter-frequency measurements procedure with gaps - Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1 - FFS whether to define at least one accuracy test case in FR1 and in FR2-1

Issue 1-1-5: Scenarios for the definition of the test cases:

Accuracy is not affected by MG cancellation; therefore, there is no valid reason to request such a test case. Thus, it is unnecessary to define a test case for accuracy requirements.

- RAN4 RRM not to define test cases for accuracy requirements for NR-SA, and FR1 and FR2-1.

## Discussion on test case configuration

### Issue 1-2-1: SSB time index detection <Way forward> - Proposal 1: For measurement procedure, define test cases without SSB time index detection. - Proposal 2: Use SSB index detection for intra-/inter-frequency measurement TCs, use no SSB index detection with lower priority for intra-/inter-frequency measurement TCs.

Issue 1-2-1: SSB time index detection:

For measurement procedure, define test cases without SSB time index detection.

- For measurement procedure, define test cases without SSB time index detection.

## Discussion on MG cancellation model

### Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps <Way forward> Proposals for discussion - Option 1: Measurement gap cancelation is modelled as a fixed pattern for the test cases. - Option 2: Measurement gap cancelation is determined randomly by the test equipment. ### Issue 2-3-3: Cancelation ratio <Way forward> Proposals for discussion - Proposal 1: Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests. - Proposal 2: For Type 1 MGs test cases: - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

Issue 1-3-1: Cancelation model for XR test cases: conditions for Type 1 gaps and Type 2 gaps:

On the topic of which measurement gaps to be cancelled during test cases, measurement gap cancelation is determined randomly by the test equipment.

- The measurement gap cancelation is determined randomly by the test equipment.

Issue 2-4-3: Cancelation ratio:

On the topic of which measurement gaps to be cancelled during test cases, ratio of gap occasions to be cancelled during the measurement period is 20% in the tests.

- The ratio of gap occasions to be cancelled during the measurement period is 20% in the tests.

## Discussion on DCI aspects

### Issue 1-4-1: Toffset between DCI and start of MG <Way forward> - Option 1: The processing time 3ms/5ms shall be verified in the test cases. - Option 1a: Define a single test case that includes time offset of 5ms and 3ms with UE capability used as a condition - Option 2: Define test case for 5ms offset only. ### Issue 1-4-2: Types of DCI with cancellation indication <Way forward> - Option 1: In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation. - Option 2: Define RAN4 test cases using DCI 1-1 for the canceling indication. ### Issue 1-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’ <Way forward> - Proposal 1: RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. - Proposal 2: RAN4 to agree that UE behavior of option 1 and option 2 for indication value ‘0’ should be tested. Further details are as follows: - Test Option 2 in every test case. - For testing Option 1, the same method as for Option 2 can be defined but the UE configuration may be changed depending on the UE capability.

Issue 1-4-1: Toffset between DCI and start of MG:

Toffset between DCI and start of MG, 5ms offset is sufficient for defining test cases,

- RAN4 RRM define a single test case that include time offset of 5ms.

Issue 1-4-2: Types of DCI with cancellation indication:

Both DCI 0-1 and DCI 1-1 are mandatory to be used. Some test cases can be define with DCI 0-1 and other TCs with DCI 1-1 without duplicating the test cases.

- RAN4 RRM define some test cases can be define with DCI 0-1 and other TCs with DCI 1-1 without duplicating the test cases.

Issue 1-4-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’:

For indication of value ‘0’, there is no need to complicate the test cases with such indication.

- RAN4 RRM not to define test cases to cover indication of value ‘0’.

# Summary

In this contribution, we have the following proposals:

Proposal 1: RAN4 RRM not to define test cases for accuracy requirements for NR-SA, and FR1 and FR2-1.

Proposal 2: For measurement procedure, define test cases without SSB time index detection.

Proposal 3: The measurement gap cancelation is determined randomly by the test equipment.

Proposal 4: The ratio of gap occasions to be cancelled during the measurement period is 20% in the tests.

Proposal 5: RAN4 RRM define a single test case that include time offset of 5ms.

Proposal 6: RAN4 RRM define some test cases can be define with DCI 0-1 and other TCs with DCI 1-1 without duplicating the test cases.

Proposal 7: RAN4 RRM not to define test cases to cover indication of value ‘0’.

# References

- R4-2514869, “WF on NR_XR_Ph3_RRM”, Nokia, Prague, CZ, Oct, 2025.