3GPP TSG-RAN WG4 Meeting # 117	R4-2522078

# Dallas TX, USA,  2025-11-17 - 2025-11-21

Source:		Ericsson

Title:			NTN Ku bands – SAN conformance testing aspects

Agenda item:		5.10.4

Document for:	Discussion

# Introduction

In last RAN4#116 meeting, RAN4 finalized the core aspects of the NTN Ku-bands WI ([1]).

In this contribution we present several aspects related to the SAN conformance testing, in addition to the ones already presented in [2].

# Discussion

## General

With introduction of Ku-bands we have mixed the FR1-NTN and FR2-NTN frequency ranges with impact on the formulation of several requirements.

For SAN downlink both FR1-NTN and FR2-NTN have been extended with the same 10,7 – 12,75 GHz frequency range:

Table 5.2-1: Satellite operating bands in FR1-NTN

| Satellite operating band1 | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n252 | 2000 MHz – 2020 MHz | 2180 MHz – 2200 MHz | FDD |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127502,3 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127502,3 MHz | FDD |
| NOTE 1: Satellite bands are numbered in descending order from n256. NOTE 2: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 3: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT |  |  |  |

Table 5.2-2: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n5121 | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n5112 | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n5103 | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 12750 MHz4,5 | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz4,5 | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

As a consequence the definitions of frequency ranges has been modified accordingly:

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands within this frequency range are regarded as a FR1 band when references from other specifications. NOTE 2: NTN bands within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Due to the overlapping part in FR1-NTN and FR2-NTN the correspondence to traditional Type 1-O and 2-O is affected, one of the implications being that a number of requirements were defined for a new category: SAN Type 1-O operating above 10 GHz.

For Ku-band we need to introduce NTN-FR2-like requirements in FR1-NTN clauses. Current test conditions, procedures, requirements have a clear split between 1-O and 2-O, but this needs to change as now we have: “1-O below 10GHz”,  “1-O above 10GHz” and “2-O”.

Manufacturer declarations table needs an update, splitting the SAN Type 1-O in the sub-types corresponding to above and below 10 GHz.

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O Below 10 GHz | SAN type 1-O Above 10 GHz | SAN type 2-O |

Test clauses experience a similar modification. If we look at the example of ACS, we have 2 types of SAN Type 1-O and we reuse 2-O manufacturer declaration together with the new requirements for “1-O above 10 GHz”.

10.5.1.2        Minimum requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 10.5.1.2.

For SAN type 2-O, the minimum requirement is in TS 38.108 [2], clause 10.5.1.3.

10.5.1.3        Test purpose

The test purpose is to verify the ability of the SAN receiver filter to suppress interfering signals in the channels adjacent to the wanted channel.

10.5.1.4        Method of test

10.5.1.4.1            Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier:

-    M; see clause 4.9.1.

SAN RF Bandwidth edge position to be tested for multi-carrier :

-    MRFBW in single-band operation, see clause 4.9.1;

Directions to be tested:

-    For SAN type 1-O, receiver target reference direction (D.26),

-    For SAN type 2-O, OTA REFSENS receiver target reference direction (D.44),

10.5.1.4.2            Procedure

1)  Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.4.2.

2)  Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)  Align the SAN with the test antenna in the declared direction to be tested.

4)  Align the SAN so that the wanted signal and interferer signal is polarization matched with the test antenna(s).

5)  Configure the beam peak direction for the transmitter according to the declared reference beam direction pair for the appropriate beam identifier.

6)  For FDD operation, set the SAN to transmit beam(s) of the same operational band as the OSDD or OTA REFSENS RoAoA being tested according to the appropriate test configuration in clauses 4.7 and 4.8.

7)  Set the test signal mean power so that the calibrated radiated power at the SAN Antenna Array coordinate system reference point is as follows:

a)  Set the signal generator for the wanted signal to transmit as specified in table 10.5.1.5.2-1 for SAN type 1-O and table 10.5.1.5.3-1 for SAN type 2-O.

b)  Set the signal generator for the interfering signal at the adjacent channel frequency of the wanted signal to transmit as specified in table 10.5.1.5.2-1 for SAN type 1-O and table 10.5.1.5.3-1 for SAN type 2-O.

8)  Measure throughput according to annex A.1 as well as annex A.14 of TS 36.181 [23] for a SAN declared to be capable of NB-IoT operation in NTN NR in-band (D.60) for each supported polarization, for multi-carrier operation the throughput shall be measured for relevant carriers specified by the test configuration specified in clauses 4.7.2 and 4.8.

10.5.1.5        Test requirement

10.5.1.5.1            General

The test requirement is calculated from the OTA wanted signal mean power level offset by the OTA ACS Test Tolerance specified in clause 4.1.

10.5.1.5.2            Test requirements for SAN type 1-O

10.5.1.5.3            Test requirements for SAN type 2-O

# Observations and proposals

Observation 1: SAN Type 1-O need a split in two types: “1-O operating above 10 GHz” and “1 -O operating below 10 GHz”. Use of these new terms will affect the readability of the specification, as they need to be utilized in many places.

Proposal 1: Find a shorter name for the new SAN Type 1-O sub-types. Suggestion is to use SAN Type 1-OL and 1-OH, (from Low and High part of the band) together with a proper definition.

Proposal 2: Decide on a work-split for implementation of changes related to Ku-Band in the SAN Conformance Testing Specifications

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2514113 NTN Ku bands – SAN conformance testing aspect, Ericsson, Thales