3GPP TSG-RAN4 Meeting # 117	R4-2522091

# Dallas, USA, 17th Nov – 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftbigCR | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft big CR on demodulation performance requirements for NR LP-WUS/WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No demodulation performance requirements and related applicabiltiy rules currently exist in the specification for NR LP-WUS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced demodulation performance requirements for WUS by merging the following endorsed draft CRs: R4-2522701 - WUS demodulation requirements for FR2 FDD (Huawei) R4-2522702 - RMC for WUS demodulation requirements (Ericsson) R4-2522703 - WUS demodulation requirements for FR1 FDD (Samsung) R4-2522704 - Applicability rules for WUS demodulation requirements (Apple) R4-2522705 - Common test parameters for WUS demodulation requirements in FR1 and FR2 (Apple) R4-2522794 - WUS demodulation requirements for FR1 TDD (Nokia) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Demodulation performance requirements for NR LP-WUS will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.1.3, 7.1.1.3, 5.X (new clause), 7.X (new clause), A3.X (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

========= Start of Change 1 (R4-2522703) ==========

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.

DL BWP: DL bandwidth part as defined in TS 38.213 [11].

EN-DC: E-UTRA-NR Dual Connectivity as defined in clause 4.1.2 of TS 37.340 [13].

Enhanced Receiver Type 1: SU-MIMO interference mitigation advanced receiver [14]

-	R-ML (reduced complexity ML) receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas

-	R-ML (reduced complexity ML) receiver with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas

Enhanced Receiver Type 2: MU-MIMO interference mitigation advanced receiver [14]

-	Enhanced Receiver Type 2-1: R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is explicitly signaled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212 [10].

-	Enhanced Receiver Type 2-2a: R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled.

-	Enhanced Receiver Type 2-2b: R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX when the co-scheduled UE information with DCI index 6 in Table 7.3.1.2.2-12 of TS38.212 [10] is signalled.

FR1: Frequency range 1 as defined in clause 5.1 of TS 38.101-3 [8].

FR2: Frequency range 2 as defined in clause 5.1 of TS 38.101-3 [8].

RedCap: A UE with reduced capabilities as defined in clause 4.2 in TS 38.306 [14].

eRedCap: A UE with reduced peak data rate, with or without reduced baseband bandwidth in FR1 as defined in clause 4.2.22 in TS 38.306 [14].

Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

SSB: SS/PBCH block as defined in clause 7.8.3 of TS 38.211 [9].

WUS:	Wake-up signal as defined in clause 7.4.4 of TS 38.211 [9].

========= End of Change 1 (R4-2522703) =========

========= Start of Change 2 (R4-2522704) ==========

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  | The requirements apply only when maxNumberTCI-states-r16 = 2. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1-1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 1-3 in clause 5.3.2.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1-1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.3.2.2.1. |  |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1-1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 1-3 in clause 5.3.3.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1-1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.3.3.2.1. |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |  |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |  |
| 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-8) Clause 5.2.3.1.1 (Test 1-8) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-12) Clause 5.2.3.2.1 (Test 1-12) |  |  |
|  |  | SDR | Clause 5.5.1 Clause 5.5A.1 | 1024QAM MCS indexes are used only if UE supports 1024QAM for FR1 DL. |  |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.18 Clause 5.2.3.1.17 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.19 Clause 5.2.3.2.18 |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |  |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.20 Clause 5.2.3.1.19 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.21 Clause 5.2.3.2.20 |  |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.21 Clause 5.2.3.1.20 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.22 Clause 5.2.3.2.21 |  |  |
| Support for PDCCH with intra-slot repetition (mTRP-PDCCH-Repetition-r17) | FR1 FDD | PDCCH | Clause 5.3.2.1.5 Clause 5.3.3.1.4 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.5 Clause 5.3.3.2.4 |  |  |
| Support for TDD-TDD intra-band non-colocated NR-CA deployment (intraBandNR-CA-non-collocated-r18) | FR1 TDD | PDSCH | Clause 5.2A.2.6 | The requirements apply on in case the UE indicates support of 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) |  |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML), when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212 [10]. (advReceiver-MU-MIMO-r18) | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-1) Clause 5.2.3.1.16 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.1.16 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.1.16. If the Test 3-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.1.16. If the Test 4-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.1.16. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-1) Clause 5.2.3.2.17 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.2.17 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.2.17. If the Test 3-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.2.17. If the Test 4-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.2.17. |  |
| Support for MU-MIMO Interference Mitigation advanced receiver with modulation order detection or MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh in 38.306 [14] | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) | Test 4-2 is only applicable for UE supporting ‘MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh’ in TS38.306 [14]. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |  |
| Baseline 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 1-1,3-1,5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 1-1,3-1,5-1) |  |  |
| Simplified 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 2-1,4-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 2-1,4-1) |  |  |
| Support for enhanced DMRS (pdsch-TypeA-DMRS-r18) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 2-3) Clause 5.2.3.1.1 (Test 4-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 2-3) Clause 5.2.3.2.1 (Test 4-2) |  |  |
| Reception of NR PDCCH candidates overlapping with LTE CRS REs (nr-PDCCH-OverlapLTE-CRS-RE-r18) | FR1 FDD | PDCCH | Clause 5.3.2.1.6 Clause 5.3.3.1.5 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.6 Clause 5.3.3.2.5 |  |  |
| Support for Dedicated Spectrum of Less than 5MHz ( support3MHz-ChannelBW-Symmetric-r18) | FR1 FDD | PDCCH | Clause 5.3.2.1.7 (Test 1-1) Clause 5.3.3.1.6 (Test 1-1) |  |  |
|  |  | PBCH | Clause 5.4.2.1 (Test 3-1) Clause 5.4.3.1 (Test 3-1) |  |  |
| Support DM-RS pattern for DL transmission with 2 symbols front-loaded DM-RS with one additional 2 symbols DM-RS (twoFL-DMRS-TwoAdditionalDMRS-DL) | FR1 FDD | PDSCH | Clause 5.2.4.1.1 (Tests 5-1) Clause 5.2A.4.1 | The Rank 8 requirements apply in case the UE indicates support of DM-RS pattern for DL transmission with 2 symbols front-loaded DM-RS with one additional 2 symbols DM-RS (twoFL-DMRS-TwoAdditionalDMRS-DL) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1 (Tests 5-1) Clause 5.2A.4.1 | The Rank 8 requirements apply in case the UE indicates support of DM-RS pattern for DL transmission with 2 symbols front-loaded DM-RS with one additional 2 symbols DM-RS (twoFL-DMRS-TwoAdditionalDMRS-DL) |  |
|  |  | SDR | Clause 5.5A.1 | The Rank 8 requirements apply in case the UE indicates support of DM-RS pattern for DL transmission with 2 symbols front-loaded DM-RS with one additional 2 symbols DM-RS (twoFL-DMRS-TwoAdditionalDMRS-DL) |  |
| Supports of LP-WUS operation in IDLE/INACTIVE mode (lpwus-OOK-r19) or CONNECTED mode (lpwus-OOK-Connected-r19) based on OOK signal | FR1 FDD | LP-WUS | Clause 5.X.1.1.1 |  |  |
|  | FR1 TDD | LP-WUS | Clause 5.X.2.1.1 |  |  |
| Support of LP-WUS operation in IDLE/INACTIVE mode (lpwus-OFDM-r19) or CONNECTED mode (lpwus-OFDM-Connected-r19) based on OFDM overlaid sequence. | FR1 FDD | LP-WUS | Clause 5.X.1.1.1 |  |  |
|  | FR1 TDD | LP-WUS | Clause 5.X.2.1.1 |  |  |

========= End of Change 2 (R4-2522704) ==========

========= Start of Change 3 (R4-2522705) ==========

## 5.X	WUS demodulation requirements

The receiver characteristics of the WUS are determined by the probability of miss-detection of WUS (Pm-wus).

The parameters specified in Table 5.X-1 are valid for all LP-WUS detection tests unless otherwise stated.

Table 5.X-1: Common test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Common serving cell parameters | Active DL BWP index |  | 1 |
|  | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
| LP-SS Configuration | OOK Waveform |  | OOK-4 |
|  | LP-SS OOK Chip Rate (MLP-SS) |  | 4 |
|  | Binary Sequence Length |  | [32] |
|  | BW | subcarriers | 132 |
|  | Periodicity | ms | [160] |
|  | Offset | ms | 0 |
| LP-WUS Configuration | OOK Waveform |  | [OOK-4] |
|  | Number of OOK symbols in an OFDM symbol (MWUS) |  | 4 |
|  | Number of overlaid sequences (Nseq) |  | 4 |
|  | Number of root sequence numbers (Nroot) |  | [1] |
|  | Root sequence number (q) |  | 1 |
|  | Periodicity | ms | 320 |
|  | EPRE ratio between WUS and SSB | dB | 0 |

========= End of Change 3 (R4-2522705) ==========

========= Start of Change 4 (R4-2522703) ==========

The receiver characteristics of the WUS are determined by the probability of miss-detection of WUS (Pm-wus).

### 5.X.1	1Rx Requirements

### 5.X.1.1	FDD

### 5.X.1.1.1	Minimum requirements

The performance requirements are specified in Table 5.X.1.1.1-2 and Table 5.X.1.1.1-3 with the addition of test parameters in Table 5.X.1.1.1-1.

Table 5.X.1.1.1-1: Test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | FDD |
| Subcarrier spacing | kHz | 15 |
| Bandwidth | MHz | 10 |
| Number of information bits (K) | Bits | Test 1-1 and 2-1: 2 Test 1-2 and 2-2: 5 |
| Number of bits after rate matching (E0) | Bits | Test 1-1 and 2-1: 12 Test 1-2 and 2-2: 32 |
| [Note 1: The Pm-wus is derived with the assumption of false-alarm rate of WUS (FAR-wus) set as 1%.] |  |  |

Table 5.X.1.1.1-2: Minimum performance requirements for OOK-based envelope detection

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-wus (%) | SNR (dB) |
| 1-1 | R.WUS.1-1.1 | 10 / 15 | TDLA30-10 | 1x1 | 1 | [TBD] |
| 1-2 | R.WUS.1-1.2 | 10 / 15 | TDLA30-10 | 1x1 | 1 | [TBD] |

Table 5.X.1.1.1-3: Minimum performance requirements for OFDM-based sequence detection

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-wus (%) | SNR (dB) |
| 2-1 | R.WUS.1-1.1 | 10 / 15 | TDLA30-10 | 1x1 | 1 | [TBD] |
| 2-2 | R.WUS.1-1.2 | 10 / 15 | TDLA30-10 | 1x1 | 1 | [TBD] |

========= End of Change 4 (R4-2522703) ==========

========= Start of Change 5 (R4-2522794) ==========

### 5.X.2.1 TDD

##### 5.X.2.1.1 Minimum requirements

The performance requirements are specified in Table 5.X.2.1.1-2 and Table 5.X.2.1.1-3, with additional test parameters in Table 5.X.2.1.1-1.

Table 5.X.2.1.1-1: Test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Subcarrier spacing | kHz | 30 |
| Bandwidth | MHz | 40 |
| [Number of information bits (K) | Bits | Test 1-1 and 2-1: 2 Test 1-2 and 2-2: 5 |
| Number of bits after rate matching (E0) | Bits | Test 1-1 and 2-1: 12 Test 1-2 and 2-2: 32 |
| [Note 1: The Pm-wus is derived with the assumption of false-alarm rate of WUS (FAR-wus) set as 1%.] |  |  |

Table 5.X.2.1.1-2: Minimum performance requirements for OOK-based envelope detection

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-wus (%) | SNR (dB) |
| 1-1 | R.WUS.1-2.1 | 40 / 30 | TDLA30-10 | 1x1 | 1 | TBD |
| 1-2 | R.WUS.1-2.2 | 40 / 30 | TDLA30-10 | 1x1 | 1 | TBD |

Table 5.X.2.1.1-3: Minimum performance requirements for OFDM-based sequence detection

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-wus (%) | SNR (dB) |
| 2-1 | R.WUS.1-2.1 | 40 / 30 | TDLA30-10 | 1x1 | 1 | TBD |
| 2-2 | R.WUS.1-2.2 | 40 / 30 | TDLA30-10 | 1x1 | 1 | TBD |

========= End of Change 5 (R4-2522794) ==========

========= Start of Change 6 (R4-2522704) ==========

#### 7.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 7.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 7.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 3-1) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 7.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR2 TDD | PDSCH | Clause 7.2.2.2.2 |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR2-1 TDD | PDCCH | Clause 7.3.2.2.3 | If the Test 3-1 in Clause 7.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 7.3.2.2.1. |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | PDSCH | Clause 7.2.2.2.1 (Test 1-4) |  |  |
| 256QAM for PDSCH (pdsch-256QAM-FR2) | FR2-1 TDD | SDR | Clause 7.5A.1 | For UE capable of pdsch-256QAM-FR2 for certain band(s), mcs-Table is configured to ‘64QAM’ for SDR test. |  |
| Support of FR2 HST operation [(FR2 UE power class PC6 signalling is used to indicate support of feature group)] | FR2-1 TDD | PDSCH | [Clause 7.2.2.2.4] |  |  |
| Support of Single Carrier operations with 120kHz SCS for FR2-2 (initialAccessSSB-120kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-1, 4-2, 4-3, 4-4) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-1, 1a-2, 1a-3) (Table 7.3.2.2.2-2, Test 3-1, 3-2) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 3) |  |  |
| Support of 480kHz SCS for FR2-2 (ul-FR2-2-SCS-480kHz-r17 and initialAccessSSB-480kHz-r17) | FR2-2 TDD | PDSCH | Clause 7.2.2.2.1 (Table 7.2.2.2.1-6: Test 4-5, 4-6) |  |  |
|  |  | PDCCH | Clause 7.3.2.2 (Table 7.3.2.2.1-2: Test 1a-4) (Table 7.3.2.2.2-2, Test 3-3) |  |  |
|  |  | PBCH | Clause 7.4.2.2 (Table 7.4.2.2-2: Test 4) |  |  |
| Support simultaneous reception with different QCL Type-D RSs (simultaneousReceptionDiffTypeD-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 Clause 7.2.2.2.6 Clause 7.2.2.2.7 |  |  |
| Single DCI based SDM transmission for simultaneous reception support (singleDCI-SDM-scheme-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 |  |  |
| Multi DCI based simultaneous reception non-overlapping support (multiDCI-MultiTRP-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.5 |  |  |
| Multi DCI based simultaneous reception fully-overlapping support (overlapPDSCHsFullyFreqTime-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.6 |  |  |
| Support of 2-port DL PTRS (supportTwoPortDL-PTRS-r16) | FR2 TDD | PDSCH | Clause 7.2.2.2.7 Test 1-2 |  |  |
| Support of FR2 HST operation (FR2 UE power class PC6 signalling is used to indicate support of feature group) with simultaneous multiRX reception | FR2-1 TDD | PDSCH | Clause 7.2.2.2.8 | FR2 HST UE should support the following optional capabilities - simultaneousReceptionTwoQCL-r18; - multiDCI-MultiTRP-r16; - overlapPDSCHsFullyFreqTime-r16; Additionally, the UE should report maxNumberActiveTCI-PerBWP > 1 |  |
| Support of LP-WUS operation in IDLE/INACTIVE mode (lpwus-OFDM-r19) or CONNECTED mode (lpwus-OFDM-Connected-r19) based on OFDM overlaid sequence. | FR2 TDD | LP-WUS | Clause 7.7.2.1.1 |  |  |

========= End of Change 6 (R4-2522704) ==========

========= Start of Change 7 (R4-2522705) ==========

## 7.X	LP-WUS demodulation requirements

The receiver characteristics of the WUS are determined by the probability of miss-detection of WUS (Pm-wus).

The parameters specified in Table 7.X-1 are valid for all LP-WUS detection tests unless otherwise stated.

Table 7.X-1: Common test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Common serving cell parameters | Active DL BWP index |  | 1 |
|  | Physical Cell ID |  | 0 |
|  | SSB position in burst |  | First SSB in Slot #0 |
|  | SSB periodicity | ms | 20 |
| LP-SS Configuration | OOK Waveform |  | OOK-1 |
|  | LP-SS OOK Chip Rate (MLP-SS) |  | 1 |
|  | Binary Sequence Length |  | 8 |
|  | BW | subcarriers | 132 |
|  | Periodicity | ms | [160] |
|  | Offset | ms | 0 |
| LP-WUS Configuration | OOK Waveform |  | OOK- |
|  | Number of OOK symbols in an OFDM symbol (MWUS) |  | 1 |
|  | Number of overlaid sequences (Nseq) |  | 2 |
|  | Number of root sequence numbers (Nroot) |  | 1 |
|  | Root sequence number (q) |  | 1 |
|  | Periodicity | ms | 320 |
|  | EPRE ratio between WUS and SSB | dB | 0 |

========= End of Change 7 (R4-2522705) ==========

========= Start of Change 8 (R4-2522701) ==========

### 7.X.1	1Rx requirements

#### 7.X.1.1	FDD

(Void)

#### 7.X.2.1	TDD

##### 7.X.2.1.1	Minimum requirements

The performance requirements are specified in Table 7.X.2.1.1-2 with the additional test parameters in Table 7.X.2.1.1-1.

Table 7.X.2.1.1-1: Test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Subcarrier spacing | kHz | 120 |
| Bandwidth | MHz | 100 |
| [Number of information bits (K) | Bits | Test 1-1: 2 Test 1-2: 5 |
| Number of bits after rate matching (E0) | Bits | Test 1-1: 12 Test 1-2: 32 |
| [Note 1: The Pm-wus is derived with the assumption of false-alarm rate of WUS (FAR-wus) set as 1%.] |  |  |

Table 7.X.2.1.1-2: Minimum performance requirements for OFDM-based sequence detection

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-wus (%) | SNR (dB) |
| 1-1 | R.WUS.2-1.1 | 100 / 120 | TDLA30-75 | 1x1 | 1 | TBD |
| 1-2 | R.WUS.2-1.2 | 100 / 120 | TDLA30-75 | 1x1 | 1 | TBD |

========= End of Change 8 (R4-2522701) ==========

========= Start of Change 9 (R4-2522702) ==========

## A.3.X	Reference measurement channels for WUS demodulation requirements

### A.3.X.1	Reference measurement channels for FR1

Table A.3.X.1-1: Wake-up Signal Reference Channel for FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.1-1.1 | R.WUS.1-1.2 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

Table A.3.X.1-2: Wake-up Signal Reference Channel for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.1-2.1 | R.WUS.1-2.2 |
| Subcarrier spacing | kHz | 30 | 30 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

### A.3.X.2	Reference measurement channels for FR2

Table A.3.X.2-1: Wake-up Signal Reference Channel

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.WUS.2-1.1 | R.WUS.2-1.2 |
| Subcarrier spacing | kHz | 120 | 120 |
| Number of bits after Rate matching (E0) | Bits | 12 | 32 |
| Number of bits after Line coding (G0) | Bits | 24 | 64 |
| Wake-up information length (K) | Bits | 2 | 5 |

========= End of Change (R4-2522702) ==========