3GPP TSG-RAN WG4 Meeting #117	R4-2522101

Dallas, USA, November 17th – November 21st, 2025

Source:	Nokia

Title:	Discussion on maintenance of XR ph3 RRM core requirements

Agenda item:	4.24.1

Document for:	Discussion

Introduction

This contribution provides Nokia’s views on open issues after RAN4#116bis on RRM core requirements for XR enhancements [1].

Discussion

Agreement from RAN4 #116 R4-2512144

### Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility <Agreement> Condition; TL1-RSRP_Measurement_Period_SSB_Inter. No DRX; Max(Treport, Ceil((M+Lcancel) * Kgap) Max(MGRP, SSB period) CSSFinter). DRX cycle ≤ 320 ms; Max(Treport, Ceil((M+Lcancel) 1.5 * Kgap) Max(MGRP, SSB period, DRX cycle) CSSFinter). DRX cycle > 320 ms; Ceil((M+Lcancel) * Kgap) DRX cycle CSSFinter.

The agreement above regarding LTM measurement delay was only captured for FR1 requirements. For FR2 requirements, the beam sweeping scaling factor needs to be considered as well.

In the agreed CR, the formula introduced for FR2 includes multiplication of the number of cancelled measurement gap occasions by the mean sweeping factor, which extends the measurement delay in an exaggerated manner. Therefore, we propose correcting this formula in a similar way as L3 measurements in FR2, as

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter |

Formulas for measurement delay extension of L1/L2 triggered mobility in FR2-1 are

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter |

Conclusion

The following Observations and Proposals were made:

Proposal 1: Formulas for measurement delay extension of L1/L2 triggered mobility in FR2-1 are

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter |

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2514869, WF on RRM requirements for NR_XR_Ph3, RAN4#116bis, Nokia

- 3GPP TS 38.133, Requirements for support of radio resource management (Release 19)

- R4-2416864, WF on RRM requirements for NR_XR_Ph3, RAN4#112-bis, Nokia

- R4-2420103, WF on RRM requirements for NR_XR_Ph3, RAN4#113, Nokia

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, RAN4#114-bis, Nokia

- R4-2508285, WF on RRM requirements for NR_XR_Ph3, RAN4#115, Nokia

- R4-2512144, WF on RRM requirements for NR_XR_Ph3, RAN4#116, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, RAN4#114bis, Nokia