3GPP TSG-RAN WG4 Meeting #117	R4-2522121

Dallas, US, November 17 – 21, 2025

Agenda Item:	6.16.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM performance requirements for LP-WUS

Document for:	Discussion

# Introduction

In this paper, we further discuss the remaining issues related to RRM perf part for R19 LPWUS and provide our view. The achieved agreements in the last meeting are captured in the WF [1].

# Discussion

In the last meeting RAN4 agreed on the following issues.

Issue 3-2-3: Test case design for entry/exit conditions for case 1/case 3 Ad hoc meeting agreement: RAN4 to define at least test cases for exit conditions. FFS on whether to define test cases for evaluation of entry conditions. Issue 3-2-3-1: Detail on cases to be verified for entry/exit conditions for case 1/case 3 Agreement: - Define test cases for evaluation of exit conditions for the following cases. - UE exit Case 1 to legacy - TC1: UE exit Case 1 to legacy where only LP-SS signal is used in the test case for OOK based LR - UE exit Case 3 to legacy - TC2: UE exit Case 3 to legacy where only PSS/SSS signal is used in the test case for OFDM based LR - TC3: UE exit Case 3 to legacy where only LP-SS signal is used in the test case for OOK based LR - The above agreement can be revisited if there no feasible way for test case design. Issue 3-2-5: Test case design for offloading case Agreement: Cell reselection is triggered immediately at the timepoint X when exit from offloading (case 1) to legacy is triggered plus MR wake up period. Cell reselection is triggered immediately at the timepoint X when exit from RRM relaxation (case 3) to legacy is triggered [plus MR wake up period].

For the FFS in Issue 3-2-3, there is no need to test the evaluation of entry requirements since the exit requirements will be tested and verified. The test case can be defined such that before the test starts UE should have entered Case 1 or Case3, and UE exit at T1, then at T2 cell reselection is performed.

Proposal 1: The test case can be defined such that before the test starts UE should have entered Case 1 or Case3, then UE exit at T1, and at T2 cell reselection is performed.

In addition, regarding the MR wake up period, RAN4 to use 500ms period in the test for the MR wake up period.

Proposal 2: RAN4 to discuss the value of MR wake up period used in test. RAN4 can assume 500ms.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: The test case can be defined such that before the test starts UE should have entered Case 1 or Case3, then UE exit at T1, and at T2 cell reselection is performed.

Proposal 2: RAN4 to discuss the value of MR wake up period used in test. RAN4 can assume 500ms.

# References

- R4-2512139, WF on RRM requirements for NR_LPWUS, vivo.