3GPP TSG-RAN WG4 Meeting #117                                                                              R4-2522140

Dallas, USA, Nov 17 – 21, 2025

Agenda Item:	10.2.1

Source: 	Huawei, HiSilicon

Title:	On the requirements of modified MPR behavior

Document for:	Approval

# 1	Introduction

As per the WF [1] from last meeting, we continue to discuss how to improve the definition of modifiedMPR-Behavior.

# 2	Discussion

Regarding which MPR/AMPR requirements should apply, the following consensus was achieved [1]:

- For the open release in which the MPR/AMPR requirements are modified, and also for the later releases, the new MPR/AMPR requirements are applied. For releases earlier than this modification, the new MPR/AMPR requirements may be applied depending on UE indication.

  - For example, if MPR requirement is changed/updated for R19 specification before Rel-19 ASN.1 freeze, the modifiedMPR-Behaviour bit “shall” be set to 1 for R19 UE. For early releases, the modifiedMPR-Behaviour bits “may” be set to 1.

- Some exceptions can be discussed case by case and no impact to on-going product development is expected.

The principle as described in the main bullet is reasonable, however, the wording to implement it is still debatable.

It has been pointed out [2] that the use of “shall be set to 1” in an open release is not only redundant but also problematic for future changes.

Use band n1 as an example. The current requirements are defined as follows.

Rel -18 (v18.11.1)

| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | This bit may be set to 1 by a UE supporting this release of the specification. |
| --- | --- | --- | --- |

Rel-19 (v19.3.1)

| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | - This bit shall be set to 1 by a UE supporting this version of the specification. |
| --- | --- | --- | --- |

Figure 1: Current definition of modifiedMPR for band n1

Hypothetically, if the AMPR for NS_05 and NS_05U is updated again in Rel-20, how to indicate the supported AMPR version, especially for a Rel-19 UE?

One possible solution is shown below.

Rel -18 (v18.a.b)

| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | This bit may be set to 1 by a UE supporting this release of the specification. |
| --- | --- | --- | --- |
|  | 1 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v20.e.f | - This bit may be set to 1 by a UE supporting this version of the specification. |

Rel-19 (v19.c.d)

| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | - This bit may be set to 1 by a UE supporting this version of the specification. |
| --- | --- | --- | --- |
|  | 1 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v20.e.f and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v20.e.f | - This bit may be set to 1 by a UE supporting this version of the specification. |

Rel-20 (v20.e.f)

| n1 | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v19.3.0 and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v19.3.0 | - This bit shall be set to 0 by a UE supporting this version of the specification. |
| --- | --- | --- | --- |
|  | 0 (leftmost bit) | Requirements for network signalling values NS_05 and NS_05U as defined in Clause 6.5.3.3.4 of 38.101-1 v20.e.f and the A-MPR as defined in clause 6.2.3.4 of 38.101-1 v20.e.f | - This bit shall be set to 1 by a UE supporting this version of the specification. |

Figure 2: Hypothetical changes to the definition of modifiedMPR for band n1 if it’s updated again in Rel-20

Another solution is to utilize the release version information reported by the UE in accessStratumRelease IE. By default, a UE supports the AMPR requirements defined in the reported release. Only when it supports the AMPR defined in a later release, the UE indicates the bit in modifiedMPR IE.

- RAN4 to discuss how to handle the case when the AMPR requirement is changed more than once over different releases. Potential solutions include:

Alt-1: use modifiedMPR IE alone;

Alt-2: combine accessStratumRelease IE and  modifiedMPR IE

Regarding the spec version number in the definition column of the table, several options are listed in the WF [1]. Our preference is to remove the version numbers as per the approach for TS 38.101-2. This approach is clear and forward compatible. For example, when Rel-20 spec inherits the table from Rel-19, no changes are needed to update the spec version numbers.

- For the spec version number, we prefer to follow the approach for TS 38.101-2, i.e. remove vx.y.z, only keep Rel-x information for the sake of forward compatibility.

Regarding the problem for band n41, the following fix can be considered.

However, the change of “may be set to 1” to “shall be set to 1” could be viewed as NBC. To avoid possible disruption to UE product development, it’s suggested to apply the changes from Rel-18.

- RAN4 to discuss the fix for band n41, and consider to apply the potential changes from Rel-18 to minimize the impact of NBC.

# 3	Conclusion

Based on our analysis on the current modifiedMPR-Behavior table in existing NR specs, we have made the following observations and proposals:

Proposal 1: RAN4 to discuss how to handle the case when the AMPR requirement is changed more than once over different releases. Potential solutions include:

Alt-1: use modifiedMPR IE alone;

Alt-2: combine accessStratumRelease IE and  modifiedMPR IE

Proposal 2: For the spec version number, we prefer to follow the approach for TS 38.101-2, i.e. remove vx.y.z, only keep Rel-x information for the sake of forward compatibility.

Proposal 3: RAN4 to discuss the fix for band n41, and consider to apply the potential changes from Rel-18 to minimize the impact of NBC.

# References

- R4-2511776 WF on Modified MPR Behavior, Xiaomi, MediaTek, OPPO, RAN4#116

- R4-2507723 (PHS_System_NRonly) On the requirements of modifiedMPR-Behavior, Huawei, HiSilicon, RAN4#115

- R4-2504558 (PHS_System_NRonly) On the requirements of modifiedMPR-Behavior, Huawei, HiSilicon, RAN4#114bis

- R4-2502236 On the requirements of modified MPR behavior, Huawei, HiSilicon, RAN4#114