3GPP RAN WG4 Meeting #117																	R4-2522145

Dallas, TX, USA, 17th – 21st November 2025

Agenda item:	8.4.3

Source:	CableLabs

Title:	Considerations on 6G coexistence studies

WI/SI:	6G RF and coexistence

Release:	Rel-20

Document for:	Approval

# 1	Introduction

The study item on 6G was approved in RAN#108 [1] and the WG 6G discussions started in RAN4#116bis meeting [2]. The WF was approved in RAN4#116bis meeting [2], including the 6G coexistence studies, as summarized below.

Issue 2-1-1: Coexistence studies Way Forward: List of candidate frequencies: 700 MHz, 2 GHz, 4 GHz, 7 GHz, 15 GHz and 30 GHz. For each exemplary frequency, companies proposing to redo coexistence studies should: - Identify the key assumptions differences (comparing to the assumptions used when RAN4 did the coex study e.g. the work done in TR 38.921 for 6.425-7.125GHz - Identify the potential impacts on the conclusions of previous coex studies (e.g. how much ACLR/ACS difference is expected). - Identify the next steps (update existing requirements? New requirements/new type? Regulation impacts?).

This contribution presents our considerations on 6G adjacent channel coexistence studies.

# 2	Considerations on coexistence study

Some companies have expressed interest in initiating the 6G coexistence study around 7 GHz band. Due to the higher propagation loss near 7 GHz compared to low- and mid-bands, the coverage area may be limited. Although a large active antenna system (AAS) could theoretically compensate for the high propagation loss, its narrow beamwidth introduces additional challenges, such as degraded performance in non-line-of-sight (NLOS) conditions and stringent requirement on beam tracking to align the narrow beams toward UE. Consequently, large AAS deployments near 7 GHz may encounter issues similar to those observed in the 28 GHz or 39 GHz mmWave bands in 5G.

From a field operation perspective, the 7 GHz band is therefore considered also suitable for urban hotspot deployments. We propose to include the urban micro (UMi) scenario, which was already been defined in TR 38.922 [3] and is being considered by ITU-R WP 5D [4]. The specific simulation parameters should follow the urban micro scenario definitions in [3] and [4].

Proposal 1: Considering the high propagation loss, cost-control constraints on large AAS, and the limited performance under NLOS condition, from an operator perspective, the urban hotspot scenario is suitable for the 7 GHz band. We therefore propose to add the urban micro (hotspot) scenario as a priority at around 7 GHz band in the 6G coexistence study. The detailed simulation parameters should leverage [3] and [4] as baselines.

The previous RAN4 coexistence studies assumed the entire channel bandwidth (CBW) is utilized for uplink (UL). In practice, UL transmissions typically occupy only a portion of the CBW, such as a limited number of physical resource blocks (PRBs). This assumption may underestimate the UL power spectral density (e.g., in dBm/MHz), and consequently underestimates the UL coverage and interference power density generated by UE.

Proposal 2: To accurately evaluate UL power density, coverage, and interference, we propose that RAN4 study an appropriate UL transmission BW in 6G coexistence studies that is smaller than the entire channel BW.

The adjacent channel leakage ratio (ACLR) and adjacent channel selectivity (ACS) specifications defined in TS 38.104 and 38.101 were introduced more than a decade ago. Most BS and UE vendor companies have claimed that their products demonstrate significantly better performance than the ACLR and ACS requirements specified in TS 38.104 and 38.101. In previous RAN4 coexistence studies, the use of realistic ACLR and ACS values has been a recurring topic of discussion. As identified in [2], we concur with the agreement that ACLR and ACS should be redefined for 6G.

Proposal 3. We concur with the agreement in [2] that RAN4 reevaluate the ACLR and ACS requirements in 6G for both BS and UE.

The ACLR and ACS concepts are primarily applicable when the victim and adjacent-channel networks operate with symmetrical channel bandwidths. In practice, however, the victim and adjacent-channel network often use different channel bandwidths. A bandwidth scaling method for ACLR and ACS was presented in TR 38.858 Annex E [5]. However, several companies have expressed concerns regarding this approach. Therefore, it is proposed that the 6G coexistence study should re-examine the ACLR and ACS values to ensure their applicability for asymmetric bandwidths between victim and adjacent-channel networks.

Proposal 4. We propose that RAN4 re-examine the ACLR and ACS definitions and scaling methods to ensure their applicability for asymmetric channel bandwidths between victim and adjacent-channel networks in 6G coexistence studies.

# 4	Conclusion

The key observations and proposals of this contribution include:

Proposal 1: Considering the high propagation loss, cost-control constraints on large AAS, and the limited performance under NLOS condition, from an operator perspective, the urban hotspot scenario is suitable for the 7 GHz band. We therefore propose to add the urban micro (hotspot) scenario as a priority at around 7 GHz band in the 6G coexistence study. The detailed simulation parameters should leverage [3] and [4] as baselines.

Proposal 2: To accurately evaluate UL power density, coverage, and interference, we propose that RAN4 study an appropriate UL transmission BW in 6G coexistence studies that is smaller than the entire channel BW.

Proposal 3. We concur with the agreement in [2] that RAN4 reevaluate the ACLR and ACS requirements in 6G for both BS and UE.

Proposal 4. We propose that RAN4 re-examine the ACLR and ACS definitions and scaling methods to ensure their applicability for asymmetric channel bandwidths between victim and adjacent-channel networks in 6G coexistence studies.

# References

- RP-251881, "Study on 6G Radio," NTT DOCOMO (Moderator), RAN#108, Prague, CZ, June, 2025.

- R4-2514638, "WF on 6G BS RF and co-existence," Feature Lead (Ericsson), RAN4#116bis, Prague, CZ, Oct. 2025.

- 3GPP TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz,” V19.1.0, June 2025.

- R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27,” RAN4#112, Maastricht, Netherlands, Aug. 2024.

- 3GPP TR 38.858, “Study on Evolution of NR Duplex Operation,” V18.1.0, March 2024.