3GPP TSG RAN WG4 #117	R4-2522166

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.6

Source: 	Ericsson

Title:	Discussion on 6G RRM requirements

Document for:	Discussion

- Introduction

In this contribution, we provide an overview of potential directions for procedure enhancements and the user experience improvements in 6G RRM.

# General

# Two-level RRM core requirements

In NR, many RRM requirements (e.g., measurement delay requirements) are too easy for UEs to meet, while at the same time it is important to define the requirements, e.g., their side conditions, so that network deployments and network configurations do not become too limited for individual features but also provide enough flexibility for the basic legacy technology. For 6G, to address the above observation and improve the UE and network performance also under typical operational conditions, it can be practically useful to introduce a more stringent requirement level for more typical radio conditions. For example, a UE would be expected to meet both levels of a core requirement such as below:

REQUIREMENT LEVEL1: <Basic requirement> in <cell-edge radio conditions>,

REQUIREMENT LEVEL2: <More stringent requirement> in <more typical radio conditions>.

Proposal 119: In addition to ensuring minimum UE performance in stressed radio conditions (e.g., at the cell edge), RAN4 considers introducing also the corresponding more stringent UE requirement level for more typical radio conditions.

# Scalable RRM requirements

In NR, many features were introduced to support UE power saving from Rel-16 to Rel-19. However, these later introduced features are all based on capabilities. To support legacy Rel-15 UE, it’s difficult for network and operators to support these enhanced UE power saving features in NR. To make 6G market success, it is imperative to provide a good end-user experience for device battery life considering different UE types, such as low power IoT devices in Day-1.

For example, in the existing specifications, different measurement delay requirements are applied for NR, RedCap, HST, NTN, ATG, and IoT UEs. Each category further includes variants arising from diverse carrier configurations, enhancement/relaxation criteria. For 6G, it is necessary to establish a unified and scalable measurement requirement framework that can accommodate different UE types in a consistent manner.

Proposal 219: RAN4 shall study and define a scalable set of requirements in 6G to ensure compatibility across different UE types and configurations and conditions.

# UE architecture

The RRM requirements are strongly connected to UE architecture. Historically in 3GPP, the balance between the use cases of a feature and what a certain UE architecture can and cannot do has been well studied. In each early new generation discussion, UE architecture was often part of the general RRM and RF requirement discussion.

From our understanding, there are several examples:

- Single or multiple LNA in RF frontend.

- Single or multiple FFT and IFFT in baseband.

- The number of searchers in baseband.

A single FFT drives all carriers towards a fraction of CP. A single LNA handling several carriers might work better of aligned at UE RX, limiting MRTD, so CP time period can be used to control LNA gain on one carrier without perturbing data throughput on the other, but this limits site to site distance for non-collocated deployments, on the other hand. The RF retuning time is also highly impacted the interruption performance. The number of FFT will impact most of the performance for L1 level procedure, such as L1 inter-frequency measurement. Another important aspect contributing to RRM requirement is the searcher assumption. In NR, RAN4 requirements assume that one searcher is dedicated to the PCell frequency, while the second searcher handles all other intra-frequency carriers. This constraint limits the UE’s ability to perform parallel measurements across multiple carriers or RATs.

Observation 1: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

In NR, several advanced features—such as multi-RX chain reception, TDCP, and CJTC reporting for non-ideal backhaul—have been introduced across different releases. However, there remains a noticeable gap between the feature design in other WGs, and the corresponding requirements defined in RAN4. When other WG develops a feature, it often assumes that the UE can support a specific architecture necessary for the feature to function. In contrast, RAN4 discussions frequently rely on Rel-15 baseline UE assumptions, focusing on worst-case or basic UE implementations.We would like to respectfully emphasize that not all features are expected to be supported by all UEs. For 6G, it would be valuable to define requirements that enable a feature to operate effectively.

In many cases, RAN4 discussions begin with whether requirements should be defined at all, rather than focusing on how they should be defined. Addressing these gaps earlier in the process would lead to more coherent and implementable specifications.

In RAN4#116bis the UERF session agreed the following in the WF [2]:

# Topic #2: UE reference architecture - Agreements in online session - Strive to agree UE reference architecture on case-by-case manner for features being studied. Discuss reference architecture together with the feature itself as necessary.(first 6G agreement in RAN4) - Base evaluations on agreed reference architecture, while resulting requirements should apply for any feasible implementation architecture as much as possible - If no reference architecture is agreed, recommend companies to mention assumed architecture together with evaluation results for easier comparisons between inputs

Based on this decision in UE RF we reiterate our proposal that also RAN4 RRM should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature. This can be done on a case-by-case manner for features being studied

Proposal 3: RAN4 RRM should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature. This can be done on a case-by-case manner for features being studied.

# Interruptions

At a very high level, it is very desirable from the performance point of view that the amount of interruptions in 6G UE due to different procedures is reduced, which is a trade-off between the UE complexity/capability and a plurality of procedures, supported bands and bandwidths, and carrier frequencies and cells under operation. However, there are many types of and reasons for interruptions, e.g., related to cell management for DC, CA-related, due to BWP switching, SRS carrier-based switching, CBW change, CGI reading, measurements on deactivated SCG, RLM/BM on deactivated PSCell, etc. Hence, the interruption length depends also on the corresponding procedure, actions in the UE as well as on the UE architecture and its capabilities. In addition, the interruption length depends on network deployment and corresponding assumptions, including synchronization, propagation delay, etc., hence a too short interruption length requirement may limit network deployments or restrict network configuration. All these aspects need to be considered in interruptions discussions and in the context of the corresponding RRM procedure, case by case.

Proposal 4: RAN4 shall strive for reducing the amount of interruptions in UEs, while considering:

- Interruption cause/purpose,

- UE actions performed during the interruption time,

- UE architecture,

- Network deployment assumption.

Proposal 5: Consider reducing the interruption lengths when possible, e.g., by changing the granularity level to symbol-level.

# Measurement Gap design

In NR Rel-15, RAN4 developed a conservative measurement gap mechanism similar to LTE. To provide flexibility in UE design, RAN4 introduced more than 20 MGPs to accommodate different UE implementations. In later releases, RAN4 developed multiple measurement gap mechanisms; however, most of these solutions are independent and complex. As a result, both UE and NW vendors have been reluctant to support these later enhanced measurement gap mechanisms.

Gap-less measurement

In NR, a typical measurement gap length(MGL) is 6ms. One reason for introducing such a long MGL is that the UE doesn’t have a spare RF chain for measurements during that time. In 6G, RAN4 should aim to design a simplified and enhanced gap mechanism. For example, the shorter gap length mainly caused by RF retuning should be supported.

In last RAN4 meeting, RAN4 made the first agreements of gap-less measurement as follows.

Issue 4-1: MG related scope Agreement in main session (Tue): Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

In NR, RAN4 developed two types of small gap mechanisms, one is small gap controlled by UE(NeedForGaps) and the other is small gap controlled by NW(NCSG). UE announces gap-less measurement in both of features. However, from NW perspective, we see uncontrolled interruptions occur during the data transmission once NW enables this feature. Therefore, to avoid possible negative impact, we suggest RAN4 should first study the NW controlled gap-less measurement in 6G. After that, RAN4 can further study whether it’s possible to define UE controlled gap-less measurement with a strict requirement.

Observation 2: In NR, NeedForGaps feature result in uncontrolled interruption occurs frequenctly.

Furthermore, we understand at least in some scenarios UE can perform measurement without any interruption. RAN4 needs to identify these real gap-less scenarios.

Observation 3: In some scenarios, UE can perform measurement without any interruption.

At the same time, we also admit that, in some extreme scenarios, UE may still need a longer gap for measurement. Therefore, a unified measurement gap mechanism to adapt different gap types should be studied.

Another possible enhancement we would like to highlight is the measurement gap control mechanism. In NR, the gap is mainly controlled by RRC. It is difficult to adapt different scenarios with different MGL and MGRP. In Rel-17, pre-configured gap is introduced for dynamic gap configuration, but the use of pre-configured gap is limited to some specific procedures, such as BWP switching and SCell activation/deactivation. In Rel-19 XR, a more dynamic gap scheduling method is introduced, but it merely focuses on the gap cancellation. Thus, when RAN4 designs the 6G measurement gap, we should consider dynamic solution to allow NW to activate/deactivate the gap on demand.

When RAN4 introduced NCSG and NeedForGaps in NR, UE will adaptively report the gap status based on band combination. It would place a higher burden on the network side if hundreds of UEs report different status. In such case, NW cannot make an optimal decision for a specific UE, since it must take all the UEs within the coverage into account. In other words, frequent and detailed gap status reporting may be better for one UE, but it would prevent the overall measurement gap design from achieving its intended objectives in real flied application. Therefore, RAN4 should study an effective gap reporting to support the adapted MG framework at the early stage of the gap-less measurement design.

Proposal 6: RAN4 to study a unified measurement gap design to consider the following dimensions:

- NW controlled gap-less measurement

  - The scenarios for measurement without spare RF chain, with spare RF chain and zero interruption measurement

  - How to setup an effective gap reporting mechanism to avoid unnecessary information exchange with NW

- Both static and dynamic gap activation/deactivation/cancellation mechanism

Parallel measurement within gap

In NR, UE can perform parallel measurements without gap. The searcher concept has been introduced to define how many measurement objects can be measured in parallel. In the beginning, UE can support 2 searchers and further enhanced to 3 searchers. However, no such parallel measurement concept is supported for measurement within gap in NR. One of the important reasons is the single RF chain assumption. However, gap-less measurement has already been agreed to be studied in 6G Day-1 which means UE can support spare RF chains. Naturally, UE can support parallel measurements within gap. At the same time, we also understand the RF architecture limitation of UE will result in the parallel measurements cannot be performed all the time. Therefore, we propose RAN4 to study the parallel meas within gap and the relevant conditions.

Proposal 7: RAN4 to study parallel measurements within gap and the relevant conditions.

NW controlled scheduling restriction

In NR, scheduling restriction may happen in any SSB occasions due to L1 or L3 measurement. The scheduling restriction will result in interruption for data transmission/reception. However, NW doesn’t know where the scheduling restriction occurs in which carrier’s measurement. Thus, NW had to assume the interruption may happen in all the SSB occasions even if some of the SSB occasions are unused by UE. Therefore, RAN4 can study how to control the scheduling restriction occasions in 6G.

Observation 4: The real measurement occasions with scheduling restriction are unknown to NW.

Proposal 8: RAN4 to study how to control the scheduling restriction occasions.

Measurement Gap Pattern Reduction

In Rel-15, a large number of measurement gap patterns were defined. UE vendors only support few of the gap patterns, such as only the mandatory gap patterns. On the other hand, the measurement gap is mainly due to RF retuning and is not related to any measurement gap repetition period (MGRP). Thus, to simplify the gap patterns, RAN4 can consider decoupling the MGL with the MGRP configuration.

Observation 5: Although NR defines a large number of measurement gap patterns, only a small subset of them are used in real network.

Proposal 9: RAN4 to study how to simplify the MGP design to avoid unnecessary patterns.

# RRM framework

RAN4 started discussing the 6G RRM measurement framework in the last meeting, and a considerable number of proposals have been received. As a first step RAN4 agreed on the following:

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided.

Based on our understanding, RAN4’s initial task is to down select or define the scope of the SI for one or more of the above sub-topics. In this section, we first discuss Measurement delay/overhead/accuracy/quantities and Unified measurements, followed by Measurement capabilities, and provide our view on what the scope of the 6G RRM study should be.

# Measurement delay/overhead/accuracy/quantities

The summary of the various proposals is captured in the following WF. We begin by revisiting the discussion from our previous paper to reiterate the motivation behind our proposals. Additionally, we provide our views on proposals from other companies, with the aim of reaching a common understanding on the scope of the study item.

- Searcher number for enhanced simultaneous measurements (OPPO, HW, Samsung, ZTE, vivo, Ericsson) - Rx beam sweeping factor reduction (QC, Ericsson) - Identification/measurement/tracking/reporting delay reduction (QC) - RRM with NW aided measurement priority (Ericsson) - Virtual RRM UE group (Apple, ZTE) - Measurement requirements depending on purpose of the configured measurement: mobility or data (CA) (Nokia) - Unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes. (Nokia) - SSB evaluation (Samsung) - Intra and inter-frequency definition

When we designed the NR NR measurement requirements, many of the foundational principles were inherited from LTE — a system conceptualized nearly two decades ago. As we now look ahead to 6G, it’s important to recognize that this generation will serve the industry for the next twenty years or more. Over the past two decades, UE processing capabilities have evolved significantly, and we expect applications and use cases to continue evolving rapidly. Therefore, it may not be optimal to use NR (and by extension, LTE) as the baseline for 6G measurement design.

In NR, the measurement delay requirements consist of three key components:

- Cell Search and Identification Delay

- SSB Measurement Period

- SSB Index Reading Time

For the purpose of this discussion, we focus on FR1. The relevant inter-frequency measurement delays are defined as follows (exact scaling factors are not considered here for simplicity):

- Cell Search and Identification Delay: max (600 ms, 8×SMTC period) ×Scaling Factor

- SSB Measurement Period: max (200 ms, 8×SMTC period) ×Scaling Factor

The RAN4 requirements are defined as minimum performance criteria, assuming that the UE shall meet them even under worst-case conditions. However, practical observations from the field data show that most UEs are designed to meet only the minimum requirements, even when operating under better conditions. This conservative UE behavior, while technically compliant, can lead to inefficiencies in the NW implementation.

Observation 6: In NR, the measurement requirements are too relaxed, even in good channel conditions.

Searcher number for enhanced simultaneous measurements

As illustrated above, the current measurement requirements are too relaxed. One contributing factor for this relaxed measurement requirement is the lower bound inherited from LTE, which may no longer be suitable for 6G scenarios. Another important factor we need to revisit is the searcher assumption for 6G. In NR, a searcher refers to a hardware or logical measurement engine in the UE responsible for performing cell search and signal measurements (e.g., SSB or CSI-RS). The number of searchers directly impacts how many frequency layers carriers the UE can measure in parallel. In Rel-19, CSSF enhancements considering three searchers have effectively reduced measurement latency, but significant limitations remain for multi-carrier configurations and optional capabilities.

Based on our observations, increasing the number of searchers can enable:

- Parallel measurement processing, reducing reporting latency, in popular measurement configurations (e.g., within MG/W.O MG, intra-frequency/inter-frequency, L1/L3).

- Faster carrier aggregation setup, particularly in connected mode

- Improved responsiveness to bursty traffic and dynamic network conditions

- Enhanced support for network-driven measurement prioritization

Furthermore, it’s worth noting that, despite its widespread reference in CSSF discussions, the searcher’s underlying concept and architecture remain insufficiently defined, it further leads to applicablilty ambiguity.

Building on the above, 6G is expected to further clarify searcher design and utilization to to enable efficient, scalable, and responsive measurements across multiple carriers, frequency layers, and various network conditions.

Observation 7: Increasing the number of searchers and removing the lower bound in the measurement period will significantly reduce measurement period and shall be considered as part of the study. Once the searcher number are agreed, how to assign those searchers for different measurements can be discussed.

Rx Beam sweeping factor reduction

In NR, RAN4 agreed beam sweeping factor as 8 for FR2 measurements. It was based on the assumption that a large number of beams would be implemented on the network side, and that UEs would be equipped with multiple antenna panels on all four sides of the device, requiring the UE to sweep across the entire spherical coverage.

However, over time in practice, this assumption did not hold. In later releases, additional UE capabilities were introduced to reduce the RX beam sweeping factor in different releases under different scenarios and features.

We suggest that RAN4 shall revisit the default RX beam sweeping factor, taking into account practical deployment realities and updated UE implementation capabilities.

Another issue we would like to highlight is the treatment of wide and narrow beams and their associated RX beam sweeping factors in the RAN4 specifications. These assumptions are currently reflected only as a fixed value (N=8) in the requirements, without a clear link to actual UE behavior. In performance testing, this aspect was not thoroughly evaluated, leaving significant margin in the gain assumptions used in test case design. As a result, RAN1, RAN2, and product implementation teams have limited visibility into actual UE behavior. At the end, the delay was scaled but we are not sure whether UE has really performed any RX beam sweeping as it was not visible in the reported L1-RSRP or L3-RSRP.

Observation 8: The RAN4 specification does not define any UE behavior for RX beam sweeping in the context of L3 and L1 measurements, apart from prescribing a fixed numerical value in the requirements.

RRM with NW aided measurement prioritization

Currently, RAN4 requirement is defined as minimum requirement with periodic measurement. In other words, the requirement aims to treat all the target measurement objects with equal priority during the measurement. However, current minimum requirement sometimes didn’t fulfil the issues in the real field. To illustrate this, let’s consider a scenario we’ve encountered in our networks. Suppose four carriers are configured. According to RAN4, the UE is considered compliant with RAN4 requirements if it reports measurements for all four carriers within 3200 ms — that’s 600 ms for cell search and 200 ms for measurement per carrier. However, in practice, we’ve seen some UEs wait the full 3200 ms before reporting anything (in other words, we observe that UE schedule the measurements based on the RAN4 minimum requirements). This delay has real consequences: it slows down the network’s ability to initiate carrier aggregation in connected mode. It can also lead to the network to assumes that a carrier is out of coverage and triggers unnecessary reconfigurations. In certain cases, we have observed that when the SCell is configured in a deactivated state and the reporting configuration includes A1/A2 events, some UEs do not report these events at all. As illustrated in the figure below, one test configuration includes four cells—one PCell and three SCells. The figure clearly shows that some UEs fail to report A1/A2 events.

Percentage of UEs

(ms)

We consider another example to highlight the issue with current measurement behaviour.

Measurements play a critical role not only in mobility but also in carrier activation (e.g., SCell activation). There are two primary approaches can be considered for a carrier activation:

- Known carrier activation: The network configures the measurement object, waits for a measurement report to assess the cells quality before activating it.

- Unknown carrier activation: The network activates the carrier/cell without waiting for a measurement report or the measurement report was before last 5 seconds, assuming the carrier is likely to be suitable. If the carrier coverage is not good, then the carrier activation may fail, and NW may need to activate the other carrier.

In NR, in scenarios where multiple SCells are activated simultaneously, the activation delay for an unknown SCell includes:

- TFirstSSB_MAX_multiple_scells

- TSMTC_MAX_multiple_scells

- TRS × N1 + TRS + 5 ms

SCell activation involves AGC, L3 measurements, and L1 measurements. In a simplified case with a single beam and no L1 measurement, and assuming an SSB and SMTC period of 20 ms per carrier, the UE can activate all four SCells in approximately: 40 ms (AGC) + 20 ms × 4 + 20 ms + 5ms= 145 ms.

However, when the same carrier is configured as a neighboring frequency for measurement, the UE may take up to 3200 ms to report measurements. In other words, there is a different behaviour from the measurements point of view when the carrier is activated, and the same carrier is configured to be measured before activating it. This discrepancy can lead the network to incorrectly assume the carrier is out of coverage, delaying or preventing carrier activation.

Additional Observations:

- SCell activation conditions typically include higher SNR side conditions (e.g., ≥ -2 dB).

- Cell search may only require a single SSB occasion under the SCell activation side condition.

Despite good channel conditions, when the carrier is configured as a measurement object, UEs still delay reporting, reinforcing the need to revisit the current measurement strategy.

Thus, we believe that currently defined minimum requirement cannot meet the real world scenarios described above. Towards this end, in Rel-18, RAN2 introduced the measurement order based indication, which allows the network to recommend the order in which intra/inter-RAT and intra/inter-frequency measurements are performed based on the NW’s current requirement.

measSequence Indicates the recommended sequence for intra/inter-RAT intra/inter-frequency measurement. Value 1 means the corresponding frequency is measured firstly. Value 2 means the corresponding frequency is measured secondly and so on. If more than one frequency is configured with the same value, it means no recommended sequence among these frequencies. If not configured, it means there is no recommended sequence for the corresponding frequency. This field is only configured for NR standalone or if the measObject is associated to the MCG.

In 6G, RAN4 should define beyond the baseline minimum measurement requirements and incorporate adaptive measurement behavior based on the priority or order indicated by the network. While preserving an enough flexibility for the typical UE measurement operations, it is equally essential to enable the UE to adaptively reschedule its measurement tasks based on network preference or evolving real-time conditions.

This flexibility becomes especially critical in scenarios such as carrier aggregation for bursty traffic, where timely and prioritized measurements can significantly enhance responsiveness and overall network performance. By enabling the network to guide measurement priorities and allowing the UE to adjust accordingly, it can be better to support the dynamic and heterogeneous nature of 6G traffic patterns.

Observation 9: NW aided RRM measurement prioritization can enable timely measurements and improve the NW performance.

Measurement requirements depending on purpose of the configured measurement: mobility or data (CA)

If we understand the proposal correctly, the UE is expected to differentiate whether the configured measurements are intended for mobility or CA, and to perform the measurements accordingly. However, we believe that making such a distinction may not always be practical.

For example, consider an operator deploying their 5G network across three frequency bands: low-band (700 MHz), mid-band (3.5 GHz), and high-band/mmWave (26 GHz). To separate carriers for mobility or CA purposes, the operator would likely need to pre-assign certain bands or component carriers exclusively for one role. In practice, this kind of static classification may not be feasible in all the cases. Traffic load can vary significantly throughout the day, making it difficult to reserve specific bands or carriers solely for mobility or CA. Moreover, what serves as a PCell for one UE might be an SCell for another. In a heavily loaded system, the network must retain the flexibility to perform load balancing and dynamically distribute UEs across different carriers.

Given these practical constraints, enforcing a classification of measurements as either for mobility or CA may not be realistic under all conditions or across all markets. That said, we agree with the intended objective of the proposal. We believe the same goal can be achieved more effectively through network control mechanisms or by leveraging measurement prioritization.

Observation 14: It seems enforcing a classification of measurements as either for mobility or CA is not feasible in all conditions.

Virtual RRM UE group

It has been proposed by some companies to discuss the idea of forming a virtual RRM UE group, where devices from the same vendor are grouped together to reduce the measurement burden and thereby improve power consumption for low-capability UEs. While this may appear to be a simple and effective concept, we believe it could have significant implications for NW implementation and other 3GPP WGs.

Firstly, this topic has been proposed within the scope of a RAN4 study, but we believe that the RAN4 6G SI should primarily focus on addressing existing 5G pain points. Any new concepts or mechanisms, such as virtual RRM UE grouping, should ideally originate in RAN1 or RAN2, as these groups are responsible for designing the underlying functionality. In contrast, RAN4 is tasked with defining the core and performance requirements to verify that functionality.

Although the initial version of the study proposes a simplified scenario, where only L3 RSRP/RSRQ measurements are shared in the connected mode, with no Tx/Rx timing sharing and no communication between the devices for RRM grouping, it is well known that 3GPP tends to evolve features over multiple releases. Starting with a simplified scenario and later extending it to more complex scenarios without a thorough study could lead to increased complexity and cost. Moreover, the benefits of the RRM UE grouping for the connected mode scenario are not obvious.

Given these considerations, we believe that studying this concept solely within RAN4 may not be appropriate. We recommend that a more comprehensive study be conducted in the relevant WGs, particularly RAN1 and RAN2, before agreeing to include it in the RAN4 study. From the RAN4 perspective, this concept may be viewed as a measurement relaxation for certain UEs. If other WGs agree to introduce the necessary functionality, we believe RAN4 can also study the feature once there is significant progress is made in other WGs.

Based on our initial analysis, supporting RRM UE grouping in a full-fledged manner could have implications across multiple WGs including other RAN-, CT-, and SA-groups.

Observation 10: Virtual RRM UE grouping affects multiple WGs. Since it's a new feature and not a current 5G pain point, RAN4 can start the discussion if other WG groups agree on this concept in their 6G study.

SSB evaluation

There is significant interest among operators in improving network energy efficiency (NW EE) by increasing the SSB periodicity. In NR, the typical SSB periodicity is 20 ms. One potential approach to reduce energy consumption is to extend the SSB periodicity.

When assessing the sparse SSB impact, it is important to consider the inter-frequency measurement delay—i.e., whether the number of measurement samples can remain at 8 or be reduced under certain scenarios and SNR conditions, such as Handover.

Additionally, RAN4 should analyse the impact on measurement accuracy based on the new SSB design by other WGs.

Observation 11: New Synch signal design and the periodicity may have impact of SSB measurement delay and accuracy.

Measurement assumptions for 7 to 15 GHz

With the introduction of a new frequency range (7 to 15 GHz) for 6G services, it is important for RAN4 to discuss assumptions regarding UE beamforming in this band. Aligning UE reference architecture for this spectrum prior to defining requirements would be beneficial. Based on our understanding, the UE antenna architecture may not rely on analog beamforming using antenna panels (as assumed in FR2) across the entire 7 to 15 GHz range. At least for frequencies below 10 GHz, the UE architecture may instead resemble that of FR1—i.e., with a digital Rx port per antenna element. Therefore, we consider that RX beam sweeping is not necessary for measurements in this frequency range, particularly below 10 GHz. For the Frequency range above 10 GHz, RAN4 should align the assumptions of UE antenna architecture.

Observation 12: RAN4 to align the UE antenna architecture assumptions for the 7 to 15 GHz.

Measurements quantities

In NR, L3-RSRP, L3-RSRQ, and L3-SINR measurement requirements are defined for both L3 measurements and L1-RSRP and L1-SINR are defined for L1 measurements. Among these, L1-SINR was introduced in Rel-16. Although RTD measurements have been discussed across multiple releases, they have not yet been introduced. We believe SINR and RTD are critical for network operation, and to maximize their utility, all devices should support them from the first release of 6G. Currently, L1-SINR is supported only for serving cells, and not for candidate cells. However, we consider support for candidate cells essential, particularly when the network relies on L1 measurements for HO decisions.

Except for the measurement quantities mentioned above, ANR measurements are also a fundamental function that supports network optimization during the initial planning and deployment phases. CGI reading helps resolving PCI confusion in the network and facilitates network planning for the operators. This feature is useful to the operators from day one of network deployment and introducing such measurement in later release would add limited value. However, in NR, RAN4 started the CGI discussion in a very late stage in Rel-15. Therefore, to ensure timely implementation of the functionality, it is essential that UE supports the ANR measurement from the beginning of 6G and RAN4 should define the CGI reading requirement in the first release.

Observation 13: It is beneficial to support the measurements quantities such as RSRP, RSRQ, SINR, RTD and ANR measurements from the first release of 6G.

Intra-frequency and inter-frequency definition

Several companies have proposed removing the definitions of intra-frequency and inter-frequency measurements. While we understand the motivation behind this suggestion, we do not think it is good idea to making such changes without clearly identified issues or pain points.

We have been using these definitions in network implementations for quite sometime for the MO configuration and HO decisions. Changing or removing these definitions without solid evidence of problems could disrupt existing implementations and introduce unnecessary complexity. Therefore, we do not agree with making such changes unless there is a very good justification.

That said, if a study is nevertheless agreed upon, we believe its focus should be on understanding why changing the definitions is considered the only viable solution. It should also explore whether any issues with the current definitions can be resolved without removing them entirely. This would ensure that any changes are well-justified and not made prematurely.

Observation 14: The study on intra and inter-frequency definitions shall be well justified.

Proposal 1010: RAN4 to agree on following as the scope of the RRM measurement framework study for the measurement delay reduction

  - Increase of searcher number for enhanced simultaneous measurements and removing the lower bound for the faster measurements.

  - RRM measurement requirements with NW aided measurement prioritization

  - Rx beam sweeping factor reduction

  - The measurements quantities to include at least RSRP, RSRQ, SINR, RTD and ANR measurements.

  - UE reference architecture for new frequency range of 7 to 15 GHz

Proposal 1110: RAN4 to start on the intra-and-inter-frequency definition study when the SSB design is clear.

# Unified measurements

In the last meeting, multiple companies expressed interest in unifying the measurement framework within RAN4. There were differing views on how this unification should be done and which measurements should be considered as part of the unified framework. These proposals are summarized in the WF provided by the FL below.

- United/integrated cross-layers measurement and/or report between L1 and L3 (Samsung, OPPO, CMCC, LGE, Xiaomi) - United/integrated cross-functions measurement and/or report for L1 (e.g., integration of MIMO and LTM, or integration of RLM/BFD/CBD) (OPPO, CMCC, Xiaomi) - RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement. Unification in terms of reusing L3 measurements for L1 before certain point (e.g., before TCI state activation) (Ericsson)

First of all, we would like to emphasize that measurement configuration and reporting are defined in RAN2, and the ongoing unification study in RAN4 should not influence the RAN2 design. From our perspective, the interest in a unified measurement framework within RAN4 arises from the observation that, although the same SSB may be configured for both L1 and L3 measurements, the RAN4 requirements and resulting UE measurement behavior differ.

Specifically, when the same synchronization signal (SS) or reference signal (RS) is configured for multiple measurement purposes (e.g., L1-RSRP and L3-RSRP), we see an opportunity for RAN4 to explore how such measurements could be unified to reduce the time to obtain measurement results.

In RAN4, the prevailing assumption is that L3 measurements use wide beams, while L1-RSRP is measured using narrow (fine) beams. One benefit of using fine beams for serving cell L1-RSRP is that, in the P2 beam management procedure, the fine beam used for SSB-based L1-RSRP can also be used for receiving CSI-RS for data beam selection.

Additionally, it is assumed that QCL acquisition occurs during L1-RSRP measurement. However, this assumption does not fully align with actual UE behavior. In practice, UEs typically do not store QCL properties for all measured RSs, but only for those associated with activated TCI states. As a result, measuring L1-RSRP using fine beams across all cells may not be meaningful unless the corresponding TCI states are activated.

This issue becomes more apparent when L1-RSRP is configured for candidate cells, where TCI states are not yet activated. In such cases, wide beam measurements may be sufficient until activation occurs. That said, one potential benefit of using fine beams for L1-RSRP—even for candidate cells—is that it can accelerate TCI state activation. However, for candidate cells, the delay in TCI state activation may not be critical. Therefore, a context-aware and flexible measurement strategy may be needed, balancing efficiency and responsiveness based on activation status and network intent.

We suggest that RAN4 study the measurement model for L3 and L1 measurements in this context.

Regarding RLM and BFD, although they are considered L1 measurements, their configuration may differ from L1-RSRP or L1-SINR. In many cases, the network may configure implicit RLM, where no dedicated RS is configured for RLM or BFD. Instead, the UE considers the RS associated with the PDCCH TCI state as the RLM RS or BFD RS. From this perspective, unifying measurements for L1-RSRP, BFD, and RLM may not be feasible, as the RSs used for each may differ.

Proposal 1210: RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement and how to unify the L1 and L3 measurements when they are configured on the same RS.

# UE measurements capabilities

UE measurement capability is a scarce resource, while it is important for the entire network performance and service continuity that the UE is capable of performing many measurements during the same time period. With the growing measurement demand, it is very much expected that the measurements demand in 6G will be at least not less than that in NR. Hence, RAN4 will have to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements and reporting criteria, and

- the number of simultaneously measured carriers, cells, and beams.

Proposal 13: RAN4 to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements and reporting criteria, and

- the number of simultaneously measured entities (e.g., RATs, carriers, cells, beams, etc.).

# CONNECTED mode mobility

In NR, various mobility procedures have been standardized across different releases. Rel-15 introduced Layer 3 handover as the baseline. Rel-16 added conditional handover to improve robustness and DAPS to minimize interruption time. Rel-18 introduced lower-layer triggered mobility (LTM) for reduced latency, and Rel-19 further enhanced robustness with conditional LTM.

Both low interruption time and high robustness are essential and should be considered from the outset in 6G. UL and DL pre-synchronization, as defined in LTM, accelerates handover execution and significantly reduces interruption time. This mechanism is beneficial and should be part of the baseline design in 6G. With advanced UE capabilities such as ltm-FastUE-Processing-r18, interruption time can be reduced to as low as 10 ms. Achieving such fast processing and shorter interruption should be a design goal for 6G, and any necessary system-level support to enable this should be included in the baseline.

Currently, RAN4 requirements rely on explicit L1-RSRP configurations for DL synchronization during pre-synchronization. We believe this imposes unnecessary constraints on network configuration. In 6G, UE design should aim to maintain DL synchronization without requiring explicit L1-RSRP configuration.

For UL synchronization, if it is completed significantly ahead of the handover or cell switch command, the network may need to trigger multiple RACH preambles. This is an inefficient approach for both UE and network. RAN4 should study mechanisms to reduce this overhead and improve the overall handover efficiency.

Proposal 14: As a baseline to 6G connected mode mobility

- RAN4 should target <=10 ms handover interruption as a baseline system design for 6G and study the necessary assistance information required.

- RAN4 should aim for low signalling overhead mechanisms to maintain DL and UL synchronization.

# RRM related Energy Efficiency

# Network energy saving

Today in NR, SSB is designated as an always-on transmission on the PCell. During initial cell search, when a UE attempts to discover and connect to its first cell, SSBs in coverage cells are required to be transmitted every 20 ms. For 6G, a sparser baseline configuration may be introduced to allow SSB transmission periodicities of up to 160 ms to enable energy savings. However, such sparse transmissions may negatively impact performance, and various solutions to address these issues are under development.

Figure 5: NW energy consumption for SSB periodicities equal to or larger than 20 ms

In last RAN1 meeting, RAN1 had a long debate about how to evaluate the NES and UE performance regarding to the long periodicities of SSB. The related agreements are as follow.

SSB design Agreement Study and evaluate NW energy savings and the impact on UE performance and user experience with respect to 20ms and longer periodicities of sync signal(s) at least for initial access with the following consideration, but not limited to: BS assumptions: - Cell-common signaling (e.g., sync signal(s), broadcast PDCCH, SIB-1, SIB, paging, PRACH), e.g., - Clustered provisioning of different cell-common signaling, - On-demand provisioning of different cell-common signaling, - UE-specific signaling (for low, light, medium loads), e.g., - Clustered provisioning with cell-common signaling, - Un-clustered provisioning with cell-common signaling, UE impact: - Cell search complexity and latency, including frequency search latency, - UE Power consumption, - Sync signal detection, coverage and tracking performance, - RRM, mobility, - Beam management, - Other properties are not precluded, - Improvements to address identified impact, e.g., - Additional sync signal needs, - Adaptation of sync signal transmission periodicity, - Sparser synch raster.

From our understanding, RAN4 can support RAN1’s RRM and mobility performance evaluation once the sparse SSB design is clear or the work can be triggerred by RAN1’s LS.

OD-SSB

During the later stages of NR standardization (Release 18 and 19), several promising network energy-saving techniques were further developed. However, the NW energy savings features to some extent cannot be achieved in NR due to certain limitations. Firstly, all these mechanisms depend on UE capabilities and the number of UEs that support such features may be very small. Secondly, techniques that could negatively impact legacy UEs were excluded. Therefore for 6G, we need to support the network energy saving techniques from Day-1 to avoid back-compatible issues in NR.

In 6G, to avoid dense SSB transmission, OD-SSB should be studied to assist more procedures for UEs in both Idle/Inactive and CONNECTED mode for any carrier. To allow the 6G design to optimally cater for both IDLE and CONNECTED mode mobility needs, separate SSB for IDLE and CONNECTED should be considered. For example, a sparse SSB can be used in IDLE mode. An OD-SSB can be applied when it is needed, such as for connection establishment, CONNECTED mode operation and CONNECTED mode mobility. In last RAN1 meeting, RAN1 also agreed to study the OD-SSB for NES as follow. Therefore, RAN4 can study the scenarios in which OD-SSBs are needed or does not needed, e.g. for IDLE/INACTIVE mode mobility, Connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

OD-SSB Agreement Study and evaluate on-demand sync signal(s) mechanisms for 6GR energy efficiency, considering, e.g.,: - On-demand sync signal(s) for single cell/carrier, multi-carrier/cell, multi-TRP, - Network-triggered and UE-triggered on-demand sync signal(s), - Idle and/or connected modes, - Other mechanisms/aspects/signals/channels are not precluded.

Proposal 15: RAN4 to identify the scenarios where OD-SSB is beneficial, such as IDLE/INACTIVE mode mobility, Connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

SSB-less SCell

In last RAN1 meeting, RAN1 achieved the following agreements related to SSB-less for NES as follow.

SSB-less Agreement Study and evaluate multi-carrier/cells/TRPs mechanisms for 6GR NES, considering, e.g.,: - Sync signal-less carriers/cells/TRPs for at least intra-band and collocated inter-band multi-carrier/cell/TRPs, including potential extensions to additional deployments and scenarios, - RRC states, - UE energy consumption and complexity, - Other mechanisms/aspects/signals/channels are not precluded.

In NR Rel-15, an intra-band SSB-less Scell capability was already introduced and RAN4 defined the related Scell activation requirements. In NR Rel-18, an inter-band co-located SSB-less Scell capability was introduced. Such SSB-less Scells can achieve good energy saving for network. UE obtains timing reference from the reference cell or an intra-band SpCell/SCell. Thus the SSB-less SCell does not need to transmit SSB. In this case, UE can perform a fast SCell activation without receiving SSB. With a new Rel-20 5G-adv WI for RRM [RP-252955], with objective on for fast SCell activation, RAN4 is expected to  support data scheduling prior to the valid CSI reporting, the overall activation delay may be further reduced. However, the SSB-less features have not yet been commercialized, since the effectiveness would be limited if some UEs do not support the feature. In our understanding, RAN4 shall start the study of the conditions whether the SSB-less Scell activation can work at least about intra-band and inter-band collocated SSB-less scenarios. Furthermore, RAN4 also needs to identify the UE behaviour and relavant NW assistant information once SSB-less conditions are not met.

Proposal 1619: RAN4 to study the SSB-less cell for NES at least as follow.

- The conditions for intra-band and inter-band colocated SSB-less Scell activation.

- The UE behaviour and relavant NW assistant information once SSB-less conditions are not met.

- Other scenarios are not precluded.

# UE energy saving

From Rel-16 to Rel-19, a large number of UE energy saving technologies were developed. For example, RRM relaxation due to low mobility and cell centre was introduced from Rel-16. Further enhancement for stationary UE and RLM/BFD relaxation were introduced in Rel-17. In Rel-19, a new type of low power receiver with further RRM relaxation is introduced. However, both network vendors and operators may be reluctant to activate these features or tend to configure the parameters in conservative manner in commercial networks. To achieve the real UE power saving, RAN4 should define a reasonable RRM relaxation solution considering both power saving and RRM mobility issues.

Proposal 1719: RAN4 to evaluate and define a simple unified RRM relaxation solution for UE power saving.

Most UEs spend long durations not actively transmitting/receiving data but still need to monitor PDCCH in CONNECTED mode or Paging in IDLE mode frequently. An efficient way to reduce the monitoring time is to request the network to wake up the UE only when needed by sending a wake-up signal (WUS). Different wake-up signals were developed in NR different releases, such as DCP, PEI and LP-WUS etc.

In NR Rel-19, a new wakeup signal is designed for both IDLE/INACTIVE and CONNECTED modes which are detected by a dedicated wake up receiver (WUR). The unified signal is based on an OOK waveform overlaid with OFDM sequences which can be detected by both OOK WUR and OFDM WUR. In RAN4, we defined two different types of requirements to support both LP-SS and SSB. The OFDM WUR provides significantly better coverage compared to the OOK WUR. Moreover, for synchronization and measurements in Idle/Inactive mode, the OOK WUR requires a dedicated low power synchronization signal (LP-SS) while the OFDM WUR can re-use the existing SSBs. Considering NW energy consumption, we should avoid the additional OOK transmission on top of SSB transmission. In last RAN1 meeting, RAN1 also achieved a similar agreements as follow.

LP-WUS Agreement Study and evaluate DL WUS of OFDM based sequence and corresponding mechanisms for 6GR EE improvement, regarding at least the following aspects: - Coverage target for DL WUS (e.g., same as PDCCH, common sync signal, or other) - Measurements and/or synchronization. - System overhead and network energy consumption/UE energy saving for UE operation with the DL WUS. - RRC states - Other functionalities

In NR, the neighbour cell SSB transmission is mainly based on 20ms, which reliefs the LP-WUS drawbacks for mobility detection. UE can perform the neighbour cell measurement based on the dense SSB periodicity within DRX cycle. However, in 6G, due to the sparse SSB transmission, a new LP-WUS receiver should be investigated, either to enhance the LP-WUR capability to measure the neighbour cell’s SSB directly or to wake up the main receiver more efficiency to support the mobility measurement and pre-synchronization for CONNECTED procedures.

At the same time, to achieve better NW energy saving, NW also doesn’t need to transmit the dense periodic signals if not need to wake up the UE, such as SSB. Instead of a dense periodic signal, the on-demand signals can be transmitted to assist UE together with the wake-up signals. Therefore, when RAN4 discusses the LP-WUR receiver from UE side, the network energy efficiency needs to also be considered. RAN4 needs to evaluate the OFDM-based LP-WUS receiver together with the network energy efficiency in IDLE and CONNECTED mode.

Proposal 18: RAN4 to study and evaluate an OFDM-based LP-WUS/WUR mobility performance together with NES in both IDLE and CONNECTED mode in 6G.

# Spectrum aggregation

For 6G, we see more spectrums will be available especially for the centimetre wave band frequency. How to utilize this new frequency resource is critical for 6G end performance.

Also in NR, the midband 3GHz and lower part of 7GHz band within FR1 carrier aggregation is already supported. The FR1 and FR2 carrier aggregation is also already supported in NR.

To our view, further introduce non-collocated inter band spectrum aggregation can provide full flexibility to address almost all the aggregation scenario wherever it is needed. Certain benefits include but not limited to:

- Deployment flexibility for lowband FDD to be fully utilized for example the low FDD bands can be aggregated with mid and high bands without being limited to the same physical location

- Independent deployment of 3-8GHz or highband (24GHz) without a collocated low band base station, this can further reduce the cost of deployment and enhance the coverage for mid and high bands.

DL/UL flexible combining

In NR, the deployment of the high/mid band TDD carrier is one step for 5G performance enhancement. However, due to the  power difference between the BS transmit power and UE transmit power, for the TDD carrier, the DL and UL coverage is imbalanced. This limits the coverage, especially the UL coverage for high band carriers.

To be fully utilized the UL coverage provided by the low band (LB in figure 6), especially the sub-GHz FDD bands, the flexible combining of the DL/UL carrier is necessary. As we can see for different UE with different accessibility to the high band (HB in figure 6), the combination of the DL and UL can be flexible to make sure the UEs always have the best UL serving link and best DL serving link available.

In NR, the UL carrier aggregation requirements were specified based on whether this Scell has been configured with uplink control channel (PUCCH)or not. In 6G, how the air interface fundamentals eg control signal has high dependency on other RAN groups. To support flexible combining of the DL/UL carrier RAN4 needs to further study the impact of the requirements when other RAN group have made sufficient progress.

Figure 6: example of DL/UL decoupling

Fast carrier setup and activation

Traffic is often bursty and one challenge in NR has been to setup and activate additional spectrum eg. SCells in time before the buffer is emptied to ensure good user performance and effective load balancing between the deployed carriers.

This would require the network to identify the relevant carrier to transmit the data on and activate those carriers quickly after the UE’s state transition to RRC Connected, see Figure 1.

Figure 7: Illustration of carrier identification

For identifying the relevant candidate carriers to be used, features like Early measurement reporting were standardized in NR. However, a UE’s dwelling time in energy saving state (RRC Idle mode or RRC Inactive state) is much shorter (under 25 seconds on an average) than the time the UE can take to perform inter-frequency measurements (up to a minute per frequency) as per RAN4 requirements while being in energy saving state.

Thus, RAN4 needs to study potential requirement enhancements in identifying the candidate carriers fast to be used for the UE upon state transition.

Once a candidate carrier has been identified, there is a need to activate those carriers early enough to be able to use them before the primary carrier becomes a bottleneck. As can be seen from the plots shown above, activating SCells quickly gives gains in bitrates for all object sizes. These results show the object bit rate of subsequent downloads, meaning that the TCP connection is kept open between subsequent downloads so that there is no slow start.

Figure 8: Simulation results showing the benefits of fast spectrum aggregation activation for a cell edge UE

The plots with CA activated after 5ms of RA procedure show significant improvement in object bitrate compared to other CA activation delays. This motivates faster CA activation. The results also show the impact of poor UL when the object size is larger. When object size is 10 MB, the performance of PCell @ 800 MHz and PCell @ 7 GHz without CA are quite similar due to poor UL on 7 GHz (plots are generated for UEs on 7GHz UL coverage border with RSRP=-110dBm). This motivates the decoupling of UL carrier and DL carrier.

Reuse of measurements across states

Currently, when a UE transitions between RRC states, such as from RRC CONNECTED to RRC IDLE or RRC INACTIVE state, it typically releases measurements from previous state. This means that, as per the RAN4 requirements, upon entering a new state, the UE is allowed to restart the same measurements from scratch (at least from the measurement delay perspective). This behavior is not optimal, especially considering the following:

UEs do not remain in a single RRC state for extended periods. UEs often switch between connected to idle or connected to inactive states based on traffic patterns and network activity. This frequent switching makes it inefficient to discard and re-initiate measurements each time.

Certain state transitions result in the release of measurement data. For example, transitioning from RRC CONNECTED to RRC IDLE may clear previously buffered measurements, even though they could still be relevant and valid in the new state. Repeating measurements after a state transition introduces latency, which can delay the UE’s ability to resume a fast CA setup.

Given these challenges, we think enabling reuse of measurements across states could offer clear benefits. However, implementing such a mechanism would require changes in how the UE handles measurement data, specifically, the ability to buffer or retain relevant measurements during state transitions. This may have implications on UE architecture, including memory management and measurement scheduling. Considering the architectural impact and potential benefits, we believe this topic is well-suited for inclusion in the 6G RRM Day-1 study.

Observation 15: Reuse of measurements across the RRC states shall be part of the 6G RRM Day-1 study.

Proposal 19: RAN4 to support collocated, non-collocated intra-band and inter-band spectrum aggregation for potential new spectrum from 6G day 1.

Proposal 20: RAN4 further study the potential requirement enhancement for fast carrier setup and activation when RAN1 and RAN2 have made sufficient progress.

Proposal 21: RAN4 to study potential requirement impact of supporting DL/UL flexible pairing when RAN1 and RAN2 have made sufficient progress.

# MIMO and mTRP operation related RRM

The main RRM requirements defined by RAN4 in the context of MIMO and multi-TRP across different NR releases have primarily focused on TCI state switching and the timing requirements such as MRTD and MTTD. In later releases, RAN4 has also introduced requirements related to TDCP and CJTC measurements.

We believe that all of these requirements are heavily dependent on the feature design and signaling framework defined by RAN1. Since the RAN1 study on these topics is scheduled to begin in the February meeting, initiating discussions in RAN4 ahead of RAN1’s input is not ideal and may not be productive. Without a clear understanding of the RAN1 design assumptions and the features that are going to be supported, it would be premature for RAN4 to start a study.

Therefore, we suggest that RAN4 discussions on these topics be deferred until RAN1 has made sufficient progress in its study, ensuring that any RRM requirements study are well-aligned with the underlying feature design.

Proposal 22: RAN4 discussions on MIMO and multi-TRP to be deferred until RAN1 has made sufficient progress in its study.

# NTN related RRM

From Rel-17 to Rel-19, RRM requirements for NTN have been introduced in alignment with the evolution of TN requirements with necessary adaptations for NTN-specific aspects.

In 6G, NTN requirements shall continue to follow the TN RRM requirement framework while considering necessary updates to account for the specific features, characteristics, and aspects of NTN.

In particular, some ongoing discussions on RRM study in 6G can be directly applied for for NTN, e.g.,

- Unified measurement framework

- Some objectives in RRM measurement framework, e.g.:

  - Searcher number for enhanced simultaneous measurements

  - RRM measurement requirements with NW aided measurement prioritization

  - Reuse of measurements across different RRC states

- Measurement gap enhancement, e.g.:

  - Unified measurement gap design

  - Parallel measurement within gap

In 6G, the convergence of NTN and TN necessitates harmonizing their requirements, in order to simplify UE implementation and ensure seamless operation across both network types.

Apparently, it requests a unified RRM framework for 6G to align TN and NTN from day 1. To achieve this, each objective in the RRM study should consider NTN scenarios from day 1, rather than focusing solely on TN requirements and later taking remedial actions for NTN.

Proposal 23: RAN4 shall consider the demands and scenarios of both TN and NTN simultaneously in the 6G RRM design from the begining.

GNSS-resilient NTN operation has been initiated for study in Release 20, while GNSS-less, i.e., no GNSS, operation is under discussion in the 6G scope. Accordingly, RAN4 needs to evaluate the impacts on the corresponding NTN RRM requirements, such as downlink/uplink timing and measurements, and study the solutions where applicable.

Proposal 24: RAN4 shall study valid and effective NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation.

# IDLE/INACTIVE mode mobility

Referring to field data analyses in NR, it is observed that a significant number of UEs camp on cells that are not intended to serve them after entering RRC_CONNECTED state. For example, currently, when UE enters RRC_IDLE/INACTIVE state, the UE is highly possible to cell reselect and camp on a FDD low-band cell even though there is a good quality TDD mid-band cell. In practice, this means that once UEs transition to RRC_CONNECTED state, the serving cell may soon initiate a handover to another cell that is better aligned with network configuration and service objectives. Such redundant mobility activities could be avoided if UEs initially camped on the cells preferred by the network.

The current priority-based cell reselection scheme, while designed to reflect network preferences, shows certain limitations in practice, e.g.,

- When the network configures lower SnonIntraSearchP, SnonIntraSearchQ thresholds to restrict measurements to higher-priority frequencies, the risk of mobility increases after RRC connection setup, as potential better-quality lower-priority cells may be excluded from consideration.

- Conversely, when the network configures higher SnonIntraSearchP, SnonIntraSearchQ thresholds, the UE is required to measure all inter-frequencies of higher, equal, or lower priority in most of practical scenairos. In such cases, the UE may effectively ignore the configured priority and camp on low lower priority cells. Furthermore, it extends the measurement delay for identifying high priority cells.

Generally, UE needs to spend merely one minute to finish all configured frequencies measurement based on the defined evaluation requirement shown in Table 1. It’s highly possible UE to ignore the configured cell reselection priority and camp on a lower priority cell. As an example, we observed in real field that UE is highly possible to perform cell reselection and camp on a low-band cell even though NW configured a mid-band cell with higher priority and having good quality.

Figure 2: An example of cell reselection priorities configuration with 3 frequencies

Observation 16: UE Camping Behavior Not Aligned with Network Preference under Current NR Cell Reselection Procedures

Table 1: Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tevaluate,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x N1 x 1.5 (36 x N1 x 1.5) | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | 5.12 x N1 x 1.5 (16 x N1 x 1.5) |
| 0.64 |  | 5 | 17.92x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |

For 6G, it is expected that the priority-based cell reselection mechanism will be further refined, with clear design objectives derived from practical field observations. RAN4 shall study the potential enhancement and requirements to improve UE camping behaviour in alignment with network configuration intention.

Proposal 25: RAN4 shall study possible solutions and requirements to improve UE camping behavior in RRC_IDLE/INACTIVE state and reduce redundant mobility activities.

# Timing and synchronization

MRTD

Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility when it comes to cell planning, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Figure 1: Large MRTD and path loss difference use case.

Observation 17: Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Consider whether the synchronization requirements of a feature or function really need to be specified in the standard. Often, a sync requirement cannot be adequately captured by a single requirement figure, as there generally are complex and varying input parameters associated with it. In the worst-case scenario, specifying a value based on the most challenging dimensioning can lead to increased costs and complexity, which will not be necessary in most scenarios. If feasible, opt for using a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Let us discuss an important example. In Figure 2, we know that:

RTD= TAE+ ∆RF_Propagation+Channel_dispersion

If the RAN4 requirement is specified as Maximum Receive Time difference, MRTD then it is possible to trade TAE and ∆RF_Propagation, within a total budget.

Figure 2: Receive time difference example

For requirement clarity link each requirement to specific features or functions (which may be optionally supported), clearly defining the conditions under which they apply—such as co-location scenarios, particular band combinations, or other relevant constraints, like robustness—and, where applicable, to specific UE capabilities.

Observation 18: When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Proposal 26: When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between TRP.

TDD Cell Phase Synchronization

TDD Cell Phase Synchronization time uncertainty budgets are depicted in Figure 3. The current requirement of 3 µs is based on a budget consisting of three terms, the synchronization input error, the operational limit and the holdover margin. The input error represents the uncertainty in the external clock source. The operational limit is the performance when the external clock source is running with normal operation base station uncertainties added. Finally the holdover margin is e.g. how far the clocks can drift from the operational limit until the TDD Cell Phase Synchronization Requirement is violated, i.e. ± 1.5 µs (relative 3 µs).

Observation 19: part of the current 3 µs TDD Cell Phase Synchronization requirement is for holdover margin.

For a network with a base level of TDD Cell Phase Synchronization we can conclude that:

- A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network.

- A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics.

- We get a higher level of redundancy as more alternative synchronization sources can be used (including GNSS independent sources). This leads to a more robust network.

- It´s impact on overhead in a TDD system is very small

For a network with a strict level of TDD Cell Phase Synchronization we can conclude that we get:

- Lower holdover margin and a less robust network when it comes to temporary loss of synchronization.

- A strict requirement on input synchronization limits deployment flexibility..

- We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

Figure 5: System robustness and resilience in relation to Cell Phase Sync

Observation 20: A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

Observation 21: A lower holdover margin makes the network less robust when it comes to temporary loss of synchronization. A strict requirement on input synchronization lowers the probability that the base stations and the network can operate in certain deployments. The number of possible base station configurations will be more limited regarding how network components are connected with switched fronthaul. We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

As highlighted above, we consider robustness, deployment flexibility, and the ability to leverage existing infrastructure to be more important than marginal TDD overhead reductions. Therefore, we propose to maintain the TDD Cell Phase Synchronization requirement at the same level as in NR NR, i.e., 3 μs. Similarly, we believe that synchronization enhancements are better utilized to improve overall system robustness rather than to achieve the aforementioned minor TDD overhead reductions.

Proposal 27: Keep TDD Cell Phase Synchronization requirement the same as in NR NR.

NTA_offset

The RRM requirementin RRM specification is related to Cell Phase Synchronization and TDD transient times in RF specifications through the formula:

NTA offset≥Cell_Phase_Synchronization+Transmitter transient period

It also needs to accommodate sufficient time for TX<=>RX switching. This means that, Cell Phase Synchronization and TDD transient times are inter dependent and has to be discussed together.

Observation 22: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

In existing NR specification (TS 38.133, Table 7.1.2-2) there are several coexistence cases listed, since the RRM requirement NTA offset affects UL timing. There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

For example Multi RAT Spectrum Sharing (MRSS) between 5G and 6G is a key example of a feature where inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 23: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 24: Multi RAT Spectrum Sharing (MRSS) between 5G and 6G is a key example of a feature where inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

UE TX timing

In [3] the UE TX time mechanism is discussed. The existing mechanism has two inputs a DL time reference and a TA command from the network. This mechanism has proven to be very robust for baseline systems of 4G and 5G and even before, for a very wide array of use cases. We therefore propose that also 6G system shall use the existing 4G and 5G mechanism for mainline terrestrial use cases.

Observation 25: The existing mechanism has two inputs a DL time reference and a TA command from the network. This mechanism has proven to be very robust for baseline systems of 4G and 5G and even before, for a very wide array of use cases.

Proposal 28: We therefore propose that also 6G system shall use the existing 4G and 5G mechanism for mainline terrestrial use cases, with a DL time reference and a TA-command from the network.

Moreover, also in [3] the issue of PRACH and timing is discussed. It is true that we in the 5G baseline have a large set of PRACH formats and each format has different preambles, guard periods and cyclic prefix. The main intention in 5G is to be able to trade maximum cell radius and Doppler performance at initial access.

However the UL time requirements have to sustain an ongoing connection and then we rely on the CP for the user data

Observation 26: We already have specific requirements for PRACH, in the form of supported maximum cell radius and Doppler performance in 5G, for different long and short preamble formats.

Observation 27: However the UL time requirements have to sustain an ongoing connection and then we rely on the CP for the user data and not the performance of the initial PRACH.

# Summary

In this contribution, we initially discussed the 6G RRM-related aspects, including lessons learnt from NR, potential procedure enhancements from NR and new user experience improvements in 6G. Based on these discussions, we have made the following proposals and observations:

Observation 1: Aspects of UE architecture like FFT, searcher and LNA implementations put conditions on what kind of requirements UE can be achieved.

Observation 2: In NR, NeedForGaps feature result in uncontrolled interruption occurs frequenctly.

Observation 3: In some scenarios, UE can perform measurement without any interruption.

Observation 4: The real measurement occasions with scheduling restriction are unknown to NW.

Observation 5: Although NR defines a large number of measurement gap patterns, only a small subset of them are used in real network.

Observation 6: In NR, the measurement requirements are too relaxed, even in good channel conditions.

Observation 7: Increasing the number of searchers and removing the lower bound in the measurement period will significantly reduce measurement period and shall be considered as part of the study. Once the searcher number are agreed, how to assign those searchers for different measurements can be discussed.

Observation 8: The RAN4 specification does not define any UE behavior for RX beam sweeping in the context of L3 and L1 measurements, apart from prescribing a fixed numerical value in the requirements.

Observation 9: NW aided RRM measurement prioritization can enable timely measurements and improve the NW performance.

Observation 10: Virtual RRM UE grouping affects multiple WGs. Since it's a new feature and not a current 5G pain point, RAN4 can start the discussion if other WG groups agree on this concept in their 6G study.

Observation 11: New Synch signal design and the periodicity may have impact of SSB measurement delay and accuracy.

Observation 12: RAN4 to align the UE antenna architecture assumptions for the 7 to 15 GHz.

Observation 13: It is beneficial to support the measurements quantities such as RSRP, RSRQ, SINR, RTD and ANR measurements from the first release of 6G.

Observation 14: The study on intra and inter-frequency definitions shall be well justified.Observation 14: The study on intra and inter-frequency definitions shall be well justified.

Observation 15: Reuse of measurements across the RRC states shall be part of the 6G RRM Day-1 study.

Observation 16: UE Camping Behavior Not Aligned with Network Preference under Current NR Cell Reselection Procedures

Observation 17: Deployments can be colocated and non-colocated. Non colocated deployments, adds flexibility, but this drives requirements related to MRTD in RRM and maximum power difference in RF.

Observation 18: When feasible, in timing requirement, use a total budget that allows flexible allocation among subcomponents instead of specifying sub-requirements on part of the system.

Observation 19: part of the current 3 µs TDD Cell Phase Synchronization requirement is for holdover margin.

Observation 20: A large holdover margin makes the system more robust to temporary loss of synchronization, which, in turn, enables a more robust network. A more relaxed input synchronization requirement enhances deployment flexibility, enabling the network to operate across a variety of configurations including switched networks with varying characteristics. We get a higher level of redundancy as more alternative synchronization sources can be used. This leads to a more robust network.

Observation 21: A lower holdover margin makes the network less robust when it comes to temporary loss of synchronization. A strict requirement on input synchronization lowers the probability that the base stations and the network can operate in certain deployments. The number of possible base station configurations will be more limited regarding how network components are connected with switched fronthaul. We get a low level of redundancy, since few or no alternative synchronization sources can be used, resulting in a less robust network.

Observation 22: NTA offset (RRM) , Cell Phase Synchronization (RRM) and TDD transient times (RF) are inter dependent and has to be discussed together, since NTA offset≥Cell_Phase_Synchronization+Transmitter transient period.

Observation 23: There exist various configuration and inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 24: Multi RAT Spectrum Sharing (MRSS) between 5G and 6G is a key example of a feature where inter RAT co-existence dependencies which motivate keeping NTA offset the same as in NR FR1 and FR2.

Observation 25: The existing mechanism has two inputs a DL time reference and a TA command from the network. This mechanism has proven to be very robust for baseline systems of 4G and 5G and even before, for a very wide array of use cases.

Observation 26: We already have specific requirements for PRACH, in the form of supported maximum cell radius and Doppler performance in 5G, for different long and short preamble formats.

Observation 27: However the UL time requirements have to sustain an ongoing connection and then we rely on the CP for the user data and not the performance of the initial PRACH.

Proposal 1: In addition to ensuring minimum UE performance in stressed radio conditions (e.g., at the cell edge), RAN4 considers introducing also the corresponding more stringent UE requirement level for more typical radio conditions.

Proposal 2: RAN4 shall study and define a scalable set of requirements in 6G to ensure compatibility across different UE types and configurations and conditions.

Proposal 3: RAN4 RRM should define 6G feature requirements based on realistic UE architecture assumptions, rather than relying solely on basic UE profiles that may not support the intended feature. This can be done on a case-by-case manner for features being studied.

Proposal 4: RAN4 shall strive for reducing the amount of interruptions in UEs, while considering:

- Interruption cause/purpose,

- UE actions performed during the interruption time,

- UE architecture,

- Network deployment assumption.

Proposal 5: Consider reducing the interruption lengths when possible, e.g., by changing the granularity level to symbol-level.

Proposal 6: RAN4 to study a unified measurement gap design to consider the following dimensions:

- NW controlled gap-less measurement

  - The scenarios for measurement without spare RF chain, with spare RF chain and zero interruption measurement

  - How to setup an effective gap reporting mechanism to avoid unnecessary information exchange with NW

- Both static and dynamic gap activation/deactivation/cancellation mechanism

Proposal 7: RAN4 to study parallel measurements within gap and the relevant conditions.

Proposal 8: RAN4 to study how to control the scheduling restriction occasions.

Proposal 9: RAN4 to study how to simplify the MGP design to avoid unnecessary patterns.

Proposal 10: RAN4 to agree on following as the scope of the RRM measurement framework study for the measurement delay reduction

  - Increase of searcher number for enhanced simultaneous measurements and removing the lower bound for the faster measurements.

  - RRM measurement requirements with NW aided measurement prioritization

  - Rx beam sweeping factor reduction

  - The measurements quantities to include at least RSRP, RSRQ, SINR, RTD and ANR measurements.

  - UE reference architecture for new frequency range of 7 to 15 GHz

Proposal 11: RAN4 to start on the intra-and-inter-frequency definition study when the SSB design is clear.

Proposal 12: RAN4 to study the flexible and adaptive measurement behaviour for L1 measurement and how to unify the L1 and L3 measurements when they are configured on the same RS.

Proposal 13: RAN4 to discuss, agree on, and specify minimum requirements on 6G UE capability in terms of:

- the number of parallel measurements and reporting criteria, and

- the number of simultaneously measured entities (e.g., RATs, carriers, cells, beams, etc.).

Proposal 14: As a baseline to 6G connected mode mobility

- RAN4 should target <=10 ms handover interruption as a baseline system design for 6G and study the necessary assistance information required.

- RAN4 should aim for low signalling overhead mechanisms to maintain DL and UL synchronization.

Proposal 15: RAN4 to identify the scenarios where OD-SSB is beneficial, such as IDLE/INACTIVE mode mobility, Connection establishment, CONNECTED mode operation and CONNECTED mode mobility.

Proposal 16: RAN4 to study the SSB-less cell for NES at least as follow.

- The conditions for intra-band and inter-band colocated SSB-less Scell activation.

- The UE behaviour and relavant NW assistant information once SSB-less conditions are not met.

- Other scenarios are not precluded.

Proposal 17: RAN4 to evaluate and define a simple unified RRM relaxation solution for UE power saving.

Proposal 18: RAN4 to study and evaluate an OFDM-based LP-WUS/WUR mobility performance together with NES in both IDLE and CONNECTED mode in 6G.

Proposal 19: RAN4 to support collocated, non-collocated intra-band and inter-band spectrum aggregation for potential new spectrum from 6G day 1.

Proposal 20: RAN4 further study the potential requirement enhancement for fast carrier setup and activation when RAN1 and RAN2 have made sufficient progress.

Proposal 21: RAN4 to study potential requirement impact of supporting DL/UL flexible pairing when RAN1 and RAN2 have made sufficient progress.

Proposal 22: RAN4 discussions on MIMO and multi-TRP to be deferred until RAN1 has made sufficient progress in its study.

Proposal 23: RAN4 shall consider the demands and scenarios of both TN and NTN simultaneously in the 6G RRM design from the begining.

Proposal 24: RAN4 shall study valid and effective NTN RRM requirements remain under both GNSS-resilient and GNSS-less operation.

Proposal 25: RAN4 shall study possible solutions and requirements to improve UE camping behavior in RRC_IDLE/INACTIVE state and reduce redundant mobility activities.

Proposal 26: When feasible, specify MRTD (RRM) as a total budget and avoid stating TAE (BS RF) between TRP.

Proposal 27: Keep TDD Cell Phase Synchronization requirement the same as in NR NR.

Proposal 28: We therefore propose that also 6G system shall use the existing 4G and 5G mechanism for mainline terrestrial use cases, with a DL time reference and a TA-command from the network.

# References

[1]. RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

[2]. R4-2514642, WF for [116bis][102] 6G general RF and UE RF, Qualcomm

[3]. R4-2513039, Views on 6G RRM, MediaTek Inc.

[4]. R4-2514624, WF on 6G RRM, Apple