3GPP TSG-RAN WG4 Meeting #117						         	       		R4-2522168

Dallas, USA, Nov. 17-21, 2025

Title: 	On UE PA and transmitter model for 6G RAN1 and RAN4 studies

Source: 	Skyworks Solutions, Inc.

Agenda Item:	8.2.1	Waveform [FS_6G_Radio]

Document for:	Approval

# 1	Background

In RAN#116bis, a way forward [1] captured several guidance and agreements for the 6G system parameters study. In his contribution, we discuss the required level of PA and transmitter model complexity needed for 6G evaluations in RAN1 and RAN4 and provide our recommendations.

# Discussion

## 2.1 Way forward input

In RAN4#116bis, a way forward [1] captured several guidance and agreements for the 6G system parameter study, the parts relevant to waveform and PA modelling aspects are copied here in italics.

Waveforms

- The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1

- To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects

  - Tx assumption including PA model

  - Related RF requirements which should be taken into consideration

  - Both UL and DL are considered

  - Take the candidate waveforms identified/discussed in RAN1 as the baseline

  - Identify the evaluation metric, e.g. net gain

  - Implementation constraints

- Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

  - Evaluation cases could be selected step-by-step, based on RAN1 progress

  - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

PA model

- Consider PA modelling at least for the following aspects for RAN4 discussion

  - Memory effects for UE supporting larger CBW

  - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3)

  - Calibration conditions

  - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc.

  - PA models for different device types, handheld UE is prioritized

  - Applicable requirements

    - 5G-A requirements as starting point

  - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled

In this contribution we will cover the aspects highlighted in yellow and more specifically limitations in PA modelling and simulations.

## 2.2 UE waveforms evaluations in RAN1 and RAN4

In our understanding, the use of simulations for waveform evaluation in RAN1 and RAN4 have different objectives and thus different requirements in terms of PA modelling.

PA model for RAN1

RAN1 evaluates the benefit of different modulations and waveforms by comparing their attributes in terms of in band performance while keeping an eye on OOB performance.

- However, OOB performance can only be done fully by RAN4 as OOB requirements, SU, other impairments than PA non-linearity like transmission BW, PA efficiency enhancement techniques impact, impact of PA coupling for muti-Tx cases need to be taken into account.

- For 6G UE Tx the focus in RAN1 is low PAPR waveforms, 1024QAM and ISAC waveforms.

- The simulation is static as power control is not used for assessing waveforms, only a simple backoff is applied.

Based on this, the PA simulations for RAN1 should:

- Properly represent the PA saturation characteristics for low PAPR waveforms

- Properly model the in-band distortions for high order modulations

- Include proper IQ impairments and LO phase noise for high order modulations

- Use proper calibration point to ensure results are comparable

- Be representative of PAs up to at least 15GHz

In our view, for the above criteria, using properly calibrated AM/AM and AM/PM based PA models is sufficient, any higher complexity in the modelling will be difficult to align and thus only create doubts in comparing results. Especially, memory effects should not dominate in band as it mostly creates imbalance in the upper and lower frequency OOB emissions and only results in significant errors for narrow allocations.

Nevertheless, some caution should be taken when interpreting the results: some AM/AM AM/PM models are not extracted far enough in saturation and thus do not represent properly the signal is clipping, but also it does not demonstrate the steep AM/PM that occurs in deep PA saturation. This is not to mention that in some cases when the PA is saturated too far, the output power may actually reduce with increased input power.

For accurate PA calibrations, the usual RAN4 calibration is still valid for AM/AM and AM/PM models as they do not have any bandwidth or carrier frequency limitations. A simple AM/PM curve scaling may prove adequate.

In order to account for the frequency of operation, the main parameter to adjust is the AM/PM curve rather than the AM/AM curve as the AM/PM distortion increases with frequency as the transistor technology limitations start to show.

Finally, for IQ impairments, RAN1, can simply assume an IQ level that is in proportion to the desired in band SNR for a given modulation order. For example, by assuming a 6 to 10dB better IQ impairment than the target SMR for a given modulation order. For LO phase noise, scaling with frequency of operation is a valid approach.

Proposal for 6G UE PA modelling for RAN1 6G evaluation:

- AM/AM and AM/PM based PA models are adequate for RAN1 evaluations. The addition of memory effects may only blur the comparison of results for the evaluation of waveforms.

- AM/AM and AM/PM curves must be extracted far enough into compression to properly evaluate low PAPR waveforms

- Increasing the AM/PM curve distortion with frequency by simple scaling may prove sufficient to represent limitations linked to the frequency of operation

- For high order QAM evaluation adding IQ imbalance contribution at a 6 to 10dB lower than the targeted in band SNR is a simple approach.

- PA calibration is more important than the PA model accuracy to allow fair comparisons and the current RAN4 5G PC3 calibration point is sufficient for waveform comparisons even if improved linearity/MPR and higher power classes is foreseen in 6G as it would apply equally to all waveforms. The calibration point to be verified without any linearization technique applied is MPR1 at 30dB ACLR with a 20MHz QPSK DFT-s-OFDM 100RB0 waveform.

  - For 1024QAM assessment, proper back should be found to reach the targeted SNR

  - Conversely, for low PAPR waveforms, proper boosting should be found to reach the targeted EVEM and in-band and out-of-band emissions.

PA model for RAN4

Before we discuss the UE PA model attributes for 6G evaluations in RAN4, it is important to set the baseline target performance.

In our UE discussions, we propose that UL256QAM should be the baseline target, at least up to 5GHz, and thus IQ impairments of 34dB can be assume for any waveform evaluation other than UL1024QAM, where improved IQ performance should first be agreed in the UE thread together with LO phase noise and digital+analog BB SNR.

For RAN4 simulations, there has been proposal to account for:

- Memory effects

- ET/APT operation

- Use of CFR and DPD

From our experience of various ET and APT implementations with various DPD algorithms, bandwidth and training strategies (number of frequency points, of waveforms, power levels…), the overall performance when using the same PA can vary significantly over dynamic range, frequency and temperature so the system performance should not be dimension based on ideal linearization behaviour.

Based on this experience, we suggest that a state of the art and reasonable SNR for the PA+ET+DPD can target up to 38dB at least for the upper 20dB of output power. For large targeted SNR like for 1024QAM, the PA backoff must be high enough to ensure enough intrinsic PA linearity to maintain good performance with unavoidable DPD errors or BW limitations and this needs to be maintained over a large power control/backoff range.

If this in essence will improve the performance at low back off levels compared to the traditional fixed bias backed off PA and 30dB ACLR, at larger backoffs, OOB emission will only improve dB/dB while the current RAN4 approach improves ~3dB/dB.

In our experience and measurements, at 3dB backoff a linearized ET/APT PA is equivalent to the current 3GGP fixed biased PA calibrated at 30dB ACLR, beyond this its OOB emissions are worse.

It should be noted that ET and DPD may create BW limitations for 6G UL as state-of-the-art implementations provide good transmit SNR improvements at 100MHz modulation BW and can probably be extended to 200MHz modulation BW but will have difficulties to go beyond while the PA AM/PM and memory effect will be more significant.

If memory effects are indeed a concern for wider BW support as it creates imbalance between the upper and lower OOB emissions, its effect is more acute on narrow RB allocations than fully allocated RBs. Because of this, the effect might be unique to a model and at a specific position in the spectrum due to AM/PM and memory effect combinations. If it is definitively useful to acknowledge the margins required to account for it (especially for A-MPR) it is unlikely that different PA models or PA measurements will exhibit the same behaviour.

Finally, after many attempts, we have not been able to properly model PA coupling in multi-Tx cases in its related RIMD as it is mainly a mixing phenomenon that has a different behaviour for ET, APT or fixed bias PAs. In the case of multi-stream MIMO, the signals are not correlated, so it is unlikely that DPD can efficiently combat that source of distortion.

All in all, from the above it seems unlikely to us that RAN4 can agree on a model that would:

- Incorporate ET/APT, CFR and DPD linearisation improved performance at low backoff but degraded

- Accounts for AM/AM, AM/PM and memory

- While also modelling multi-Tx PA coupling and related reverse IMD.

Furthermore, it will certainly blur comparisons as a proper calibration of the transmitter intrinsic linearity cannot be performed like we currently do with the MPR1 waveform and the details of the DPD performance/limitations is not likely to be disclosed.

All of the above may sound as a long list of bad news, but in fact diversity in linearisation techniques and PA models is a good thing for the opportunity to find issues and limitations. There would no worse thing than every company simulating the same thing and thus having the same results.

For all the above, we still think that the use of the current AM/PM based fixed bias PA model simulations and measurements should still be the workhorse in transmitter performance evaluation as it provides an unambiguous comparison through an agreed calibration waveform and requirement. For multi-Tx case and large backoffs at least one measurement should be provided in the set of results to derive requirements.

Nevertheless, companies should be encouraged to provide the impact (good or bad) of real-life limitations or improvements linked to PA efficiency management and linearization techniques or modelling of more complex phenomenon like memory effect and reverse IMD via modelling and simulation or measurements.

Still, the one thing to remember is that RAN4 evaluation is at best typical, and millions of devices needs to be produced and deliver over frequency, temperature, filter and/or antenna mismatch while ignoring some other contributors in the RF front-end: reasonable margin is essential.

Proposal for 6G UE PA modelling for RAN4 6G evaluation:

- RAN4 encourages the use of more elaborate PA models, PA efficiency enhancement and linearisation techniques in simulations to provide insights on potential improvements or limitations in transmitter performance.

  - However, it is unlikely that the parameters and characteristics of such approach is agreed as a reference in RAN4.

  - Finding an applicable linearity calibration level may prove difficult, blurring any comparison.

- Thus, AM/AM and AM/PM based PA models and lab measurements based on fixed bias PA should stay the reference for RAN4 evaluations and fair comparisons based on an agreed linearity level calibration.

  - Re-evaluating the calibration waveform, BW and MPR level for 6G is welcomed

  - Re-assessing the results in light of simulation of more elaborate PA models, PA efficiency enhancement and linearisation techniques is welcomed provided the results clearly state the intrinsic PA linearity, the techniques used and any known limitations in the result.

- For any UL modulation order other than 1025QAM, 256QAM support can be assumed with 34dB IQ impairments at least up to 5GHz, more discussions are need for up to 15GHz.

# Conclusion

In this contribution, we provide some guidance on PA modelling for 6G waveform evaluation in both RAN1 and RAN4.

Proposal for 6G UE PA modelling for RAN1 6G evaluation:

- AM/AM and AM/PM based PA models are adequate for RAN1 evaluations. The addition of memory effects may only blur the comparison of results for the evaluation of waveforms.

- AM/AM and AM/PM curves must be extracted far enough into compression to properly evaluate low PAPR waveforms

- Increasing the AM/PM curve distortion with frequency by simple scaling may prove sufficient to represent limitations linked to the frequency of operation

- For high order QAM evaluation adding IQ imbalance contribution at a 6 to 10dB lower than the targeted in band SNR is a simple approach.

- PA calibration is more important than the PA model accuracy to allow fair comparisons and the current RAN4 5G PC3 calibration point is sufficient for waveform comparisons even if improved linearity/MPR and higher power classes is foreseen in 6G as it would apply equally to all waveforms. The calibration point to be verified without any linearization technique applied is MPR1 at 30dB ACLR with a 20MHz QPSK DFT-s-OFDM 100RB0 waveform.

  - For 1024QAM assessment, proper back should be found to reach the targeted SNR

  - Conversely, for low PAPR waveforms, proper boosting should be found to reach the targeted EVEM and in-band and out-of-band emissions.

Proposal for 6G UE PA modelling for RAN4 6G evaluation:

- RAN4 encourages the use of more elaborate PA models, PA efficiency enhancement and linearisation techniques in simulations to provide insights on potential improvements or limitations in transmitter performance.

  - However, it is unlikely that the parameters and characteristics of such approach is agreed as a reference in RAN4.

  - Finding an applicable linearity calibration level may prove difficult, blurring any comparison.

- Thus, AM/AM and AM/PM based PA models and lab measurements based on fixed bias PA should stay the reference for RAN4 evaluations and fair comparisons based on an agreed linearity level calibration.

  - Re-evaluating the calibration waveform, BW and MPR level for 6G is welcomed

  - Re-assessing the results in light of simulation of more elaborate PA models, PA efficiency enhancement and linearisation techniques is welcomed provided the results clearly state the intrinsic PA linearity, the techniques used and any known limitations in the result.

- For any UL modulation order other than 1025QAM, 256QAM support can be assumed with 34dB IQ impairments at least up to 5GHz, more discussions are need for up to 15GHz.

# References

[1] R4-2514641 WF on [116bis][101] 6G system parameter, Huawei RAN4#116b