# 3GPP TSG-RAN WG4 Meeting #117	R4-2522175

# Dallas, USA, Nov. 17-21, 2025

Title: 	Concluding on remaining issues on the preferred OTA test methods and TRP simplifications

Agenda Item:	6.8.4.2.2

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

In this contribution we provide suggestions on how to conclude on remaining issues on the preferred OTA test methods and TRP simplifications.

# 2	Discussion

# 2.1	CR to TS 38.141-2

Based on content provided in R4-2514107 [2], we have reviewed all the proposed modifications and provided feedback in draft CR to TS 38.141-2 in [3]. That Draft CR provides further refinements to the TRP annex modifications, focusing on the General part, and pre-scan method corrections. Still, any modification to the pre-scan is seen as applicable to the previous releases as well. Therefore, it is not clear if any modifications to the pre-scan shall be captured in Rel-19, or to Rel-15 under Maintenance agenda.

Other modifications proposed in R4-2514107 [2] are considered to belong to the Maintenance agenda and not NR_BS_RF_req_evo WI.

Proposal 1: For the TRP annex modifications, focus at least on the General part of the Annex; most of other proposed corrections belong to Maintenance agenda (Rel-15 onwards).

Proposal 2: For the TRP annex modifications on the pre-scan method, first decide if those modifications shall be capture in Rel-19, it rather from Rel-15 onwards (under Maintenance agenda).

Within the pre-scan method, one of the modifications proposed in [2] was introduction of the sentence on noise floor level limit, with the value of 30 dB below the specified TRP limit. However, to derive that limit, the TRP value would need to be known first. Therefore, it is not clear how that threshold is supposed to be derived. More discussion maybe necessary. Otherwise, that modifications does not seem to be mature enough right now and is proposed to be dropped.

# 2.2	CR to TS 37.145-2

During RAN4#116bis meeting, a Draft CR in R4-2515074 [1] was Endorsed. It has captured the preferred test method corrections. There were no modifications captured to the TRP annex. For this meeting, we have updated draft CR in [1] also include the proposed improvements to the TRP annex, based on discussion in clause 2.1.

# 2.3	Consideration of the ERC Recommendation 74-01

During RAN4#116bis discussion, it was proposed to reuse text from ERC recommendation 74-01 for the selection of proper measurement methodology. Namely the following highlighted notes from Table 6 in Annex 2 of ERC Recommendation 74-01 were proposed to be reused:

However, when reviewed, it was found that the source text was somehow modified. To ease reaching consensus, it is suggested to reuse text from ERC recommendation 74-01 in a transparent manner, with no additional modifications made.

Proposal 3: For the General section of the TRP annex, reuse guidance text for the the selection of proper measurement methodology from Table 6 in Annex 2 of ERC Recommendation 74-01, with no additional modifications introduced.

Furthermore, the above proposed 74-01 text talks about the so-called systematic error. Before we introduce the above proposed 74-01 text into RAN4 spec, it shall be clarified whether the systematic error is the same as the already defined ΔTRP correction factor.

Proposal 4: Clarified whether the “systematic error” term used in ERC Recommendation 74-01 is the same as the ΔTRP correction factor already used in the TRP annex.

# 2.2	LS to ETSI MSG TFES

The following information was captured in [6]:

We were drawing attention to the EC feedback related to the alternative test methods, which was part of the discussion on the TRP simplification topic during previous meetings. The above referred feedback said that for ETSI Harmonized Standard, a preferred test method now always needs to be identified – otherwise, the Harmonized Standard may not be listed in the Official Journal (OJ). Therefore, this issue was of high importance to all companies who care about efficient use of ETSI Harmonized Standards for products conformance testing and their smooth introduction to the market.

RAN4 conformance testing specifications are the baseline for the ETSI MSG TFES work on Harmonized Standards. Therefore, it was advised that RAN4 seriously considers the above EC feedback (otherwise, decisions on the preferred test methods for BS Harmonized Standards will have to be taken in ETSI MSG TFES, which is not seen as effective an approach).

As the “preferred” test method topic solution was Endorsed during last RAN4#116bis meeting in [1], and considering the ETSI MSG TFES timeline (AAS BS and NR BS Harmonized Standards for Rel-17 are about to be submitted to HAS review under ETSI Standardization Request Deliverable Approval Process (SRdAP) during January 2026 meeting), this is the last RAN4 meeting to inform TFES on fruitful conclusion on the “preferred” test method, so that it could be incorporated already in Rel-17 HS for AAS BS and NR BS.

Proposal 5: Send LS to ETSI MSG TFES, information that Rel-19 specifications TS 38.141-2 and TS 37.145-2 will incorporate agreements on the “preferred” test method for the TRP measurements.

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: For the TRP annex modifications, focus at least on the General part of the Annex; most of other proposed corrections belong to Maintenance agenda (Rel-15 onwards).

Proposal 2: For the TRP annex modifications on the pre-scan method, first decide if those modifications shall be capture in Rel-19, it rather from Rel-15 onwards (under Maintenance agenda).

Proposal 3: For the General section of the TRP annex, reuse guidance text for the the selection of proper measurement methodology from Table 6 in Annex 2 of ERC Recommendation 74-01, with no additional modifications introduced.

Proposal 4: Clarified whether the “systematic error” term used in ERC Recommendation 74-01 is the same as the ΔTRP correction factor already used in the TRP annex.

Proposal 5: Send LS to ETSI MSG TFES, information that Rel-19 specifications TS 38.141-2 and TS 37.145-2 will incorporate agreements on the “preferred” test method for the TRP measurements.

Related draft LS was submitted in [5].

# 4	References

[1]	R4-2515074		Draft CR to TS 37.145-2: implementation of the preferred test method solution, Huawei, RAN4#116bis

[2]	R4-2514107		CR to TS 38.141-2: Improvements on Annex I - TRP measurement procedures, Ericsson, RAN4#116

[3]	R4-2522176	Draft CR to TS 38.141-2: Improvements on Annex I on TRP measurement procedures, Huawei, RAN4#117

[4]	R4-2522177		Draft CR to TS 37.145-2: implementation of the preferred test method and TRP annex improvements, Huawei, RAN4#117

[5]	R4-2522178 	Draft LS to ETSI MSG TFES on implementation of the preferred test method for TRP measurements of the BS, Huawei, RAN4#117

[6]	TFES(25)83027, Open issues discussed with the European Commission – updates,  https://docbox.etsi.org/msg/tfes/05-CONTRIBUTIONS/2025/TFES(25)83027_Open_issues_discussed_with_the_European_Commission_-_updates.docx

[7]