# 3GPP TSG-RAN WG4 Meeting #117	R4-2522179

# Dallas, USA, Nov. 17-21, 2025

Title: 	Tx spurious emission co-existence simulation results

Agenda Item:	6.8.3.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

In RAN4#116bis the calibration results for the different contributions were compared and methodologies to find a suitable interference level for 6.4GHz were discussed, this is captured in the WF [2].

Whilst some progress was made aligning the simulators there remain a number of important open issues with respect to agreeing a suitable requirement.

# 2	Discussion

Some of the critical open issues identified in [1] are:

- Antenna / array gain considerations: FFS

- Correlation factor for OoB: FFS

- Performance metrics: CDF %-tile value to be used for the requirement derivation to be agreed at RAN4#117.

## 2.1	Antenna array gain / correlation factor

To some extent these 2 issues are related.

In this specific case we are looking at frequencies a long way away from the wanted signal i.e., 6.4 GHz with a 2 GHz interfering BS and a 3 GHz interfering BS. These frequencies have been selected because they are significantly separate from the victim but also because the operating bands in question potentially have 2nd and 3rd harmonics in the victim band.

Generally, a long way outside the operating band we would expect the emission to be non-harmonically related and noise-like, as such the correlation could be assumed to be 0. However, the harmonics are generated due to non-linearities generating multiples of the wanted signal. These harmonics may not have the same phase relationship as the fundamental signal however they potentially are correlated to the fundamental. Outside the obvious harmonics (2nd, 3rd etc.) it is also possible that spurious emissions are due to harmonics of clocks, digital interfaces etc mixed with the fundamental, these emissions may be correlated or at least partially correlated.

Hence it is difficult to have definitive answer on correlation, however as we are specifically looking at the harmonics in this case we should consider at least partial correlation.

As to the antenna model, whilst out of band performance is difficult to predict the current model is based on fixed element separation and as such can be easily used to look at out of band array performance.

Considering the sub-array model:

The separation between the elements (and sub-arrays) is a fixed physical distance based on the design frequency, for example an antenna with a 2 GHz design frequency and 0.7λ separation between elements, the elements will be separated by 10.5cm (in air). This separation will not change if the antenna is excited by a different frequency. When we are considering in-band performance we generally simplify so that the excitation frequency and the design frequency are the same. However, as we are now deliberately considering the out of band performance, this simplification is no longer valid.

Observation 1: The simplification that design frequency and excitation frequency are the same is not valid in this case.

Proposal 1: Use aggressor frequency as the design frequency and victim frequency as the excitation frequency.

In addition to the separation of the elements (which is clearly a fixed physical parameter) we also have the weighted phase shifts applied to the excitation signal to consider. How these behave out of band depends somewhat on how the phase shifts are generated for the wanted signal.

For the sub-arrays the phase shifts are applied in the RF in front of the antenna elements and can be considered fixed (or potentially slowly variable with a RET type system). In most cases as this is done after the PA with a passive delay line type shifter. In this case the phase shift would be linked to the length of the delay line and would be frequency dependent. For example, a 5° phase shift at 2 GHz would generate a 15° phase shift at 6 GHz. The sub-array model is capable of incorporating this modification quite simply.

Proposal 2: Use aggressor frequency as the sub-array phase shifter design frequency and victim frequency as the excitation frequency.

For the main array if we consider the phases shift is applied to the fundamental in the BB or low power RF, at this stage the harmonic signal does not exist. In a simplistic approach we can consider the harmonic is generated by the PA non-linearity. A PA will generate harmonics which will look like a power series, the 3rd harmonic for example will be represented as a3x3, as x is a complex vector when x is cubed the associated phase shift is multiplied by 3. For the antenna model as we equalise the radiation pattern to calculate directivity the amplitude component can be ignored but the modification to the phase shift to the weighted signals can easily be modelled.

Clearly there is a difference between the generation of the harmonic signal from the PA non-linearity and the scaling of the antenna physical parameters, the element separation and the phase shifter delay length are directly scalable by the ratio of the excitation frequency and the design frequency, however the generation of the harmonics (and associated phase shifts) only occurs at the harmonics of the excitation frequency (this can be linked to the discussion about the correlation). However, as we have specifically picked aggressor and victim systems to be harmonics, in this case the 2 are the same and we can use 3 (for 2 GHz) and 2 (for 3 GHz).

If the unwanted excitation frequency is considered to be non-correlated then the weighting vector phase shift is not relevant.

Proposal 3: Cube the wanted signal weighting vector to generate the harmonic excitation weighting vector.

Proposal 4: As victim frequency is 6.425 GHz, use 2.14 GHz as aggressor fundamental frequency.

The last part of the antenna model is the element pattern, the element is generally a resonant element, when excited with signal below its design frequency then it behaves like a short antenna and the radiation pattern tends to become wider, when exited by a higher frequency then it becomes narrower and eventually will split as it has multiple resonances, the split pattern may have nulls in the previous forward directions. However in reality element behaviour is complex and depends on patristics, mutual coupling and many other parameters which are difficult to model. The existing model is a generic model which is not intended to precisely simulate any particular element as such without any additional information it remains as suitable worst case.

Proposal 5: Continue using the existing element model.

## 2.2	Performance metrics

Existing limits are based primarily on worst case MCL analysis for the defined scenario with non-AAS (static) base stations. Preliminary AAS requirements used the same requirements based on the following assumptions:

- From in band co-existence studying ACLR we found that average AAS interference was similar for correlate and non-correlated systems and average radiated power followed the element pattern.

- Interference was directly related to TRP (or conducted power) not an instantaneous EIRP value.

- An AAS offered no more interference than a non-AAS system if the TRP was the same.

Based on these findings the existing co-existence limits was translated directly to a TRP requirement.

As technology is advancing AAS systems are more complex and often have larger antennas and greater beam steering ability than initially studied. Whilst this is thought not to affect the average performance significantly it can affect the worst case and low probability cases. In addition, new operating bands are sufficiently far away from the operating bands studies that potentially the assumption that in-band performance can be assumed a reasonable worst-case assumption for out of band performance come under question.

In this specific case we are looking at a 6.4 GHz victim which is potentially being interfered with by a 2 GHz or 3 GHz aggressor. This has a number of key differences to the existing analysis:

- The PL at 6.4 GHz is significantly higher,

- The victim is assumed to be an AAS with high gain (narrow beam) antenna (so it can operate on the same grid as the lower band systems),

- The aggressor also is assumed to have a high gain AAS antenna.

Because of these differences the scenario is no longer static and finding a worst case MCL is difficult, as such it has been suggested that we simulate a network and use the results to identify the worst case.

This opens up a number of issues the appropriate performance metric being a critical one.

In RAN4 we do a number of simulations where parameters are selected from CDF curves at different statistical values and it is often discussed what the correct value is, but it is difficult to decide on this figure unless the metric being analysed in understood.

In the current simulator calibration process we are comparing the coupling loss of every path between aggressor and victim BS including the aggressor and victim antenna gain and the path loss. This is perhaps a suitable method to find an MCL value which can be slotted into the methodology currently used to calculate the emissions requirement. However it is not really a true indication of the interference the victim BS is subjected to or how the performance of the victim BS is impacted.

If we consider the 19 site analysis we are using, there are 19*3=57 aggressor BS, it is the sum of the these which are interfering with the victim BS not just the largest. As the beam directions are dynamic the total interference is more likely to be affected by them than in the static case we considered when the requirement was initially derived.

In addition we are considering the victim BS to have a dynamic antenna pattern and using this to identify the worst case. However the dynamic nature of the victim not only affects the interferer but also the wanted signal. This was not the case when looking at a static system.

Observation 2: As the aggressor beam is variable the total interference is not directly related to the MCL.

If we look at the 8 worst case PL cases in figure 2.2-1:

Figure 2.2-1. Example of the 8 worst case UE placements (BS-BS, fundamental antenna model, rho=1)

The worst case MCL figures occur when the victim antenna is pointing at the aggressor in azimuth but also it can be noted they also occur when the UEs are quite close to the BS so the beam is not pointed at the aggressor in elevation.

It should be noted that the direction of the victim antenna and the level of the wanted signal are directly related to each other and also the level of the interference is directly related to the direction of the victim antenna. Hence the level of the wanted signal and the level of the interferer are directly related. For example if the victim system UE was at the edge of the cell then it would also not be receiving the worst-case interferer. The dynamic nature of the victim system must be applied to both the interference but also the wanted signal.

Observation 3: The level of the wanted signal and the level of the aggressor are directly related.

It seems clear that analysis and simulation of a fully dynamic aggressor and victim networks cannot directly fit into the current MCL approach.

Proposal 6: MCL is not an appropriate metric to represent the interference of a dynamic AAS to AAS network.

As such if MCL is used as a metric then only beam pointing cases where the wanted signal is at a low level should be considered, or the extreme cases should be discounted by selecting a probability lower than 100%.

Observation 4: If MCL is used as a metric then only beam pointing cases where the wanted signal is at a low level should be considered or a lower percentage probability should be selected.

### 2.2.1 Throughput loss

As it is not possible to separate the level of the interferer from the level of the wanted signal we could consider a metric which includes both the wanted signal and the interference level. Potentially SNR (Eb/No) level, degradation of SNR or throughput loss (which is derived from SNR).

In SNIR simulations for identifying in-band adjacent channel performance we use throughput degradation as a metric which is perhaps ultimately the most important parameter. In these simulations we consider:

Average 5% throughput loss -indicating the effect on total network throughput,

5%-tile 5% throughput loss – indicating the impact for low SNR (edge of cell) UE’s.

In this case as we are considering BS-BS interference so perhaps the 5%-tile (or lower) is a more appropriate metric.

How the noise component of the interferer for an Eb/No or throughput loss calculation is done has not been discussed, there are a number of options:

- Consider the co-existence BS aggressor interference and the victim BS thermal noise,

- Consider the co-existence BS aggressor interference on top of the existing ACIR interference,

- Consider the co-existence BS aggressor interference, the ACIR interference and the co-channel interference.

The most sensitive to the effect of the co-existence BS aggressor interference will be the 1st one where only the noise and the BS BF is considered (this also has some similarity to the existing MCL approach where only the desensitisation due to the BS interference is considered). This is also the simplest addition to the co-existence simulation so can be done more easily in the short time. However we are open to discussing a different baseline case.

The simulation is updated to include the signal from the victim network UE to the victim BS. The UMa path loss model is used and power control applied to the UE (as per TR 38.922 – 6.1.2.5). As such the wanted signal level can be recorded for each drop and the signal to noise ratio calculated. The reference SNR is based on the victim BS NF alone.

The following plots are using the 2GHz aggressor with the fundamental antenna model to maintain consistency with the calibration case. This is for calibration only when discussing results we have used the harmonic antenna model.

Figure 2.2.1-1. UL wanted signal power, total interferer power and SNR CDF (Aggressor BS, 2GHz, fundamental antenna model, Pspur = -52dBm/MHz)

From the SNR the throughput can be calculated, and the aggressor power level swept to see its effect on throughput degradation.

Sweeping the level of the aggressor BS interferer level and observing the effect on throughput in figure 2.2.1-2.

Figure 2.2.1-2. 5%-tile, 1%-tile and 0.1%-tile throughput loss, BS-BS (Aggressor BS, 2GHz, fundamental antenna model)

The current spurious emissions power level is -52dBm/MHz, it can be seen that at this level there is no throughput loss at all even at the 0.1%-tile. If 5% throughput loss at the 5%-tile is used then then there is >30dB margin in the current level.

Proposal 7: Use a low percentile throughput degradation metric to find the suitable interference level requirement for the aggressor BS.

## 2.3	Results

As the BS-BS has shown itself to be the worst case (BS-UE seems to display a lot of margin using current simulation assumptions) we have concentrated on the BS-BS analysis.

### 2.3.1	BS-BS

#### 2.3.1.1	Calibration

The latest calibration data set shows that the 4 CDF curves from contributing companies are very similar.

As we are most interested in the low probability end of the CDF we can see there is approximately 2dB difference at 99% (approx. 5dB @50%).

Figure 2.3.1.1-1 BS-BS, pathloss plus correlated antenna gain

#### 2.3.1.2	Simulation

Looking at the results for the 2 GHz (with 3rd harmonic) and 3 GHz (with 2nd harmonic) we have the throughput loss and MCL values in table 2.3.1.2-1:

Table 2.3.1.2-1: Throughput loss (and MCL) results (2k drops)

| Aggressor Freq (GHz) |  | 2 |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| antenna model |  | Harmonic |  |  | Harmonic |  |  |
| Metric |  | correlation |  |  | correlation |  |  |
| type | Percentile | 1 | 0.5 | 0 | 1 | 0.5 | 0 |
| 5% TP Loss | Average | -4 | -9 | -9 | -10 | -9 | -9 |
|  | 5% | -15 | -20 | -20 | -20 | -19 | -19 |
|  | 1% | -26 | -30 | -30 | -30 | -28 | -28 |
|  | 0.10% | -37 | -40 | -40 | -43 | -38 | -38 |
| MCL (dB) | 99 | -98 | -91.4 | -91.4 | -91.4 | -88.6 | -88.6 |
|  | 99.9 | -80.6 | -76 | -76 | -75.1 | -75.8 | -76 |
|  | 100 | -51.4 | -64.2 | -64.1 | -53.7 | -66.6 | -67 |

Note the worst case MCL results require a larger number of drops to be stable so may show some deviation.

If we consider the current emission level is -52 dBm and the current MCL assumption is 67 dB, it is easier to compare the results by looking at the margin over the current specification.

Table 2.3.1.2-2: Throughput loss (and MCL) margin (2k drops)

| Aggressor Freq (GHz) |  | 2 |  |  | 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| antenna model |  | Harmonic |  |  | Harmonic |  |  |
| Metric |  | correlation |  |  | correlation |  |  |
| type | Percentile | 1 | 0.5 | 0 | 1 | 0.5 | 0 |
| 5% TP Loss | Average | 48 | 43 | 43 | 42 | 43 | 43 |
|  | 5% | 37 | 32 | 32 | 32 | 33 | 33 |
|  | 1% | 26 | 22 | 22 | 22 | 24 | 24 |
|  | 0.10% | 15 | 12 | 12 | 9 | 14 | 14 |
| MCL (dB) | 99 | 31 | 24.4 | 24.4 | 24.4 | 21.6 | 21.6 |
|  | 99.9 | 13.6 | 9 | 9 | 8.1 | 8.8 | 9 |
|  | 100 | -15.6 | -2.8 | -2.9 | -13.3 | -0.4 | 0 |

As discussed, the worst case (100%) MCL results are not realistic as they cannot occur at the same time as a low power level wanted signal.

The usual metric used for outlier cases is the 5%-tile however in this case it can be seen there are huge margins even with this metric, the 1%-tile and even 0.1%-tile also show significant margin.

Observation 5: Using even very strict throughput loss metrics at least 9 dB margin can be seen over the current requirement.

It can be noted that a similar level of margin is achieved if the 99.9% MCL metric is used.

It seems that when the actual performance of the dynamic AAS networks are considered there is significant margin in the existing spurious emissions levels.

## 2.4	Discussion on the original issue

The original task was to answer if the higher PL found at 6.4GHz had any effect on the co-existence in the same geographical area spurious emissions requirements.

In order to answer that question it became necessary to consider methodology used to derive the original requirement and apply that to the higher frequency use case.

In order to do this it was necessary to consider that both the aggressor network and the victim network were based on dynamic AAS deployments and it has become clear that it is this as much as the frequency which leads to the current static MCL approach being unsuitable to derive the requirement.

Whilst the path loss due to frequency increases with frequency in order to maintain the same deployment grid it has become necessary to use AAS systems where the antenna gain can overcome the increased pathloss, in general the link budget is maintained so when looking at worst case antenna gain and worst-case path loss we derive results which are comparable with the existing specifications (in some cases as AAS is used to enhance the link budget the worst case is actually higher). However this performance is achieved by using the dynamic nature of the AAS, it is clear that the worst-case interference level and the worst case wanted signal cannot occur at the same time, leading to the MCL approach (which assumes all wanted signals are worst case) providing incorrect results.

So, the answer to the original question is that the higher frequency and frequency based PL do not directly lead to a different requirement, however the dynamic AAS networks required to operate at higher frequencies do lead to a different requirement.

Observation 6: The higher frequency and frequency-based PL do not directly lead to a different requirement, however the dynamic AAS networks required to operate at higher frequencies do lead to a different requirement.

Proposal 8: Based on initial throughput loss simulations the current emissions levels do appear to be over specified. As we consider higher frequencies and potentially higher gain AAS systems we should develop a methodology to correctly set these limits based on throughput degradation.

# 3	Summary

Considering the open issues in the WF for the issue of the antenna model and the correlation factor we have the following observations and proposals

Observation 1: the simplification that design frequency and excitation frequency are the same is not valid in this case.

Proposal 1: Use aggressor frequency as the design frequency and victim frequency as the excitation frequency

Proposal 2: Use aggressor frequency as the sub-array phase shifter design frequency and victim frequency as the excitation frequency.

Proposal 3: Cube the wanted signal weighting vector to generate the harmonic excitation weighting vector.

(simply multiple the phase shifts by 3 in the model if this is more practical)

Proposal 4: As victim frequency is 6.425GHz use 2.14GHz as aggressor fundamental frequency.

Proposal 5: continue using the existing element model.

On the issue of performance metrics we find:

Observation 2: As the aggressor beam is variable the total interference is not directly related to the MCL.

Observation 3: the level of the wanted signal and the level of the aggressor are directly related.

Proposal 6: MCL is not an appropriate metric to represent the interference of a dynamic AAS to AAS network

Observation 4: If MCL is used as a metric then only beam pointing cases where the wanted signal is at a low level should be considered or a lower percentage probability should be selected.

Proposal 7: Use a low percentile throughput degradation metric to find the suitable interference level requirement for the aggressor BS.

Based on observation 7 simulations were conducted based on the harmonic antenna model for 2GHz (3rd harmonic) and 3GHz (2nd harmonic) systems, the following was found:

Observation 5: Using even very strict throughput loss metrics at least 9dB margin can be seen over the current requirement.

Finally it is necessary to consider the original goal of this task which was to answer a question about the validity of the emission level limits at higher frequencies where we find:

Observation 6: the higher frequency and frequency-based PL do not directly lead to a different requirement, however the dynamic AAS networks required to operate at higher frequencies do lead to a different requirement.

Proposal 8: Based on initial throughput loss simulations the current emissions levels do appear to be over specified as we consider higher frequencies and potentially higher gain AAS systems we should develop a methodology to correctly set these limits based n throughput degradation.

# 4	References

[1]	R4-2515171 - Way Forward for [116bis][305] NR_BS_RF_Part1_SE_CLTA Huawei

[2]	Simulation assumptions for Coexistence spurious emission requirements_R4	 ZTE