# 3GPP TSG-RAN WG4 Meeting #117	R4-2522187

# Dallas, USA, Nov. 17-21, 2025

Title: 	Introduction of 6MHz channel bandwidth: BS RX aspects

Agenda Item:	7.1.5.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

Rel-20 WI on UE RF enhancements for NR FR1 (Phase 5) in [1] includes objectives on 6 MHz channel bandwidth, as extracted below:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

In this contribution we provide further discussion on introduction of 6MHz channel bandwidth, with the focus on BS RX requirements as per WF in R4-2515084.

# Discussion

# 2.1	BS RF requirements: ACS

For ACS requirement, despite adding a new channel BW value, there is need to define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. Based on the pre-defined formula captured in the TR 38.817-2, the following was derived:

Table 8: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 6 | ±2.5125 |  |
| 7 | ±2.5075 |  |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

Proposal 1: For ACS, define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as ±2.5125 MHz.

# BS RF requirements: ICS

As discussed in R4-2507437, selection of the RB number for the Interfering signal should consider TS 38.211 specification, where the range of RB values for the PUSCH is limited to a predefined set which is due to the DFT process principles. The bandwidth of the PUSCH in terms of resource blocks () shall fulfil the following equation:

where  is a set of non-negative integers.

For our discussion, and limiting it by the interferer RB values for 5MHz and 10MHz wanted channels (i.e., 10RB and 25RB, respectively), we can derive the range of allowed values to the following:

MRBPUSCH=[10,12,15,16,18,20,24,25]

Currently, the interfering signal for 5 MHz wanted signal is set to 10 RB, while for 7 MHz wanted signal the interfering signal is set to 15 RB. Therefore, for 6 MHz wanted signal it may be reasonable to select value of 12RB for the interfering signal.

Proposal 2: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [12RB].

Depending on the final decision, the ICS requirement for 6 MHz channel bandwidth is expected to look as presented in table 9:

Table 9: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 6 | 15 | G-FR1-A1-7 | -100.6 | [-81.4] [-80.6] [-79.6] | DFT-s-OFDM NR signal, 15 kHz SCS, [10, 12, or 15] RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 13: For ACS, define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as ±2.5125 MHz.

Proposal 14: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [12RB].

# 4	References

[1]	RP-252942	UE RF enhancements for NR FR1, Phase 5, WID

[2]	R4-2502870	WF on requirements for 7MHz channel bandwidth, RAN4#114