3GPP TSG-RAN WG4 Meeting #117	R4-2522200

Dallas, USA, Nov. 17-21, 2025

Agenda item:	6.18.2

Source:	Qualcomm Incorporated

Title: 		RRM performance requirements for XR enhancements

Document for:	Discussion

# Introduction

RAN4 completed the core part of the Work Item (WI) on XR enhancements during the last meeting [1]. The key enhancement introduced in this WI is the extension of cell detection and measurement delay requirements, facilitated by DCI-based Measurement Gap (MG) cancellation.

During the last meeting, RAN4 made a good progress and achieved many agreements on the list of test-cases, test configurations and parameters etc. In this contribution, we provide our views on some open issues for the performance part.

# Test-cases list

In the last meeting, RAN4 agreed to define test case for:

- Inter-frequency measurements procedure with gaps

- Define test case for: Inter-frequency L1-RSRP measurements procedure with MG for FR1 and FR2-1

- FFS whether to define at least one accuracy test case in FR1 and in FR2-1

We believe that defining measurement accuracy test cases for this scenario is unnecessary, since the measurement gap cancellation feature only impacts delay requirements, not accuracy requirements. It should be noted that all measurement accuracy requirements are tested under AWGN channel conditions. Therefore, any potential degradation in measurement accuracy due to extended delays would not be captured in the test environment. As a result, defining accuracy test cases in this context would not yield meaningful insights and may be redundant.

Observation 1: RAN4 has not defined any new measurement accuracy requirements for the MG cancellation feature.

Observation 2: Measurement accuracy tests are conducted under AWGN conditions and cannot reflect potential degradation caused by extended measurement delays.

Proposal 1: RAN4 should not introduce measurement accuracy test cases for the measurement gap cancellation feature, as they are unlikely to provide useful validation.

# Cancelation ratio and DCI aspects for tests

During the last RAN4 meeting, there was discussion on how many measurement gaps (MGs) should be cancelled in the test cases—whether to cancel one or two gaps, or to adopt a gap cancellation ratio (e.g., 20%, 40%, 50%).

We believe the primary objective of the work item is to verify whether the UE can respond in a timely manner to the gap cancellation DCI and successfully cancel the next scheduled measurement gap, while still meeting the extended measurement delay requirements.

To achieve this, cancelling one or at most two measurement gaps is sufficient. Since most measurement delay requirements rely on multiple samples across multiple gap occasions, the Test Equipment (TE) can randomly choose to cancel one or two gap occasions with equal probability. This approach balances test coverage with practical feasibility, as scenarios involving more than two cancellations would significantly increase test duration and complexity.

Observation 3: A large number of gap cancellations increases test duration and complexity.

Proposal 2: RAN4 should define test cases with a single or at most two MG cancellations.

Proposal 3: In the test cases, the TE may randomly cancel one or two MG occasions with equal probability.

Regarding the verification of the time offset between the gap cancellation DCI and the measurement gap, we propose that test cases be defined using a 5 ms offset only. This is because 5 ms is the baseline requirement and 3 ms is an optional UE capability, not a minimum requirement.

Proposal 4: Define test case using 5ms offset only.

# Test-case design

Use incremental test-case design approach to specify the new test cases, where only additional parameters and configurations relative to the baseline test case are defined, along with the corresponding test requirements. This approach has been adopted in other WIs and has proven to be effective in minimizing redundancy, simplifying documentation, and facilitating easier interpretation and implementation of test cases.

Proposal 5: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

# Conclusion

Observation 1: RAN4 has not defined any new measurement accuracy requirements for the MG cancellation feature.

Observation 2: Measurement accuracy tests are conducted under AWGN conditions and cannot reflect potential degradation caused by extended measurement delays.

Proposal 1: RAN4 should not introduce measurement accuracy test cases for the measurement gap cancellation feature, as they are unlikely to provide useful validation.

Observation 3: A large number of gap cancellations increases test duration and complexity.

Proposal 2: RAN4 should define test cases with a single or at most two MG cancellations.

Proposal 3: In the test cases, the TE may randomly cancel one or two MG occasions with equal probability.

Proposal 4: Define test case using 5ms offset only.

Proposal 5: RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

# References

[1] R4-2512144, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#116