3GPP TSG-RAN WG4 Meeting #117	R4-2522655

Dallas, USA, Nov. 17-21, 2025

Title: 	Way Forward for [117][224] NR_ENDC_RF_Ph4_Demod_6Rx

Agenda Item:	6.1.1

Source: 	MediaTek, Huawei, HiSilicon

Document for:	Approval

# 1 General

Issue 1-1: How to explicitly capture the capability rules that “6 layers requirements are only applicable for FWA UE

- Proposals:

  - Option 1: Capture only in the test applicability section (section 5.1.1) of TS 38.101-4. Use the following as the starting point

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| Supported maximum number of 6 PDSCH MIMO layers for FWA UE (maxNumberMIMO-LayersPDSCH-v1900) | FR1 TDD | PDSCH | Clause xxx (new 6 Layer requirements) |  |

  - Option 2: Capture as a Note in the PDSCH requirement section (section 5.2)

    - Note: the requirement is applicable for FWA UE.

Issue 1-2: Applicability rules for UE supporting different Rx

- Agreement:

  - If UE supports both 8Rx and 6Rx, 8Rx tests shall be applied and 6Rx tests shall be skipped

  - If UE passes 6Rx tests, 4Rx with basic rank4 (Rel-15) test can be skipped

Issue 1-3: Specification structure

- Agreement

- Add a new clause for 6Rx after the current 8Rx performance requirements

# 2 PDSCH and SDR requirements

## 2.1 SDR requirements

Issue 2-1-1: MCS value for 64QAM with factor 1

- Agreement

  - MCS 26

Issue 2-1-2: MCS value for 256QAM with factor 1

- Agreement

  - MCS 25

Issue 2-1-3: MCS value for 1024QAM with factor 1

- Agreement

  - MCS23

## 2.2 PDSCH requirements

Issue 2-2-1: MIMO correlation for 6Rx 4 MIMO layers

- Agreement

- Confirm α = 0 and β = 0.07179

# 3 CQI requirements

Issue 3-1: SNR values selection for CQI requirements under static channel

- Agreement

- (5,6) and (11,12) dB