3GPP TSG-RAN WG4 Meeting #117	                                                        R4-2522667

Dallas, USA, November 17th – 21st, 2025

Agenda item:			7.12.1

Source:	Moderator (OPPO)

Title:	Ad-hoc minutes for NR_AIML_Mob_RRM

Document for:	Approval

Introduction

This document provides the ad-hoc discussions for [117][221] NR_AIML_Mob_RRM which includes the papers in agenda 7.12.2, 7.12.3, and 7.12.4.

The issues discussed in the ad-hoc:

- Issue 1-2-1

Topic #1: General aspects for AI mobility

Sub-topic 1-2: Simulation assumption for AI mobility

Sub-topic description:

In this sub-topic the company proposals about the simulation assumption for RRM measurement prediction and indirect measurement event prediction are summarized.

Issue 1-2-1: Simulation assumption for measurement prediction and [indirect measurement event prediction]

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - Use the simulation assumptions in R4-2521345 as baseline including SLS assumption, LLS assumption and simulation procedure.

    - Use Figure 9 as starting point for the simulation procedure of measurement prediction.

    - Figure 9: Illustration of simulation procedure for measurement prediction

  - Proposal 1a (QC):

    - RAN4 to consider the following changes from the simulation procedure suggested in R4-2513096:

      - In Step 1 and Step 2, L3-RSRP should be updated to L1-RSRP as the L3 filtering will be applied at Step 5.

      - In Step 7, whether to apply dataset filtering based on SINR condition should be left optional.

      - In Step 8, the dataset to be used for inference needs to be linked to 7th step.

  - Proposal 1b (MTK, ZTE, Nokia):

    - Use the SLS assumptions in Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744 as a basis.

  - Proposal 1c (Nokia):

    - For simulation assumptions, RAN4 to consider observation window (OW) length of 200ms and prediction window (PW) length of 100ms with a sampling period of 40ms for both FR1 and FR2.

    - For FR2, RAN4 to consider similar link-level simulation (LLS) parameters as used by RAN4 AIML BM for the BB error. For example, error model and corresponding values could be used similar as agreed in RAN4 AIML BM.

  - Proposal 1d (vivo):

    - For performance evaluation, the impact of OW length, PW length and UE speed need to be considered.

    - For MRRT and Skipping pattern for Case B, it depends on how RAN2 designs the SSB configuration to indicate the timing of the NW's SSB transmission.

    - For performance evaluation, RAN4 to consider the non-sliding window as the baseline, and the sample number reuses the sample number defined by the legacy L1 filtering.

  - Proposal 2 (Ericsson):

    - Do not define explicit simulation assumptions for indirect event prediction evaluations. Accuracy of predicted event(s) based on predicted RRM measurement(s) shall be considered as one of the performance metrics to be reported by companies in their simulation results.

- Recommended WF

  - For measurement prediction:

    - Use the SLS assumptions in RAN2 SI evaluation (i.e., Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744) as baseline.

    - Use the LLS assumption used in RAN4 AIML BM as baseline for BB error derivation. The detailed parameters can be further discussed.

    - Use the simulation procedures in R4-2521345 as baseline and accommodate the discussions on issue 1-2-2 and issue 1-2-6.

- Companies to generate ideal intra-frequency or inter-frequency SS-RSRP dataset from the SLS assumption. - Use the LLS assumptions to generate intra-frequency SS-RSRP difference as the baseband errors. - Use the Simulation assumptions for RF error to generate RF errors. - Add the measurement error into the ideal SLS dataset in 1st step to derive the dataset with measurement errors. - Both baseband errors and RF errors are included. - Measurement error assumptions follow 2nd step and 3rd step. - Add L3 filtering to the dataset with added measurement error generated from the 4th step. - Use subset samples of the dataset generated from the 5th step for training models. - Filter UEs in the other subset samples of the dataset with added measurement error (non-overlapped samples with the subset dataset in 6th step) based on SINR value (e.g., SINR>= -6dB as the starting point) to derive the dataset for inference. - Use the dataset with measurement error in 7th step for inference to evaluate the performance metrics (predicted L3-RSRP) assuming that the ground truth is with the measurement error. Note: Intra-frequency or inter-frequency SS-RSRP in 1st and 2nd step means the Cell level RSRP measurement without L3 filtering, which is same as that in RAN4 legacy SS-RSRP accuracy requirements.

Agreements:

- Companies to generate ideal L1-RSRP dataset from the SLS assumption. - Note: The dataset is filtered based on SINR value (i.e., -6dB <= SINR). - Use the LLS assumptions to generate L1-RSRP difference as the baseband errors [based on one shot measurement]. - Use the RF error model assumption to generate RF errors. - Note: this is based on option A for RF error addition. If using option B, the RF error will be added after L3 filtering in 5th step. - Add the measurement error into the ideal SLS dataset in 1st step to derive the dataset with measurement errors. - Both baseband errors and RF errors are included. - Measurement error assumptions follow 2nd step and 3rd step. - Note: this is based on option A for RF error addition. If using option B, the RF error will be added after L3 filtering in 5th step. - Add L1 filtering and L3 filtering to the dataset with added measurement error generated from the 4th step. - Use subset samples of the dataset generated from the 5th step for training models. - Use the other subset samples of the dataset with added measurement error (non-overlapped samples with the subset dataset in 6th step) to derive the dataset for inference. - Use the dataset with measurement error in 7th step for inference to evaluate the performance metrics (predicted L3-RSRP) assuming that the ground truth is with the measurement error. Note: L1-RSRP in 1st and 2nd step means the L1 beam level RSRP measurement without L1 filtering and L3 filtering.

Agreement:

  - For measurement prediction:

    - Use the SLS assumptions in RAN2 SI evaluation (i.e., Table 5.1-1, Table 5.3.1-1 and Table 5.3.1-2 in TR38.744) as baseline.

      - The options used in the simulation are up to companies report.

Tentative Agreement:

    - Use the following LLS assumption as baseline for BB error derivation.

| Parameter | Value for FR1 |
| --- | --- |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 30kHz |
| Channel bandwidth | 20MHz |
| Channel model | Fading channel - Baseline: - TDL-C, DS=100ns - Optional: - CDL-C, DS=100ns |
| BS antenna configurations | Same as SLS; |
| UE antenna configurations | Same as SLS; 2Rx is assumed |
| UE moving speed | 30,60,90, 120 km/h |
| Reference signal | SSB |
| DRX | No |
| Number of samples in L1 filtering | 1 sample |