3GPP TSG-WG4 Meeting #117	R4-2522752

# Dallas, USA,  November 17, 2025 - November 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on minimum requirement for DL MIMO for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce minimum requirements for minimum requirements for DL MIMO for ATG |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce minimum requirements for minimum requirements for DL MIMO for ATG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The demodulation requirement for DL MIMO for ATG is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.22, 5.2.2.2.23, 5.2.3.1.21, 5.2.3.2.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2521128 |  |  |  |  |  |  |  |  |

==============First change==============

##### 5.2.2.1.22	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.2.1.22-3, with the addition of test parameters in Table 5.2.2.1.22-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.1.22-1.

Table 5.2.2.1.22-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3. 2-1, 2-2 |

Table 5.2.2.1.22-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.22-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | AWGN | 220Hz | 2x2 | 70 | 6.0 |
| 1-2 | R.PDSCH.1-3.6 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | 13.6 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | 22.5 |

Table 5.2.2.1.22-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.x FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x2 | 70 | [16.8] |
| 2-2 | R.PDSCH.1-4.x FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x2 | 70 | [25.7] |

==============Next change==============

##### 5.2.2.2.23	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.2.2.23-3, with the addition of test parameters in Table 5.2.2.2.23-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.23-1.

Table 5.2.2.2.23-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2, 2-3, 2-4 |

Table 5.2.2.2.23-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | Test 1-1, 1-2, 1-3, 2-1, 2-2: 8 Test 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-3, 2-4: 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.2.2.23-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-2 | R.PDSCH.2-3.6 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | 22.6 |
| 1-4 | R.PDSCH.2-31.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-5 | R.PDSCH.2-32.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-6 | R.PDSCH.2-33.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 22.6 |
| 1-7 | R.PDSCH.2-40.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 6.0 |
| 1-8 | R.PDSCH.2-41.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 13.5 |
| 1-9 | R.PDSCH.2-42.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | 22.6 |

Table 5.2.2.2.23-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | [16.8] |
| 2-2 | TBD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x2 | 70 | [25.4] |
| 2-3 | TBD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | [16.8] |
| 2-4 | TBD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x2 | 70 | [25.4] |

==============Next change==============

##### 5.2.3.1.21	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.3.1.21-3, with the addition of test parameters in Table 5.2.3.1.21-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.1.21-1.

Table 5.2.3.1.21-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 2-1, 2-2 |

Table 5.2.3.1.21-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.3.1.21-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | AWGN | 220Hz | 2x4 | 70 | 3.1 |
| 1-2 | R.PDSCH.1-3.6 FDD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | 10.6 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | 19.6 |

Table 5.2.3.1.21-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBD | 10 / 15 | 64QAM, 0.65 | AWGN | 220Hz | 2x4 | 70 | [13.8] |
| 2-2 | TBD | 10 / 15 | 256QAM, 0.82 | AWGN | 220Hz | 2x4 | 70 | [22.7] |

==============Next change==============

##### 5.2.3.2.22	Minimum requirements for ATG

The performance requirements are specified in Table 5.2.3.2.22-3, with the addition of test parameters in Table 5.2.3.2.22-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.22-1.

Table 5.2.3.2.22-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE PDSCH performance under the ATG scenario | 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-1, 2-2, 2-3, 2-4 |

Table 5.2.3.2.22-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | Test 1-1, 1-2, 1-3, 2-1, 2-2: 8 Test 1-4, 1-5, 1-6, 1-7, 1-8, 1-9, 2-3, 2-4: 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Table 5.2.3.2.22-3: Minimum performance for ATG with rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-2.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-2 | R.PDSCH.2-3.6 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-3 | R.PDSCH.2-4.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | 19.6 |
| 1-4 | R.PDSCH.2-31.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-5 | R.PDSCH.2-32.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-6 | R.PDSCH.2-33.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 19.6 |
| 1-7 | R.PDSCH.2-40.1 TDD | 40 / 30 | 16QAM, 0.48 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 3.1 |
| 1-8 | R.PDSCH.2-41.1 TDD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 10.6 |
| 1-9 | R.PDSCH.2-42.1 TDD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | 19.6 |

Table 5.2.3.2.22-4: Minimum performance for ATG with rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Frequency offset | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | TBD | 40 / 30 | 64QAM, 0.65 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | [13.7] |
| 2-2 | TBD | 40 / 30 | 256QAM, 0.82 | FR1.30-1 | AWGN | 500Hz | 2x4 | 70 | [22.4] |
| 2-3 | TBD | 40 / 30 | 64QAM, 0.65 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | [13.7] |
| 2-4 | TBD | 40 / 30 | 256QAM, 0.82 | FR1.30-8 | AWGN | 500Hz | 2x4 | 70 | [22.4] |

==============End of change==============