3GPP TSG-RAN4 Meeting # 117	R4-2522844

# Dallas, USA, Nov.17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft Big CR for RRM performance requirements of NR MIMO Phase 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is the draft big CR to capture the endorsed draft CRs in RAN4#117: - R4-2522779, DraftCR TC for Intra-cell SSB-based UEIBM reporting in FR1, Nokia - R4-2522778, DraftCR on TC Event-2, FR1, Inter-cell SSB based, no window time based, mode-A (TC5), CMCC - R4-2522781, TC on Event-2, FR2, Intra-cell SSB based, no window time based, mode-A, Ericsson - R4-2522780, DraftCR on TC2 for Rel-19 MIMO UEIEBM CSI-RS based event-2, vivo - R4-2522777, Draft CR on test case of Event-2, FR2, Inter-cell SSB based, no window time based, mode-A, Samsung - R4-2522815, Draft CR for Mapping Tables for CJTC reporting, Apple |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the performance part of Rel-19 MIMO RRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance part of Rel-19 MIMO RRM are incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.4.X1 A.6.6.4.X2 A.6.6.4.X3 A.7.6.3.X1 A.7.6.3.X2 A.7.6.3.X3 10.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change R4-2522779==============

A.6.6.4.X1	Event Triggered Reporting for UE initiated beam management without eventDetectionTimeWindowLength-r19

A.6.6.4.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations in table A.6.6.4.X1.1-1.

Table A.6.6.4.X1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

A.6.6.4.X1.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.X1.2-1 and table A.6.6.4.X1.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A, and without eventDetectionTimeWindowLength-r19 configuration. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0 which is the current beam. During T1, SSB#1 is not detectable. At the beginning of T2, SSB#1 becomes stronger than the SSB#0 and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.X1.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| resourcesForChannelMeasurement | 1~3 |  | SSB#1 |
| eventThreshold-r19 | 1,2,3 | dB | 4 |
| reportTransmissionMode-r19 | 1,2,3 |  | ModeA |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | disabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | sl4, 3 |
|  | 3 |  | sl8, 4 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.X1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.4.X1.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, no later than 20 ms + X slots from the beginning of T2, where:

- Config 1 and 2: X = 5

- Config 3: X = 8

The UE shall not send event triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522779==============

A.6.6.4.X2	Event Triggered Reporting for UE initiated beam management with eventDetectionTimeWindowLength-r19

A.6.6.4.X2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management, when eventDetectionTimeWindowLength-r19 is configured. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations in table A.6.6.4.X2.1-1.

Table A.6.6.4.X2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

A.6.6.4.X2.2 	Test parameters

There is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.6.4.X2.2-1 and table A.6.6.4.X2.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A, and with eventDetectionTimeWindowLength-r19 configuration. The test consists of two successive time periods, with time duration of T1 and T2, respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During test period T1, the UE receives from SSB#0 which is the current beam. During T1, SSB#1 is not detectable. At the beginning of T2, SSB#1 becomes stronger than the SSB#0 and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.6.6.4.X2.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NPRB,c = 52 |
|  | 2 |  | 10: NPRB,c = 52 |
|  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| eventThreshold-r19 | 1,2,3 | dB | 4 |
| reportTransmissionMode | 1,2,3 |  | ModeA |
| eventDetectionTimeWindowLength-r19 | 1,2,3 | ms | 80 |
| eventInstanceCount-r19 | 1,2,3 |  | 3 |
| nrofReportedRS-UE-IBR | 1~3 |  | 1 |
| currentBeamReport | 1~3 |  | enabled |
| pucch-Resource-r19: periodicityAndOffset | 1,2 | slot | sl4, 3 |
|  | 3 |  | sl8, 4 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table A.6.6.4.X2.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.64 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -79.65 |
|  | 3 |  | -91.64 | -91.64 | -Infinity | -76.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -51.56 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -60.59 | -45.46 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 15 |
| NOTE 1: The resources for first uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.4.X2.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, no later than 60 ms + X slots from the beginning of T2, where:

- Config 1 and 2: X = 5

- Config 3: X = 8

The UE shall not send event triggered measurement reports as long as the reporting criteria with eventDetectionTimeWindowLength-r19 and eventInstanceCount-r19 according to the configuration in Table A.6.6.1.Y.2-1 is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522778==============

A.6.6.4.X3	Event triggered reporting for UE initiated beam management for UE configured with Inter-cell SSB based L1-RSRP measurement on FR1 when DRX is not used

A.6.6.4.X3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event based on L1-RSRP measurement . This test will partly verify the event triggered reporting for UE initiated beam management in clause 9.13.3.4 configured with event2-r19, modeA-r19 and without eventCountWindow-r19 configuration.

Table A.6.6.4.X3.1-1: Applicable NR configurations for inter-cell SSB based Event triggered reporting for UE initiated beam management in FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.6.6.4.X3.2	Test Parameters

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a cell with different PCI from Cell 1 (Cell 2). The test parameters are given in table A.6.6.x.1.2-1and table A.6.6.x.1.2-2 below.

SSB#0 is transmitted on Cell 1. SSB#1 is transmitted on Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on the SSB#1. SSB#0 is for current beam measurement and SSB#1 is for new beam measurement. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#1 starts transmission and the UE is configured for L1-RSRP measurement on SSB#1, and the RSRP configuration of SSB#1 is better than SSB#0 to trigger the event-2 reporting. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the the SSB#0 for Cell 1.

Table A.6.6.4.X3.2-1: General test parameters

| Parameter |  | Config | Unit | Value |
| --- | --- | --- | --- | --- |
| Active cell |  | 1~3 |  | Cell 1 |
| A cell with additional PCI |  | 1~3 |  | Cell 2 |
| RF Channel Number |  | 1~3 |  | 1: Cell 1 and Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 |
| Duplex mode |  | 1 |  | FDD |
|  |  | 2 |  | TDD |
|  |  | 3 |  | TDD |
| TDD Configuration |  | 1 |  | N/A |
|  |  | 2 |  | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NPRB,c = 52 |
|  |  | 2 |  | 10: NPRB,c = 52 |
|  |  | 3 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  |  | 2 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  |  | 3 |  | SSB.4 FR1 for Cell 1 and Cell 2 |
| OCNG Patterns |  | 1~3 |  | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 |
| TRS Configuration |  | 1 |  | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD |
| DRX configuration |  | 1~3 |  | off |
| eventTypeUE-IBR-r19 |  | 1~3 |  | event2-r19 |
| eventThreshold-r19 |  | 1~3 |  | 1 |
| reportTransmissionMode-r19 |  | 1~3 |  | modeA-r19 |
| currentBeamReport-r19 |  | 1~3 |  | enabled |
| Timing offset between two SSBs associated to different PCI |  | 1~3 |  | <CP |
| T1 |  | 1~3 | s | 5 |
| T2 |  | 1~3 | s | 1 |
| CP length |  | 1~3 |  | Normal |
| PUCCH-Resource-r19 | periodicityAndOffset | 1~2 |  | sl4, 3 |
|  | pucch-Cell |  |  | Cell 1 |
|  | periodicityAndOffset | 3 |  | sl8, 4 |
|  | pucch-Cell |  |  | Cell 1 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition |  | 1~3 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.6.6.4.X3.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 of Cell 1 |  | SSB#1 of Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15 kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 7 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -87.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -84.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 7 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.6.4.X3.3	Test Requirements

The UE shall send a first PUCCH message using the new UCI type, no later than 20 ms plus X slot from the beginning of time period T2.

- X is 4 slots for Config 1 and 2.

- X is 8 slots for Config 3.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522781==============

#### A.7.6.3.X1	Event Triggered Reporting for the UE initiated beam management

##### A.7.6.3.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes the correct reporting of event triggered L1-RSRP measurement for the UE initiated beam management. This test will partly verify the L1-RSRP reporting requirements in clause 9.5.3.4, with the testing configurations for NR cells in table A.7.6.3.X1.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Config 1 is used in this test, and it is given by, Config 1: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode.

##### A.7.6.3.X1.2	Test parameters

There is one cell in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.6.3.X1.2-1 and table A.7.6.3.X1.2-2 below.

In CSI resource configuration, UE is indicated to perform L1-RSRP measurement on the resourcesForChannelMeasurement and the resourcesForChannelMeasurement consists of SSB. In the CSI report configuration, UE is configured with event-triggered reporting, with the event 2 and Mode-A. The test consists of two successive time periods, with time duration of T1 and T2 respectively. The test has higher layer parameter timeRestrictionForChannelMeasurements configured.

During the T1, SSB 1 is not detectable. At the beginning of T2, SSB 1 becomes stronger than the SSB 0 by offset amount and the event entering condition is met.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

Table A.7.6.3.X1.2-1: General test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| SSB ARFCN |  | freq1 |
| Duplex mode |  | TDD |
| TDD Configuration |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 66 |
| PDSCH Reference measurement channel |  | SR.3.2 TDD |
|  |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD |
|  |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD |
|  |  | CCR.3.7 TDD |
| SSB configuration |  | SSB.1 FR2 |
| eventThreshold-r19 | dB | 4 |
| OCNG Patterns |  | OP.1 |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration |  | SMTC.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration |  | TCI.State.2 |
| DRX configuration |  | Off |
| reportConfigType |  | periodic |
| reportQuantity |  | ssb-Index-RSRP |
| Number of reported RS |  | 2 |
| L1-RSRP reporting period | slot | 320 |
| PUCCH-Cell |  | Cell 1 |
| periodicityAndOffset | slots | Sl5, 4 |
| T1 | s | 5 |
| T2 | s | 2 |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |
| Propagation condition |  | AWGN |

Table A.7.6.3.X1.2-2: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1 |  | Rough |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96 |  |  |  |
|  | 1 | dB | [-32.03] | [-32,03] | -Infinity | [-34.53] |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | [-92] |
| Io Note3 | 1 | dBm/95.04 MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1 | dB | 0 | 0 | -Infinity | 4 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.3.X1.3	Test Requirements

In the test, the UE shall send a first PUCCH message using the new UCI type, within 160ms plus 5 slots from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522780==============

A.7.6.3.X2	CSI-RS based UE-initiated/event-driven beam management of event2

A.7.6.3.X2.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of event2 UE initiated beam management. This test will partly verify the Event Triggered Reporting for the UE initiated beam management requirements in clause 9.5.3.4, with the testing configurations for NR cells in table A.7.6.3.X2.1-1.

Table A.7.6.3.X2.1-1: Applicable NR configurations for FR2 CSI-RS based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

A.7.6.3.X2.2	Test parameters

There is one cells in the test, the FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.6.3.X2.2-1 and table A.7.6.3.X2.2-2 below.

In CSI measurement and report configurations, CSI-ReportConfig is configured with the higher layer parameter eventType-r19 set to ‘event2, and newBeamResourceSet is configured as NZP-CSI-RS-ResourceSet configured with repetition.

The test consists of two time period T1 and T2. UE is also configured to measure L1-RSRP based on SSB.There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD based on the SSBs.

Table A.7.6.3.X2.2-1: General test parameters

| Parameter |  | Config | Unit | Value |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 |  | freq1 |
| Duplex mode |  | 1 |  | TDD |
| TDD Configuration |  | 1 |  | TDDConf.3.1 |
| BWchannel |  | 1 | MHz | 100: NPRB,c = 66 |
| PDSCH Reference measurement channel |  | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.3.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR2 |
| CSI-RS configuration |  | 1 |  | CSI-RS.3.5A TDD |
| OCNG Patterns |  | 1 |  | OP.1 |
| Initial BWP Configuration |  | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1 |  | SMTC.1 |
| TRS Configuration |  | 1 |  | TRS.2.1 TDD |
| TCI Configuration |  | 1 |  | DLorJoint TCI.State.0 |
| newBeamResourceSet |  | 1 |  | CSI-RS.3.5A TDD |
| DRX configuration |  | 1 |  | Off |
| reportQuantity |  | 1 |  | cri-RSRP |
| Number of reported RS |  | 1 |  | 2 |
| qcl-Info |  | 1 |  | SSB#0 for resource#0 |
|  |  |  |  | SSB#1 for resource#1 |
| Propagation condition |  | 1 |  | AWGN |
| reportTransmissionMode-r19 |  | 1 |  | ModeA |
| eventThreshold |  | 1 |  | 0 |
| PUCCH-Resource-r19 | periodicityAndOffset | 1 |  | sl5, 4 |
|  | pucch-Cell-r19 | 1 |  | Cell 2 |
| T1 |  | 1 | s | 1 |
| T2 |  | 1 | ms | 100ms |
| EPRE ratio of PSS to SSS |  | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.3.X2.2-1: CSI-RS specific test parameters

| Parameter | Config | Unit | CSI-RS#1 |  | Resource #4 in TRS resource set 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 | 1 |  | Rough |  | Rough |  |
| Note1 | 1 | dBm/15 kHz | -105 |  |  |  |
| Note1 | 1 | dBm/SSB SCS | -95.97 |  |  |  |
|  | 1 | dB | 0 | 9 | 0 |  |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97 | -86.97 | -95.97 |  |
| Io Note2 | 1 | dBm/95.04 MHz | -63.97 | -57.47 | -63.97 |  |
|  | 1 | dB | 0 | 9 | 0 |  |
| NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.3.X2.3	Test Requirements

The UE shall send one Event triggered measurement report, no later than 20 ms plus 5 slots from the beginning of time period T2.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522777==============

A.7.6.3.X3	Event triggered reporting for UE initiated beam management for UE configured with Inter-cell SSB based L1-RSRP measurement on FR2 when DRX is not used

A.7.6.3.X3.1	Test Purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event based on L1-RSRP measurement. This test will partly verify the event triggered reporting for UE initiated beam management in clause 9.13.3.4 configured with event2-r19, modeA-r19 and without eventCountWindow-r19 configuration.

The AoA setup of FR2 cell for this test is Setup 3 as defined in clause A.3.15.

Table A.7.6.3.X3.1-1: Applicable NR configurations for inter-cell SSB based Event triggered reporting for UE initiated beam management in FR2

| Config | Description |
| --- | --- |
| 1 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 5 | NR PCell 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 6 | NR PCell 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode NR SCell 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

A.7.6.3.X3.2	Test parameters

There are two cells in the test, Cell 1 is the serving cell in CA, including a FR1 PCC and FR2 SCC. Cell 2 is a FR2 cell with different PCI from Cell 1. The test parameters for Cell 1 are given in table A.7.6.3.X3.2-1. The test parameters for FR2 Cell (Cell 2) are given in table A.7.6.3.X3.2-2 and table A.7.6.3.X3.2-3.

SSB#0 is transmitted on Cell 1. SSB#1 and SSB#2 are transmitted on FR2 Cell 2. In CSI measurement configuration, UE is indicated to perform L1-RSRP measurement on SSB#1 and SSB#2, where SSB#1 is for current beam measurement and SSB#2 is for new beam measurement. The test consists of two successive time periods, with time duration of T1 and T2 respectively. At the beginning of T2, SSB#2 starts transmission and the UE is configured for L1-RSRP measurement on SSB#2 and the RSRP configuration of SSB#2 is better than SSB#1 to trigger the event-2 reporting. The test has higher layer parameter timeRestrictionForChannelMeasurements configured in CSI-ReportConfig and additionalPCIList configured in CSI-SSB-ResourceSet.

There is no measurement gap configured in the test. Before the test, UE is configured to perform RLM and BFD on Cell 1 in FR1 and perform L1-RSRP measurements on the SSB#0in FR2.

Table A.7.6.3.X3.2-1: Cell specific test parameters for FR1 PCell

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1~6 |  | freq1 |
| Duplex mode | 1,4 |  | FDD |
|  | 2,5 |  | TDD |
|  | 3,6 |  | TDD |
| TDD Configuration | 1,4 |  | N/A |
|  | 2,5 |  | TDDConf.1.1 |
|  | 3,6 |  | TDDConf.2.1 |
| BWchannel | 1,4 | MHz | 10: NPRB,c = 52 |
|  | 2,5 |  | 10: NPRB,c = 52 |
|  | 3,6 |  | 40: NPRB,c = 106 |
| PDSCH Reference measurement channel | 1,4 |  | SR.1.1 FDD |
|  | 2,5 |  | SR.1.1 TDD |
|  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1,4 |  | CR.1.1 FDD |
|  | 2,5 |  | CR.1.1 TDD |
|  | 3,6 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1,4 |  | CCR.1.1 FDD |
|  | 2,5 |  | CCR.1.1 TDD |
|  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration | 1,4 |  | SSB.3 FR1 |
|  | 2,5 |  | SSB.3 FR1 |
|  | 3,6 |  | SSB.4 FR1 |
| OCNG Patterns | 1~6 |  | OP.1 |
| Initial BWP Configuration | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~6 |  | SMTC.1 |
| CSI-RS for tracking | 1,4 |  | TRS.1.1 FDD |
|  | 2,5 |  | TRS.1.1 TDD |
|  | 3,6 |  | TRS.1.2 TDD |
| DRX configuration | 1~6 |  | Off |
| SSB index assigned as RLM RS | 1~6 |  | 0,1 |
| SSB Index assigned as BFD RS (q0) | 1~6 |  | 0 |
| SSB Index assigned as CBD RS (q1) | 1~6 |  | 1 |
| EPRE ratio of PSS to SSS | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note2 | 1~6 | dBm/15 kHz | -104 |
|  | 1~6 | dB | 17 |
|  | 1~6 | dB | 17 |
| SS-RSRP Note3 | 1~6 | dBm/15 kHz | -87 |
| IoNote3 | 1,2,4,5 | dBm/ 9.36 MHz | -58.96 |
|  | 3,6 | dBm/ 38.16 MHz | -52.86 |
| Propagation condition | 1~6 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for Noc to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselvess. |  |  |  |

Table A.7.6.3.X3.2-2: Cell specific test parameters for FR2 SCell

| Parameter |  | Config | Unit | Value |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1~6 |  | Freq2 |
| Duplex mode |  | 1~6 |  | TDD |
| TDD Configuration |  | 1~2 |  | TDDConf.3.1 |
| BWchannel |  | 1~6 | MHz | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 1~6 |  | 66 |
| PDSCH Reference measurement channel |  | 1,2,3 |  | SR.3.2 TDD |
|  |  | 4,5,6 |  | SR.3.3 TDD |
| RMSI CORESET Reference Channel |  | 1,2,3 |  | CR.3.1 TDD |
|  |  | 4,5,6 |  | CR.3.2 TDD |
| Dedicated CORESET Reference Channel |  | 1,2,3 |  | CCR.3.1 TDD |
|  |  | 4,5,6 |  | CCR.3.7 TDD |
| SSB configuration |  | 1,2,3 |  | SSB.1 FR2 |
|  |  | 4,5,6 |  | SSB.2 FR2 |
| OCNG Patterns |  | 1~6 |  | OP.1 |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration |  | 1~6 |  | SMTC.1 |
| TRS Configuration |  | 1~6 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration |  | 1~6 |  | TCI.State.2 |
| DRX configuration |  | 1~6 |  | Off |
| reportConfigType |  | 1~6 |  | periodic |
| reportQuantity |  | 1~6 |  | ssb-Index-RSRP |
| Number of reported RS |  | 1~6 |  | 2 |
| SSB index associated to a PCI different from serving cell |  | 1~6 |  | 0,1 |
| SSB index associated to indicated TCI |  | 1~6 |  | 0 |
| SSB index associated to newBeamResourceSet |  | 1~6 |  | 1 |
| eventTypeUE-IBR-r19 |  | 1~6 |  | event2-r19 |
| eventThreshold-r19 |  | 1~6 |  | 4 |
| reportTransmissionMode-r19 |  | 1~6 |  | modeA-r19 |
| currentBeamReport-r19 |  | 1~6 |  | disabled |
| PUCCH-Resource-r19 | periodicityAndOffset | 1~6 |  | sl5, 4 |
|  | pucch-Cell-r19 | 1~6 |  | Cell 2 |
| Timing offset between two SSBs associated to different PCI |  | 1~6 |  | <CP |
| T1 |  | 1~6 | s | 5 |
| T2 |  | 1~6 | s | 2 |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition |  | 1~6 |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.6.3.X3.2-3: SSB specific test parameters for FR2 SCell

| Parameter | Config | Unit | SSB#1 |  | SSB#2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration | 1~6 |  | Setup 3 according to A.3.15.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 | 1~6 |  | Rough |  | Rough |  |
| Note2 | 1~6 | dBm/15 kHz | -105 |  | -105 |  |
| Note2 | 1~3 | dBm/SSB SCS | -96 |  | -96 |  |
|  | 4~6 |  | -93 |  | -93 |  |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| SSB_RP Note3 | 1 | dBm/SSB SCS | -96 | -96 | -Infinity | -87 |
|  | 2 |  | -93 | -93 | -Infinity | -84 |
| Io Note3 | 1 | dBm/95.04 MHz | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 2 |  | -63.97 | -63.97 | -66.98 | -57.47 |
|  | 1~2 | dB | 0 | 0 | -Infinity | 9 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

A.7.6.3.X3.3	Test Requirements

The UE shall send a first PUCCH message using the new UCI type, no later than 160 ms plus 5 slots from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

==============Next change R4-2522815==============

### 10.1.X CJT Calibration Report Mapping

##### 10.1.X.1 CJT Calibration Delay Offset Measurement Report Mapping

The reporting range of delay offset is defined from 0 to AD, where AD∈0.5CP,CP. The number of levels is defined by MD, where MD∈{32, 64, 128, 256}. The mapping of measured quantity is defined in tables 10.1.4X-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.X-1 Mapping of cjtc-Dd

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC_D_00 | 0 ≤ CJTC-Dd < AD MD-1 | Fraction of CP |
| CJTC_D _01 | AD MD-1≤ CJTC-Dd < 2*AD MD-1 | Fraction of CP |
| CJTC_D _02 | 2*AD MD-1≤ CJTC-Dd < 3*AD MD-1 | Fraction of CP |
| … | … | … |
| CJTC_D _MD-2 | (MD-2)*AD MD-1≤ CJTC-Dd < AD | Fraction of CP |
| CJTC_D _MD-1 | AD £ CJTC-Dd | Fraction of CP |

##### 10.1.X.2 CJT Calibration Frequency Offset Measurement Report Mapping

The reporting range of frequency offset is defined from 0 to AFO, where AFO∈0.1ppm,0.2ppm. The number of levels is defined by MFO, where MFO∈{16, 32, 256}. The mapping of measured quantity is defined in tables 10.1.Y-1. The range in the signaling may be larger than the guaranteed accuracy range.

Table 10.1.Y-1 Mapping of cjtc-F

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CJTC_F_00 | 0 ≤ CJTC-F < AFO 2(MFO-1) | ppm |
| CJTC_F _01 | AFO 2(MFO-1)≤ CJTC-F < 3*AFO 2(MFO-1) | ppm |
| CJTC_F _02 | 3*AFO 2(MFO-1)≤ CJTC-F < 5*AFO 2(MFO-1) | ppm |
| … | … | … |
| CJTC_F _MFO-2 | (2*MFO-5)*AFO 2(MFO-1)≤ CJTC-F | ppm |
| CJTC_F _MFO-1 | 'Invalid' | ppm |

==============End of change==============