3GPP TSG-RAN WG4 Meeting #117                                                                   R4-2523058

Dallas, USA, 17 – 21 November 2025	                   (revision of R4-2520269, revision of R4-2512550)

Title:	(Revised) LS Reply on precompensation for NB-IoT NTN TDD mode

Response to:	LS (R1-2508064) on precompensation for NB-IoT NTN TDD mode

Release:	Release 19

Work Item:	Introduction of IoT-NTN TDD mode (IoT_NTN_TDD-Core)

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact person:	Andjela Savoia

andjela.savoia@iridium.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: N/A

# 1	Overall description

Working assumption

For precompensation, from RAN1 perspective:

- The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

  - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

- FFS: whether spec impact is in RAN1, RAN4 or both

RAN4 received from RAN1 the following LS (R1-2503142) with the following RAN1 working assumptions:

Then RAN1 asked for another clarification under LS RAN1 R1-2508064:

RAN1 has reached the following agreement on segmented precompensation, related to the working assumption on the previous RAN1 LS: Agreement Confirm the following working assumption with modifications: For precompensation, from RAN1 perspective: - The UE may adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - The UE may adjust its time/frequency pre-compensation at the beginning of an NPUSCH/NPRACH transmission (same behavior as Rel-18) - Segmented precompensation is not supported. - It is not supported to perform precompensation within the set of 8 consecutive uplink subframes other than at the beginning of an NPUSCH/NPRACH transmission - FFS: whether spec impact is in RAN1, RAN4 or both. NOTE: RAN1 may revisit this agreement if RAN4 reply LS shows concerns, including concerns on meeting the requirements without segmented precompensation The RAN4 CR may not be aligned with the latest RAN1 agreement. RAN1 would like to ask RAN4 if they have concerns on the updated agreement, and: - If RAN4 has concerns on the RAN1 agreement, RAN1 will modify its specifications to introduce segmented precompensation within the set of 8 consecutive uplink subframes. - If RAN4 has no concerns on the RAN1 agreement, we recommend RAN4 to update their specifications to clarify that segmented precompensation is not supported in IoT NTN TDD mode.

RAN4 would like to thank RAN1 for informing RAN4 about the agreement regarding UE behaviour in IoT NTN TDD system regarding the pre-compensation. RAN4 does not see any issues with the agreement and has agreed to incorporate the agreement as recorded in the 36.133 CR (R4-2521735) and 36.102 CR (R4-2522036).

RAN4 confirms that pre-compensation gap is not applicable for 249 for both frequency error requirement (TS 36.102) and timing error requirement (TS 36.133). RAN4 has no concerns on the RAN1 agreements, and the current specifications are currently taking into account RAN1 assumptions.

# 2	Actions

To RAN1

ACTION: 	RAN4 confirms that RAN1 agreements and assumptions with respect to NTN NB-IoT UL pre-compensation for 249 frequency band have been taken into account.

# 3	Dates of next WG RAN4 meetings

RAN4#118	 2026-02-09 - 2026-02-13	 Stor-Göteborg