3GPP TSG-RAN WG4 Meeting #117     														R4-2523217

Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0242 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.101-5 Corrections on NTN VSAT minimum requirements in Ku-band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek, Thales, CHTTL, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The present context leads to contractions in the definitions of minimum EIRP, maximum EIRP, and the direction of the beam peak with RAN4 discussions |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.2.1.1: Change 'Minimum peak EIRP' to 'Minimum guaranteed EIRP' and update the corresponding definition and Table 9.2.1.1-1. 9.2.1.2: Change 'Minimum peak EIRP' to 'Minimum guaranteed EIRP' and update the corresponding definition and Table 9.2.1.2-1. 10.3.2 : Like Tx part, Rx part needs to be modified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Confusion of terminology: in 38.101-2, “minimum peak EIRP” refers to the minimun requirement for highest EIRP among all testing angles, whereas in 38.101-5 it means the minimun requirement for worst-case among all supported angles. The terminology should be changed to avoid confusion. 2. The incorrect definition for min EIRP: The minimum EIRP based on RAN4 discussions refers to the worst-case EIRP within the the declared minimum elevation angle, but based on current context it refers to the highest EIRP direction (beam peak direction). 3. Incorrect testing direction for max EIRP: The maximum EIRP should be measured in beam peak direction (the direction with highest EIRP) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.1, 9.2.1.2 and 10.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 9.2.1.1	Minimum requirements for Fixed VSAT

The following requirements define the maximum output power radiated by the Fixed VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.1-1. The EIRP of each direction (Link= Satellite direction, Meas=Link angle) should be verified within the declared minimum elevation angle supported for transmitting, which can be achieved by mechanical steering and/or electronic steering according to VSAT Type. The supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.1-1 below is taken as an example.

Figure 9.2.1.1-1 Example measurement grid for min guaranteed EIRP with the declared supported minimum elevation angle

Table 9.2.1.1-1: Minimum guaranteed EIRP for Fixed VSAT

| Operating band | UE Type | EIRPmin (dBm) |
| --- | --- | --- |
| n512, n511, n510 | 1 | 70 (NOTE 1) |
|  | 2 | 70 (NOTE 1) |
|  | 3 | 61 (NOTE 1) |
| NOTE 1: The value corresponds to a 100 MHz CBW configuration with 66 RBs and 120 kHz SCS. Minimum guaranteed EIRP is defined in the direction of the satellite and VSAT is configured to transmit with maximum output power. Minimum guaranteed EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP for fixed VSAT are specified in Table 9.2.1.1-2 below. The requirement should be verified with test metrics of EIRP (Link= Satellite direction, Meas=Link angle).

Table 9.2.1.1-2: Maximum output power limits for Fixed VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 1 | 35 | 76.2 |
|  | 2, 3 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

#### 9.2.1.2	Minimum requirements for Mobile VSAT

The following requirements define the maximum output power radiated by the Mobile VSAT for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). The minimum output power values for EIRP are specified in Table 9.2.1.2-1.

The EIRP of each direction (Link= Satellite direction, Meas=Link angle) should be verified within the declared minimum elevation angle supported for transmitting, which can be achieved by mechanical steering and/or electronic steering according to VSAT Type. The supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 9.2.1.2-1 below is taken as an example.

Figure 9.2.1.2-1 Example measurement grid for min guaranteed EIRP with the declared supported minimum elevation angle

Table 9.2.1.2-1: Minimum guaranteed EIRP for Mobile VSAT

| Operating band | UE Type | EIRPmin (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 (NOTE 1) |
|  | 5 | 70 (NOTE 1) |
| n509, n508 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| n248, n247 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| NOTE 1: The value corresponds to a 100 MHz CBW configuration with 66 RBs and 120 kHz SCS. Minimum guaranteed EIRP is defined in the direction of the satellite and VSAT is configured to transmit with maximum output power. Minimum guaranteed EIRP is defined as the lower limit without tolerance. |  |  |

The maximum output power values for TRP and EIRP for mobile VSAT are specified in Table 9.2.1.2-2 below. The requirement should be verified with test metrics of EIRP (Link= Satellite direction, Meas=Link angle).

Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.21 |
|  | 5 | 43 | 76.21 |
| n509, n508 | 4 | 47 | 88.22,4 |
|  | 5 | 47 | 88.22,4 |
|  | 6 | 40 | 76.62,4 |
| n248, n247 | 4 | 47 | 88.23,4 |
|  | 5 | 47 | 88.23,4 |
|  | 6 | 40 | 76.63,4 |
| NOTE 1: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. NOTE 2: Maximum EIRP is defined using 6RBs allocation with 120kHz SCS. NOTE 3: Maximum EIRP is defined using 27RBs allocation with 30kHz SCS or using 54RBs allocation with 15kHz SCS. NOTE 4: For land ESIM operating in CEPT regions, the upper limits of EIRPmax is 84.5 dBm. |  |  |  |

==============Next change==============

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the QPSK reference measurement channels as specified in Annexes A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 with peak reference sensitivity specified in Table 10.3.2-1. EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. Vendor can also declare EISREFSENS_10M , in which case the EISREFSENS_50M = EISREFSENS_10M +7dBm. The requirement is verified with the test metric of EIS (Link= Satellite direction, Meas=Link angle).

The EIS of each direction (Link= Satellite direction, Meas=Link angle) should be verified within the declared minimum elevation angle supported for receiving, which can be achieved by mechanical steering and/or electronic steering according to VSAT Type. The supported minimum elevation angle shall be declared by manufacturer and within the range of 3°≤minimum elevation angle≤75°, and it can be expressed as (90-θ) if the coordinate systems in Figure 10.3.2-1 below is taken as an example.

Figure 10.3.2-1 Example measurement grid for EIS with the declared supported minimum elevation angle

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 10, 15, 20, 25, 35, 50, 70, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | ≤ -122 |
| n248, n247, n509, n508 | Mobile VSAT | 4, 5 | ≤ -126 |
| n248, n247, n509, n508 | Mobile VSAT | 6 | ≤ -122 |

==============End of change==============