3GPP TSG-RAN WG4 Meeting #115	R4-2508727

Malta, MT, May 19th – 23rd, 2025

Title: 	Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx

Agenda Item:	7.1.4

Source: 	Huawei, HiSilicon

Document for:	Approval

1 General

Issue 1-1: Test applicability rule for 6Rx tests

- Proposals

- Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2.

- Option 2: Introduce test applicability rule only for 6 layers cases

- Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design

Issue 1-2: Test applicability rule for different number Rx antenna ports

- Proposals

- Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass.

- Other options

Issue 1-3: Specification structure

- Proposals

- Option 1: Add a new clause for 6Rx

- Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification

2 PDSCH requirements for 6Rx

2.1 Test scenario

Issue 2-1-1: Test scenarios for PDSCH performance requirements

- Agreement

- Test scenarios without interference: PDSCH Mapping Type A in Rel-19

2.2 Performance requirements for 4 Layers

Issue 2-2-1: MIMO correlation for 4 MIMO layers for Handheld UE testing

- Agreement

- Introduce correlation matrix for 6Rx in the specification:

RUE6Rx=1β125β425β925β1625ββ125*1β125β425β925β1625β425*β125*1β125β425β925β925*β425*β125*1β125β425β1625*β925*β425*β125*1β125β*β1625*β925*β425*β125*1

- Proposal

- Option 1: with α = 0 and β = 0.07179

- Option 2: with α = 0.3 and β = 0.07179

- Option 3: with α = 0 and β = 0

Issue 2-2-2: Propagation channel for 4 MIMO layers

- Agreement

  - TDLA30-10 for both FWA and Handheld UE

Issue 2-2-3: Test cases for 4 MIMO layers for FWA and Handheld UE

- Proposals

- Option 1: Define one single test case

  - Option 1a: One single test case for 4 MIMO layers that is applicable for both FWA and Handheld UE

  - Option 1b: One single test case for 4 MIMO layers corresponding to a Handheld UE

- Option 2: Define two sets of 4 layers tests with different MIMO correlation

Issue 2-2-4: Antenna configuration for 4 MIMO layers case

- Agreement

- 4Tx6Rx

Issue 2-2-5: MCS for 4 MIMO layer case for FWA/Handheld UE with TDLA30-10 ULA Low

- Agreement: Down select the following two options in the next RAN4 meeting

- Option 1: MCS 20 (Table 2)

- Option 2: MCS 17 (Table 1)

Issue 2-2-6: MCS for 4 MIMO layer case for Handheld UE with ULA medium (If agreed)

- Agreement

- MCS 13 (Table 1)

2.3 Performance requirements for 6 MIMO Layers for FWA

Issue 2-3-1: Test applicability rule for FWA

- Proposals

- Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests

- Other options

Issue 2-3-2: Propagation channel and MIMO correlation for 6 MIMO layers

- Agreement

- TDLA 30-10 ULA Low

Issue 2-3-3: DMRS configuration for 6 MIMO layers case

- Proposals

- Option 1: Rel-15 Type 1 DMRS 2+2

- Option 2: Rel-18 eType1 DMRS 1+1

Issue 2-3-4: Whether to consider 256QAM for 6 MIMO layers case

- Agreement

- Not consider 256QAM

Issue 2-3-5: MCS for 6 MIMO layers case

- Agreement

- MCS17 (Table 1)

2.4 Summary of test parameters for 6Rx tests

Issue 2-4-1: Test parameters for 6Rx PDSCH tests

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Propagation channel |  |  | 4 Layers: Refer to Issue 2-2-1 and 2-2- - TDLA30-10 ULA Low - TDLA30-10 ULA Medium 6 Layers: TDLA30-10 ULA Low |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| MCS |  |  | 4 Layers - ULA Low: Option 1: MCS20 (Table 2); Option 2: MCS17(Table 1) - ULA Medium: MCS13 (Table 1) 6 Layers: MCS17(Table 1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: Refer to Issue 2-3-3 |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |

3 SDR requirements for 6Rx

Issue 3-1-1: Static channel and precoding for 6Rx

- Agreement

- For 1T6R (Random precoding)

H=111111

- For 2T6R (Random precoding):

H=111j-jj111-jj-j

- For 4T6R (Random precoding):

- H=111111111-1-1-122j-2j22j3010j305j-33010j-22j2j-22j-3010j305j-3010j

- For 8T6R (Fixed precoding with i1,1=i1,2=i2=0):

    - Option 1:

- H=11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

    - Option 2:

- H=34*11111-j1111-1j1-j1-11-1-1j1-11-111-1-11-j11-1-1-1j-1j1-1-111-j1-1-11

Issue 3-1-2: SDR requirements for 1024QAM with 4 layers

- Proposals

- Option 1: Not Define SDR requirements for 1024QAM with 4 layers

- Option 2: FFS SDR 1024QAM 4-layer requirements after 6Rx static channels are determined

Issue 3-1-3: DM-RS configuration for SDR 6 Layers requirements

- Agreement

- Rel-15 DM-RS Type 1: 2+2

Issue 3-1-4: SDR requirements

- Agreement

- Decide the MCS values as per the agreed static channel matrix in Issue 3-1-1 and parameters in Table 5.5A-1/2/3 of TS 38.101-4

  - QPSK/16QAM/64QAM/256QAM with 6 MIMO layers as below table

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | TBD |
| 6 | 8 | 0.8 | TBD |
| 6 | 8 | 0.75 | TBD |
| 6 | 8 | 0.4 | TBD |
| 6 | 6 | 1 | TBD |
| 6 | 6 | 0.8 | TBD |
| 6 | 6 | 0.75 | TBD |
| 6 | 6 | 0.4 | TBD |
| 6 | 4 | 1 | TBD |
| 6 | 4 | 0.8 | TBD |
| 6 | 4 | 0.75 | TBD |
| 6 | 4 | 0.4 | TBD |
| 6 | 2 | 1 | TBD |
| 6 | 2 | 0.8 | TBD |
| 6 | 2 | 0.75 | TBD |
| 6 | 2 | 0.4 | TBD |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

  - 1024QAM with 2 MIMO layers as below table

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | TBD |
|  | 2 | 10 | 0.8 | TBD |
|  | 2 | 10 | 0.75 | TBD |
|  | 2 | 10 | 0.4 | TBD |
| Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5 |  |  |  |  |

4 CSI requirements for 6Rx

Issue 4-1-1: CQI test for 6Rx

- Proposals

- Option 1: Not define CQI test

- Option 2: Only define one CQI requirements with rank 4 under static channel

Issue 4-1-2: RI test for only Rank 6

- Proposals

- Option 1: Not define RI tests for 6Rx

- Option 2: Define RI test for 6Rx