3GPP TSG-RAN WG4 Meeting # 117				       			       			                  R4-2520838

Dallas, USA, Nov. 17-21, 2025

Agenda item:			6.15.1

Source:	Moderator (Huawei)

Title:	Topic summary for [117][213] Netw_Energy_NR_enh_RRM_Part2

Document for:	Information

Introduction

This topic summary includes RRM performance requirements for Rel-19 network energy saving (AI 6.15.2.2 and 6.15.2.3).

Recommended Topics to be treated online (in order of decreasing priority):

Topic #1: Adaptation of common signal/channel transmission

Issue 1-1: Whether to verify DCI processing time in L1 based SSB adaptation test

Issue 1-2: How many SSB index in L1 based SSB adaptation test

Issue 1-3: Details in test procedure for L1 based SSB adaptation test

Topic #2: OD-SIB1

Issue 2-1: Ending point for cell reselection TC with longer TSI-NR for OD-SIB1

Issue 2-2: Test requirement if option 1 is agreed in issue 2-1

Issue 2-3: Test requirement if option 2 is agreed in issue 2-1

Issue 2-4: One direction or two directions for cell reselection TC with longer TSI-NR for OD-SIB1

Issue 2-5: Verify known target cell case for cell reselection TC

Topic #1: Adaptation of common signal/channel transmission

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520928 | Huawei, HiSilicon | Proposal 1: For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows: - Treport=20ms (Periodic L1-RSRP reporting); only SSB#0 is configured and observed; timeRestrictionForChannelMeasurements is configured - DCI 2-9 indicates SSB adaptation: SSB periodicity from 80ms to 20ms - Condition of serving Cell is changed at the same time as DCI 2-9 indication - UE shall report L1-RSRP result with new value based on indicated SSB from the first SSB after DCI decoding plus 2 ms. |
| R4-2520929 | Huawei, HiSilicon, China Telecom | Draft CR |
| R4-2521684 | Samsung | Draft CR |
| R4-2521685 | Samsung | Proposal 1: For DCI format 2_9 indication based SSB adaptation, additional processing time (i.e., 2 ms or 3 ms) to determine the first SSB burst for L1-RSRP measurement should be considered in the test requirements. Purpose 2: To verify the UE supporting ssb-BurstPeriodicityAdaptation-r19 makes correct reporting of L1-RSRP measurement when SSB periodicity changes due to SSB adaptation - Only one SSB (i.e., SSB#0) is configured |
| R4-2521778 | Ericsson | Proposal 1: For L1-based SSB adaptation, RAN4 shall define the test case to verify the DCI processing time. |
| R4-2521956 | ZTE Corporation, Sanechips | Draft CR |

Open issues summary

Sub-topic 1 SSB adaptation

Issue 1-1: Whether to verify DCI processing time in L1 based SSB adaptation test

- Proposals

  - Option 1 (Huawei, Samsung, Ericsson): yes

- Recommended WF

  - DCI processing time is to be verified in L1 based SSB adaptation test

Issue 1-2: How many SSB index in L1 based SSB adaptation test

- Proposals

  - Option 1 (Huawei, Samsung): only SSB#0 is configured and observed

- Recommended WF

  - Only SSB#0 is configured and observed in L1 based SSB adaptation test

Issue 1-3: Details in test procedure for L1 based SSB adaptation test

- Proposals

  - Option 1 (Huawei): For TC of L1-RSRP/SINR measurement of SSB adaptation, the details in test procedure are proposed as follows

    - Treport=20ms (Periodic L1-RSRP reporting) for FR1

    - timeRestrictionForChannelMeasurements is configured

    - DCI 2-9 indicates SSB adaptation:

L1-RSRP TC: SSB periodicity from 20ms to 80ms, and from 80ms to 40ms

L1-SINR TC: SSB periodicity from 80ms to 20ms

    - Condition of serving Cell is changed at the same time as DCI 2-9 indication

    - UE shall report L1-RSRP result with new value based on indicated SSB from the first SSB after DCI decoding plus OD-SSB post processing time

To figure out the test procedure, companies can conduct discussion based on the below figure (from R4-2520928) which illustrates one-way SSB adaption (from 80ms to 20ms).

Figure cited from R4-2520928

- Recommended WF

  - Further discussion.

Topic #2: OD-SIB1

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2521101 | vivo | Proposal 1: For TC of OD-SIB1, the ending point of the TC is defined as the UL WUS on NES Cell. UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR. Proposal 2: In the new Cell reselection TC, only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC. |
| R4-2521102 | vivo | Draft CR |
| R4-2521245 | Nokia, Nokia Shanghai Bell | Observation 1: If UE is performing cell reselection to an OD-SIB1 based cell, the cell reselection delay must additionally consider the time for requesting and acquiring OD-SIB1. Proposal 1: Specify the new test case with longer TSI-NR to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell. Proposal 2: Specify the new test case for known target cell only. Proposal 3: Specify the new test case to verify the cell reselection behaviour to an OD-SIB1 supporting NES cell and thereafter cell reselection back to non-NES cell with two time periods T1 and T2. Proposal 4: The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1 and set to [5]s with further study being needed to verify this figure. |
| R4-2521246 | Nokia, Nokia Shanghai Bell | Draft CR |
| R4-2521779 | Ericsson | Proposal 1: RAN4 to define a new OD-SIB1 test case to verify the cell reselection behaviour together with the same TSI-NR as legacy. - UE transmits UL-WUS transmission - UE receives and decodes RAR - UE receives OD-SIB1 within OD-SIB1 window with TSI-NR - UE triggers the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility |

Open issues summary

Sub-topic 2 TC for OD-SIB1

Issue 2-1: Ending point for cell reselection TC with longer TSI-NR for OD-SIB1

- Proposals

  - Option 1(vivo): The ending point of the TC is defined as the UL WUS on NES Cell.

  - Option 2 (Nokia, Ericsson): The ending point of the TC is defined as legacy (i.e., starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2)

- Recommended WF

  - Discuss the above options

Issue 2-2: Test requirement if option 1 is agreed in issue 2-1

- Proposals

  - Option 1(vivo): UE shall be able to transmit UL WUS no later than: Tdetect, NR_Intra + TSI-NR + TUL-WUS, where TSI-NR is 1280ms as defined in existing test cases, and TUL-WUS is the time to the first UL WUS occasion after TSI-NR.

- Recommended WF

  - Discuss the above option

Issue 2-3: Test requirement if option 2 is agreed in issue 2-1

- Proposals

  - Option 1 (Nokia): The cell reselection delay shall additionally consider the time for requesting and acquiring OD-SIB1 (i.e., Tevaluate, NR_Intra + TSI-NR + TOD-SIB1), where

    - The additional cell reselection delay due to OD-SIB1 is named TOD-SIB1. TOD-SIB1 comprises below three components:

      - UE transmits UL-WUS transmission

      - UE receives and decodes RAR

      - UE receives OD-SIB1 within OD-SIB1 window

    - TOD-SIB1 set to [5]s with further study being needed to verify this figure.

  - Option 2 (Ericsson): verify the cell reselection behaviour together with the same TSI-NR as legacy.

    - UE transmits UL-WUS transmission

    - UE receives and decodes RAR

    - UE receives OD-SIB1 within OD-SIB1 window with TSI-NR

    - UE triggers the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility

- Recommended WF

  - Discuss the above options.

Issue 2-4: One direction or two directions for cell reselection TC with longer TSI-NR for OD-SIB1

- Proposals

  - Option 1(vivo): Only verify the case that UE reselect from Cell 1 to Cell2 (NES Cell), and no need to verify the case the UE reselect back from Cell2 (NES Cell) to Cell 1 as designed in legacy Cell reselection TC

  - Option 2 (Nokia): Verify the cell reselection behaviour to an OD-SIB1 supporting NES cell and thereafter cell reselection back to non-NES cell with two time periods T1 and T2.

- Recommended WF

  - Discuss the above options.

Issue 2-5: Verify known target cell case for cell reselection TC

- Proposals

  - Option 1(Nokia): Specify the new test case for known target cell only

- Recommended WF

  - Further discussion.