3GPP TSG-RAN WG4 Meeting #117                                                               R4-2522365

Dallas, USA, 17th – 21st November, 2025

Agenda item:			8.1

Source:	Moderator (Samsung)

Title:	Topic Summary for [117][107] 6G AI

Document for:	Information

# Introduction

In RAN#108, RAN plenary approved a new RAN working group level SID on 6G Radio [RP-251881], which has been further revised into [RP-252912] in RAN#109. Specifically, AI/ML is the indispensable enabler for 6G RAN, for which at least AI/ML framework and new AI/ML use cases are highlighted as key aspects to be studied, with objectives provided as follows [RP-252912]:

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this topic summary, it is intended to include the summary for 6G RAN4 AI topic.

Note: In the meeting, the updated running summary [R4-2520371] is provided to include the status update for 6G RAN4 AI/ML issues and summary of RAN4 and other WGs’ updated key agreements.

# Pre-meeting FL Summary

## Topic #1: RAN4 AI/ML framework and other general aspects

### Issue 1-1: Post-deployment enhancement

[Background Summary]

The following agreement is achieved in RAN4#116bis [R4-2520371].

Issue 2-2: Post-deployment enhancement Agreement: - RAN4 will discuss the potential post-deployment testing enhancements scope in 6G

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| CATT | R4-2520153 | Observation 1: RAN4 spent much of time discussing post-deployment testing in R18/R19 and performance monitoring based option was prioritized. Proposal 1: Before starting detailed discussion, the issues that should be addressed by post-deployment testing should be clearly identified. Proposal 2: The feasibility/procedures of possible solutions and the impacts of these solutions on spec need to be clarified at the beginning. |
| Samsung | R4-2520372 | - Observation 1: There are different ways to guarantee AI/ML performance which has been identified in 5G-A AI/ML work items. Ways to guarantee AI/ML performance; Time; Where to perform; Comment. 1; Pre-deployment conformance test; Before cell-phone shipped into market; Testing lab for conformance testing; Same as existing conformance testing. 2; Post-deployment pre-activation functionality test; After cell-phone shipped into market, but before new AI/ML functionality activated; UE vendors’ lab; Similar as product testing. 3; Post-deployment post-activation functionality testing based on performance monitoring; After new AI/ML functionality activated; In-field practical network environment; No yet introduced in Rel-19.. - Observation 2: If post-deployment pre-activation functionality test is not part of conformance test, the potential specification impact and the necessity to be discussed in RAN4 should be clarified by the proponent firstly. Proposal 2: Post-deployment post-activation functionality testing based on performance monitoring can be further discussed in 6G scope for the identified 6G use cases, as per-use case basis. |
| CMCC | R4-2520442 | Proposal 1: for post-deployment, 5G study can be used as baseline. And option 2 (Post-deployment post-activation functionality testing based on performance monitoring) can be used as starting point for 6G study. Proposal 2: for post-deployment based on performance monitoring, it is proposed to study whether monitoring procedures can be established in a general way which can be used for all use cases or need to be discussed separately in a case by case manner. |
| Apple | R4-2520625 | Proposal 1: RAN4 to introduce a proactive post-deployment framework for AI/ML model management. |
| Nokia | R4-2520710 | Observation 1: New aspects related to post-deployment testing to be considered in 6G are: 1) Significant update to the in-lab test framework, test requirements, conformance testing and certification [RAN4]. 2) A new framework for in-field post-deployment validation [RAN2, RAN4] 3) Updates to ML management procedures required to support in-field validation of AI/ML functionalities [RAN2] Proposal 1: RAN4 to further discuss and consider studying post-deployment testing and validation enhancements in 6G in collaboration with RAN2 from early phases of the 6GR SI. |
| Ericsson | R4-2521209 | Observation 5 LCM procedures like model training, model retraining, and model update are critical to ensure reliability of the AI/ML model performance. Observation 6 LCM procedures like model training, model retraining, and model update impose complexity to testing procedures. Proposal 2 RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment. Observation 7 For functionalities to be supported by UE-sided model, choice of the AI/ML model for deployment can be up to UE implementation. Observation 8 For functionalities to be supported by UE-sided model, different flavors of AI/ML model may be deployed by different UE vendors for the same functionality. Proposal 3 RAN4 to study and identify monitoring metric relevant for UE sided model monitoring. Details of monitoring metric can be studied on a per use case basis. |

[Proposal Summary]

- General study objectives for post-deployment enhancement:

  - Issues to be addressed by post-deployment testing should be clearly identified.

- Clarification on existing pre-deployment conformance test and two options of post-deployment enhancement:

|  | Ways to guarantee AI/ML performance | Time | Where to perform | Comment |
| --- | --- | --- | --- | --- |
| (1) | Pre-deployment conformance test | Before cell-phone shipped into market | Testing lab for conformance testing | Same as existing conformance testing |
| (2) | Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) | After cell-phone shipped into market, but before new AI/ML functionality activated | UE vendors’ lab | Similar as product testing |
| (3) | Post-deployment post-activation functionality testing based on performance monitoring (Option 2 in Rel-19 discussion) | After new AI/ML functionality activated | In-field practical network environment | No yet introduced in Rel-19. |

- Proposal related to pre-deployment conformance testing, i.e., above (1):

  - Significant update to the in-lab test framework, test requirements, conformance testing and certification [RAN4].

- Proposal related to post-deployment pre-activation functionality testing, i.e., above (2):

  - A new framework for in-field post-deployment validation [RAN2, RAN4]

  - RAN4 to study testing aspects related to performance validation of the UE based AI/ML model that is updated after deployment and the model that is trained after deployment.

- Proposal related to post-deployment post-activation functionality testing based on performance monitoring, related to above (3):

  - Updates to ML management procedures required to support in-field validation of AI/ML functionalities [RAN2]

  - RAN4 to study and identify monitoring metric relevant for UE sided model monitoring. Details of monitoring metric can be studied on a per use case basis.

  - Study whether monitoring procedures can be established in a general way which can be used for all use cases or need to be discussed separately in a case by case manner.

- New proposal for post-deployment enhancement:

  - Proactive post-deployment framework for AI/ML model management, illustrated by the figure from R4-2520625:

[FL Recommended Discussion Point]

- Discuss the table for clarification on existing pre-deployment conformance test and two options of post-deployment enhancement.

- Discuss all proposals related firstly.

### Issue 1-2: Model standardization and deployment

[Background Summary]

Model standardization and deployment was discussed, including effective model design and constraints on model deployment, while no agreement achieved in RAN4#116bis.

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| CATT | R4-2520153 | Proposal 3: RAN4 consider balance between AI/ML performance and complexity when developing standardized AI/ML models, which improves the possibility of commercial use of AI/ML models standardized by RAN4. Observation 2: “Constraints on Model Deployment” is more like a UE implementation issue. Proposal 4: Provide more clarification and inputs to facilitate the discussion on the constraints on model deployment. |
| OPPO | R4-2521242 | "Simple model”; "Complex model". Num of model parameter; Low, e.g., <1M; High, e.g., 1M ~10M. Model computing complexity; Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs]; High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs. Model size; Small, e.g., <1MB; Large, e.g., 1MB ~10MB. [Inference latency]; Low; High. [Training data requirement]; Low; High. [Generalization]; Sufficient for simple tasks; High potential for complex tasks. Proposal 2: RAN4 should define AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization. Proposal 3: AI models with different complexities could be supported by UEs with different AI categories. |

[Company Proposal Summary]

- See above observations/proposals directly.

[FL Recommended Discussion Point]

FL: Because AI UE category related discussion is agreed to deferred to Q2 2026, suggest only discuss model complexity, while “constraints on model deployment” should also be deferred.

- For model complexity,

  - Discuss the following study direction proposed by OPPO:

    - RAN4 should define AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization.

  - If model complexity is agreed to be one study direction, FFS the following example provided by OPPO to differentiate “Simple” and “Complex” models:

|  | "Simple model” | "Complex model" |
| --- | --- | --- |
| Num of model parameter | Low, e.g., <1M | High, e.g., 1M ~10M |
| Model computing complexity | Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs] | High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs |
| Model size | Small, e.g., <1MB | Large, e.g., 1MB ~10MB |
| [Inference latency] | Low | High |
| [Training data requirement] | Low | High |
| [Generalization] | Sufficient for simple tasks | High potential for complex tasks |

### Issue 1-3: AI/ML framework for RAN4-driven use cases

[Background Summary]

In 5G-A AI/ML discussion, AI/ML framework is established for RAN1-driven use cases such as AI-BM, AI-positioning and AI-CSI. It is expected RAN4 could identify new RAN4-driven use cases in 6G, and the applicability of the AI/ML framework for RAN4-driven use cases can be considered in per-use case study.

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Samsung | R4-2520372 | - For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not: - UE-side and NW-side data collection if needed - Applicability report - Associated ID to indicate additional conditions that may not be explicitly configured - Performance monitoring - Dedicated AI/ML processing unit (APU) and timeline - Testing and Performance Requirements |

[Company Proposal Summary]

- See above proposal directly.

[FL Recommended Discussion Point]

- Discuss the following proposal can be adopted or not:

- For RAN4 use case(s) to be identified in Rel-20, RAN4 should at least consider the following AI/ML framework aspects be applicable or not:

  - UE-side and NW-side data collection if needed

  - Applicability report

  - Associated ID to indicate additional conditions that may not be explicitly configured

  - Performance monitoring

  - Dedicated AI/ML processing unit (APU) and timeline

  - Testing and Performance Requirements

### Issue 1-4: Principle for use cases selection and discussion

[Background Summary]

The following agreement on principle for AI/ML use case selection is achieved in RAN4#116bis.

Issue 3-1: Principle for AI/ML use cases selection Agreement: - Principles for AI/ML RAN4-driven use cases selection - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work. - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope. - Typical RAN4-driven use case shall - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics - Prioritize the use case by taking into account: - use cases with gain over non-AI counterpart - gain may include performance gain or gain from other aspects, like overhead reduction. - trade-off between performance and complexity/overhead should also considered. - use cases with potential standard impact. - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Huawei | R4-2520331 | Proposal 1: RAN4 driven use cases need to be justified based on the following aspects, which include value scenarios, the motivation for introducing AI, the urgency of starting to study, and whether the use case is intended to address issues for mandatory features. |
| vivo | R4-2521439 | Proposal 1: For those use cases that fall under the scope of other WGs-led in 5GA or are extensions of such features, they will be regarded as low-priority use cases in the discussions on RAN4 6G AI. RAN4 will postpone the decision on whether to continue discussing them until the stage when the selection of RAN1/2 use cases becomes clearer. |
| ZTE | R4-2521536 | Proposal 1: Use cases that were already discussed during the 5G-A stage shall be excluded from the selection of RAN4-driven use cases. Observation 1: It is believed that RAN4 should proactively align with the outcomes from other working groups by closely tracking their agreements or conclusions. Observation 2: RAN1 is selecting top-prioritized use cases and RAN2 will commence use cases discussion in Feb meeting 2026. Proposal 2: RAN4 shall not initial discussion or evaluation of use cases with high similarity in AI functionality to those already under discussion in other working groups. Proposal 3: RAN4 shall prioritize discussing or evaluating use cases that have not been identified by other working groups which belong to RAN4 core responsibility. Proposal 4: For performance gain, the following aspects shall be considered:  Improvement of throughput  Latency reduction  Coverage enhancement  RS overhead reduction |

[Company Proposal Summary]

FL: For performance gain, it is included in the agreed principle already, and more detailed metrics can be considered in per-use case study.

- In addition to the principle for AI/ML use cases selection agreed in RAN4#116bis, the following aspects are provided:

  - The value of scenarios,

  - the motivation for introducing AI,

  - the urgency of starting to study,

  - whether the use case is intended to address issues for mandatory features

- For use cases under other WG-led 5G-A or the extension of these use cases,

  - RAN4 treat them with low priority or directly exclude them from RAN4 discussion.

[FL Recommended Discussion Point]

- Discuss the above additional aspects can be adopted

- Discuss the above principle for use cases under other WG-led 5G-A or the extension of these use cases can be adopted.

### Issue 1-5: Interoperability for one-sided and two-sided models

[Background Summary]

Relevant proposal on interoperability for one-sided and two-sided models was provided in RAN4#116bis, while no agreement has been achieved. Interoperability for two-sided and one-sided models are one of the key focused topics in 5G-A study and work item. It should be noted that the Rel-20 5G-A WI will specify the interoperability (by considering Direction C and Direction A with both Option 3a-1 and Option 4-1) and it is expected that the conclusion can be followed in 6G study.

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Ericsson | R4-2521209 | Observation 1 In 6G, for the same functionality, network needs to support a UEs with different flavours of AI/ML model implementation. Observation 2 In 6G, for the same functionality, network needs to support a mix of UEs that can use AI/ML model to infer measurements and UEs that do not support AI/ML based inferencing. Observation 3 If the AI/ML model performance varies between different UEs and their models, then the network needs to know which UE it is interacting with, and when/where it can configure AI/ML functionality to this UE and the performance that can be expected from this UE. Needing to have such knowledge for each of the UEs will compromise interoperability and increase the complexity of deploying the AI/ML based functionality in 6G. Proposal 1 RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G. Observation 4 Interoperability for two-side model is being addressed for CSI compression in 5G-A Rel. 20 WI. Learnings from this WI and the approaches identified to handle interoperability issues in two-side AI/ML model deployment can potentially be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment. |

[Company Proposal Summary]

- Interoperability for one-sided model,

  - RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G.

- Interoperability for two-sided model,

  - No more discussion in 6G study, while the to-be-specified mechanism for CSI compression in Rel-20 5G-A WI can be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

[FL Recommended Discussion Point]

- For interoperability for one-sided model, the proposed WF is very general, and encourage proponent companies to give more detailed proposal.

- For interoperability for two-sided model, the proposed WF seems straightforward and can be adopted.

### Issue 1-6: Channel model for evaluation

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Ericsson | R4-2521209 | Proposal 4 If the test framework for AI based receiver is using test signal and channel emulator, following aspects should be considered for channel model application: 1) Model variants depending on scenarios covered by AI feature 2) Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities 3) Balance between channel model richness and test complexity. 4) How to avoid test signals being used for AI model training. Observation 9 It might be possible to take channel coefficients from system level simulations to create data set for standardization tests. Observation 10 The alignment method and the performance metrics could be different from legacy tests. Proposal 5 Study the feasibility and methodology of taking channel coefficient from system/link level simulations as data set for further tests. Proposal 6 The TE limitations, such as storage space, number of channel filter taps, coefficient precision etc., should be studied and improved for AI if possible. Proposal 7 Study feasible performance metrics based on channel model with rich randomness. |

[Company Proposal Summary]

- For channel model for evaluating AI-based receiver, the following aspects are proposed:

  - Model variants depending on scenarios covered by AI feature

  - Model variants depending on time, frequency and spatial drifting of the channel and UE capabilities

  - Balance between channel model richness and test complexity.

  - How to avoid test signals being used for AI model training.

- Channel model for defining AI-ML based Demod requirements:

  - The applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible, at all, should be discussed in the demod session first.

- For the detailed methods to generate channel model coefficient:

  - Study the feasibility and methodology of taking channel coefficient from system/link level simulations as data set for further tests.

- Testability issues for AI/ML:

  - The TE limitations, such as storage space, number of channel filter taps, coefficient precision etc., should be studied and improved for AI if possible.

  - Study feasible performance metrics based on channel model with rich randomness.

[FL Recommended Discussion Point]

- For the proposed aspects for channel model for evaluating AI-based receiver,

  - RAN4 to discuss the proposed 4 aspects can be adopted or not.

- Particularly for AI-Demod,

  - Discuss the applicability of CDL model.

- For the detailed methods to generate channel model coefficient,

  - RAN4 can further discuss based on Rel-19 AI-ML and AI-CSI to see the experience can be followed.

- Testability issues for AI/ML,

  - After RAN4-driven use cases are selected, these testability issues can be discussed in testability thread, based on RAN4 chair guidance.

### Issue 1-7: Other topics with low priority

[Background Summary]

The following proposals involves topics (including model generalization, AI/ML UE category) which are agreed to begin from Q2 2026.

Issue 2-4: Model generalization Agreement: - For AI/ML functionality generalization, RAN4 shall study generalization of UE and BS based AI/ML functionality for 6G and identify the potential standard impact. - The study will begin no early than Q2 2026. - Study and identify the typical parameters/aspects that are expected to be generalized by AI/ML functionality for each use case firstly, and then discuss how to test the generalization for the corresponding use case. - RAN4-led use cases will be prioritized. - The related study on other WG-led use cases will be based on the requests from the corresponding WG. Issue 2-6: AI/ML UE category Agreement: - Regarding metrics related to AI/MI complexity supported by UE/NW, it can be discussed if a study is needed in 6G no earlier than Q2 2026. - The definition and the context of AI/MI complexity needs to be further clarified if the study is agreed.

[Relevant Proposals in Companies’ Tdocs]

Model generalization:

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520043 | Observation 1: An AI/ML model’s ability to generalize can be truly tested in scenarios where channel parameters (e.g., Doppler in AI/ML based CSI prediction and temporal beam prediction) change during the tests. Proposal 1: RAN4 should study if and how to change channel parameters during a performance test to check the generalization aspects of AI/ML models. Proposal 2: RAN4 waits until the down-selection of RAN4 led use cases before deciding the parameters that need to be generalized by AI-ML functionality. |
| Apple | R4-2520625 | Proposal 2: Study on how semantic abstract of the deployment characteristics can help to address the model generalization issues. |

AI/ML UE category:

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520043 | Observation 2: Aspects like energy consumption, location of computing resources and memory tiers are fully under UE/network implementation, and they are transparent to external entities. Observation 3: Aspects like energy/power consumption of an AI/ML model would be hard to define. Observation 4: RAN plenary is expected to discuss UE device types for 6G. Proposal 3: RAN4 should postpone any discussion on categorization based on AI-ML until RAN plenary settles UE device types for 6G. |
| OPPO | R4-2521242 | Proposal 1: For 6G, RAN4 should define UE AI categories for different devices. Details related to UE AI categories could be postponed to Q2 2026. |

[Company Proposal Summary]

- See above observations/proposals directly.

[FL Recommended Discussion Point]

- The above proposals involve topics which are agreed to begin no early than Q2 2026. Therefore, no further discussion in RAN#117.

- This sub-topic #1-7 can be closed in this meeting.

## Topic #2: AI/ML use cases for RF issues (Per-use case discussion)

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| NTT DoCoMo | R4-2521396 | Observation 1: AI-DPD/DPoD techniques may impact compliance with national radio regulations such as Japanese regulation, particularly regarding MPR reduction and RF testing complexity, which require further evaluation. Proposal 1: Initiate discussion in RAN4 from study to assess regulatory implications of AI-DPD/DPoD and explore alignment with future regulation including certification frameworks and RF emission standards. |
| Qualcomm | R4-2520044 | Observation 1: In RAN4, non-AI based DPD at the UE is not standardized. Proposal 1: We propose not to study AI based DPD at the UE in the context of Issue 4-1b. Observation 2: Based on SID, power enhancement with relaxed UE Tx EVM and advanced BS receiver is defined for UL. Proposal 2: Power enhancement on the DL i.e AI based DPD at the NW and AI based DPoD at the UE is out of scope of this study item. Observation 3: Non-linearity model(s) of transmission signals is discussed in 6G General RF and UE RF session. Proposal 3: Evaluation of AI based DPoD should remain on hold until the model of UE TX non-linearity model(s) gets agreed in the 6G General RF and UE RF session. Observation 4: In RAN4, non-AI based DPD at the UE is not standardized. Proposal 4: We propose not to study AI based DPD at the UE in the context of Issue 4-1d. |
| CATT | R4-25120154 | Proposal 5: RAN4 should consider the spec impacts of the selected AI based RF use cases when making down- selection. Use cases have no spec impacts do not need discussion in RAN4. |
| Huawei | R4-2520332 | Proposal 3: For AI DPoD, performance implications of different characteristics of PA need to be considered. |
| MediaTek | R4-2520345 | Proposal 1: Deprioritize the study on AI-nonlinearity compensation for DL before the motivation is well justified. Proposal 2: Open to have a joint session or separate session of AI and non-AI DPoD at NW. Proposal 3: AI DPoD at NW shall be compared with non-AI DPoD in the end. Proposal 4: Not to study AI based DPD at UE. |
| Samsung | R4-2520373 | Observation 1: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. Observation 2: The goal of the work in 6G is to enable low-end PA and extreme high modulation to transmit higher power, leveraging the capabilities of advanced BS receiver. Proposal 1: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions. Proposal 2: Adopt the following working steps for feasibility study for both AI and Non-AI solutions. - Study item phase (demod centric) - Step-1: Selection of Non-linearity model(s) of transmission signals - UE RF session firstly to determine the non-linearity model(s) - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure - Note: For feasibility study, demodulation relevant evaluation is performed in UE RF session for non-AI based solution, and demodulation relevant evaluation is performed in AI session for AI based solution - Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM - For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER - Performance evaluation - Case 1: Conventional receiver including non-linearity TX model: SNR-a - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b - Case 3: Conventional receiver without any non-linearity TX model: SNR-c - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance - Performance comparison - Compare SNR-d and SNR-c, the results should be comparable - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver - Perform evaluation with different Tx EVM values, find/determine the maximum Tx EVM with the target SNR-d derived based on Case 4 to meet RAN4 requirement or test metric and comparable with the target SNR-c derived based on Case 3. - Perform different PA models, if needed - WI phase (UE RF centric) - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation and elevate the Net Gain : - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged - For instance for 256QAM Tx EVM, 3.5% -> 6%, - For selected PA model(s), evaluate the allowed TX power boosting/gain (i.e., MPR reduction) for different power class - Net Gain [dB] = (Tx power boosting/gain relative to reference) − (SNR degradation relative to reference at 10% BLER) - FFS the necessity for new EVM model for RAN4 demodulation test cases. Note: Above approaches are applicable for both 5G waveforms and 6G waveforms |
| CMCC | R4-2520443 | Proposal 3: it is proposed to consider the use case of AI/ML based PA non-linearity compensation for UL. |
| Nokia | R4-2520711 | Observation 1: UE-sided AI-based solutions compensation show potential for coping with PA nonlinearity. Proposal 1: RAN4 to consider studying AI-based-DPD (UE sided model) as a RAN4-driven use case. |
| Ericsson | R4-2521210 | Proposal 1 RAN4 should study AI based UL-Post distortion compensation at BS receiver and understand the feasibility of creating RAN4 core and performance requirements. Observation 1 Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off. Proposal 2 RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion, which could further help to evaluate UE transmitter model in the study. Proposal 3 In the candidate sub-use cases, we propose prioritizing “NW: AI-DPoD” for the RAN4 study. Observation 2 Accurate modeling of UEs PA and RF-front end is necessary for this feature. However, transmitter characteristics have many variations due to other RF impairments, frequency range, RB allocation etc. Therefore, UE RF front-end needs to be evaluated by RAN4, so that reliable and representative data from the RF front-end can be obtained to effectively train the AI/ML model. Proposal 4 RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models. Observation 3 There is no reference study on NW-sided DPoD, therefore both AI and non-AI approach would be needed for performance and complexity evaluation. Proposal 5 We propose to study UE transmitter model in the UE RF session initially and further evaluate the non-linearity compensation in both AI thread (for AI-based approach) and BS Demodulation thread (for non-AI approach). Proposal 6 RAN4 could consider the extent of adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit such as compensating the increased distortion at the receiver. Observation 4 The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions. Observation 5 AI/ML-based uplink PA digital post-distortion (DPoD) could in certain conditions achieve performance similar to that of ideal PA without power backoff, resulting in a potentially higher UE output power and/or more power efficient operation for higher order modulations. Proposal 7 For AI-based NW-sided DPoD, we observe reasonable gain under high-SNR and high modulation order conditions with a simple UE PA model and complete investigation with realistic UE-Tx model is needed. Observation 6 To characterize multiple UE transmitters, either a fine-tuned offline model or less precise and adaptive online approach can be feasible, considering the UE power consumption needed for model training and potential latency issues. Observation 7 Offline trained AI/ML models may need to be updated in a certain time period to include new UE transmitter models being implemented by UEs. Proposal 8 RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible. Proposal 9 RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included. Proposal 10 RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies. Proposal 11 The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation. Proposal 12 RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations. Observation 8 As part of the study, RAN4 should decide the appropriate complexity level of the reference AI/ML model. Proposal 13 RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its impact on UL coverage and UE transmit energy efficiency. Observation 9 In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits. Proposal 14 RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station. Observation 10 The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance. Proposal 15 UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply. Proposal 16 A common UE transmitter model with non-linearities should be evaluated in UE RF session, which could further be used for different AI and non-AI DPoD model. |
| OPPO | R4-2521243 | Proposal 1: For 6G, NW side DPoD (Digital Post-Distortion) should be studied in RAN4. Proposal 2: For NW side DPoD, following sub cases could be considered in RAN4 - DPoD-opt1: NW side NC by a separate AI DPoD module + Non-AI demod - DPoD-opt2: NW side NC by a separate AI DPoD module + AI demod - Note1: AI DPoD module could be updated through online training/tuning - Note2: Channel estimation may/may not be included Proposal 3: RAN4 should consider and clarify whether the time-varying characteristic of PA non-linearity should be considered in PA non-linearity modeling. Proposal 4: Based on companies' inputs, during or after RAN4#117, RAN4 should have on an aligned simulation assumption for further study. Observation 1: According to the simulation results, the proposed AI-DPoD scheme achieves a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM. Observation 2: When EVM increases to 8%, the proposed AI- DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels. Observation 3: For AI-DPD, the simulation results indicate that a simple AI-DPD model could be utilized to achieve PA non-linear compensation, delivering better performance. Observation 4: Implementing DPD at the UE side using a pre-trained PA non-linear compensation model seems has low/less standardization impact. |
| vivo | R4-2521440 | Observation 1: AI demonstrates significant potential for accurate DPD modelling while maintaining lower computational complexity and hardware cost. Observation 2: The output power and PAPR of UL signals for data collection significantly impact the performance of the resulting model. Observation 3: Both AI/ML-based DPD and DPoD demonstrate promising potential in compensating for PA non-linearity with similar complexity. Proposal 1: RAN4 study AI based solution for PA non-linearity compensation in 6G, including both AI DPD and AI DPoD. Proposal 2: Update the evaluation procedure to capture both AI DPD and AI DPoD, as listed below: - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation. - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure - FFS a joint session or separate session of AI and non-AI non-linearity compensation. - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to - For both AI DPD and AI DPoD - FFS suitable AI/ML model, type of training. - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD and DPD performance and ensuring comparability across different implementations - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models), measurement-based datasets (from multiple UEs). - For AI DPD - All modulation orders are expected to be evaluated - FFS on detailed baseline assumption for UE requirements, including ACLR, SEM, IBE, EVM, etc. - For AI DPoD - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity. - Focusing on higher modulation orders focusing on high-SNR conditions - Other details are not precluded - Step-3: For AI DPoD, under a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. For AI DPD, under a given baseline assumption of UE emission requirements, conduct TX MPR and power boosting simulation. - The potential MPR reduction and power boosting enabled by AI-DPoD /AI DPD are evaluated. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged - The potential FBR bandwidth reduction of AI DPD can also be considered. - The impact on UL coverage and UE transmit energy efficiency is evaluated. |
| ZTE | R4-2521537 | Proposal 1: RAN4 to prioritize the DPoD study for AI-nonlinearity compensation use case. Proposal 2: For DPoD evaluation, two alternatives can be considered for PA model derivation: -Alternative 1: To determine a common PA model for further evaluation -Alternative 2: multiple PA models can be used for further evaluation as long as the relaxed EVM requirements can be derived reasonably |

### Use Case #1: AI-nonlinearity compensation

#### Issue 2-1: AI-nonlinearity compensation (sub-)use case selection/prioritization

[Company Proposal Summary]

| Sub-use case | Compensation for UL or DL | Model location and training | Be positive | Be negative |
| --- | --- | --- | --- | --- |
| NW-sided DPoD (Digital Post-Distortion) and receiver | UL | NW-sided model | *(4) Samsung, Ericsson, OPPO, ZTE ‡(1) CMCC |  |
| UE-sided DPD (Digital Pre-Distortion) | UL | UE-sided model | *(3) Nokia, vivo, OPPO (better to be further justified) | (2) Qualcomm, MediaTek |
| UE-sided DPoD (Digital Post-Distortion) and receiver | DL | UE-sided model |  | (2) Qualcomm, MediaTek |
| NW-sided DPD (Digital Pre-Distortion) | DL | NW-sided model |  | (2) Qualcomm, MediaTek |
| Note: for companies show supporting, with and without numerical results in the Tdocs are separated counted. * with numerical results in Tdocs ‡ without numerical results in Tdocs (x) in which x is the number of companies for that category |  |  |  |  |

For UL DPoD@BS and DPD@UE

Qualcomm: DPD should not be standardized in AI since it is not for non-AI case.

Ericsson: prefer to prioritizing DPoD

Samsung: prioritize DPoD. It is related to BS demod performance.

Vivo: prioritize DPD at UE. No strong justification has been provided to back up DPoD.

MTK: hard to justify the gain of AI based DPD since there is no obvious reference to compare with.

Apple: similar view as Qualcomm/MTK.

Nokia: similar view as vivo

CMCC: both DPoD and DPD should be considered

Ericsson: it is OK to continue the discussion on AI-based DPD.

Apple: what’s the standard impact of DPD?

Vivo: RF requirements can be different. New DL RS and calibration gap may be needed.  DPoD and DPD are supposed to address the same issue.

Samsung: on DPoD, RF requirement can be different and new BS feature. No common understanding on UL RS part.

Qualcomm: gain of DPD is questionable.

Samsung: DPoD hasn’t been implemented in the existing BS.

Agreement:

RAN4 will study the following 6G AI use cases

Case 1.1: AI-based DPoD+no DPD

Case 1.2: AI-based DPoD+non-AI DPD

Case 2: no DPoD+AI based DPD

benchmark: 1) non-AI DPD only 2)non-AI DPoD only

- The interested companies can choose to compare with no DPD/DPoD and non-AI DPD(UE)+non-AI DPoD(BS)

Training: TBD

Evaluation metric: TBD

[FL Recommended Discussion Point]

- Discuss the sub-use case selection/prioritization in main session.

  - Before showing positive/negative views, companies are encouraged to check the key information for use case description.

#### Issue 2-2: General regulatory aspects for evaluation

[Company Proposal Summary]

- Proposal (NTT DoCoMo): Study to assess regulatory implications of AI-DPD/DPoD and explore alignment with future regulation including certification frameworks and RF emission standards.

[FL Recommended Discussion Point]

- The proposal should be considered in evaluation procedure if the AI-nonlinearity compensation use case(s) are adopted.

#### Issue 2-3: AI-DPoD in gNB – key information for use case description

[Company Proposal Summary]

|  |  | Ericsson | OPPO | Samsung | ZTE |
| --- | --- | --- | --- | --- | --- |
| AI model input | Input in training | The model was trained using labeled IQ samples obtained after the IFFT in DFT-s-OFDM | Received signal after channel estimation and equalization | The estimated RS signal time domain sample with 1st, 3rd terms | The estimated data signal time domain sample with 1st, 3rd, 5rd terms |
|  | Input in inference | The estimated IQ samples after IFFT in DFT-s-OFDM | Received signal after channel estimation and equalization | The estimated data signal time domain sample with 1st, 3rd terms | The estimated data signal time domain sample with 1st, 3rd, 5rd terms |
| AI model output | Label in training (if applicable) | Known transmitted coded bits available at the receiver side | Received signal before demodulation [or before channel decoding] | original/known RS signal | original/known data signal |
|  | Output in inference | Soft bits | Received signal before demodulation [or before channel decoding] | data signal with non-linear distortion compensated | data signal with non-linear distortion compensated |
| Assumption on training | Training type | Offline Training | offline training [FFS online training, tuning] | Online training/finetune | offline training |
|  | Label construction (if applicable) | Based on transmitted bits, which can be obtained at the receiver by relying on - bits decoded after successful cyclic redundancy check (CRCs), - transmission of bit sequences known to both transmitter and receiver. | May need data collection for offline training [FFS online training, tuning] | DMRS with 1st, 3rd terms | Data signal time domain with 1st, 3rd, 5rd terms |
| Model location for inference |  | NW-sided model | NW-sided model | NW-sided model | NW-sided model |
| Collaboration/interaction between UE and NW |  | The NW may request the UE to - adjust PA back-off associated with relaxed EVM requirements, - generate and provide data to support training and refinement of AI/ML models at the NW side, - provide measurement reports and receive updated configuration from the NW. | For inference, no collaboration/interaction For training, collaboration/interaction may be needed for date collection | UE capability and network indication could be required | N/A |
| Evaluation methodology |  | Link level simulation | Link level simulation | Link level simulation | Link level simulation |
| Evaluation assumption |  | See R4-2521210 | See R4-2521243 | See R4-2520373 | See R4-2521537 |
| Evaluation KPI |  | BLER, throughput | BLER gain @ EVM(3.5%, 8%) | BLER, MPR, EVM, throughput | BLER, MPR, EVM, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD with PA distortion Legacy receiver without DPoD without PA (upper bound performance) Receiver with AI-DPoD | Conventional receiver (without non-linearity compensation) | Legacy receiver without DPoD Receiver with non-AI based DPoD, GMP-NC | Legacy receiver without DPoD |
| Preliminary evaluation results |  | Performance benefit: 100% link-level throughput gain at mid/high SNR levels at BLER = 10-1 | NW DPoD achieve a performance gain of approximately 1 dB at a BLER of 10% with 3.5% EVM. When EVM increases to 8%, the proposed AI-DPoD schemes still achieve 10% BLER at an SNR of 22.2 dB, whereas the benchmark is unable to achieve 10% BLER even at higher SNR levels. | Performance benefit: AI-NC provides 1-2dB improvement at 10% BLER when using 256QAM, 1024QAM and 4096QAM for the given EVM values. | Tx EVM can be relaxed to 4.04% |
| High level potential specification impact |  | - Adjusted in-band distortion (e.g. EVM) and impact to other requirements (e.g. MPR, A-MPR), - UE capability exchange, - Data collection using know bit sequences generated at the UE side by the mean of a shared seed between the UE and the BS, under different hardware operating conditions. | EVM/[MPR] relaxing in RAN4 requirement | RAN4 requirements, e.g, EVM, MPR reporting Signalling/procedure related to LCM for NW-sided model | UE capability report on supported MPR relaxing RAN4 measurement requirement on MPR delta |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Complexity: 100 MFLOPs | Model: AI DPoD + AI Demod FLOPs: 86.4M + 11.1M Trainable parameters: 44K | Low complexity neural network for online tuning | Complexity: FCN is used: Number of input neurons is 30 Number of output neurons is 2 Number of hidden layers is 4 Number of neurons in each hidden layer is 64 FLOPs: 0.59G, Params: 0.01M Performance benefit: AI-NC provides 2.5dB MPR improvement when using 256QAM for the given SNR value(SNR=16). |

[FL Recommended Discussion Point]

- Discuss the above key information provided by different companies for use case description, by focusing on following differences in companies’ proposals:

  - AI model input/output

  - Offline training vs. online training/finetune

#### Issue 2-4: AI-DPoD in gNB – evaluation related proposals

[Background] The following agreement is achieved for the evaluation procedure for AI-DPoD in RAN4#116bis:

Issue 4-1c: Evaluation for AI-DPoD Agreement: - Non-linearity model(s) of transmission signals: - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible. - Using separate models for AI and non-AI DPoD is not precluded. - Evaluation Procedures and some detailed discussion points - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation. - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure - FFS a joint session or separate session of AI and non-AI non-linearity compensation. - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity. - FFS suitable AI/ML model, type of training. - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs). - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations - Focusing on higher modulation orders focusing on high-SNR conditions - Other details are not precluded - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. - The potential MPR reduction enabled by AI-DPoD is evaluated. - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged - The impact on UL coverage and UE transmit energy efficiency is evaluated. - Other design principle to be considered: - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

[Company Proposal Summary]

- Proposals related to Step-1 (Non-linearity model(s) of transmission signals is determined to be used in the evaluation):

  - Samsung:

    - Proposal 1: Non-linearity model(s) of transmission signals (selected for the evaluation of BS compensation capabilities and UE MPR reduction limits) can be determined in 6G UE RF session, which should be common for BS compensation with AI and Non-AI solutions.

  - Huawei:

    - Proposal: For AI DPoD, performance implications of different characteristics of PA need to be considered.

  - OPPO:

    - Proposal: RAN4 should consider and clarify whether the time-varying characteristic of PA non-linearity should be considered in PA non-linearity modeling.

  - Ericsson:

    - Proposal: RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models.

  - ZTE:

    - two alternatives can be considered for PA model derivation:

      - Alternative 1: To determine a common PA model for further evaluation

      - Alternative 2: multiple PA models can be used for further evaluation as long as the relaxed EVM requirements can be derived reasonably

- Proposals related to Step-2 (Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation):

  - Qualcomm:

    - Proposal: Evaluation of AI based DPoD should remain on hold until the model of UE TX non-linearity model(s) gets agreed in the 6G General RF and UE RF session.

  - MediaTek:

    - Proposal 1: Open to have a joint session or separate session of AI and non-AI DPoD at NW.

    - Proposal 2: AI DPoD at NW shall be compared with non-AI DPoD in the end.

  - Ericsson:

    - Proposal 1: We propose to study UE transmitter model in the UE RF session initially and further evaluate the non-linearity compensation in both AI thread (for AI-based approach) and BS Demodulation thread (for non-AI approach).

    - Proposal 2: RAN4 could consider the extent of adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit such as compensating the increased distortion at the receiver.

    - Proposal 3: For AI-based NW-sided DPoD, we observe reasonable gain under high-SNR and high modulation order conditions with a simple UE PA model and complete investigation with realistic UE-Tx model is needed.

    - Proposal 4: RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible.

    - Proposal 5: RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included.

    - Proposal 6: RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies.

    - Proposal 7: The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation.

    - Proposal 8: A common UE transmitter model with non-linearities should be evaluated in UE RF session, which could further be used for different AI and non-AI DPoD model.

  - Samsung:

    - Proposal 1: Including the non-linearity model(s) into the RAN4 PUSCH demodulation test case(s) for both 64QAM and 256QAM.

    - Proposal 2: For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

      - Performance evaluation

        - Case 1: Conventional receiver including non-linearity TX model: SNR-a

        - Case 2: Non-AI based non-linearity receiver including non-linearity TX model: SNR-b

        - Case 3: Conventional receiver without any non-linearity TX model: SNR-c

        - Case 4: AI based non-linearity (AI-NC) receiver with non-linearity TX model: SNR-d

        - Note: Case 3 is considered as upper bound performance, Case 1 is expected to be low bound performance

      - Performance comparison

        - Compare SNR-d and SNR-c, the results should be comparable

        - FFS the criteria to be claimed as “comparable” (e.g., within certain x dB), and the target SNR-c should be feasible for network deployment

        - Compare SNR-d and SNR-b, for AI-NC advantage over non-AI receiver

- Proposals related to Step-3 (For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation):

  - Ericsson:

    - Proposal 1: RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations.

    - Proposal 2: RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its  impact on UL coverage and UE transmit energy efficiency.

    - Proposal 3: RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

    - Proposal 4: UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

  - Samsung:

    - Proposal: The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged.

[FL Recommended Discussion Point]

- Discussion on the above proposals firstly.

#### Issue 2-5: AI-DPD in UE – key information for use case description

[Company Proposal Summary]

|  |  | vivo |  |  | Nokia |
| --- | --- | --- | --- | --- | --- |
|  |  | Direction 1 | Direction 2 | Direction 3 |  |
| AI model input | Input in training | Time domain samples after PA distortion and PA config e.g. Vcc etc. |  | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples before pre-distortion |
|  | Input in inference | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples after PA distortion and PA config e.g. Vcc etc. | Time domain samples before PA distortion and PA config e.g. Vcc etc. | Time domain samples after pre-distortion |
| AI model output | Label in training (if applicable) | Time domain samples w/o distortion |  | Time domain samples after PA distortion | Time domain samples |
|  | Output in inference | Time domain samples after pre-distortion | LUT coefficients | Time domain samples after PA distortion |  |
| Assumption on training | Training type | Offline training；Online finetune |  |  | Offline training Online training/finetune2 |
|  | Label construction (if applicable) |  |  |  |  |
| model location for inference |  | UE-sided |  |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | As for UE side model |  |  | Similar to UE-sided model as NR |
| Evaluation methodology |  | link level simulation |  |  |  |
| Evaluation assumption |  | See R4-2521440 |  |  |  |
| Evaluation KPI |  | BLER, EVM, MPR |  |  | BLER, EVM, MPR |
| Evaluation benchmark |  | No DPD |  |  | No DPD |
| preliminary evaluation results |  | See 2.1.1.2 in R4-2521440 |  |  |  |
| High level potential specification impact |  | See 2.1.1.3 in R4-2521440 and summarized as below: 1. RAN4 requirements, e.g. MPR, power boosting 2. SRS and/or PUSCH enhancement/Tx power determination for data collection 3. Signaling/ procedure related to LCM for UE-sided model |  |  | 1. RAN4 requirements, e.g. EVM 2. Tx power determination 3. Signaling/ procedure related to LCM for UE-sided model |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Inference complexity |  |  |  |
|  |  | Depend on AI model complexity | similar as LUT-based DPD | similar as LUT-based DPD |  |

[FL Recommended Discussion Point]

- Discuss the above key information provided by different companies for use case description, the three possible directions proposed by vivo.

#### Issue 2-6: AI-DPD in UE – evaluation related proposals

[Background] The following agreement is achieved for the evaluation procedure for AI-DPD in RAN4#116bis:

Issue 4-1d: Evaluation for AI-DPD Agreement: - Non-linearity model(s) of transmission signals: - Discussed in other 6G agenda, which is intended to discuss the common model as much as possible.

[Company Proposal Summary]

- Proposals related to evaluation of AI-DPD in UE:

  - vivo:

    - Proposal 1: RAN4 study AI based solution for PA non-linearity compensation in 6G, including both AI DPD and AI DPoD.

    - Proposal 2: Update the evaluation procedure to capture both AI DPD and AI DPoD, as listed below:

      - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

      - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

        - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

        - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

          - For both AI DPD and AI DPoD

            - FFS suitable AI/ML model, type of training.

            - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD and DPD performance and ensuring comparability across different implementations

            - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models), measurement-based datasets (from multiple UEs).

          - For AI DPD

            - All modulation orders are expected to be evaluated

            - FFS on detailed baseline assumption for UE requirements, including ACLR, SEM, IBE, EVM, etc.

          - For AI DPoD

            - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

            - Focusing on higher modulation orders focusing on high-SNR conditions

          - Other details are not precluded

      - Step-3: For AI DPoD, under a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation. For AI DPD, under a given baseline assumption of UE emission requirements, conduct TX MPR and power boosting simulation.

        - The potential MPR reduction and power boosting enabled by AI-DPoD /AI DPD are evaluated.

          - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

        - The potential FBR bandwidth reduction of AI DPD can also be considered.

        - The impact on UL coverage and UE transmit energy efficiency is evaluated.

[FL Recommended Discussion Point]

- Discussion on the above proposals firstly.

## Topic #3: AI/ML use cases for Demod issues (Per-use case discussion)

### Use Case #2: AI-SRS-assisted channel reconstruction

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520044 | Observation 5: UE can compensate for insertion loss by boosting its transmit power as long as it has power headroom. Observation 6: UE cannot boost its SRS Tx power and compensate for insertion loss when it does not have power headroom. This can only happen at the cell edge where the role of SRS is somewhat diminished due to the smaller link coverage. Observation 7: Insertion loss induced imbalance is one relatively minor aspect of the overall topic of channel reconstruction through the combination of CSI-RS and SRS. Observation 8: If 3GPP decides to study CSI-RS and SRS based two-sided channel reconstruction, 3GPP will have to discuss how network can share parameters regarding encoder input and decoder input to train UE encoder. Here, decoder input contains both encoder output and channel obtained from SRS. The details of this parameter sharing can only be done in RAN1. Proposal 5: SRS assisted channel reconstruction, if needed, should be a RAN1 led AI-ML topic. |
| Nokia | R4-2520711 | Observation 2: In Rel-19 NR_ENDC_RF_Ph4 WI, RAN4 discussed solutions like SRS IL imbalance UE self-compensation and SRS IL imbalance UE assistance and reporting. Proposal 2: If non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1, then there is no need to rely on AI/ML for SRS IL compensation. |
| Samsung | R4-2520373 | Observation 3: SRS based DL CSI acquisition for large channel dimensions is challenged by factors including SRS coverage or channel ageing and SRS power imbalance. Observation 4: Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions. Proposal 3: Study SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement at least for NW-sided model. |
| Huawei | R4-2520332 | Observation 1: NW is constrained in obtaining high-accuracy CSI from CSI feedback for a larger number of CSI-RS ports supported by 6GR, as it may heavily rely on UE’s AI capabilities. Observation 2: The power consumption of AI-based CSI feedback for a larger number of CSI-RS ports is higher than that of SRS transmission. Observation 3: SRS is an attractive complement to, or even a potential substitute for, CSI-RS in numerous 6G scenarios. Observation 4: If SRS power imbalance is well compensated, the coverage of antenna switching SRS can be extended, without relying on CSI-RS. Observation 5: If SRS channel aging is well resolved, the coverage of antenna switching SRS can be extended, without relying on CSI-RS. Observation 6: As 6G evolves toward higher frequency bands and a larger number of UE Rx, the magnitude of SRS power imbalance induced by insertion loss will consequently increase. Observation 7: Investigating compensation for SRS power imbalance to improve the accuracy of CSI is urgent for 6GR to support a larger number of CSI-RS ports. Observation 8: SRS power imbalance issue have been confirmed in both RAN4 and RAN1; however, RAN4-identified solutions for supporting non-AI based compensation method failed to achieve consensus in RAN1. Observation 9: For UEs with more antennas in higher-frequency bands that cannot maintain power balance, compensation at NW is beneficial to enhancing 6G system performance. Observation 10: Non-AI compensation has two key issues: one is the signaling overhead caused by dynamic UE reporting of residual power imbalance, and the other is that under low SNR conditions, even with such reporting, high-precision channel recovery cannot be achieved. Observation 11: AI based SRS power imbalance compensation has the potential to avoid reporting actual values of residual power imbalance, thereby reducing signaling overhead, and to improve compensation performance even under low SNRs. Observation 12: AI-based SRS residual power imbalance compensation delivers desirable performance even under medium-to-low SNR conditions. Moreover, it provides an error tolerance of at least 1 dB for reporting—meaning there is no need to report updates to the SRS residual power imbalance when changes fall within this 1 dB range. Proposal 1: Details of AI-based SRS residual power imbalance compensation are summarized in Table 2. Proposal 2: In RAN4 6G AI, consider supporting AI based SRS residual power imbalance compensation. The evaluation procedures are listed as below. - Step-1: Assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation. - Step-2: Evaluation methodology by modelling SRS residual power imbalance to baseband evaluation with the following procedure - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to - Training dataset and testing dataset construction - Evaluation metric - AI models and training type - Other details are not precluded |
| Ericsson | R4-2521210 | Proposal 17 Further investigation could be on “training-data consistency”, “reporting strategy” and “performance vs complexity trade-off” Proposal 18 Further clarification is needed on: • Benefit of IL-range reporting method vs zero-reporting method • The level of UE implementation transparency for IL-range reporting method • How to align the AI model performance • Detailed parameter assumptions |
| ZTE | R4-2521537 | Observation 1: The motivation of SRS-assisted channel reconstruction is to reduce the CSI-RS overhead, especially when deploying antennas on the larger scale. Observation 2: The SRS-IL power imbalance shall be included in the SRS-AI assist channel reconstruction. Accurate channel estimation is a prerequisite for this reconstruction. Proposal 3: Propose to postpone the related AI SRS assist channel reconstruction discussion in RAN4, to be led by RAN1. |

#### Issue 3-1: AI-SRS-assisted channel reconstruction (sub-)use case selection/prioritization

[Company Proposal Summary]

| Sub-use case | Model location and training | Be positive | Be neutral or postpone | Be negative (or led by RAN1) |
| --- | --- | --- | --- | --- |
| AI-based SRS residual power imbalance compensation and AI-based SRS+CSI-RS based channel reconstruction | NW-sided model | *(2) Huawei, Samsung | (1) Ericsson | (3) Nokia, Qualcomm, ZTE |
| Note: for companies show supporting, with and without numerical results in the Tdocs are separated counted. * with numerical results in Tdocs ‡ without numerical results in Tdocs (x) in which x is the number of companies for that category |  |  |  |  |

[FL Recommended Discussion Point]

- Discuss the sub-use case selection/prioritization in main session.

  - Before showing positive/negative views, companies are encouraged to check the key information for use case description.

  - The following aspects are mentioned to consider AI-based SRS residual power imbalance compensation:

    - SRS IL compensation if power headroom is available

    - SRS power imbalance issue severity if operating frequency is higher in 6G

    - SRS power imbalance issue severity if the number of UE antenna is larger in 6G

    - Whether or not non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1

  - The following aspects are mentioned to consider AI-based SRS+CSI-RS based channel reconstruction:

    - RAN1 or RAN4-led if 3GPP decides to study CSI-RS and SRS based two-sided channel reconstruction

#### Issue 3-2: AI-SRS-assisted channel reconstruction – key information for use case description

[Company Proposal Summary]

|  |  | Huawei | Samsung |
| --- | --- | --- | --- |
| AI model input | Input in training | UL measured channel matrix from SRS with residual power imbalance | CSI measurement and channel matrix feedback for subset of CSI-RS ports Estimated channel based on SRS |
|  | Input in inference | UL measured channel matrix from SRS with residual power imbalance | CSI measurement and channel matrix feedback for subset of CSI Estimated channel based on SRS |
| AI model output | Label in training (if applicable) | CSI measurement without power imbalance, candidate solutions could be: - CSI feedback based on CSI-RS - SRS measurements that are processed via time domain filtering to achieve high enough SNR - SRS measurements with perfect pre-compensation at UE | Ideal CSI measurement with full channel dimension |
|  | Output in inference | Ideal DL channel matrix | The reconstructed CSI with full channel dimension |
| Assumption on training | Training type | Offline training | Offline |
|  | Label construction (if applicable) | Dataset construction: A total of 100,000 TTIs (Transmission Time Intervals) are allocated as follows: - 70,000 TTIs: Only S slots are used for model training; - 10,000 TTIs: Both S slots and D slots are used for model validation; - 20,000 TTIs: Both S slots and D slots are used for model testing. | Ideal CSI measurement with full channel dimension |
| model location for inference |  | NW-sided AI | Network-sided |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS residual power imbalance for model training and model selection 2. Data collection/monitoring: UE reporting/assistance information for constructing labels, e.g., CSI feedback based on CSI-RS or SRS measurements with perfect pre-compensation | Similar to NW-sided model in NR |
| Evaluation methodology |  | Link level simulation | Link level simulation |
| Evaluation assumption |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5] dB. [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. A common AI model is trained which applies to each channel matrix extracted from 4RBs. The model input/output is the channel matrix with the dimension [M×N], where M denotes the number of CSI-RS ports and N denotes the number of UE Rx. Channel aging issue is not considered in this use case. | Parameter; Value; Parameter; Value. M (measured CSI-RS ports); 64; Channel; UMa Dense-urban. N (NW antenna ports); 256; SRS measurement interval; 30ms. Nr (number of UE receive ports); 4; Mean power imbalance (4 antennas); [0 -3 -6 -9]dB. Carrier frequency/BW; 7 GHz/ 20MHz 13 SBs; Power imbalance distribution; Uniform [-1,1] dB centered on the mean.. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model testing - First, obtain full-bandwidth ideal channel matrix for D slot - Then perform SVD decomposition for each subband to derive eigenvectors - Subsequently, find the full-bandwidth channel matrix constructed from AI-recovered CSI at the S slots closest to the D slot, and also perform SVD decomposition for each subband to derive its eigenvectors - Finally, compare these eigenvectors (from the AI-recovered CSI) with the eigenvectors obtained from the ideal channel matrices to calculate SGCS | SGCS, Tput |
| Evaluation benchmark |  | 1) SRS measurements without residual power imbalance 2) non-AI based SRS residual power imbalance compensation | SRS without IL imbalance SRS only based NW-sided model |
| preliminary evaluation results |  | •AI/ML based SRS residual power imbalance compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. •AI/ML is capable of compensating without actual values reporting, meaning there is no need to report updates to the SRS residual power imbalance when changes fall within reasonable range. | See R4-2520373 |
| High level potential specification impact |  | 1) UE assistance information/reporting related to SRS residual power imbalance 2) Potential UE RF related testing | - Signalling/procedure to support lower overhead or simpler explicit CSI feedback - Signalling/ procedure related to LCM for NW-sided model Potential UE RF related testing |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |  |

Qualcomm: this should be RAN1-driven

ZTE; this should be RAN1-driven.

Huawei: SRS-IL issue can be resolved by AI in RAN4.

[FL Recommended Discussion Point]

- Discuss the above key information provided by different companies for use case description, by focusing on following differences in companies’ proposals:

  - AI input/output: with or without CSI-RS based channel matrix feedback

  - How the labels are obtained.

#### Issue 3-3: AI-SRS-assisted channel reconstruction – evaluation related proposals

[Company Proposal Summary]

- Proposals related to evaluation procedure:

  - Huawei:

    - Step-1: Assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation.

    - Step-2: Evaluation methodology by modelling SRS residual power imbalance to baseband evaluation with the following procedure

      - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

        - Training dataset and testing dataset construction

        - Evaluation metric

        - AI models and training type

        - Other details are not precluded

  - Ericsson:

    - Proposal 1: Further investigation could be on “training-data consistency”, “reporting strategy” and “performance vs complexity trade-off”

    - Proposal 2: Further clarification is needed on:

      - Benefit of IL-range reporting method vs zero-reporting method

      - The level of UE implementation transparency for IL-range reporting method

      - How to align the AI model performance

      - Detailed parameter assumptions

[FL Recommended Discussion Point]

- Discussion on the above proposals firstly.

### Use Case #3: AI-enabled PRACH receiver

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520044 | Observation 9: In AI based PRACH, it is not clear how a UE would know its corresponding TA and UL grant if multiple UL grants are tied with the same random access preamble index (RAPID) within one random access response. Observation 10: If network can detect multiple UEs within a PRACH preamble, it may mean that the spacing between two consecutive PRACH preambles, in terms of cycle shifts, is large enough to contain more UEs. If that is the case, network can configure smaller cyclic shift and include more PRACH preambles within the same time-frequency resource. Observation 11: The evaluation of multi-user detection within same PRACH preamble sequence and time-frequency resource completely depends on PRACH sequence of 6G, which is expected to be studied in RAN1. Proposal 6: Multi-user detection within the same PRACH preamble via AI-based PRACH receiver, if required, is more suitable to be a RAN1 led topic. The proponents should clarify how UE could resolve its ambiguities while selecting Msg3 UL grant and why network cannot instead use smaller cyclic shift to increase the number of users per PRACH time-frequence resources before 3GPP starts studying this topic. |
| Samsung | R4-2520373 | Observation 5: From PRACH preamble detection perspective, it is necessary to investigate effective preamble detection algorithm to operate well in the multiple users Observation 6: From random access procedure perspective, it is necessary to investigate the early contention resolution Observation 7: For RAN4 requirement, both demodulation requirement and RRM requirement for random access are specified. - For PRACH requirement, only single preamble detection performance without considering the multi-user collision was specified. - For RRM requirement, RAN4 defined the corresponding random-access requirement with following the random-access procedure specified in RAN1 and RAN2 for both contention and non-contention based random access. Observation 8: For PRACH detection performance perspective, it is feasible to improve the detection performance for single UE and multiple-UE collision scenario with AI/ML-enabled advanced receiver Proposal 4: RAN4 study AI/ML-enable PRACH receiver for 6GR, including both single-user and multi-user collision scenario based on existing PRACH preamble formats as starting point, including LCM procedure, data collection, performance monitoring, and model interference pending on the assumption of AI/ML receiver. Proposal 5: Pending on the study of random-access procedure to enable the early contention resolution with AI/ML-enabled PRACH receiver, RRM requirement for random access also need to be studied, including the UE correct behavior when receiving random access response. |
| Ericsson | R4-2521210 | Observation 11 Based on the agreement in RAN4#116bis meeting for RAN4 driven use-case selection, RAN4 may prioritize those use-cases which have minimum to no impact on other WGs. Observation 12 The AI/ML-enabled PRACH receiver use case depends on new random-access procedures, multi-user detection mechanisms, and potential updates to PRACH preamble design, which are within RAN1’s scope. Current RAN4 requirements only address single-user PRACH detection and cannot assess performance for multi-user collision scenarios. Proposal 19 RAN4 may not prioritize this use-case at this stage and should be driven by RAN1, and its study outcome could be further considered for creating RAN4 performance requirements. |
| ZTE | R4-2521537 | Observation 3: Some potential technical issues need to be discussed in RAN1, and the RACH access procedure may need to be discussed in RAN1 and RAN2. Proposal 4: Propose to postpone the related AI based RACH receiver discussion in RAN4, to be led by RAN1. |

#### Issue 3-4: AI-enabled PRACH receiver use case selection/prioritization

[Company Proposal Summary]

| Sub-use case | Model location and training | Be positive | Be neutral or postpone | Be negative (or led by RAN1) |
| --- | --- | --- | --- | --- |
| AI-enabled PRACH receiver | NW-sided model | *(1) Samsung ‡(0) |  | (3) Qualcomm, Ericsson, ZTE |
| Note: for companies show supporting, with and without numerical results in the Tdocs are separated counted. * with numerical results in Tdocs ‡ without numerical results in Tdocs (x) in which x is the number of companies for that category |  |  |  |  |

[FL Recommended Discussion Point]

- Discuss the sub-use case selection/prioritization in main session.

  - Before showing positive/negative views, companies are encouraged to check the key information for use case description.

  - The following aspects are mentioned to consider AI-enabled PRACH receiver:

    - The proponents should clarify how UE could resolve its ambiguities while selecting Msg3 UL grant and why network cannot instead use smaller cyclic shift to increase the number of users per PRACH time-frequency resources before 3GPP starts studying this topic.

    - New random-access procedures, multi-user detection mechanisms, and potential updates to PRACH preamble design

#### Issue 3-5: AI-enabled PRACH receiver – key information for use case description

[Company Proposal Summary]

| Use Case 3# |  | Samsung |
| --- | --- | --- |
| AI model input | Input in training | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
|  | Input in inference | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
| AI model output | Label in training (if applicable) | Ground-truth number of UEs that transmitted the same preamble |
|  | Output in inference | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Ground-truth number of UEs that transmitted the same preamble |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | Similar to NW-sided model in NR |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameter; Values. Channel type; AWGN/TDLC300-100. PRACH format; C0. Ncs; 0/23. Logical index; 0/1. Preamble ID; 0/5. Number of Preamble; 64. Number of UE; 1/2. Doppler Shift; 2334Hz for restrict set B. Number of Tx/Rx; 1/2. Timing offset between 2UE; 20samples. |
| Evaluation KPI |  | Miss detection rate and false detection rate |
| Evaluation benchmark |  | Legacy receiver with threshold-based method |
| preliminary evaluation results |  | For multi-UE scenario, when the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%. For single UE scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches And also, for HST scenario, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region |
| High level potential specification impact |  | 1. RAN 4 requirements including both demodulation requirement and RRM requirement for random access 2. Signaling/procedure related to random access for more than one UEs with selected the same PRACH preamble 3. Signaling/procedure related to LCM for NW-sided model |
| Feasibility, complexity/cost |  | CNN-based AI model |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

### Other AI-demod issues

#### Issue 3-6: Channel model for defining AI-ML based Demod requirements

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520044 | Observation 12: Existing CDL models are not suitable in case of AI/ML based receivers since such a receiver can learn the determinism of the channel easily and generate too optimistic results. Proposal 7: The applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible, at all, should be discussed in the demod session first. |

[Company Proposal Summary]

- Channel model for defining AI-ML based Demod requirements:

  - The applicability of CDL models for AI/ML receivers and potential enhancements of CDL, if possible, at all, should be discussed in the demod session first.

[FL Recommended Discussion Point]

- Put the proposal together with other points in Issue 1-6.

- Considering the applicability of CDL channel model can be potentially important to AI-demod use case(s), still keep this issue, but suggest discuss the proposal in details after the AI-Demod use case is selected.

#### Issue 3-7: Other proposals for AI Receiver

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |
| Qualcomm | R4-2520044 | Observation 13: It is not possible to study model driven methods for any module as long as the signal structure like waveform, reference signals, modulation and channel codes has not been defined precisely by RAN1 Proposal 8: We propose to postpone any discussion on model driven approaches for the AI receiver till RAN1 has defined the physical layer for 6GR. |
| R4-2521537 | R4-2521537 | Observation 6: The data driven approach regards multiple functional modules of the wireless communication system as an unknown and black box, replaces them with an AI network, and then relies on a large amount of training data input to output training. Observation 7: The model driven approach, based on the existing technology of the wireless communication system, does not change the model structure of the wireless communication system, but uses a deep learning network to replace a certain module or train related parameters to improve the performance of a certain module. Proposal 5: Propose to discuss whether RAN4 can lead the discussion on AI receiver in RAN4, such as legacy release discussed MMSE-IRC and advanced receivers. |

[Company Proposal Summary]

- Model driven approach for AI Receiver:

  - Postpone any discussion on model driven approaches for the AI receiver till RAN1 has defined the physical layer for 6GR.

- Leading group for AI-receiver (if adopted in 6G study):

  - RAN4 discuss whether RAN4 can lead the discussion on AI receiver in RAN4.

[FL Recommended Discussion Point]

- Discuss the proposals firstly.

## Topic #4: AI/ML use cases for RRM issues (Per-use case discussion)

### AI-RRM (sub-)use case selection/prioritization

#### Issue 4-1: AI-RRM (sub-)use case selection/prioritization

[Background] Totally 12 sub-use cases are provided in last meeting, which are captured in [R4-2514637] for information.

[Company Proposal Summary]

FL: To avoid confusion, the table from last meeting is reused to collect companies’ preference

| Index | use cases | Model location (Training method) | Positive | Negative (deprioritized, or wait for other WG-led study) | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | L1 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Spatial domain (inter-cell) - Time and spatial domain (inter-cell) | NW and UE sided model (Offline training) | OPPO (c,e) | Samsung, Qualcomm, MediaTek, LGE, Ericsson | Measurement reduction; HO/LTM performance improvement Gap reduction | Rel-19 AI-BM |  |
|  | L3 beam measurement: - Time domain (intra-cell) - Time and spatial domain (intra-cell) - Freq. and spatial domain (inter-cell) - Time and freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) - Freq. domain prediction for N carriers single cell | NW and UE sided model (Offline training) | Samsung (only multi domain), LGE (only non-collocated), CTC (e), Apple OPPO (c,d), Nokia (a, b, c, d ,e) | Qualcomm, MediaTek, Ericsson | Measurement reduction; HO/LTM performance improvement Gap reduction | Rel-20 AI-Mob |  |
|  | L3 cell measurement: - Time domain (intra-cell) - Freq. domain (inter-cell) - Freq. domain prediction for non-collocated cells (inter-cell) | NW and UE sided model (Offline training) | LGE (only non-collocated), Apple (post-processing based on 2) Nokia (c) | Samsung, Qualcomm, MediaTek, ZTE, Ericsson | Measurement reduction; HO performance improvement Gap reduction | Rel-20 AI-Mob |  |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction | UE sided model (Offline training) | CMCC (non-collocated), Samsung*, LGE, vivo, CTC, CATT | MediaTek, Nokia | MG reduction | Rel-20 AI-Mob | Identify which use cases can serve the purpose of measurement gap reduction |
| 5a | Dynamic Adaptation of Measurement Procedure | UE sided model | Qualcomm*, MediaTek*, Samsung, Xiaomi (AI cell meas. selection), LGE, vivo*, CATT | Ericsson (clarify non-AI counterpart first) | - Measurement reduction - Improved mobility: HO/LTM | Prediction performance: Rel-20 AI-Mob | How to define UE measurement requirements in a way that AI/ML based UE measurement adaptation can be spec-compliant. |
| 7a | Spatial domain RX beam sweeping reduction (L1/L3) | UE sided model | CMCC, Samsung, Qualcomm*, Xiaomi, LGE, vivo*, CTC, Nokia | MediaTek, CATT (coordinate with RAN1) | Measurement delay reduction | Minor dependence on other WG-led use cases. FR2 RX beam sweep factor is transparent to RAN1/RAN2 spec. | Benefits and feasibility of the use of AI-ML to reduce spatial domain RX beam sweeping factor |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction | UE-sided model | Samsung, LGE, vivo | MediaTek |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction | UE-sided model | OPPO | MediaTek (merge to L3 prediction), LGE |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism | UE-sided model | LGE | MediaTek (FFS merge to 5a) |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction | UE-sided model | ZTE* | MediaTek, LGE |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction | UE-sided model | ZTE* | MediaTek, LGE |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving | NW-sided model | CATT | MediaTek, LGE | facilitate data scheduling and network power saving |  |  |
| Note: for companies show supporting, with and without using the template table for use case description * with use case description otherwise: without use case description |  |  |  |  |  |  |  |

[FL Recommended Discussion Point]

- Discuss the sub-use case selection/prioritization in main session.

  - Suggestion to down-scope to 1-2 AI-RRM use cases for more focused discussion.

### AI-RRM prediction models (i.e., AI-RRM use case 1/2/3)

#### Issue 4-2: General view on how to proceed for AI-RRM prediction models

[FL Recommended Discussion Point]

  - For AI-RRM prediction models (AI-RRM use case 1, 2 and 3),

    - Direction 1: by avoiding overlapping with 5G-A use cases

      - L1 beam measurement

        - For FR2, reuse Rel-19 RAN2-led AI-Mobility conclusion.

        - For other sub-use cases, no need to discuss in RAN4 AI-RRM, to avoid overlapping scope with other WGs

      - L3 beam measurement

        - FFS the necessity to study in RAN4 AI-RRM

      - L3 cell measurement

        - No need to discuss in RAN4 AI-RRM.

        - Reuse Rel-20 RAN2-led AI-Mobility conclusion.

ZTE: support the proposal

OPPO: clarification on the coordination between WG

Chair: we should avoid the overlapping between 5G and 6G.

Apple

Nokia

Chair: let’s firstly outline the new use cases which are not part of 6G scope.

[FL Recommended Discussion Point]

- Discuss the above direction firstly.

#### Issue 4-3: L3 Mobility prediction framework and extension

[Company Proposal Summary]

FL: the following L3 mobility prediction framework is proposed by Apple.

- L3 Mobility prediction framework for 6G RRM

  - Study AI/ML-based L3 mobility prediction as a structured framework covering beam-level and cell-level L3 RSRP prediction across time, frequency, and spatial domains, prioritized for progressive investigation. The objective is to reduce measurement gaps, improve mobility, and enable predictive RRM operations through multi-domains. The study will proceed as follows:

    - Step 1: Establish single-domain prediction (time, frequency, spatial) for intra/inter-cell RSRP at beam and cell levels.

      - Time: L3 beam level, UE sided model, intra-cell

      - Frequency: non-collocated for both beam and cell level, cluster based

      - Spatial: Joint TX-RX

    - Step 2: Extend to joint-domain prediction (time + frequency / time + spatial)

    - Step 3: Investigate full multi-domain prediction (time + frequency + spatial) to reconstruct complete L3 RSRP sets from measurements.

    - Step 4: Apply prediction outputs to RRM procedures such as dynamic measurement-gap reduction, CFRA mobility optimization, and Scell operations.

    - Step 5: Quantify end-to-end performance gains, throughput, latency, power efficiency, and mobility robustness, as derived benefits.
The prediction models will be treated as enabling tools, with RAN4 focusing on defining principles, input/output metrics, and accuracy evaluation before benefit assessment in later phases.

- L3 Mobility prediction framework is extended to L1 beam level mobility:

  - Extend the AI/ML-based L3 beam-level mobility prediction framework to L1 beam-level (LTM) mobility, as both share similar temporal, spatial, and frequency characteristics. The study will reuse L3 prediction principles to predict L1 beam evolution for mobility enhancement.

[FL Recommended Discussion Point]

- Discuss the above proposals firstly

### Application scenarios for AI-RRM prediction models (key information)

#### Issue 4-4:  AI-RRM use case 4: Multi-Domain L3 level Prediction for Measurement-Gap Reduction

[Company Proposal Summary]

|  |  | Samsung | Apple |
| --- | --- | --- | --- |
| AI model input | Input in training (Multi-domain: Joint time and frequency) | Measured RSRPs for all candidate frequency layers/cells for time series until t | Time sequence of L3 RSRP for subset beams on f₁ from one or more cells Assistance information: deployment geometry, frequency scaling, codebook class, TRP metadata Optional features: UE speed, Doppler, beam index, PCI |
|  | Input in inference | Measured L3 RSRPs corresponding to multi-dimensional minimal selected Set B | Sliding window of L3 RSRP for subset beams (e.g., last 3-5 samples per beam) with assistance information Input tensor: RSRP[cell, beam, time] on f₁ → [M cells × B beams × T samples] |
| AI model output | Label in training (if applicable) | Legacy measured L3 RSRPs | Future L3-RSRP measurements (legacy measurements at future time) |
|  | Output in inference | Predicted L3 RSRPs corresponding to multi-dimensional complete Set A | Top-1 / Top-K predicted future best beam(s) on f₂ Target cell: Cell-B or Cell-C (may differ from input Cell-A) Predicted metric: Future L3-RSRP or probability score |
| Assumption on training | Training type | Offline training | Offline training |
|  | Label construction (if applicable) | Through legacy measurement | Through legacy measurement |
| model location for inference |  | UE-sided model | UE-sided model |
| Collaboration/interaction between UE and NW |  | TBA | Assistance Information for non-collocated |
| Evaluation methodology |  | System-level simulation | System-level simulation |
| Evaluation assumption |  | TBA | TBA |
| Evaluation KPI |  | Intermediate KPIs：RSRP difference as baseline Performance KPI: e.g., QoS related KPI and/or HO performance KPI | Intermediate KPIs：RSRP statistical difference as baseline Performance KPI: KPI for HO performance, CFRA success rate, |
| Evaluation benchmark |  | Time domain: Sample and hold Frequency domain: Predict based on co-located/ non-collocated cell measurement with pathloss modification | Time domain : sample and hold |
| Preliminary evaluation results |  | TBA | TBA |
| High level potential specification impact |  | TBA | TBA |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Measurement delay requirements and scheduling requirements | Measurement delay requirements and scheduling requirements |
| Expected benefits |  | MG reduction Measurement delay reduction HO performance enhancement | Proactive handover FR1→FR2 offloading CA activation Multi-layer mobility optimization, MG Reduction, Measurement delay reduction, CFRA enhancement |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

#### Issue 4-5:  AI-RRM use case 5a: Dynamic Adaptation of Measurement Procedure

[Company Proposal Summary]

|  |  | Apple | Qualcomm | MediaTek | vivo |
| --- | --- | --- | --- | --- | --- |
| AI model input | Input in training |  Primary serving cell (f1 macro): RSRP, RSRQ,etc  Secondary serving frequency (f4): Signal quality metrics  Mobility context: Timing Advance, velocity, mobility state (stationary/mobile)  Historical context: Past frequency detections (e.g., "f2 detected 3 times in last hour", "f6 never detected in 30 days") | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells | RSRP of serving cell and neighbor cells | Necessary input: - Current signal quality data of serving cell (such as RSRP, etc.) Other possible input: network-configured measurement thresholds (such as ssb-RSRP, csi-RSRP, if any) |
|  | Input in inference | Minimal selected Set B: Multi-dimensional subset spanning reduced Tx/Rx beam pairs, Serving cell measurements from f1 Compact fingerprint: Input tensor example: RSRP[cell, beam, time] on f1 → [M cells × B beams × T samples] + Assistance Information vector Assistance information: Deployment geometry, codebook class etc, | Beam and/or cell level RSRP measurements from serving cell and/or neighbor cells | RSRP of serving cells and neighbor cells | Necessary input: - Current signal quality data of serving cell (such as RSRP, etc.) Other possible input: network-configured measurement thresholds (such as ssb-RSRP, csi-RSRP, if any) |
| AI model output | Label in training (if applicable) |  Complete Set A: Full measurement results from exhaustive legacy procedures  Future L3-RSRP measurements at time t+Δt for all target frequencies (f2, f3, f5, f6, f7, f8, f9)  Binary coverage labels: Whether each hotspot frequency was detectable (0/1)  Per-frequency RSRP values when cells detected | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells | Suppose inter-f measurement on f2 is configured. The label is whether any cell on f2 is [detectable] | Necessary: - Signal quality data of serving cell (such as RSRP, etc.) Other possible input: Measurement decision labels based on network-configured measurement thresholds |
|  | Output in inference | Per-frequency predictions (f2, f3, f5, f6, f7, f8, f9): Coverage likelihood: 0-100% probability score for each target frequency Prediction confidence: 0-100% uncertainty estimate per prediction Top-1 / Top-K predicted best beams on target frequency f2 (future time) Decision output: MEASURE, DOWNSCALE, ON-DEMAND, SKIP Adaptive Set B sizing:Recommended number of Tx/Rx beam pairs to measure Suggested time window length (number of samples) | Beam and/or cell level RSRP measurements from serving cell and neighbor cells which may or may not be co-located with the measurement cells | Whether any cell on f2 is [detectable] | Necessary: - Predicted value of future signal quality of serving cell Other possible input: Measurement decision |
| Assumption on training | Training type |  Network-side option (Approach A): Training at gNB or RAN Intelligent Controller (RIC) using aggregated data from multiple UEs  UE-side option (Approach B): Local training on device using UE's own measurement history | offline training, potentially site-specific online finetuning | Offline training | Offline training/Online training |
|  | Label construction (if applicable) |  Ground truth collected via traditional exhaustive inter-frequency measurements  UE performs full measurement sweeps on all configured frequencies during training phase | Collected measurement data | Through legacy inter-f measurement | - Based on system-level simulation Based on filed data |
| model location for inference |  | UE sided | UE-sided model | UE-sided model | UE-sided model |
| Collaboration/interaction between UE and NW |  | TBA Assistance information | With or without collaboration/interaction, depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW. | No collaboration/interaction | - NW side configures basic parameters for the UE, potential including measurement thresholds (such as ssb-RSRP), measurement parameter related (such as SMTC DRX), etc.; - After UE side generates predicted measurement results (or measurement decisions), it reports the results as required by the network; NW can schedule UE according to measurement decisions |
| Evaluation methodology |  | SLS | System level simulation (a new simulation might not be necessary, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged) | System level simulation | System level simulation and link level simulation |
| Evaluation assumption |  |  f1 (macro): Continuous coverage everywhere (baseline)  f2 (hotspot): Dense urban areas, shopping centers, transportation hubs only (~30-40% geographic coverage)  f3 (hotspot): Similar to f2 but slightly smaller footprint | May or may not be needed, depending on how much out of Rel-20 AI/ML-based mobility can be leveraged | Serving cell is on f1. To-be-predicted frequency is on f2. Set up a scenario that f1 has a full coverage but f2 does not. | The evaluation assumption on AI/ML PHY BM and AI mob time domain RRM measurement prediction can be as the starting point |
| Evaluation KPI |  | RSRP prediction accuracy | Measurement period optimization for more relevant cells (in terms of RSRP), compared to the RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. | number of preditions that is righttotal number of preditions | RSRP prediction accuracy and other possible performance metrics (e.g., decision prediction accuracy) |
| Evaluation benchmark |  | Measurement following legacy requirements | RAN4 minimum requirements, which mandate TDM-based measurements across all cells and frequencies in the configured measurement object. | Measurement following legacy requirements | Non-AI measurement initiation mechanism, such as traditional S-measure measurement mechanism and cell reselection measurement mechanism |
| Preliminary evaluation results |  |  | TR 38.744 “Technical Report on Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR” |  |  |
| High level potential specification impact |  | Measurement period requirements Dynamic MG control mechanisms UE measurement reporting enhancements | Overall measurement period requirements in RAN4 specification. Potentially, RAN2 signalling support (depending on whether and to what extent the UE measurement adaptation decision needs to be communicated to the NW) | New measurement delay requirements to allow dynamic adaptation of measurement procedure. | Potential new requirements: - Prediction delay requirements - Measurement period measurement in the AI-assisted measurement scenarios - RSRP prediction accuracy and other possible performance metrics (e.g., decision prediction accuracy) - Potential scheduling requirements Impact to other WG’s specifications: - Necessary signaling Necessary procedure based on AI prediction results |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Model complexity and UE computational requirements: Training data collection and model deployment: Network and UE coordination: | No big concern compared to other AI/ML use cases. | The model complexity is expected to be not high as it is just a classification model. | Similar to AI mob use case- time domain prediction with one cell |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

#### Issue 4-6:  AI-RRM use case 7a: Spatial domain RX beam sweeping reduction (L1/L3)

[Company Proposal Summary]

| Sub-use case 7a |  | vivo | Qualcomm | Apple |
| --- | --- | --- | --- | --- |
| AI model input | Input in training | Measurements of target reference signals using a reduced set of RX beams with corresponding subset/full set of Tx beams | L1-RSRP of a subset (<8) of RX beams | L1-RSRP of a subset of Rx beam with corresponding subset/full set of Tx beams |
|  | Input in inference | Measurements of target reference signals using a reduced set of RX beams with corresponding subset/full set of Tx beams | L1-RSRP of a subset (<8) of RX beams | A reduced set of RX beams with corresponding subset/full set of Tx beams |
| AI model output | Label in training (if applicable) | Properties of the full set of RX beams (with corresponding set A Tx beams) for the reference signals measured with subset RX beams (with corresponding set A or set B Tx beams) | L1-RSRP of “full set” (=8) of RX beams | Full set of RX beams (with corresponding set A Tx beams) |
|  | Output in inference | Tx-Rx beam pair prediction result | L1-RSRP of “full set” (=8) of RX beams | Tx-Rx beam pair prediction result |
| Assumption on training | Training type | offline training | Offline training | Offline training |
|  | Label construction (if applicable) | Labeled with actual full RX beams (with corresponding set A Tx beams) and the subset RX beam measurements (with corresponding set A or set B Tx beams) of the same reference signals | Measurement data | Labelled measured data of Full set of RX beams (with corresponding set A Tx beams) |
| model location for inference |  | UE-sided model | UE-sided model | UE-sided model |
| Collaboration/interaction between UE and NW |  | NW informs UE deployment configuration, e.g., Application ID | UE informs network the required number of RX beams it needs to measure to predict the properties of full set of RX beams. | NW informs UE ASSOCIATED ID |
| Evaluation methodology |  | system level simulation, and link level simulation. | system level simulation | SLS |
| Evaluation assumption |  | The evaluation assumption on AI/ML PHY BM can be as the starting point | As an example, the framework of Rel-18 and Rel-19’s evaluation assumptions for AI-ML beam management (mentioned in table 6.3.1-1 of 38.843) can be used. This framework needs to be slightly modified to focus on RX beam prediction, instead of TX beam prediction. Other examples should also be considered, especially to evaluate RX beam sweeping factor reduction in L3. | AI/ML BM as starting point |
| Evaluation KPI |  | - RSRP prediction accuracy - Beam prediction accuracy | Accuracy of predicted RSRP | RSRP prediction accuracy Beam prediction accuracy |
| Evaluation benchmark |  | Compare the performance of AI/ML-based reduced RX beam sweeping with legacy full RX beam sweeping in FR2 | Radial basis function interpolation | Compare with legacy full RX beam sweeping in FR2 |
| Preliminary evaluation results |  |  |  |  |
| High level potential specification impact |  | Potential new requirement - Reduced measurement period for FR2 measurement due to reduced Rx beam sweeping factor - TCI state known condition - Performance requirements, including beam prediction accuracy and RSRP accuracy | UE capability regarding the required number of RX beams. | Reduced measurement period for FR2 measurement due to reduced Rx beam sweeping factor TCI state known condition Performance requirements, including beam prediction accuracy and RSRP accuracy |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Model complexity: - UE-side computational load and memory resource complexity for beam pair prediction Testability issue | No big concern compared to other AI-ML use cases | Feasibility : Extend this study to L1-L3 beam and cell level for mobility |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

#### Issue 4-7:  AI-RRM use case 12: AI/ML based AGC through temporal/frequency domain prediction

[Company Proposal Summary]

|  |  | ZTE |
| --- | --- | --- |
| AI model input | Input in training | AGC values in historical measurements (t1) |
|  | Input in inference | AGC values in historical measurements (t2, where t2>t1) |
| AI model output | Label in training (if applicable) | / |
|  | Output in inference | Corrected AGC values |
| Assumption on training | Training type | offline training |
|  | Label construction (if applicable) | / |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | no collaboration/interaction |
| Evaluation methodology |  | system level simulation |
| Evaluation assumption |  | FFS |
| Evaluation KPI |  | - Difference between predicted AGC and measured AGC - Maximum difference among predicted AGCs |
| Evaluation benchmark |  | Use additional sample(s) to finish the AGC |
| Preliminary evaluation results |  | FFS |
| High level potential specification impact |  | Reduce measurement delay, reduce RS overhead and power saving |
| Feasibility issues, including complexity, and other aspects related to implementation |  | FFS |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

#### Issue 4-8:  AI-RRM use case 13: AI/ML based DL sync through temporal/frequency domain prediction

[Company Proposal Summary]

|  |  | ZTE |
| --- | --- | --- |
| AI model input | Input in training | Location of UE and two cells |
|  | Input in inference | Received Time difference |
| AI model output | Label in training (if applicable) | / |
|  | Output in inference | Corrected Received Time difference |
| Assumption on training | Training type | offline training |
|  | Label construction (if applicable) | / |
| model location for inference |  | UE-sided model |
| Collaboration/interaction between UE and NW |  | no collaboration/interaction |
| Evaluation methodology |  | system level simulation |
| Evaluation assumption |  | FFS |
| Evaluation KPI |  | - Difference between predicted RTD and measured RTD 2. Maximum difference among predicted received Times |
| Evaluation benchmark |  | Use additional sample(s) to finish the DL sync |
| Preliminary evaluation results |  | FFS |
| High level potential specification impact |  | Reduce measurement delay, reduce RS overhead and power saving |
| Feasibility issues, including complexity, and other aspects related to implementation |  | FFS |

[FL Recommended Discussion Point]

- Discuss if questions are raised over the information provided.

### Other directions

#### Issue 4-9:  Other directions

[Company Proposal Summary]

- The following new use cases are proposed by Apple:

  - SSB packing and MGL reduction : RAN4 is invited to study AI/ML-based SSB subset prediction and packing to reduce MGL by reconstructing the full SSB set (Set A) from a smaller measured subset (Set B)

  - Dynamic MG control: RAN4 is invited to study AI/ML-driven dynamic measurement-gap control with adaptive Set-B resizing for MGL reduction

  - Cell-identification for noncollocated: Study mechanisms to enable cell-identification for AI/ML inter-frequency prediction in non-collocated 6G deployments.

  - CFRA optimization with beam level prediction: Study the extension of AI/ML-based mobility prediction from cell-level to beam/TRP-level, addressing the critical gap where correct cell prediction still fails if the wrong SSB beam is used at CFRA execution.

  - CFRA enhancement in asymmetric DL-UL: RAN4 is invited to study an AI/ML-based CFRA enhancement where the UE, using only DL measurements, predicts the optimal UL TRP in asymmetric multi-TRP deployments.

  - UL-aware mTRP mobility prediction: RAN4 is invited to study UL-aware, TRP-aware inter-frequency mobility prediction that combines DL metrics with UL-reachability indicators to ensure the selected target is not just strongest but uplink-viable.

  - multi-TRP beam prediction: Investigate an AI/ML-based framework for multi-TRP beam prediction that supports both coherent and non coherent mode.

[FL Recommended Discussion Point]

- Discuss the proposals firstly

# 1st Round Offline Discussion

Note: Placeholder for xx round discussion

## Topic #1: RAN4 AI/ML framework and other general aspects

### Issue 1-1: Post-deployment enhancement

[FL Recommended Discussion Point]

- FL: Check the views on the following clarification on existing pre-deployment conformance test and two options of post-deployment enhancement:

  - If there is different view, pls. provide it.

|  | Ways to guarantee AI/ML performance | When to perform | Where to perform | Comment |
| --- | --- | --- | --- | --- |
| (1) | Pre-deployment conformance test | Before cell-phone shipped into market | Testing lab for conformance testing | Same as existing conformance testing |
| (2) | Post-deployment pre-activation functionality test (Option 1 in Rel-19 discussion) | After cell-phone shipped into market, but before new AI/ML functionality activated | UE vendors’ lab | Similar as product testing |
| (3) | Post-deployment post-activation functionality testing based on performance monitoring (Option 2 in Rel-19 discussion) | After new AI/ML functionality activated | In-field practical network environment | No yet introduced in Rel-19. |

- FL: For (1), check the views on (1) should be included in the scope for 6G post-deployment enhancement.

- FL: For (2) (3), check the views on (2) and (3) should be included in the scope for 6G post-deployment enhancement.

  - Note: it is noted that (3) (i.e., Option-2 in Rel-19) is not introduced in Rel-19, but here companies’ view on (3) for 6G study are encouraged to check by re-thinking it is the 6G study.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Nokia | In (2) Post-deployment pre-activation functionality test, it shouldn’t be tested in UE vendors’ lab. It should be rather tested in testing lab for conformance testing. (2) is not a full conformance testing and is tailored to testing specific ML feature(s) only, therefore it is much faster than (1) and helps in-field validation (3). |
| ZTE | For (1) we do not think it should have a dedicated discussion in 6G. Since this is pre-deployment conformance test, as comments form FL, this is the same as existing conformance testing. For (2), we don’t know why the conformance test performs in UE vendor’s lab. For (3), this is kind of much flexible. We are fine to continue to consider it in 6G since (3) is not introduced in R19. Actually, do we really need (2)? We have two kind of performance tests: pre-deployment and post deployment. For pre-deployment, we could follow legacy. In 6G day 1, post deployment with (3) is much more clear to us. We would like to study post deployment conformance test step by step. RAN1/RAN2 is discussing online training, we think only performance monitoring like legacy is not enough. How to prevent the quick degradation and how to prevent the wrong decision on performance is the key point in 6G study. |

### Issue 1-2: Model standardization and deployment

[FL Recommended Discussion Point]

FL: Because AI UE category related discussion is agreed to deferred to Q2 2026, suggest only discuss model complexity below, while “constraints on model deployment” should also be deferred.

- For model complexity,

  - RAN4 should define AI model complexities, by considering: number of model parameters, model computing complexity, model size, data requirement, inference latency, and model generalization.

  - If model complexity is agreed to be one study direction, FFS the following example to differentiate “Simple” and “Complex” models:

|  | "Simple model” | "Complex model" |
| --- | --- | --- |
| Num of model parameter | Low, e.g., <1M | High, e.g., 1M ~10M |
| Model computing complexity | Low, e.g., <1M FLOPs, or [1M FLOPs ~10M FLOPs] | High, e.g., [1M FLOPs ~10M FLOPs], or 10M FLOPs ~100M FLOPs |
| Model size | Small, e.g., <1MB | Large, e.g., 1MB ~10MB |
| [Inference latency] | Low | High |
| [Training data requirement] | Low | High |
| [Generalization] | Sufficient for simple tasks | High potential for complex tasks |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| ZTE | The complexity issues shall be discussed case by case not in general part like legacy. For example, for a certain use case, its actual worst-case scenario is actually less severe than the overall specified worst-case scenario. However, if the evaluation result falls between the actual worst-case scenario and the overall specified worst-case scenario, this may lead to ambiguity. |

### Issue 1-3: AI/ML framework for RAN4-driven use cases (closed)

FL: Considering the proposal can only be discussed in-depth based on particular use case, and it is hard to get some general principle now, it is suggested to close this issue in this meeting and this discussion can be re-triggered by relevant identified use cases in future meetings.

### Issue 1-4: Principle for use cases selection and discussion (closed)

FL: There is the existing agreement on the principle for AI/ML use cases selection in RAN4#116bis. Furthermore, to select use cases should depends on per-use case analysis and overall discussion, in which more additional principles may not be helpful. By considering that, this issue is closed for this meeting.

### Issue 1-5: Interoperability for one-sided and two-sided models

[Company Proposal Summary]

- Interoperability for one-sided model,

  - RAN4 to study interoperability issues arising from the deployment of UE sided AI/ML models for agreed functionalities in 6G.

- Interoperability for two-sided model,

  - No more discussion in 6G study, while the to-be-specified mechanism for CSI compression in Rel-20 5G-A WI can be used to address interoperability issues in AI/ML based 6G features that require two-side model deployment.

[FL Recommended Discussion Point]

- For interoperability for one-sided model, the proposed WF is very general, and encourage proponent companies to give more detailed proposal.

- For interoperability for two-sided model, the proposed WF seems straightforward and can be adopted.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| ZTE | We are fine to second bullet. But how to understand interoperability on one-side model. For two-side model, R20 discussion could be await. |

### Issue 1-6: Channel model for evaluation (closed)

FL: Considering most of the proposals should be discussed based on particular use case, and it is hard to get some general principle now, it is suggested to close this issue in this meeting and this discussion can be re-triggered by relevant identified use cases in future meetings.

### Issue 1-7: Other topics with low priority (closed)

FL: The proposals listed in this issue involve topics which are agreed to begin no early than Q2 2026. Therefore, no further discussion in RAN#117.

## Topic #2: AI/ML use cases for RF issues (Per-use case discussion)

### Use Case #1: AI-nonlinearity compensation

#### Issue 2-1: AI-nonlinearity compensation (sub-)use case selection/prioritization (closed)

FL: the issue is closed with following agreement achieved during Tuesday online session.

Agreement:

- RAN4 will study the following 6G AI use cases

- Case 1.1: AI-based DPoD+no DPD

- Case 1.2: AI-based DPoD+non-AI DPD

- Case 2: no DPoD+AI based DPD

- benchmark: 1) non-AI DPD only 2)non-AI DPoD only

  - The interested companies can choose to compare with no DPD/DPoD and non-AI DPD(UE)+non-AI DPoD(BS)

- Training: TBD

- Evaluation metric: TBD

#### Issue 2-2: General regulatory aspects for evaluation (closed)

FL: The proposal will be considered in evaluation procedure.

#### Issue 2-3: AI-DPoD in gNB – key information for use case description (closed)

FL: for information only.

#### Issue 2-4: AI-DPoD in gNB – evaluation related proposals

[FL Recommended Discussion Point]

Ad-hoc discussion:

- Proposals related to Step-1 (Non-linearity model(s) of transmission signals is determined to be used in the evaluation):

  - Considering the non-linearity model(s) of transmission signals are agreed to be considered in other 6G agenda (e.g., system parameter agenda and UE RF agenda), suggest to hold the discussion now.

- Proposals related to Step-2 (Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation):

  - FFS Joint session or separate session of AI and non-AI non-linearity compensation

  - Analyze on the feasibility for both online and offline training

  - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).For a given Tx EVM value derived based on the non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target SNR to meet RAN4 requirement or test metric, e.g., 70% of maximum throughput or 10% BLER

Qc: the use cases could be dependent on other session’s conclusion.

    - Case 1.1: AI-based DPoD + non-linearity TX model(s) (no DPD)

    - Case 1.2: AI-based DPoD + non-linearity TX model(s) (non-AI DPD)

    - Case 1.3: non-AI-based DPoD + non-linearity TX model(s) (no DPD)

    - Case 1.4: non-AI-based DPoD + non-linearity TX model(s) (non-AI DPD)

Samsung: for non-AI DPD, how we can get the non-linearity model for non-AI DPD case.

Qc: it is still under UE RF session discussion to discuss the non-linearity by considering non-AI DPD.

    - Benchmark

      - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD)

      - Additional benchmark: No DPoD + non-linearity TX model (no DPD)

    - FFS which RAN4 use case should be used.

    - FFS the possibility to align non-AI and AI-based DPoD

    - FFS how to proceed the work before non-linearity model is not agreed in other sessions.

      - FFS how to proceed if there are multiple non-linearity models from 6G system parameter and UE RF sessions.

    - FFS TxEVM values

    - Detailed RF transmission scheme and configuration:

      - Use 5G NR existing supported scheme and configuration as starting point, and will be finally determined based on the conclusion from 6G system parameter and UE RF sessions

- Proposals related to Step-3 (For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation):

  - Hold the discussion until Step-2 is clarified.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

#### Issue 2-5: AI-DPD in UE – key information for use case description (closed)

FL: for information only.

#### Issue 2-6: AI-DPD in UE – evaluation related proposals

[FL Recommended Discussion Point]

Ad-hoc discussion:

- For Step-2:

Vivo: For AI DPD, EVM is not the only requirement to be considered in the evaluation. In addition, since there is no DPoD at BS side in case2, so the impact on SNR doesn’t need to be considered, so we suggest to modify the sub-bullet.

Samsung: why no SNR considered?

- For Step-2:

  - For a given non-linearity models, evaluate the performance with following assumptions to find the target output power (after applying AI-based DPD) to meet RAN4 Tx requirements, including ACLR/SEM/IBE/EVM

    - FFS how to perform the evaluation on the impact of remaining non-linearity after AI-DPD.

    - Case 2: No DPoD + non-linearity TX model (AI-based DPD)

    - Benchmark

      - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD)

      - Additional benchmark: No DPoD + non-linearity TX model (no DPD)

    - FFS how to proceed the work before non-linearity model is not agreed in other session.

      - Strive to have aligned model as DPoD evaluation.

      - FFS how to proceed if there are multiple non-linearity models from 6G system parameter and UE RF sessions.

    - FFS Tx requirements values

    - Detailed RF transmission scheme and configuration:

      - Use 5G NR existing supported scheme and configuration as starting point, and will be finally determined based on the conclusion from 6G system parameter and UE RF sessions

- Proposals related to Step-3 ( For given Tx requirements, conduct Tx MPR and power boosting simulation.):

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| vivo | For AI DPD, EVM is not the only requirement to be considered in the evaluation. In addition, since there is no DPoD at BS side in case2, so the impact on SNR doesn’t need to be considered, so we suggest to modify the sub-bullet: - For a given non-linearity models, evaluate the performance with following assumptions on selected RAN4 test cases to find the target output powerto meet RAN4 Tx requirements, including ACLR/SEM/IBE/EVM - Case 2: No DPoD + non-linearity TX model (AI-based DPD) - Benchmark - Benchmark 1): No DPoD + non-linearity TX model (non-AI DPD) - Additional benchmark: No DPoD + non-linearity TX model (no DPD) - FFS which RAN4 use case should be used. - FFS how to proceed the work before non-linearity model is not agreed in other session. - FFS Tx requirements values - Proposals related to Step-3 ( For given Tx requirements, conduct Tx MPR and power boosting simulation.): Hold the discussion until Step-2 is clarified. |

## Topic #3: AI/ML use cases for Demod issues (Per-use case discussion)

### Use Case #2: AI-SRS-assisted channel reconstruction

#### Issue 3-1: AI-SRS-assisted channel reconstruction (sub-)use case selection/prioritization

FL: Based on latest RAN1 agreement, AI-based SRS power imbalance compensation has been confirmed “Not RAN1-led”. Therefore, the concern on the clear worksplitting between RAN1 and RAN4 can be resolved for AI-based SRS power imbalance compensation. Therefore, the relevant issues are brought back for discussion.

Copied latest RAN1 agreement for information:

RAN1 Agreement

From RAN 1 perspective, the following use cases can be matched to the identified primary agendas of RAN1

| Use cases |  |  | Primary agendas |
| --- | --- | --- | --- |
| Low overhead CSI-RS or CSI prediction with AI/ML |  |  | CSI-RS and CSI acquisition |
| Low overhead DMRS with AI/ML receiver |  |  | UL & DL DMRS associated with PUSCH/PDSCH Note: Sub-Case C: DMRS-free may be related to modulation |
| CSI compression and feedback |  |  | CSI acquisition Note: this may be related to uplink control |
| AI/ML for beam management and extension |  |  | Initial access for Sub-case D Beam management for other sub-cases Note: sub-case A/B/D maybe related to mobility |
| AI/ML for SRS |  | Low overhead SRS with AI/ML | SRS |
|  |  | Low PAPR SRS sequence design |  |
|  |  | AI/ML based SRS power imbalance compensation | Not RAN1-led |
| AI-enabled UL precoder indication |  |  | UL MIMO |
| AI-based non-linearity handling at transmitter or receiver |  |  | Not RAN1-led Note: this may be related to DMRS, SRS, Power control in RAN 1 |
| AI/ML for (de)modulation |  |  | modulation Note: Sub-case B may be related to MIMO Note: assuming no change to DMRS |
| AI/ML based waveform for PAPR reduction |  |  | Waveform |
| AI/ML based HARQ-ACK feedback |  |  | Channel coding Note: this may be related to uplink control |
| PDCCH related |  | Prior-Information-Aided DCI Decoding | DL control |
|  |  | Lossless DCI Compression |  |
| Power control related |  | UL closed-loop power control | Power control |
|  |  | Pathloss prediction |  |
| RACH related design |  | Early contention resolution in RACH | Random access/PRACH |
|  |  | Low PAPR PRACH sequence design |  |
| Site Specific Learning for AI/ML using RAN Digital Twin |  |  | Depending on corresponding use case where site specific learning is applicable, e.g., DMRS |
| Digital twin construction related use cases | AI/ML-enabled RAN digital twin with distributed model |  | ISAC |
|  | Sensing based RAN digital twin construction with NW-side AI/ML model |  | ISAC |
| AI for positioning |  |  | Positioning related agenda, if any |

| To accommodate AI/ML service, e.g., token traffic | if any impact, most likely scheduling/HARQ |
| --- | --- |

[FL Recommended Discussion Point]

- Discuss the sub-use case selection/prioritization in main session.

  - Before showing positive/negative views, companies are encouraged to check the key information for use case description.

  - The following aspects are mentioned to consider AI-based SRS residual power imbalance compensation:

    - SRS IL compensation if power headroom is available

    - SRS power imbalance issue severity if operating frequency is higher in 6G

    - SRS power imbalance issue severity if the number of UE antenna is larger in 6G

    - Whether or not non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1

Ad-hoc discussion:

QC: if so, there should be no problem for SRS IL compensation.  In Huawei contribution, it is said UE can report the range of IL range. The considered IL values are not practical.

Huawei: Agree with QC that there should be no problem for SRS IL compensation if there is power headroom. Four values are for four RX ports. The IL range report can be per-port.

QC: RAN1 only preclude SRS power imbalance use case from RAN1. What if the RAN1 discussion on the remaining use cases can impact RAN4 study?

Huawei: It is not harmful to discuss this in RAN4.

Nokia: IL issue could be also discussed in 6G UE RF session for non-AI scheme.

Huawei: not against non-AI solution but still prefer to discuss AI-based solution. Non-AI solution has been discussed over several releases but no conclusion.

Nokia: can we wait till UE RF session has conclusion?

QC: limited TU limitation.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Huawei, Hisilicon | To reply “SRS IL compensation if power headroom is available”: - SRS IL imbalance issue has already been recognized by RAN4 and later confirmed by RAN1 during previous releases. Keeping debating whether SRS power imbalance is much of an issue is a waste of time. Anyone who does not have this background can sync up with their colleagues from RF session or review Tdocs back then. - Regarding “SRS imbalance issue should only occur when the UE is operating at, or close to, maximum Tx power”, based on our understanding, the existence of IL imbalance issue comes from the RF architecture rather than the specific power setting. - Some companies believe that if power headroom is available, the SRS imbalance issue can be resolved by UE self compensation. - > Even if that’s the case, we do not think the discussion should be based on any spec-transparent hypothesis unless a consensus have been reached. Whether and how to define UE behavior is out of the scope of this AI based compensation study. However, if UEs are not willing to compensate due to power saving or the compensation is not perfect, then compensation at NW will help. - If power headroom is not available, SRS power imbalance occurs because UE cannot fully compensate for the loss. - > This is the case we are focusing on, where residual power imbalance persists. To reply: “SRS power imbalance issue severity if operating frequency is higher in 6G” and “SRS power imbalance issue severity if the number of UE antenna is larger in 6G” - True. That’s the intention to introduce this use case to help improve the quality of CSI from SRS for 6G. To reply: “Whether or not non-AI solutions for SRS IL compensation will be discussed and agreed in 6G in RAN4/RAN1” - In R19, for CSI prediction and beam prediction, 3GPP has never touched non-AI-based methods before the study. If many companies hold the view that this is a critical issue and non-AI algorithms are really needed, then what harm could there be in our discussing AI here? Moreover, if companies also show interest in non-AI-based solutions, it is up to them to propose and discuss such solutions, or even use them as a benchmark to investigate whether AI is beneficial on top of that. - If we stop here just because there are no non-AI-based solutions specified in 3GPP, waiting and hoping that non-AI-based methods may be come up by some companies in somewhere, then what would we do if no non-AI solutions work in the end? Would we start after then? We are afraid that it would be too late. - To be honest, if non-AI works, this would not be the situation where, after struggling for 2 releases, there is still nothing. |

#### Issue 3-2: AI-SRS-assisted channel reconstruction – key information for use case description (closed)

FL: for information only. See AI-based SRS power imbalance compensation part.

#### Issue 3-3: AI-SRS-assisted channel reconstruction – evaluation related proposals

[Company Proposal Summary]

- Proposals related to evaluation procedure:

  - Huawei:

    - Step-1: Assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation.

    - Step-2: Evaluation methodology by modelling SRS residual power imbalance to baseband evaluation with the following procedure

      - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

        - Training dataset and testing dataset construction

        - Evaluation metric

        - AI models and training type

        - Other details are not precluded

  - Ericsson:

    - Proposal 1: Further investigation could be on “training-data consistency”, “reporting strategy” and “performance vs complexity trade-off”

    - Proposal 2: Further clarification is needed on:

      - Benefit of IL-range reporting method vs zero-reporting method

      - The level of UE implementation transparency for IL-range reporting method

      - How to align the AI model performance

      - Detailed parameter assumptions

[FL Recommended Discussion Point]

- Discussion on the above proposals firstly.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |
| Huawei, Hisilicon | To reply “Benefit of IL-range reporting method vs zero-reporting method”: - We will bring this result next meeting. To reply “The level of UE implementation transparency for IL-range reporting method” - UE can compensate or not compensate, it’s up to UE’s implementation. However, the remaining/ residual power imbalance related information is required to facilitate NW-side compensation. To reply “How to align the AI model performance” - Reference model can be used just for alignment purpose. To reply “Detailed parameter assumptions” - Please find it in Issue 1.3.1. |

### Use Case #3: AI-enabled PRACH receiver (closed)

FL: considering online discussion on Tuesday, suggest to hold the discussion till more clear worksplitting between RAN1 and RAN4 can be made to confirm RAN4 study scope on this use case.

## Topic #4: AI/ML use cases for RRM issues (Per-use case discussion)

### AI-RRM (sub-)use case selection/prioritization

#### Issue 4-1: AI-RRM (sub-)use case selection/prioritization

FL: As guided by RAN4 chair, RAN4 should provide more detailed information for use case 1/2/3 firstly (treated in Issue 4-2), and as suggested by some companies, the group can still further discuss the selection/prioritization for other use cases, as below.

[Company Proposal Summary]

FL: L1/L3 cell/beam measurement prediction use cases will be treated in Issue 4-2 and there are the other proposals:

| Index | use cases |  | Model location (Training method) | Positive for the sub-use case to be included in 6G study with priority | Positive for the use case to be included in 6G study after the prioritized sub-use cases are concluded | Expected benefits (e.g., RS reduction, MG reduction etc.) | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Multi-Domain L3 level Prediction for Measurement-Gap Reduction |  | UE sided model (Offline training) | CMCC (non-collocated), Samsung*, LGE, vivo, CTC, CATT |  | MG reduction | Rel-20 AI-Mob | Identify which use cases can serve the purpose of measurement gap reduction |
| 5a | Dynamic Adaptation of Measurement Procedure |  | UE sided model | Qualcomm*, MediaTek*, Samsung, Xiaomi (AI cell meas. selection), LGE, vivo*, CATT |  | - Measurement reduction - Improved mobility: HO/LTM | Prediction performance: Rel-20 AI-Mob | How to define UE measurement requirements in a way that AI/ML based UE measurement adaptation can be spec-compliant. |
| 7a | Spatial domain RX beam sweeping reduction (L1/L3) |  | UE sided model | CMCC, Samsung, Qualcomm*, Xiaomi, LGE, vivo*, CTC, Nokia |  | Measurement delay reduction | Minor dependence on other WG-led use cases. FR2 RX beam sweep factor is transparent to RAN1/RAN2 spec. | Benefits and feasibility of the use of AI-ML to reduce spatial domain RX beam sweeping factor |
|  | AI/ML based RLM/BFD/CBD, LTM by measurement prediction |  | UE-sided model | Samsung, LGE, vivo |  |  |  |  |
|  | AI/ML based SCell related enhancement by measurement prediction |  | UE-sided model | OPPO |  |  |  |  |
|  | AI-based prediction for enhanced s-measure mechanism |  | UE-sided model | LGE |  |  |  |  |
|  | AI/ML based AGC through temporal/frequency domain prediction |  | UE-sided model | ZTE* |  |  |  |  |
|  | AI/ML based DL sync through temporal/frequency domain prediction |  | UE-sided model | ZTE* |  |  |  |  |
|  | AI abstraction of the wireless environment to facilitate data scheduling and network power saving |  | NW-sided model | CATT |  | facilitate data scheduling and network power saving |  |  |

[FL Recommended Discussion Point]

- FFS the selection/prioritization.

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

### AI-RRM prediction model

#### Issue 4-2: General view on how to proceed for AI-RRM prediction models

[FL Recommended Discussion Point]

| use case | Sub use cases | Positive for the sub-use case to be included in 6G study with priority | Positive for the use case to be included in 6G study after the prioritized sub-use cases are concluded | Dependence with other WG-led use cases (in 5GA or 6G) | Which specific aspect RAN4 should study |
| --- | --- | --- | --- | --- | --- |
| 1) L1 beam-level prediction for TX beam | (1a) FR1/FR2-1 spatial domain (intra-cell) |  |  | FR2-1 is already in Rel-19 AI-BM (RAN1), FR1 not |  |
|  | (1b) FR1/FR2-1 time domain (intra-cell) |  |  |  |  |
|  | (1c) FR1/FR2-1 spatial + time domain (intra-cell) |  |  | FR2-1 is already in Rel-19 AI-BM (RAN1), FR1 not |  |
|  | (1d) FR1/FR2-1 spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
|  | (1e) FR1/FR2-1 freq. + spatial domain (inter-cell, co/non-collocated) |  |  | [RAN1 agree to be studied in 6G study] |  |
|  | (1f) FR1/FR2-1 time + spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
| 2) L3 beam-level prediction for TX beam | (2a) FR1/FR2-1 spatial domain (intra-cell) | Nokia, vivo, Apple, OPPO (for both FR1 and FR2-1, FFS RAN2 or RAN4 driven) |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2b) FR1/FR2-1 time domain (intra-cell) |  |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. |  |
|  | (2c) FR1/FR2-1 spatial + time domain (intra-cell) | Nokia, vivo, OPPO (for both FR1 and FR2-1, FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2d) FR1/FR2-1 frequency domain (inter-cell, collocated) | Apple, OPPO (FFS RAN2 or RAN4 driven) |  | Already in Rel-19 AI-Mobility study (RAN2) but not enter into Rel-20 work item. |  |
|  | (2e) FR1/FR2-1 spatial domain (inter-cell, co/non-collocated) | Nokia, Apple (FFS RAN2 or RAN4 driven) |  |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2f) FR1/FR2-1 freq. + spatial domain (inter-cell, non-collocated) | Nokia, OPPO (FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2g) FR1/FR2-1 time + spatial domain (inter-cell, co/non-collocated) | Nokia, OPPO (FFS RAN2 or RAN4 driven) | Apple |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements. Impacts on ML Management and performance monitoring should also be considered. |
|  | (2h) FR1/FR2-1 time + freq. + spatial domain (inter-cell, co/non-collocated) |  |  |  |  |
| 3) L3 cell-level prediction | (3a) FR1/FR2-1 time domain (intra-cell) |  |  | Already in Rel-20 AI-Mobility (RAN2) Need clarification for FR2-1 beam level measurement can be used for model input |  |
|  | (3b) FR1/FR2-1 freq. domain (inter-cell, collocated) |  |  | Already in Rel-20 AI-Mobility (RAN2) Need clarification for FR2-1 beam level measurement can be used for model input |  |
|  | (3c) FR1/FR2-1 freq. domain (inter-cell, non-collocated) | Nokia, LGE, CMCC, Apple, Samsung, ZTE (FFS RAN2 or RAN4 driven) |  |  | Nokia: Impact on prediction accuracy (accuracy requirements), reporting delay requirements and measurement gaps configuration and scheduling restrictions. |

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

#### Issue 4-3: L3 Mobility prediction framework and extension (closed)

FL: As guided by RAN4 chair, RAN4 should provide more detailed information for use case 1/2/3 firstly, therefore the issue 4-3 will be closed for this meeting by focusing on Issue 4-2 only in this meeting.

### Application scenarios for AI-RRM prediction models (closed)

FL: As guided by RAN4 chair, RAN4 should provide more detailed information for use case 1/2/3 firstly, therefore the issues under this agenda will be closed for this meeting by focusing on Issue 4-2 only in this meeting.

### Other directions (closed)

FL: As guided by RAN4 chair, RAN4 should provide more detailed information for use case 1/2/3 firstly, therefore the issues under this agenda will be closed for this meeting by focusing on Issue 4-2 only in this meeting.

# For information

## Contact information

FL: It is encouraged to add contact information here, which could be used for triggering offline discussion or sharing information in small group.

| Company | Delegate(s) | Email address |
| --- | --- | --- |
| Feature Lead | He Wang (Jackson) | h0809.wang@samsung.com |
| Samsung | Yunchuan Yang Yanze Fu Yuanyuan Zhang (Tina) Dan Liu | yc0301.yang@samsung.com yanze.fu@samsung.com tina55.zhang@samsung.com dan1992.liu@samsung.com |
| OPPO | Wenqiang Tian Qiuge Guo | tianwenqiang@oppo.com guoqiuge@oppo.com |
| Qualcomm | Nazmul Islam CH Park Pushkar Kulkarni Stefan Brueck | mislam@qti.qualcomm.com chparkqc@qti.qualcomm.com pushkark@qti.qualcomm.com sbrueck@qti.qualcomm.com |
| Ericsson | Deep Shrestha Navuday Sharma Nicholas Pu | deep.shrestha@ericsson.com navuday.sharma@ericsson.com nicholas.pu@ericsson.com |
| Nokia | Fahad Syed Muhammad Gilsoo Lee Salwa Saafi Sai Sree Rayala | fahad.syed_muhammad@nokia.com gilsoo.lee@nokia.com salwa.saafi@nokia.com sai_sree.rayala@nokia.com |
| vivo | Minhua Zheng Hao Du | minhua.zheng@vivo.com duhao.txyjy@vivo.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| CATT | Han Li | lihan2@cictmobile.com |
| ZTE Corporation, Sanechips | Derrick Chao Yan Wu Kun Yao Wubin Zhou Jie Chen | derrick.chao@zte.com.cn wu.yan7@zte.com.cn yao.kun1@zte.com.cn zhou.wubin@zte.com.cn chen.jie122@zte.com.cn |

## Reference for RAN4#116bis (Oct. 2025, Prague)

R4-2514636		Topic Summary for [116bis][107] 6G AI						Moderator (Samsung)

R4-2514637		WF on 6G AI														Samsung

R4-2513031		Discussion on AI/ML-Based 6G RRM Use Cases				Apple

R4-2513042		On RAN4-driven AI/ML aspects of 6GR						Qualcomm Technologies Ireland

R4-2513049		Discussion on AI framework and use cases for 6GR			Samsung

R4-2513060		RAN4 study on 6G AI/ML										Ericsson AB.

R4-2513072		6G Considerations on AI										Nokia

R4-2513079		Views on 6G AI/ML												MediaTek Inc.

R4-2513125		Discussion on AI/ML for 6GR study							CMCC

R4-2513229		Discussion on RAN4 driven AI use cases						CATT

R4-2513283		Overview for 6GR RAN4 centric AI use cases 				Xiaomi

R4-2513286		Views on AI study in 6G										ZTE Corporation, Sanechips

R4-2513293		6G RAN4 Study on AI											NTT DOCOMO, INC.

R4-2513298		Views on 6G AI study RAN4 aspects							China Unicom

R4-2513310		Consideration on RAN4 AI study for 6G						Huawei, HiSilicon

R4-2513318		RAN4 driven AI based use cases								OPPO

R4-2513324		Initial discussion on 6G AI for RAN4 driven cases			vivo

## Reference for RAN4#117 (Nov. 2025, Dallas)

R4-2520371	Running Summary of 6G AI										Samsung

R4-2521396	Views on 6G AI														NTT DOCOMO, INC.

R4-2520043	Further comments on the RAN4-driven AI/ML framework of 6GR	Qualcomm Technologies Ireland

R4-2520153	Discussion on general parts of RAN4 driven AI use cases		CATT

R4-2520331	Discussion on general aspects for RAN4 6G AI					Huawei, HiSilicon

R4-2520372	Discussion on AI/ML general aspects for 6GR					Samsung

R4-2520442	Discussion on general aspects for 6G AI/ML						CMCC

R4-2520625	Discussion on 6G AI/ML Framework and General Issues		Apple

R4-2520710	6G AI/ML General Aspects											Nokia

R4-2521209	Discussion on general issues related to AI/ML for 6G study		Ericsson

R4-2521242	General issues for AI/ML in RAN4									OPPO

R4-2521439	General aspect on 6G AI											vivo

R4-2521536	Discussion on 6G AI general part									ZTECorporation,Sanechips

R4-2520044	Further comments on RAN4-driven AI/ML use cases of 6GR	Qualcomm Technologies Ireland

R4-2520154	Discussion on RAN4 driven AI use cases							CATT

R4-2520332	Discussion on RAN4-driven AI/ML use cases						Huawei, HiSilicon

R4-2520345	Views on 6G AI/ML													MediaTek Inc.

R4-2520373	Discussion on RAN4-driven AI/ML use cases for 6GR			Samsung

R4-2520443	Discussion on use cases for 6G AI/ML								CMCC

R4-2520492	Overview for 6GR RAN4 centric AI use case						Xiaomi

R4-2520624	Discussion on AI/ML-Based 6G Use Cases						Apple

R4-2520711	AI/ML use cases in 6G												Nokia

R4-2520721	Discussion of RAN4-driven AI/ML use cases for 6G AI			LG Electronics Inc.

R4-2521047	Discussion on 6G AI/ML study for RAN4 RRM driven use cases	China Telecom

R4-2521210	Discussion on AI/ML use cases for 6G study						Ericsson

R4-2521243	Use cases for AI/ML in RAN4										OPPO

R4-2521440	Initial discussion on 6G AI for RAN4 driven cases				vivo

R4-2521537	Views on AI study in 6G RAN4-driven use cases					ZTECorporation,Sanechips

## Template for FL usage

[Background Summary]

[Relevant Proposals in Companies’ Tdocs]

| Company | Tdoc No. | Proposal |
| --- | --- | --- |

[FL Recommended Discussion Point]

- AAA

  - BBB

    - CCC

      - DDD

        - EEE

[View Collection on above Discussion Point]

Note: Companies’ view can be provided on the above discussion point recommended by FL. Comments from companies are not mandatory but greatly encouraged to facilitate the discussion.

| Company | Comments |
| --- | --- |

[Ad-Hoc Minutes]

- AAA

[Ad-Hoc Agreement]

- AAA