3GPP TSG-RAN WG4 Meeting #117	R4-2522828

# Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR 38.133 TC2: OD-SIB1 based Cell reselection FR2 to FR2 target Cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR_enh-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Rel-19 work item on network enery saving enhancements specifies the objective On-demand SIB1 (OD-SIB1) applicable to RRC_IDLE and RRC_INACTIVE. Thus, RRM performance requirements related to test of extended cell reselection delay due to OD-SIB1 acquisition in NES cell need to be specified for cell reselection to FR2 target cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM performance requirements related to test of extended cell reselection delay, due to OD-SIB1 acquisition in NES cell, for intra-frequency / inter-frequency NR measurements in FR2 are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing RRM performance requirements for FR2 for the objective OD-SIB1 of the Rel-19 WI on network energy saving enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.1.1.X (new), A.7.1.1.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2521246. |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### A.7.1.1.X	Cell reselection to FR2 intra-frequency NR case for FR2 cell supporting On-demand SIB1

##### A.7.1.1.X.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE capable of on- demand SIB1 acquisition and reselecting to an NES cell as described in [2, TS 38.331].

##### A.7.1.1.X.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.X.2-1, A.7.1.1.X.2-2 and A.7.1.1.X.2-3. The test consists of one time period with time duration of T1. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. UE has not registered with network for the tracking area containing cell 2. The UE has received the SIB26 IE to assist OD-SIB1 acquisition to cell 2 according to [2, TS 38.331]. During T1 the UE reselects to cell 2 supporting on-demand SIB1.

Table A.7.1.1.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.X.2-2: General test parameters for FR2 intra-frequency NR cell re-selection test case for UE reselecting to OD-SIB1 based cell

| Parameter |  |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  |  | 1, 2 | Cell 1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  |  | 1, 2 | Cell 2 | The UE has received SIB26 with OD-SIB1 configuration information for cell 2. |
| T1 end condition (final condition) | Active cell |  |  | 1, 2 | Cell 2 | The UE reselects to cell 2 during T1 period performing PRACH transmission and OD-SIB1 request with PRACH transmission and OD-SIB1 acquisition according to [2, TS 38.331]. Upon OD-SIB1 acquisition it performs RACH transmissions for Tracking Area update. |
|  | Neighbour cells |  |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  |  | 1, 2 | Not configured |  |
| T1 |  | PC2/3/4 | s | 1, 2 | [40] | T1 needs to be long enough to allow cell re-selection to already known cell. |
|  |  | PC1 |  | 1, 2 | [70] |  |

Table A.7.1.1.X.2-3: Cell specific test parameters for FR2 intra-frequency NR cell re-selection test case in AWGN for UE reselecting to OD-SIB1 based cell

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T1 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 | TDDConf.3.1 |
| PDSCH RMC |  | 1 | SR.3.1 TDD | SR.3.1 TDD |
| configuration |  | 2 | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET |  | 1 | CR.3.1 TDD | CR.3.1 TDD |
| RMC configuration |  | 2 | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD | CCR.3.1 TDD |
| RMC configuration |  | 2 | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration |  | 1 | SSB.3 FR2 | SSB.7 FR2 |
|  |  | 2 | SSB.4 FR2 | SSB.8 FR2 |
| OCNG Pattern |  | 1, 2 | OP.4 | OP.4 |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 1, 2 | 66 | 66 |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 | DLBWP.0.1 |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 | ULBWP.0.1 |
| RLM-RS |  | 1, 2 | SSB | SSB |
| Qrxlevmin | dBm/SCS | 1 | -138 | -138 |
|  |  | 2 | -135 | -135 |
| Pcompensation | dB | 1, 2 | 0 | 0 |
| Qhysts | dB | 1, 2 | 0 | 0 |
| Qoffsets, n | dB | 1, 2 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP | SS-RSRP |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 | Setup 1 defined in A.3.15.1 |
| Beam assumptionNote 4 |  | 1,2 | Rough | Rough |
| Es/Iot BB Note 5 | dB | 1 | -3.55 | 0.95 |
|  |  | 2 |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |
|  |  | 2 | -90 |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |
|  |  | 2 |  |  |
|  | dB | 1 | -3 | 1.5 |
|  |  | 2 |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -96 | -91.5 |
|  |  | 2 | -93 | -88.5 |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -67.40 | -65.34 |
|  |  | 2 | -64.40 | -62.34 |
| Treselection | s | 1, 2 | 0 | 0 |
| SSearchThresholdP |  | 1, 2 | 35 | 35 |
| SintrasearchP | dB | 1, 2 | 50 | 50 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

##### A.7.1.1.X.3	Test Requirements

The cell reselection delay to the already detected OD-SIB1 based cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected OD-SIB1 based cell shall be less than [28]s for PC2/3/4 and [54]s for PC1 for the reselection from cell 1 to cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected OD-SIB1 based cell can be expressed as: Tevaluate, NR_Intra + TSI-NR + TOD-SIB1,

Where:

Tevaluate, NR_ intra	See table 4.2.2.3-1 in clause 4.2.2.3

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

TOD-SIB1   Maximum time for performing OD-SIB1 request and OD-SIB1 acquisition; 1s is assumed in this case.

For the cell re-selection delay excluding TOD-SIB1 to an already detected OD-SIB1 based cell during T1 in this test case, this gives

-	26.88 s, allow 27s, for PC2/3/4 devices

-	52.48 s, allow 53s, for PC1 devices

This gives a total of 27.88s for PC2/3/4 and 53.48s for PC1, allow 28s for PC2/3/4 and 54s for PC1 for the cell re-selection delay including TOD-SIB1 to an already detected cell for UE accessing an OD-SIB1 based cell during T1 in this test case.

#### A.7.1.1.Y	Cell reselection to FR2 inter-frequency NR case for FR2 cell supporting On-demand SIB1

##### A.7.1.1.Y.1	Test Purpose and Environment

This test is to verify the requirement for the inter frequency NR cell reselection requirements for UE capable of on- demand SIB1 acquisition and reselecting to an NES cell as described in [2, TS 38.331].

##### A.7.1.1.Y.2	Test Parameters

The test scenario comprises of 2 cells (cell 1 and cell 2) on 2 different NR carriers respectively as given in tables A.7.1.1.Y.2-1, A.7.1.1.Y.2-2 and A.7.1.1.Y.2-3. The test consists of one time period with time duration of T1. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. UE has not registered with network for the tracking area containing cell 2. The UE has received the SIB26 IE to assist OD-SIB1 acquisition to cell 2 according to [2, TS 38.331]. During T1 the UE reselects to cell 2 supporting on-demand SIB1.

Table A.7.1.1.Y.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.Y.2-2: General test parameters for FR2 inter-frequency NR cell re-selection test case for UE reselecting to OD-SIB1 based cell

| Parameter |  |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  |  | 1, 2 | Cell 1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  |  | 1, 2 | Cell 2 | The UE has received SIB26 with OD-SIB1 configuration information for cell 2. |
| T1 end condition (final condition) | Active cell |  |  | 1, 2 | Cell 2 | The UE reselects to low priority cell 2 during T1 period performing PRACH transmission and OD-SIB1 request with PRACH transmission and OD-SIB1 acquisition according to [2, TS 38.331]. Upon OD-SIB1 acquisition it performs RACH transmissions for Tracking Area update. |
|  | Neighbour cells |  |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  |  |  | 1, 2 | 1,2 |  |
| Time offset between cells |  |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  |  | 1 | SSB.1 FR2 |  |
|  |  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  |  | 1, 2 | Not configured |  |
| T1 |  | PC2/3/4 | s | 1, 2 | [40] | T1 needs to be long enough to allow cell re-selection to already known cell. |
|  |  | PC1 |  | 1, 2 | [70] |  |

Table A.7.1.1.Y.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN for UE reselecting to OD-SIB1 based cell

| Parameter | Unit | Test configuration | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T1 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 | TDDConf.3.1 |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD | CR.3.1 TDD |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD | CCR.3.1 TDD |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 | OP.1 defined in A.3.2.1 |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| Data PRBs allocated |  | 1, 2 | 66 | 66 |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 | DLBWP.0.1 |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 | ULBWP.0.1 |
| RLM-RS |  | 1, 2 | SSB | SSB |
| Qrxlevmin | dBm/SCS | 1 | -140 | -140 |
|  |  | 2 | -137 | -137 |
| Pcompensation | dB | 1, 2 | 0 | 0 |
| Qhysts | dB | 1, 2 | 0 | 0 |
| Qoffsets, n | dB | 1, 2 | 0 | 0 |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP | SS-RSRP |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 | Setup 1 defined in A.3.15.1 |
| Beam assumptionNote 4 |  | 1, 2 | Rough | Rough |
| Es/Iot BB Note 5 | dB | 1, 2 | 9.95 | -11.05 |
| Note2 | dBm/SCS | 1 | -93 | -93 |
|  |  | 2 | -90 | -90 |
| Note2 | dBm/15 kHz | 1, 2 | -102 | -102 |
|  | dB | 1, 2 | 10.5 | -10.5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -82.5 | -103.5 |
|  |  | 2 | -79.5 | -100.5 |
| Io | dBm/95.04 MHz | 1,2 | -53.14 | -63.64 |
| SSearchThresholdP |  | 1, 2 | 35 | 29 |
| TreselectionNR | s | 1, 2 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 | Not sent |
| Threshx, highP | dB | 1, 2 | 48 | 48 |
| Threshserving, lowP | dB | 1, 2 | 44 | 44 |
| Threshx, lowP | dB | 1, 2 | 50 | 50 |
| Propagation Condition |  | 1, 2 | AWGN | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |

##### A.7.1.1.Y.3	Test Requirements

The cell reselection delay to a lower priority OD-SIB1 based cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to an already detected lower priority OD-SIB1 based cell shall be less than [28]s for PC2/3/4 and [54]s for PC1 for the reselection from cell 1 to cell 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected lower priority OD-SIB1 based cell can be expressed as: Tevaluate, NR_Inter + TSI-NR + TOD-SIB1,

Where:

Tevaluate, NR_ inter	See table 4.2.2.4-1 in clause 4.2.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case.

TOD-SIB1   Maximum time for performing OD-SIB1 request and OD-SIB1 acquisition; 1s is assumed in this case.

For the cell re-selection delay excluding TOD-SIB1 to an already detected OD-SIB1 based cell during T1 in this test case, this gives

-	26.88 s, allow 27s, for PC2/3/4 devices

-	52.48 s, allow 53s, for PC1 devices

This gives a total of 27.88s for PC2/3/4 and 53.48s for PC1, allow 28s for PC2/3/4 and 54s for PC1 for the cell re-selection delay including TOD-SIB1 to an already detected lower priority cell for UE accessing an OD-SIB1 based cell during T1 in this test case.

--- End of change 1 ---